3GPP TSG RAN WG1 Meeting #98																    R1-1908638

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	NR V2X Sidelink Physical Layer Procedures

Agenda item:	7.2.4.5

Document for:	Discussion and Decision

Introduction

This contribution provides discussion, analysis, and design details with regards to sidelink HARQ, CSI acquisition, and power control. Other aspects of NR V2X design are discussed in companion contributions [2]-[8].

HARQ Retransmission Procedure

PSFCH Timing

Agreements: - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s) - N is configurable, with the following values - 1 - At least one more value >1 - FFS details - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool Agreements: - For the period of N slot(s) of PSFCH resource, N=2 and N=4 are additionally supported. Agreements: - For a PSSCH transmission with its last symbol in slot n, when the corresponding HARQ feedback is due for transmission, it is expected to be in slot n+a where a is the smallest integer larger than or equal to K with the condition that slot n+a contains PSFCH resources. - FFS details of K

At the last meeting it was agreed, that the PSFCH is allocated in slot ‘n+a’ where ‘a’ is the smallest integer larger than or equal to ‘K’ with condition that slot ‘n+a’ contains PSFCH resources. K here is a function of processing time of PSSCH to prepare PSFCH in response.

K value is similar to N1, however it may be larger to support concurrent unicast/groupcast processes at a UE. Note, that N1 is currently a function of a subcarrier spacing, and therefore K may be different in different configurations. Further, N1 is described in symbols, while K should be a minimum integer number of slots to accommodate the PSSCH processing and PSFCH preparation time.

Another important processing time aspect is a number of symbols after the end of PSFCH and start of corresponding retransmission PSSCH, which may be denoted as K'. This value may be similar to N2, however depending on PSFCH design, it may even be smaller if PSFCH detection does not require blind decoding using polar decoders. Note, that N2 is currently a function of a subcarrier spacing, and therefore K' may be different in different configurations.

An open aspect is whether K is measured in logical slots (i.e. slots with available SL symbols or resource pool slots) or in physical slots. Although the processing time is described in physical symbols/slots, consideration of physical offset may introduce complication into HARQ-ACK codebook design. Since N period of PSFCH is mapped over SL / resource pool slots, it may be possible that single PSFCH resource need to carry mode than N HARQ-ACK bits if physical slots are counted by K. This may be resolved if K is counted in logical slots of the resource pool, that provides no more than N HARQ-ACK bit for PSFCH periodicity N. An example of the issue is illustrated in Figure 1, where assuming N = 2, K = 2 due to unavailable slots in the resource pool (DL or outside of resource pool), it is possible that a PSFCH need to convey 3 bit, while logical counting provides no more than 2 bit HARQ-ACK.

Figure 1. Physical vs logical slot counting for PSFCH

  - K is counted within slots of a given SL resource pool

  - K is calculated as a minimum integer number of slots to accommodate PSSCH processing time in a given sub-carrier spacing

  - Introduce K' as a minimum preparation time between PSFCH reception and corresponding PSSCH ReTX

System wide PSFCH resource allocation

Sidelink feedback transmission corresponding to multiple PSCCH/PSSCH resource pools should be mapped to single system wide region of PSFCH allocation. Multiple or single PSFCH pool can be allocated in this region.

  - PSFCH resources are multiplexed in time with PSCCH/PSSCH resources from system perspective

    - UE does not expect configuration of resource pools and PSFCH leading to PSCFH being allocated in symbols where PSCCH/PSSCH transmission or reception is possible

Feedback Multiplexing in PSFCH Occasion

Agreements: - At least for the case when the PSFCH in a slot is in response to a single PSSCH: - Implicit mechanism is used to determine at least frequency and/or code domain resource of PSFCH, within a configured resource pool. At least the following parameters are used in the implicit mechanism: - Slot index (FFS details) associated with PSCCH/PSSCH/PSFCH - Sub-channel(s) (FFS details) associated with PSCCH/PSSCH - Identifier (FFS details) to distinguish each RX UE in a group for Option 2 groupcast HARQ feedback - FFS detailed applicability of the above parameters - FFS: Other parameters (e.g. SL-RSRP/SINR, Layer-1 source ID, location information, etc.) - Conclusion: - Study further whether/how to handle/avoid the following cases for PSFCH transmission and reception: - Case 1 (PSFCH TX/RX overlap): A UE transmitted a PSSCH and received SCI scheduling another PSSCH where PSFCH resources corresponding the two PSSCHs appear in the same slot. - Case 2 (PSFCH TX to multiple UEs): A UE received SCI from different UEs and the associated PSFCHs appear in the same slot. - Case 3 (PSFCH TX with multiple HARQ feedback to the same UE): A UE received multiple SCI from the same UE and the associated PSFCHs appear in the same slot.

Frequency-code Resource Determination

Having PSFCH region every N slots, it may not be possible to rely on 1:1 mapping between PSSCH and PSFCH frequency resources. For example, as illustrated in Figure 2, as UE-2 chooses to transmit different TBs in the same sub-channel, there is an issue with 1:1 mapping between PSFCH and PSSCH/PSCCH since it leads to two different PSFCH resources. Moreover, when UE-1 chooses to transmit different TBs in different sub-channels, it would cause two PSFCH transmissions from the RX UE in different PSFCH resources of different sub-channels. Therefore, indication in SCI is necessary to multiplex feedbacks from multiple transport blocks in a single PSFCH resource.

Figure 2. Need for PSFCH resource indication due to periodically allocated PSFCH resources.

In order to describe the PSFCH determination rule, the following components of PSFCH resource determination within a PSFCH occasion are considered (as also illustrated in Figure 3):

- F – a total set of PSFCH frequency resources within a PSFCH time occasion

- NF – number of PSFCH resources in F

- Fs – a set of PSFCH resources associated with a sub-channel ‘s’

- NFs – number of PSFCH resources in Fs

- Fsn – a set of PSFCH resources associated with a sub-channel ‘s’ within a slot ‘n’ of a resource pool

- NFsn – number of PSFCH resources in Fsn

- C – number of different code sequences in a PSFCH frequency resource

- Fsnc – a set of different frequency-code resources associated with a sub-channel ‘s’ within a slot ‘n’

- C × NFsn – number of different frequency-code resources associated with a sub-channel ‘s’ within a slot ‘n’

Figure 3. Multiplexing of different PSFCH in the same sub-channel.

First, a 1:1 mapping rule may be defined assuming single TB transmission within PSFCH period and then may be extended to handle collision/multiplexing cases for multiple TBs:

- Fs is determined as all PSFCH frequency resources within PRBs used by the sub-channel containing associated PSCCH transmission

- Fsn is determined by dividing Fs into N (= PSFCH periodicity) equal-size groups and selecting the group index equal to (n mod N) index where ‘n’ is the slot index containing the associated PSCCH/PSSCH counted within associated resource pool.

- Frequency resource within Fsn and code resource within C is determined by:

  - For unicast and groupcast Option 1 – as a hashing function from L1 source ID

  - For groupcast Option 2 – as a hashing function from L1 source ID and {UE ID within a group}

If N = 1, the sets Fs and Fsn are equal, and there is no need to involve slot index into the mapping.

If N > 1, a UE determines Fsn from explicit signalling carried by SCI. The explicit signalling indicates which sub-channel and which slot is used in the 1:1 mapping rule. The signalling is derived from a separate field of size ceil(log2(Num_Subch*N)).

  - A mapping rule to determine PSFCH resource based on associated PSCCH/PSSCH transmission is defined as follows:

    - A set of PSFCH resources associated with a sub-channel ‘s’, Fs, is determined as all PSFCH frequency resources within PRBs used by the sub-channel containing associated PSCCH transmission

    - A set of PSFCH resource associated with a sub-channel ‘s’ within a slot ‘n’, Fsn, is determined by dividing Fs into N (= PSFCH periodicity) same-size groups and selecting the group index equal to (n mod N) where ‘n’ is the slot index containing the associated PSCCH/PSSCH counted within associated resource pool

    - Frequency-code resource within Fsn is determined by:

      - For unicast and groupcast Option 1 – as a hashing function from L1 source ID

      - For groupcast Option 2 – as a hashing function from L1 source ID and {UE ID within a group}

    - FFS details of hashing functions

If ‘s’ and ‘n’ are explicitly signaled in SCI, then the UE should not expect PSFCH resources to be different for different TBs which are being multiplexed, otherwise due to possibility of missed SCI, there may be ambiguity and efficiency loss since TX UE and RX UE may have different understanding of PSFCH frequency resource set.

  - When PSFCH periodicity N > 1, the mapping rule to determine PSFCH resource uses slot index ‘n’ and sub-channel index ‘s’ derived from explicit signalling in SCI

    - First PSCCH+PSSCH transmission associated with a given PSFCH occasion points to the same sub-channel ‘s’ and same slot ‘n’ as being used for this PSSCH+PSSCH transmission

    - Other than the first PSCCH+PSSCH transmission associated with a given PSFCH occasion indicate the same sub-channel ‘s’ and same slot ‘n’ as being used for the first PSSCH+PSSCH transmission

Multiplexing of HARQ-ACK for different TBs in a single PSFCH

Under the assumptions of single TB for rank-2 transmission and possibility to multiplex feedbacks from 1, 2, 4 slots, PSFCH needs to carry up to 4 bits. More details on the physical structure of this PSFCH is described in [2].

Multiplexing of HARQ-ACK from different UEs in a group

Agreements: - In HARQ feedback for groupcast, - When Option 1 is used for a groupcast transmission, it is supported - all the receiver UEs share a PSFCH - FFS: a subset of the receiver UEs share a PSFCH - FFS: all or a subset of receiver UEs share a pool of PSFCH. - When Option 2 is used for a groupcast transmission, it is supported - each receiver UE uses a separate PSFCH for HARQ ACK/NACK. - FFS: all or a subset of receiver UEs share a PSFCH for ACK transmission and another PSFCH for NACK transmission - FFS on which entity and how to allocate PSFCH resource to the receiver UE(s) - FFS whether or not to additionally support a mixture of option 1 and option 2 for a groupcast transmission - Note: Each PSFCH is mapped to a time, frequency, and code resource.

RAN1 discussed last time that UEs may be divided into groups which transmit on different resources. At this point it is not very clear what the potential benefits are. In order to avoid IBE effects the most constructive way would be to have TDM of UEs, however it is not possible in current slot structure assumptions. Thus, single resource for NACK transmission by all UEs is still preferred.

  - All UEs use same PSFCH resource for NACK-only transmission in response to a given associated PSCCH

    - Subset based PSFCH resource sharing is not supported for groupcast option 1 feedbacks

Therefore, for Option 1 groupcast, SFN manner transmission should be assumed. The resource determination should follow the procedure described in section 2.2.1.

If more than one TB is sent by single UE in the group, multiplexing of NACKs may be problematic. One option is to restrict NACK-only to single HARQ-ACK bit scenarios. Another option is to assign orthogonal frequency resources within the same sub-channel. This may also lead to multi-cluster transmission if one of the TBs has been successfully received, however may be OK if at most two resources are used.

  - For groupcast with NACK-only reporting (Option 1) RAN1 to select from the following alternatives

    - Alt. 1: Restrict to one HARQ-ACK bit cases (i.e. one TB mapped to PSFCH occasion)

    - Alt. 2: Restrict to two HARQ-ACK bits cases by PSFCH transmitted in different adjacent PRBs

For the case of ACK/NACK indication (Option 2), it was agreed that each receiver UE uses a separate PSFCH for HARQ ACK/NACK. Under these baseline assumptions, transmissions of PSFCH from UEs in the group can be seen as multiple unicast feedbacks multiplexed preferably in FDM manner.

In the same time, RAN2 sent an LS [R1-1907995] with mixed feedback about Option 2 support which may be problematic if higher layer signaling exchange between UEs is required.

Question: Does RAN1 assume the UE needs to be aware of each of the other UEs in the group for support of Option 2? In addition, RAN2 expects that if supported, RAN1 would provide any progress related to the resource configuration of PSFCH for Option 1 and/or Option 2.

In our understanding, the whole concept of Option 2 works only in case each UE in the group can have a unique resource. Therefore, UE-unaware procedures of PSFCH resource determination for Option 2 are not preferred. If UE-aware procedure is not possible in aspects related to RAN2 and above, then it is better to drop Option 2 support and pursue only Option 1.

  - For groupcast with ACK and NACK reporting (Option 2), unique PSFCH resource ID within a group should be known to each UE

Handling of Potential Collision Cases

In addition to the implicit and explicit resource determination rules, the collision cases also need to be decided. At the last meeting, three different cases were identified:

  - Case 1 - PSFCH TX/RX overlap

  - Case 2 - PSFCH TX to multiple UEs

  - Case 3 - PSFCH TX with multiple HARQ feedback to the same UE

It should be clarified, that Case 3 in cellular systems is usually handled by multiplexing using HARQ-ACK codebooks, so that dropping of HARQ bits is avoided. Similar approach should be used in NR V2X, i.e. the multiple feedbacks to the same UE should be multiplexed. This is described in details in previous sub-sections of this section. Meanwhile, the main focus of this sub-section is to discuss Cases 1&2.

Case 1:

Transmission and reception in the same PSFCH occasion may be first of all avoided by proper resource selection, when known occasions of PSFCH and associated PSSCH may be excluded from resource selection.

Alternatively, if PSFCH TX/RX is already scheduled, the following can be considered:

- Priority may be used to decide which PSFCH to transmit or receive. When same priority is associated with transmission and reception, other tie-breaking criteria can be applied:

  - When operating in groupcast option 1 (NACK only), the missed sending of NACK may be dangerous since it is then interpreted as ACK. Thus, it is not appropriate to always prioritize RX over TX in tie-breaking cases. Always prioritize TX over RX in tie-breaking cases may be acceptable since a UE may consider to automatically retransmit the unacknowledged data at expense of some efficiency loss.

- TX may be always prioritized over RX regardless of priority value

  - When PSFCH are pending for transmission and reception at a UE in the same PSFCH occasion

    - A priority value associated with a given PSFCH (derived from associated SCI scheduling) is used to decide which procedure to perform

      - In case of equal priority, PSFCH transmission is prioritized over PSFCH reception

Case 2:

In this case it is impossible to avoid such collisions by resource selection. Therefore, only dropping rules can be used. One way is to use priority values to decide which PSFCH to transmit. When same priority is associated with multiple transmissions, other criteria can be applied:

- As discussed in context of Case 1, if among the pending PSFCH transmissions there is one in accordance to groupcast Option 1, it is better to select the groupcast NACK-only transmission due to ACK assumption when it is not transmitted. When there are multiple such feedbacks, it may be up to UE which one to actually transmit.

  - When more than one PSFCH is pending for transmission at a UE in the same PSFCH occasion

    - A priority value associated with a given PSFCH (derived from associated SCI scheduling) is used to decide which one to transmit

      - In case of equal priority, groupcast Option 1 type of feedback is prioritized. It is up to UE how to down-select between multiple Option 1 type feedbacks in this case.

HARQ Feedback Enabling/Disabling

It was also agreed that HARQ feedback for unicast and groupcast may be disabled at least by configuration. From signalling perspective, in this case, it is natural to support the case when SFCI resource is not provided that effectively means no HARQ feedback requested. The decision on HARQ feedback disabling may be based on multiple aspects, which include the following:

QoS control: some traffic types or services may not require high reliability so that HARQ feedback may not be useful. Some other services may require ultra-low latency transmission where there is no time budget for feedback based retransmissions. All these considerations may be realized by mapping particular QoS attribute combinations to enabling/disabling HARQ setting.

The feedback from RX may also be transmitted subject to conditions:

TX-RX or TX-TX collisions with other procedures. In that case, whether to perform SFCI/PSFCH transmission may be subject to dropping rules based on QoS indicators and congestion control. The QoS indicators here are compared from the traffic requesting feedback transmission and the QoS of the other procedure in conflict, which may be either monitoring of higher priority traffic / signal or transmitting higher priority traffic / signal.

  - When HARQ feedback operation is configured for a TX UE, actual feedback request may not be sent to the RX UE based on QoS control functions

  - When HARQ feedback operation is configured for a RX UE, actual feedback may not be sent to the TX UE based on QoS control functions

Distance-based filtering

Working assumption: - Regarding the use of TX-RX geographical distance and/or RSRP in determining whether to send HARQ feedback for groupcast - Support at least the use of TX-RX geographical distance - FFS whether or not to additionally use L1-RSRP - Companies are encouraged to perform additional evaluations/analysis Agreements: - For at least option 1 based TX-RX distance-based HARQ feedback for groupcast, - A UE transmits HARQ feedback for the PSSCH if TX-RX distance is smaller or equal to the communication range requirement. Otherwise, the UE does not transmit HARQ feedback for the PSSCH - TX UE’s location is indicated by SCI associated with the PSSCH. - Details FFS - The TX-RX distance is estimated by RX UE based on its own location and TX UE location. - The used communication range requirement for a PSSCH is known after decoding SCI associated with the PSSCH - FFS implicit or explicit - FFS how to define location

With regards to additional usage of RSRP, it is of high importance that radio measurements are NOT used to decide about HARQ feedback reporting since those are subject to channel variations and interference and therefore may not be correlated with communication range requirements. Thus, additional usage of RSRP is not recommended.

  - Do not introduce RSRP criteria to determine whether to send PSFCH when there is no collision with TX or RX of other PSFCH

    - FFS usage of RSRP to resolve collision cases

To support distance based HARQ feedback filtering, SCI should carry information which may be used by receiver UEs to estimate a range towards the source of the SCI. There are in general two approaches:

- Timing measurement converted to propagation distance. In this case, if UEs have good timing synchronization (that can be achieved especially in GNSS–based sync), the propagation delay can be estimated by simplest first-arrival-path measurements. The achieved accuracy may be sufficient for the purposes of HARQ feedback filtering with tens of meters accuracy.

- Distance calculation based on coordinates of TX UE encoded into SCI. Approaches such as zone ID signaling or LSB of absolute longitude/latitude may be introduced.

It should be stressed, that explicit signaling in SCI may consume large chunk of payload. Therefore, adoption of explicit signaling motivates 2-stage SCI design where the second stage may carry such large payload while the first stage may be used for pure channel access purposes.

Further, it is assumed that there may be cases when TX UE location is not reliably known / or unknown. In that case, the signaling should be able to indicate such situation by a separate code-point, and the receivers should assume 0 distance to this UE during filtering.

  - The explicit signalling of TX UE location in SCI should have a code-point indicating that TX UE location is not available

HARQ Processes

TX processes

The total number of TX processes should be bounded by UE transmission capability. Currently in NR UL, it is 16 processes. Furthermore, depending on PSFCH timing and processing values NF2S, NS2F the number of HARQ processes for each unicast connection may also vary.

It may be assumed that UE should manage its own transmit processes also taking into account the RX UE capability. For sidelink, UE may adapt/vary number of TX processes depending on the number of sessions and processing delay, receiver capability.

  - Sidelink TX process number is bounded by 16

  - UE manages TX processes based on its implementation

RX processes

The total number of RX processes should be bounded by UE capability which is mostly concerned with UE soft buffer dimensioning. Currently in NR DL, it is up to 16. However, it should be carefully considered how many simultaneous feedback-based HARQ processes are supported at RX UE. Even in DL, the number of simultaneously generated feedbacks is very limited due to pipelining assumption and limitation of unicast PDSCH processing. In order to not impose substantial deviations in UE implementation from Uu assumptions, the number of simultaneous feedback-based HARQ processes should be also limited to e.g. 2 processes per carrier (as required for example for two-CW cases).

  - RAN1 to further discuss and agree on the following RX processes related capabilities

    - Total number of RX processes

    - Number of simultaneous feedback-based RX processes

Relation of Blind and Feedback-based Retransmissions

Throughout the contribution it is assumed that blind retransmissions or repetitions are always applicable to feedback-less and feedback-based cases. When repetitions are configured, the PSSCH-to-PSFCH time gap should be calculated from the last repetition so that other assumptions are not dependent on repetition schemes. Note, it is not possible to change PSFCH multiplexing behaviour in this case since repetition factor may be UE-specific and packet specific.

As it is also discussed in [4], there may be different implications of HARQ feedback operation on resource selection and vice versa. For example, if resource selection operates in PSFCH-resources unaware manner, there may be cases when resource for initial transmission and for retransmission are selected back-to-back (within HARQ RTT) and therefore it is not possible to exchange PSFCH in between them. In this case, the retransmission may be considered as blind, i.e. does not require feedback in between. Regardless of these options, it should be possible to send HARQ feedback after the last retransmission.

  - In case of combination of blind retransmissions / repetitions and feedback-based retransmissions, HARQ feedback should be sent at least after the last retransmission

CSI Acquisition

Agreements: - Support at least Sidelink CSI-RS for CQI/RI measurement - Sidelink CSI-RS is confined within the PSSCH transmission

When both UEs are capable of using and measuring CSI, the CSI measurement may be triggered. CSI request/trigger may be combined with the indication of CSI presence in this transmission. The indication in a form of at least one bit may be conveyed by SCI (unicast part of SCI).

  - 1-bit CSI measurement trigger + CSI-RS presence field is conveyed in unicast-related content of SCI

Then, the UE detected CSI request, assumes CSI-RS presence according to pre-configured parameters in this slot. After CSI measurement, a UE should send the CSI measurement report using PSSCH and regular resource allocation procedures for data, as it was agreed in RAN#83. The following two main alternatives are discussed:

- MAC CE in PSSCH

  - This approach relies on conventional TB transmission procedure. The presence of MAC CE may only be determined after successful TB reception. Both transmission and reception of such MAC CE should be directly possible without additional hardware in UE since it reuses PSSCH transmission/reception flow.

  - When MAC CE is conveyed in a TB, it should also have association with a resource where the measurement was performed, therefore the resource indication should be carried in the MAC CE.

- CSI piggybacking on PSSCH in same way as for UCI piggybacking on PUSCH

  - Although such mechanism is already employed in Uu, only transmission part of such mechanism is implemented in a UE. It also requires UE to be able to decode CSI using polar (currently it is UL polar code) in addition to the decoding of PSSCH itself. Such additional implementation complexity is not desirable. Moreover, presence of CSI report should be indicated in SCI since it is impossible to guarantee fixed timing relating between CSI request and CSI report.

Based on discussion, MAC CE is preferred as having minimum UE implementation impact.

  - Sidelink CSI report is conveyed via a MAC CE

  - Send LS to RAN2 asking to specify MAC CE for CSI reporting

Power Control

The following agreements were made with respect to sidelink power control during the study item phase.

Agreements: For unicast RX UEs, SL-RSRP is reported to TX UE - For sidelink open loop power control for unicast for the TX UE, TX UE derives pathloss estimation - Revisit during the WI phase w.r.t. whether or not there is a need regarding how to handle pathloss estimation for OLPC before SL-RSRP is available for a RX UE Agreements: - For sidelink transmit power control, - Total sidelink transmit power is the same in the symbols used for PSCCH/PSSCH transmissions in a slot. - FFS whether/how to handle simultaneous transmission of sidelink and uplink - The maximum SL transmit power is (pre-)configured to the TX UE. - FFS on details (e.g., whether the maximum power is dependent of parameters such as the priority of PSCCH/PSSCH) Agreements: - For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss. - When the SL open-loop power control is configured to use both DL pathloss and SL pathloss, - The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken. - (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss.

SL-RSRP Measurement and Reporting

At the last meeting, reporting of SL-RSRP to another UE for the purpose of power control has been agreed. It should be noted, that according to work item description [1], CSI reporting is going to be conveyed in PSSCH using resource allocation procedures for data. Therefore, combining CSI reporting and SL-RSRP reporting which may also be used for link adaptation, is a natural choice.

Further, SL-RSRP for power control is a L3-filtered measurement which assumes at least some averaging of measurements over several observations. Depending on traffic model, it may not be an easy task. Then, RAN1 may assume that there are “keep alive” messages exchanged between UEs which may be used for maintaining measurements. It is also important for supporting RRM/RLM procedures studied by RAN2.

  - L3-filtered SL-RSRP is measured using all PSSCH transmissions on a given UE-UE link

  - L3-filtered SL-RSRP is at least reported in a MAC CE

Power Control Setting Considerations

The working assumption should be confirmed as there are completely different assumptions on target SNR at receiver side of gNB and UE.

  - Confirm the working assumption on separate configuration of OLPC parameters for DL pathloss based TX power calculation and SL pathloss based TX power calculation

PSFCH Power Control

There is also a special case of PSFCH power control. Assuming PSCCH and PSFCH coverage should match, when PSCCH is power controlled, PSFCH may also be power controlled. However, due to asymmetric and non-reciprocal interference at both sides of the link, this assumption may lead to unpredictable performance.

Moreover, it should be possible to set different P0, alpha for PSFCH comparing to PSCCH/PSSCH, since gNB pathloss compensation may lead to large difference in power at two sides of the link. For PSFCH, a gNB may provision resources for high power PSFCH transmissions.

One scenario where PSFCH power control may be crucial is the groupcast feedback in a form of both ACK and NACK. In this case, as discussed in section 2.2, it is assumed that group members should transmit their feedback in FDMed manner. Depending on propagation conditions, this may create severe near-far and IBE problem at the UE receiving feedbacks. Thus, potential way to avoid it is to apply power control using pathloss to the TX UE so that feedbacks from different receivers arrive with similar power.

Groupcast power control for PSFCH would need another mechanism of pathloss calculation based on SL-RSRP measurement and signalled TX power. It is simply more efficient to signal TX power once instead of SL-RSRP reporting to each group member.

  - P0 and alpha values are separately (pre-)configured for DL pathloss based PSCCH/PSSCH power control and DL pathloss based PSFCH power control

Conclusions

In this contribution, we have discussed L1 procedures for sidelink HARQ, CSI, and power control. In addition, specific issues of resource allocation procedures related to unicast and groupcast were also considered. Based on the discussion the following proposals are made:

  - K is counted within slots of a given SL resource pool

  - K is calculated as a minimum integer number of slots to accommodate PSSCH processing time in a given sub-carrier spacing

  - Introduce K' as a minimum preparation time between PSFCH reception and corresponding PSSCH ReTX

  - PSFCH resources are multiplexed in time with PSCCH/PSSCH resources from system perspective

    - UE does not expect configuration of resource pools and PSFCH leading to PSFCH being allocated in symbols where PSCCH/PSSCH transmission or reception is possible

  - A mapping rule to determine PSFCH resource based on associated PSCCH/PSSCH transmission is defined as follows:

    - A set of PSFCH resources associated with a sub-channel ‘s’, Fs, is determined as all PSFCH frequency resources within PRBs used by the sub-channel containing associated PSCCH transmission

    - A set of PSFCH resource associated with a sub-channel ‘s’ within a slot ‘n’, Fsn, is determined by dividing Fs into N (= PSFCH periodicity) same-size groups and selecting the group index equal to (n mod N) where ‘n’ is the slot index containing the associated PSCCH/PSSCH counted within associated resource pool

    - Frequency-code resource within Fsn is determined by:

      - For unicast and groupcast Option 1 – as a hashing function from L1 source ID

      - For groupcast Option 2 – as a hashing function from L1 source ID and {UE ID within a group}

    - FFS details of hashing functions

  - When PSFCH periodicity N > 1, the mapping rule to determine PSFCH resource uses slot index ‘n’ and sub-channel index ‘s’ derived from explicit signalling in SCI

    - First PSCCH+PSSCH transmission associated with a given PSFCH occasion points to the same sub-channel ‘s’ and same slot ‘n’ as being used for this PSSCH+PSSCH transmission

    - Other than the first PSCCH+PSSCH transmission associated with a given PSFCH occasion indicate the same sub-channel ‘s’ and same slot ‘n’ as being used for the first PSSCH+PSSCH transmission

  - All UEs use same PSFCH resource for NACK-only transmission in response to a given associated PSCCH

    - Subset based PSFCH resource sharing is not supported for groupcast option 1 feedbacks

  - For groupcast with NACK-only reporting (Option 1) RAN1 to select from the following alternatives

    - Alt. 1: Restrict to one HARQ-ACK bit cases (i.e. one TB mapped to PSFCH occasion)

    - Alt. 2: Restrict to two HARQ-ACK bits cases by PSFCH transmitted in different adjacent PRBs

  - For groupcast with ACK and NACK reporting (Option 2), unique PSFCH resource ID within a group should be known to each UE

  - When PSFCH are pending for transmission and reception at a UE in the same PSFCH occasion

    - A priority value associated with a given PSFCH (derived from associated SCI scheduling) is used to decide which procedure to perform

      - In case of equal priority, PSFCH transmission is prioritized over PSFCH reception

  - When more than one PSFCH is pending for transmission at a UE in the same PSFCH occasion

    - A priority value associated with a given PSFCH (derived from associated SCI scheduling) is used to decide which one to transmit

      - In case of equal priority, groupcast Option 1 type of feedback is prioritized. It is up to UE how to down-select between multiple Option 1 type feedbacks in this case.

  - When HARQ feedback operation is configured for a TX UE, actual feedback request may not be sent to the RX UE based on QoS control functions

  - When HARQ feedback operation is configured for a RX UE, actual feedback may not be sent to the TX UE based on QoS control functions

  - Do not introduce RSRP criteria to determine whether to send PSFCH when there is no collision with TX or RX of other PSFCH

    - FFS usage of RSRP to resolve collision cases

  - The explicit signalling of TX UE location in SCI should have a code-point indicating that TX UE location is not available

  - Sidelink TX process number is bounded by 16

  - UE manages TX processes based on its implementation

  - RAN1 to further discuss and agree on the following RX processes related capabilities

    - Total number of RX processes

    - Number of simultaneous feedback-based RX processes

  - In case of combination of blind retransmissions / repetitions and feedback-based retransmissions, HARQ feedback should be sent at least after the last retransmission

  - 1-bit CSI measurement trigger + CSI-RS presence field is conveyed in unicast-related content of SCI

  - Sidelink CSI report is conveyed via a MAC CE

  - Send LS to RAN2 asking to specify MAC CE for CSI reporting

  - L3-filtered SL-RSRP is measured using all PSSCH transmissions on a given UE-UE link

  - L3-filtered SL-RSRP is at least reported in a MAC CE

  - Confirm the working assumption on separate configuration of OLPC parameters for DL pathloss based TX power calculation and SL pathloss based TX power calculation

  - P0 and alpha values are separately (pre-)configured for DL pathloss based PSCCH/PSSCH power control and DL pathloss based PSFCH power control

References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LGE, Huawei, Shenzhen, China, March, 2019

- R1-1908633, “Physical structure for NR V2X”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908634, “NR V2X sidelink communication under gNB control”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908635, “UE-autonomous resource allocation for NR V2X sidelink communication”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908636, “NR V2X sidelink synchronization” , Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908637, “In-device coexistence solutions for NR V2X sidelink” , Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908639, “QoS and congestion control for NR V2X”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908640, “LTE sidelink communication under gNB control”, Intel Corporation, Prague, Czech Republic, August, 2019