3GPP TSG RAN WG1 #98                                                                   R1-1908641

Prague, Czech Republic 26th – 30th August, 2019

Source: 	Intel Corporation

Title:	Considerations on performance evaluation for NTN

Agenda item:	7.2.5.1

Document for:	Discussion and Decision

# Introduction

At the RAN#80 meeting the new study item on Non Terrestrial Networks (NTN) was agreed [1], the study item description can be found in [2]. Objectives of the study item related to physical layer are copied below.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]: - Physical layer control procedures (e.g. CSI feedback, power control) - Uplink Timing advance/RACH procedure including PRACH sequence/format/message - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms. Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

At the last RAN1 meeting [3] very good progress was achieved on scenarios and evaluation assumptions for NTN performance evaluations. However, several details on evaluation assumptions and methodology are missing. In this contribution we provide our considerations on performance evaluation for NTN system level and link level simulations.

# Discussion

## 2.1. System level simulations

At the RAN1#87 meeting [3] the following agreement was made on satellite constellation for system level evaluations.

Agreement: Both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation.

The evaluation assumptions for single satellite simulations were agreed at the RAN1#87 meeting [3] including satellite location, all the details of beam layout and frequency reuse scheme. For multi-satellite performance evaluations many details are missing.

Multi-satellite simulations are needed to capture impact of interference coming from different satellites, hence it is desirable to focus on the NTN deployments there interference from multiple satellites is comparable or greater than interference from different beams of one satellite. As it was agreed previously, UEs considered for Ka-band in NTN have highly directional antenna pointing to the serving satellite. Hence, the interference from other satellites for Ka band is suppressed. Thus, the performance of multi-satellite system should be comparable to single-satellite system if the satellites are separated in angular domain from the UE perspective. Taking into account the above consideration, we propose to de-prioritize multi-satellite simulations in Ka-band.

Observation 1:

- Interference from different satellites is suppressed by directive UE antenna in Ka Band

Proposal 1:

- De- prioritize multi-satellite simulations in Ka-band

In order to define multi-satellite NTN deployment many parameters should be considered including satellite location, beam layout and frequency reuse scheme for each satellite. Specifying multi-satellite deployment needs a lot of effort, and can potentially result in many different scenarios. Thus, we propose to leave the details of multi-satellite NTN deployment for system level evaluations up to each company.

Proposal 2:

- Parameters of multi-satellite deployment used for multi-satellite simulations should be provided by the companies together with their simulation results

## 2.2. Link level simulations

At the last RAN1 meeting [3] table with impairments due to satellite payload and satellite movement was agreed. The table includes values of maximum Doppler shift with and without pre/post compensation at satellite payload side. The agreed values of maximum Doppler shift corresponds to the shift of DL carrier frequency from the nominal value observed at the UE receiver. Since the UE cannot generate signal with frequency equal to nominal value of UL carrier frequency, the carrier frequency of UL transmission follows carrier frequency of DL synchronization signals. Since DL synchronisation signal received at the UE experience Doppler shift, frequency of UL transmission is also shifted from its nominal value at the UE side. At the satellite receiver side carrier frequency of UL transmission also experience Doppler shift. Hence, the actual frequency shift of UL transmission from the nominal value is larger than Doppler shift. This effect is illustrated in the figure 1.

Figure 1. Illustration for the frequency shift of UL signal

Considering the effect described above, the value of carrier frequency of the UL signal at the satellite receiver can be represented by the following equation assuming UL carrier frequency synthesis via frequency multipliers and dividers based on carrier frequency of received DL signals.

,

- ̶  carrier frequency of UL transmission at the satellite receiver,

- ̶  nominal value of DL carrier frequency,

- ̶  pre-compensation of Doppler frequency shift at the satellite transmitter,

- ̶  projection of satellite speed relative to UE to the direction UE-satellite,

- ̶  speed of light,

- ̶  nominal value of UL carrier frequency,

- ̶  post-compensation of Doppler frequency shift at the satellite receiver.

After negligible terms are removed the UL carrier frequency at the satellite receiver is represented by the following equation

.

Thus, the actual frequency shift of UL signal observed at the satellite receiver is equal to double Doppler shift if no pre/post-compensation is applied. It is worth noting that the frequency shift of UL signal at the satellite receiver depends on the actual method of UL carrier frequency synthesis applied at the UE. For example, UL carrier frequency in Ka-band (20 GHz DL, 30 GHz UL) can be generation by adding 10 GHz to frequency of received DL signal using frequency mixer, in this case the the actual frequency shift of UL signal at the satellite receiver is equal to Doppler shift multiplied by 3/2. In order to consider the worst case of UL carrier frequency shift we propose the following table with values of maximum carrier frequency shift for UL transmission which are based on the above equations.

Observation 2:

- The maximum frequency shift for UL transmission is equal to double Doppler shift

Proposal 3:

- The following table captures values of maximum frequency shift for UL transmission considered for link level simulations

| Max frequency shift for UL transmission (Note 1) | Scenario A : 0.30 ppm Scenario C2/D2 : - 1200 km: 40 ppm - 600 km: 45 ppm |
| --- | --- |
| Max frequency shift for UL transmission if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : n/a Scenario C2/D2 : - Satellite alt. = 1200 km : - beam diameter = 90 km (Set 1 - S-band) : 1.81 ppm - beam diameter = 40 km (Set 1 - Ka-band): 0.81 ppm - beam diameter = 190 km (Set 2 - S-band) : 3.82 ppm - beam diameter = 90 km (Set 2 - Ka-band) : 1.81 ppm - Satellite alt. = 600 km : - beam diameter = 50 km (Set 1 - S-band) : 2.10 ppm - beam diameter = 20 km (Set 1 - Ka-band) : 0.84 ppm - beam diameter = 90 km (Set 2 - S-band) : 3.77 ppm - beam diameter = 50 km (Set 2 - Ka-band) : 2.10 ppm |
| Note 1 : Min. Elevation angle for both sat- user equipment is equal to 10 degree. |  |

# Conclusion

In this contribution considerations on performance evaluation for NTN are discussed. The following proposals were made.

Observation 1:

- Interference from multiple satellites is suppressed by directive UE antenna in Ka Band

Proposal 1:

- De- prioritize multi-satellite simulations in Ka-band

Proposal 2:

- Parameters of multi-satellite deployment used for multi-satellite simulations should be provided by the companies together with their simulation results

Observation 2:

- The maximum frequency shift for UL transmission is equal to double Doppler shift

Proposal 3:

- The following table captures values of maximum frequency shift for UL transmission considered for link level simulations

| Max frequency shift for UL transmission (Note 1) | Scenario A : 0.30 ppm Scenario C2/D2 : - 1200 km: 40 ppm - 600 km: 45 ppm |
| --- | --- |
| Max frequency shift for UL transmission if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : n/a Scenario C2/D2 : - Satellite alt. = 1200 km : - beam diameter = 90 km (Set 1 - S-band) : 1.81 ppm - beam diameter = 40 km (Set 1 - Ka-band): 0.81 ppm - beam diameter = 190 km (Set 2 - S-band) : 3.82 ppm - beam diameter = 90 km (Set 2 - Ka-band) : 1.81 ppm - Satellite alt. = 600 km : - beam diameter = 50 km (Set 1 - S-band) : 2.10 ppm - beam diameter = 20 km (Set 1 - Ka-band) : 0.84 ppm - beam diameter = 90 km (Set 2 - S-band) : 3.77 ppm - beam diameter = 50 km (Set 2 - Ka-band) : 2.10 ppm |
| Note 1 : Min. Elevation angle for both sat- user equipment is equal to 10 degree. |  |

# References

- RP-181999 Report of 3GPP TSG RAN meeting #80, La Jolla, USA, June 11 – 14, 2018

- RP-182880 Study on solutions for NR to support non-terrestrial networks (NTN), Thales

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, 13th – 17th, May 2019