3GPP TSG RAN WG1 #98                                                                   R1-1908644

Prague, Czech Republic 26th – 30th August, 2019

Source: 	Intel Corporation

Title:	Discussion on HARQ for NTN

Agenda item:	7.2.5.4

Document for:	Discussion and Decision

# Introduction

At the RAN#80 meeting the new study item on Non Terrestrial Networks (NTN) was agreed [1], the latest study item description can be found in [2]. Objectives of the study item related to physical layer are copied below.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]: - Physical layer control procedures (e.g. CSI feedback, power control) - Uplink Timing advance/RACH procedure including PRACH sequence/format/message - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms. Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In this contribution delay-tolerant re-transmission mechanisms for NTN is discussed.

# Discussion

At the last RAN1 meeting [3] it was agreed that semi-static HARQ disabling is supported.

Agreement: Network disabling of HARQ via RRC configuration should be supported. - FFS: Dynamic disabling of HARQ by gNB.

For NR DCI design it is assumed that adaptive asynchronous HARQ operation is always turned on. Fall-back and non-fall-back DCI for PUSCH scheduling have two fields which are related to HARQ operation: 2-bits redundancy version (RV) and 4 bits HARQ process ID. Fall-back DCI for PDSCH scheduling includes 2-bits RV, 4 bits HARQ ID and fields for HARQ feedback such as PDSCH-to-HARQ_feedback timing indicator (3 bits), PUCCH resource indicator (3 bits), Downlink Assignment Index (DAI, 2 bits). The number of bits for DAI in the non-fall-back DCI for PDSCH scheduling is configurable (0-4 bits).

Redundancy version field in the DCI is useless for the case where no retransmissions are used since there is no adaptive combining for HARQ retransmission. Redundancy version 0 can be used for all transmissions in this case. PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, DAI are not needed if there is no HARQ feedback for the scheduled PDSCH. It is not clear if HARQ process ID field can be removed from the DCI without any other specification impact since HARQ process ID may be needed for higher layers to identify the TB transmitted to the UE.

Observation 1:

- DCI format may be modified by removing the DCI fields for the case there HARQ is disabled via RRC

  - The following fields can be removed: Redundancy version, PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, Downlink Assignment Index.

  - It is not clear if HARQ process ID indication in the DCI is still needed

At the last RAN1 meeting [4] the following agreement was made on the impact of round trip delay (RTT) on HARQ operation.

Agreement: Evaluate impact of Satellite RTT when HARQ is enabled and potential solutions if needed - At least the following aspects should be considered if the number of HARQ processes is > 16: - DCI size - HARQ soft buffer size

In order to efficiently use HARQ operation gNB should transmit TB with new data for a HARQ process only after ACK is received for previous TB with the same HARQ process. Thus, the maximum number of TBs with new data transmitted within the RTT is limited by the number of parallel HARQ processes. In order to support large RTT of NTN with the enabled HARQ operation the number of parallel HARQ processes should be increased. As it is stated in the above agreement, at least DCI size and HARQ soft buffer size should be considered if the number of HARQ processes is > 16.

In Rel. 15 NR asynchronous HARQ operation is supported assuming that HARQ process ID is indicated to the UE in DCI for the corresponding PUSCH/PDSCH transmission. In contrast to the asynchronous HARQ operation, synchronous HARQ assumes that the HARQ process ID is derived implicitly based on slot index of PUSCH/PDSCH transmission. Thus, the number of parallel HARQ processes can be increased with the same number of DCI bits if the principle of synchronous HARQ operation is used. For example, most significant bits of HARQ process ID can be indicated by the DCI and least significant bits can be derived from the slot index of PUSCH/PDSCH transmission.

Observation 2:

- Larger number of parallel HARQ processes can be supported without increased number of DCI bits for HARQ process ID indication

Regarding the HARQ soft buffer size, it is not specified in the physical layer NR specification. The implementation of HARQ soft buffer is verified by the RAN4 performance requirements. Similar approach can be considered for the increased number of parallel HARQ processes. However, there is no physical layer features to improve the efficiency of HARQ operation for the case of limited soft buffer size. In order to improve the efficiency of HARQ operation in the case of limited UE soft buffer dynamic HARQ disabling/enabling can be used. Thus, gNB can control which TBs use HARQ soft buffer and also allows to optimise parameters of the transmission for TBs with and without HARQ (e.g. TBS, target BLER, etc.). Hence, not effective retransmissions (especially retransmissions with not self-decodable redundancy version) can be avoided for the TBs which are not saved in the soft buffer size.

Observation 3:

- Dynamic HARQ disabling/enabling can be used to improve the efficiency of HARQ operation in the case of limited UE soft buffer

Proposal:

- Study the following features for NTN

  - New DCI format for HARQ disabled via RRC

  - Support of increased number of parallel HARQ processes with the same number of bits for indication of HARQ process ID in DCI

  - Dynamic HARQ disabling for the increased number of parallel HARQ processes

# Conclusion

In this contribution re-transmission mechanisms for NTN is discussed. The following proposals were made.

Observation 1:

- DCI format may be modified by removing the DCI fields for the case there HARQ is disabled via RRC

  - The following fields can be removed: Redundancy version, PDSCH-to-HARQ_feedback timing indicator, PUCCH resource indicator, Downlink Assignment Index.

  - It is not clear if HARQ process ID indication in the DCI is still needed

Observation 2:

- Larger number of parallel HARQ processes can be supported without increased number of DCI bits for HARQ process ID indication

Observation 3:

- Dynamic HARQ disabling/enabling can be used to improve the efficiency of HARQ operation in the case of limited UE soft buffer

Proposal:

- Study the following features for NTN

  - New DCI format for HARQ disabled via RRC

  - Support of increased number of parallel HARQ processes with the same number of bits for indication of HARQ process ID in DCI

  - Dynamic HARQ disabling for the increased number of parallel HARQ processes

# References

- RP-181999 Report of 3GPP TSG RAN meeting #80, La Jolla, USA, June 11 – 14, 2018

- RP-182880 Study on solutions for NR to support non-terrestrial networks (NTN), Thales

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, 13th – 17th, May 2019