3GPP TSG RAN WG1 Meeting #98						 R1-1908647

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	Enhanced PUSCH repetitions

Agenda item:	7.2.6.3

Document for:	Discussion and Decision

# Introduction

In the last RAN1 meeting, Option 4 was agreed. However, it still has many details not settled. In this contribution, we further analyze the issues of Option 4 and suggest solutions to them. This discussion also relates to enhancements to configured grant PUSCH [1].

# Discussion

## Definition of L and K

In order to limit UE implementation to reasonable options, which cover most of the cases, the number of mini-slots / repetitions mapped to one slot may be limited to two. It can also relax other issues, e.g. DMRS insertion, orphan symbols etc.

Changes to definition of length in SLIV are not preferred, i.e. S + L > 14 should not be supported, since the same functionality can be achieved by other combination of K and L.

Proposal 1

- Support mapping of no more than two PUSCH repetitions into one slot

## Interaction with UL-DL Directions

In case of dynamic scheduling, the paradigm of following the dynamically scheduled symbols w/o implicit re-interpretation should be reused from Rel.15. For that purpose, the skipped symbols should be explicitly associated with DCI scheduling PUSCH. The mechanisms to do that are described in section 2.6. Therefore, Rel.15 procedures for handling UL-DL directions conflicts may be reused.

Regarding the conflicts of configured PUSCH, it is proposed to reuse Rel.15 behavior as much as possible, i.e. repetition is not transmitted if conflicts with dynamic DL symbols, except the first transmission after Type 2 activation, and where same handling as for dynamic PUSCH is applied.

Proposal 2

- For dynamic PUSCH

  - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s).

    - Note: this is the same as Rel-15 behavior.

  - For semi-static DL symbol(s), it is not expected that the resulting resource allocation has conflict with semi-static DL symbol(s)

- For configured PUSCH

  - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation,

    - If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted.

  - For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH

## Handling of Orphan Symbols

It is preferred that implicit rules are not introduced to handle the orphan symbols. If explicit signaling of skipping symbols is introduced, then the UE may not expect gNB scheduling to provide combination of SLIV and K so that it leads to orphans.

## Signalling of Number of Repetitions

It was also discussed how the total number of PUSCH repetitions is derived. It was widely accepted that the number of repetitions should be dynamically signaled. This may be realized either by putting the aggregation factor into a TDRA table entry (explicitly as a value or implicitly as a number of SLIVs), or by separately signaling the aggregation factor in DCI.

In our view, when aggregation factor and TDRA table are signaled separately, there may be combinations of these parameters, which are incompatible or useless. For example, two times repetition of one segment spanning almost whole slot, and single transmission of a pair of SLIVs spanning almost two slots may be redundant. In order to avoid this, a repetition/aggregation factor may be a part of the TDRA entry, so that it may only be applied to the entries, which benefit from it.

Proposal 3

- Nominal number of repetitions K is signaled as part of an enhanced TDRA table entry

- TDRA table size is increased up 32-64 entries

## DMRS in Case of Segmentation

It is preferred that DMRS insertion in case of segmentation follows Rel.15 behavior for a given mapping type and a given starting symbol and duration of the resulting segment.

Proposal 4

- In case of segmentation, DMRS insertion follows Rel.15 behavior for a given mapping type and a given starting symbol and duration of the resulting segment

## Skipping Symbols

As it is mentioned in many places within the contribution, it is crucial to have an explicit mechanism of skipping some symbols when Option 4 of enhanced repetitions is performed. More motivation for that was provided in our previous contribution [2].

For that purpose, a table of potential combinations of symbols, which are not available for PUSCH mapping in a set of slots may be composed. Then, an entry from this table may be dynamically associated with a given scheduled PUSCH. For example, it may be embedded into TDRA table entry or explicitly into DCI.

Figure 1. Illustration of skipped symbols

Proposal 5

- Dynamic indication of skipped symbols in a set of slots associated with a given scheduled PUSCH is supported

  - Alt.1: skipped symbols are associated with an entry in TDRA table

  - Alt.2: skipped symbols are signaled as a separate field in DCI

## TBS and RV Determination

When PUSCH transmission is composed from repetitions of relatively small segments and/or unequal segments, the currently specified procedure of TBS determination using initial transmission may not provide optimal performance due to modulation and/or LDPC base graph mismatch. As it can be seen in the appendix section (see Figure 3), for the optimal performance, a longer total PUSCH transmission should be assumed when determining TBS/MCS.

This may be done by proper TBS scaling or PUSCH duration scaling, e.g. as scaling = min(Nslot, Ntotal) / Nfirst, where Nslot, Ntotal, and Nfirst are number of symbols in a slot, total PUSCH duration (excluding any gaps between repetitions), and first segment duration respectively.

Proposal 6

- Use the first segment duration for TBS determination and introduce scaling for TBS determination procedure as a function of the first segment, total duration, and slot duration

Even if TBS is scaled, the RV cycling sequence may need to be optimized for a given combination of segments. For example, obviously if for a combination of 8+4 the RV sequence {0,2} provides best performance, then for a combination of 4+8 the RV sequence {2,0} should also provide best performance. This may be seen in LLS results provided in the appendix section (see Figure 2). However, such a sequence is currently not available. Therefore, it is proposed to introduce more RV cycling sequences to cover all possible combinations of splitting the total PUSCH into smaller segments.

Observation 1

- Due to unequal PUSCH durations possible with enhanced repetitions, conventional RV cycling sequences do not provide optimal performance

- It should be possible to schedule RV2 followed by RV0 to optimize cases when a first segment is shorter than a second segment

Proposal 7

- Introduce support of all permutations of the RV sequences {0,2,3,1}

## Frequency Hopping

When single segment is mapped to one slot, there is no need/justification to deviate from the already available intra- and inter- slot hopping which may be directly applied. When two (or more) segments are mapped to one slot, the intra-slot FH rules can be applied with modification. The modification may be needed when the segments have different length, where it is better to move FH boundary to the boundary between segments.

Proposal 8

- Support both inter-slot and intra-slot FH with two hops (two frequency positions) for PUSCH with more than one repetition within a slot

  - When two or more segments are mapped to one slot, the intra-slot FH boundary is moved to the boundary between segments

# Conclusions

In this contribution, we discussed enhancements to PUSCH to enable one dynamic/configured grant scheduling more than one PUSCH in a slot or across slots. The following proposals and observations are made based on the presented analysis:

Proposal 1

- Support mapping of no more than two PUSCH repetitions into one slot

Proposal 2

- For dynamic PUSCH

  - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s).

    - Note: this is the same as Rel-15 behavior.

  - For semi-static DL symbol(s), it is not expected that the resulting resource allocation has conflict with semi-static DL symbol(s)

- For configured PUSCH

  - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation,

    - If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted.

  - For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH

Proposal 3

- Nominal number of repetitions K is signaled as part of an enhanced TDRA table entry

- TDRA table size is increased up 32-64 entries

Proposal 4

- In case of segmentation, DMRS insertion follows Rel.15 behavior for a given mapping type and a given starting symbol and duration of the resulting segment

Proposal 5

- Dynamic indication of skipped symbols in a set of slots associated with a given scheduled PUSCH is supported

  - Alt.1: skipped symbols are associated with an entry in TDRA table

  - Alt.2: skipped symbols are signaled as a separate field in DCI

Proposal 6

- Use the first segment duration for TBS determination and introduce scaling for TBS determination procedure as a function of the first segment, total duration, and slot duration

Observation 1

- Due to unequal PUSCH durations possible with enhanced repetitions, conventional RV cycling sequences do not provide optimal performance

- It should be possible to schedule RV2 followed by RV0 to optimize cases when a first segment is shorter than a second segment

Proposal 7

- Introduce support of all permutations of the RV sequences {0,2,3,1}

Proposal 8

- Support both inter-slot and intra-slot FH with two hops (two frequency positions) for PUSCH with more than one repetition within a slot

  - When two or more segments are mapped to one slot, the intra-slot FH boundary is moved to the boundary between segments

# References

- R1-1908650, “Enhanced CG PUSCH transmission”, Intel Corporation, Prague, Czech Republic, August 2019

- R1-1906808, “On PUSCH enhancements for eURLLC”, Intel Corporation, Reno, USA, May 2019

# Appendix – Evaluation of RV Sequences and TBS Determination

In this section, first, LLS evaluation of different RV sequences when applied to a combination of segments of different duration, where the first segment is 4 symbols, and the second segment is 8 symbols are presented (see Figure 2). The LLS assumptions are listed in table below.

Figure 2. BLER vs SNR for different RV sequences when total PUSCH is split onto 4 + 8 symbols.

Second, LLS evaluation of different TBS determination schemes when applied to a combination of segments of different duration, where the first segment is 4 symbols, and the second segment is 8 symbols are presented (see Figure 3):

- Based on total duration (12 symbols)

- Based on bigger segment (8 symbols)

- Based on smaller segment (4 symbols)

Figure 3. BLER vs SNR for different TBS determination approached for PUSCH split onto 4 + 8 symbols.

| Parameter |  | Value |
| --- | --- | --- |
| Carrier Frequency |  | 4 GHz |
| BW, SCS |  | 40 MHz, 30 kHz |
| Allocation |  | 16 RB |
| Antenna |  | 1 x 4, low correlation |
| DMRS |  | Type B mapping, 1 symbol in beginning of every repetition; one additional symbol in case of single repetition |
| MCS |  | TBS determination follows first segment. TBS1 = 504 bit TBS2 = 2088 bit |
| Channel |  | TDL-C 300 ns DS 10 Hz max Doppler shift |
| Channel Est |  | MMSE with 2 RB bundling size |