3GPP TSG RAN WG1 #98                                                                   R1-1908652

Prague, Czech Republic 26th – 30th August, 2019

Source: 	Intel Corporation

Title:	On CSI Enhancements for MU-MIMO

Agenda item:	7.2.8.1

Document for:	Discussion and Decision

# Introduction

At the 3GPPRAN#81 meeting work item to specify enhancements for NR MIMO was approved. Objectives of the work item include the following [1].

- Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

- Perform study and, if needed, specify extension of Type II CSI feedback to rank > 2

At the RAN1#95 [2, 3] it was agreed to support DFT-based compression for Type II CSI as formulated below. Furthermore, it was agreed to extend the support of Type II CSI with DFT-based compression for rank 3 and 4 [4].

- Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH - P=2N1N2= #SD dimensions - N3= #FD dimensions - FFS value and unit of N3 - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank) - Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=Wf0,…,Wf2L-1 where Wf(i)=fki,0 fki,1…fki,Mi-1, where fki,mm=0Mi-1 are Mi size-N3×1 orthogonal DFT vectors for SD-component i=0,…,2L-1 - Number of FD-components {Mi} or i=02L-1Mi is configurable, FFS value range - FFS: choose one of the following alternatives - Alt1. common basis vectors: Wf=fk0 fk1…fkM-1, i.e. Mi=M ∀i and ki,mm=0Mi-1 are identical (i.e., ki,m=km, i=0,…,2L-1) - Alt2. independent basis vectors: Wf=Wf0,…,Wf2L-1, where Wf(i)=fki,0 fki,1…fki,Mi-1, i.e. Mi frequency-domain components (per SD-component) are selected - Note: kmm=0M-1 or ki,mm=0Mi-1,i=0,…,2L-1 are all selected from the index set {0,1,…,N3-1} from the same orthogonal basis group - FFS: If oversampled DFT basis or DCT basis is used instead of orthogonal DFT basis - FFS: Same or different FD-basis selection across layers - Linear combination coefficients (for a layer) - FFS if W2 is composed of K=2LM or K=i=02L-1Mi linear combination coefficients - FFS if only a subset K0<K of coefficients are reported (coefficients not reported are zero). - FFS if layer compression is applied so that i=02L-l-1Mi transformed coefficients are used to construct W2 for layer l (where the transformed coefficients are the reported quantity) - FFS quantization/encoding/reporting structure - Note: The terminology “SD-compression” and “FD-compression” are for discussion purposes only and are not intended to be captured in the specification

The design of the Type II CSI with DFT-based compression is almost finalized except several remaining details including the following.

- Segmentation or padding for N3 > 13

- Details on UCI parameters

- Supported values of codebook parameters

- UE capability signaling

- CSI omission rules

- Codebook subset restriction

In this contribution we discuss the remaining details of the Type II CSI with DFT-based compression.

# Discussion

## 2.1. Segmentation or padding for N3 > 13

At the RAN1 Ad-Hoc Meeting 1901 [5] the following agreement was achieved for supported values of DFT size (N3).

Agreement Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19, Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96 - Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R - Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

At the RAN1#96, RAN1#96b, RAN1#97 meetings [4, 6, 7] the following agreements were made on this issue.

Agreement On the values of N3, further discuss and clarify/refine both of the available alternatives with as the evaluation baseline Agreement On the value of N3 for (N3N3=NSB×R)>13: - For Alt1: - Identify alternatives for padding schemes in RAN1#97 (Reno) - Select one from the alternatives for padding scheme by RAN1#98 (Prague) - For Alt2: - Identify alternatives in RAN1#97 (Reno) - Select one from the alternatives by RAN1#98 (Prague) Agreement On the value of N3 N3for (N3=NSB×R) > 13: - For Alt1 (padding), consider only extrapolation-based scheme and decide on the final specific design alternatives in RAN1#97 (Reno) for down selection in RAN1#98 (Prague) - For Alt2 (two segments), the following alternatives will be considered for down selection in RAN1#98 (Prague): - Alt2.1: S1: 1, …, Y; S2: NSB×R -Y+1, …, NSB×R - Alt2.2: S1: 1, …, N3N3; S2: NS - N3+1, …, NS Agreement On Alt1 (padding, as described in R1-1907783) for N3, for evaluation purposes, select one of Alt1.1, 1.2, 1.3, and 1.4 in RAN1#98 (Prague) as described in the table of R1-1907783. - Alt1.1, 1.2, 1.3, 1.4 are described in R1-1907783

In order to compare the performance of Type II CSI DFT-based compression with padding and segmentation, system level simulations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB for 20 MHz and 50 MHz bandwidth with 30 kHz subcarrier spacing. Evaluation results are presented in figures 1, 2, 3, 4 for rank 1-2 Type II CSI DFT-based compression with L = 4, R = 2, p = {1/4, 1/2} and β = {1/4, 1/2}, evaluation results for Rel. 15 Type II codebook are presented for reference. Values of CSI configuration parameters considered for evaluations are presented in table 1. Padding scheme with repetitions corresponding to Alt1.3 from R1-1907783 [8] was selected for evaluations. The detailed evaluation assumptions can be found in the Appendix.

Table 1. Values of CSI configuration parameters

|  | 20 MHz bandwidth |  |  | 50 MHz bandwidth |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | No padding | Padding | Segmentation | No padding | Padding | Segmentation |
| NSB (# of subbands) | 13 | 13 | 13 | 17 | 17 | 17 |
| N3 (for each segment) | 26 | 27 | 15 | 34 | 36 | 18 |
| M (for each segment) | {4, 4, 7, 7} | {4, 4, 7, 7} | {2, 2, 4, 4} | {5, 5, 9, 9} | {5, 5, 9, 9} | {3, 3, 5, 5} |
| K0 (for each segment) | {8, 16, 14, 28} | {8, 16, 14, 28} | {4, 8, 8, 16} | {10, 20, 18, 36} | {10, 20, 18, 36} | {6, 12, 10, 20} |

Figure 1. Average packet throughput for padding and segmentation with 20 MHz bandwidth

Figure 2. Cell-edge packet throughput for padding and segmentation with 20 MHz bandwidth

Figure 3. Average packet throughput for padding and segmentation with 50 MHz bandwidth

Figure 4. Cell-edge packet throughput for padding and segmentation with 50 MHz bandwidth

As it can be seen from the above evaluation results, Type II CSI DFT-based compression with padding of FD coefficients provides similar performance comparing to the case without padding (N3 = NSB × R). Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead.

Observation 1:

- Type II CSI DFT-based compression with padding of FD coefficients provides similar performance comparing to the case without padding (N3 = NSB × R)

- Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead

Proposal 1:

- Support padding of FD coefficients to smallest multiple of 2, 3, or 5 which is ≥ NSB×R for N3 > 13

## 2.2 UCI parameters

The following agreements were made on the UCI parameters at the last RAN1 meeting [7].

Agreement For further details on the agreed UCI parameters in Table 1 of R1-1905629: - RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1 - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded - For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 - FFS: If alt 3-4 is supported, size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2 - The following FD basis subset selection scheme is supported: - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …, modMinitial+n,N3, n=0,1,⋯N3'-1 - The value N3'=αM where is higher-layer configured from two possible values - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2 Agreement For further details on the agreed UCI parameters in Table 1 of R1-1905629: - On SCI for RI>1 (reported in UCI part 2), down-select among the two following alternatives: - Alt3.3: Per-layer SCI, where SCIi is a log2K0-bit indicator (i=0,1,…(RI-1)i=0,1,…,(RI-1)), including further reducing the bitwidth if applicable (to log2minK0,2LMi) - Alt3.4: Per-layer SCI, where SCIi SCIi is a log22L–bit (i=0,1,…(RI-1)i=0,1,…,(RI-1)) Agreement On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table. - FFS: details on bitwidth and possible values for Minitial Minitial reporting in UCI part 2 - FFS: whether the possible value(s) for Minitial Minitial can depend on configured FD compression parameters - Up to the editor to capture this agreement Alt B. SCI for RI>1; Alt3.4: Per-layer SCI, where SCIi SCIiis a –bit (i=0,1,…(RI - 1)). The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIi=li*, and mi* is not reported. Index remapping; For layer i, the index mimi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmiassociated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3.. Combinatorial indicator for N3 ≤ 19N3≤19; log2N3-1Mi-1bits. Combinatorial indicator for N3>19N3 > 19; log2N3'-1Mi-1 bits. Minitial; Reported in UCI part 2, details on bitwidth and possible values are FFS.

The agreed design of the strongest coefficient indicator (SCI) for RI > 1 assumes that the strongest coefficient corresponds to FD vector with index 0 (after index remapping procedure) thanks to cyclic shift property of FD vectors. The index of SD vector corresponding to the strongest coefficient is reported using  bits. For RI = 1 it was agreed that the SCI is reported using  bits [4]. In order to reduce overhead for RI = 1 and align the SCI design for rank 1-4 we propose to support -bit SCI for RI = 1.

Furthermore, for -bit SCI the position of the strongest coefficient in the 2L × M matrix of coefficients is available at the gNB and no further signalling is required to indicate it. Since the strongest coefficient is always selected, size of the bitmap for coefficient downselection can be reduced to 2LM-1 bits.

Proposal 2

- Support  -bit SCI for RI = 1

- Support size-2LM-1 bitmap for coefficient downselection

As it can be seen from the above agreements, the design for FD basis subset selection indicator is different for N3 ≤ 19 and N3 > 19. For N3 ≤ 19 straightforward one-step selection with combinatorial indication for each layer is supported. For N3 > 19 two-step approach is supported, where layer-common intermediate subset of FD vectors is selected at the first step and layer-specific subsets are selected at the second step using combinatorial indication. The number of FD vectors in the intermediate subset N3’ is configured by higher layers via parameter α. The layer-common intermediate subset is window-based and fully parameterized with Minitial, where the number of bits for reporting of Minitial has not been defined yet. There are two corner cases for indication of Minitial: free selection of Minitial with  bits and fixed Minitial with 0 bits. For fixed Minitial it is assumed that significant FD vectors are concentrated around FD vector corresponding to the strongest coefficient (with index 0) and .

In order to evaluate the performance of two-step FD basis subset selection with free selection of Minitial and fixed Minitial, system level simulations were carried out for Dense Urban scenario with 16 Tx antennas at the gNB for 10 MHz with 15 kHz subcarrier spacing and 20 MHz bandwidth with 30 kHz subcarrier spacing. Evaluation results are presented in figures 5, 6, 7, 8 for rank 1-2 Type II CSI DFT-based compression with L = 4, R = 2, (p, β) = {(1/4, 1/4), (1/2, 1/4), (1/2, 1/2)}, evaluation results with one-step FD basis subset selection are presented for reference. The detailed evaluation assumptions can be found in the Appendix.

Figure 5. Average packet throughput for FD vectors selection with 10 MHz bandwidth

Figure 6. Cell-edge packet throughput for FD vectors selection with 10 MHz bandwidth

Figure 7. Average packet throughput for FD vectors selection with 20 MHz bandwidth

Figure 8. Cell-edge packet throughput for FD vectors selection with 20 MHz bandwidth

As it can be observed from the above evaluation results, the performance with fixed Minitial is similar to the performance with free selection of Minitial. Two-step FD basis subset selection with fixed Minitial has lower search complexity and overhead. Hence, we propose to support Two-step FD basis subset selection with fixed Minitial.

Proposal 3

- For N3 > 19, support two-step FD basis subset selection with fixed

At the last RAN1 meeting the following agreement was made [7].

Agreement In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer

The motivation to restrict the UE from reporting of all zeros in the bitmap for a polarization for each layer is to avoid unbalanced power load for antennas of different polarization. Depending on the antenna architecture, port virtualisation and characteristics of power amplifiers at the gNB, the performance of a transmission for a UE may be degraded if antennas of only one polarization are used for transmission of a spatial layer for the UE. According to our simulations the probability that a UE reports all zeros in the bitmap for a polarization for a layer is less than one percent for for Dense Urban scenario with 16 Tx antennas at the gNB with Type II FD compression CSI reporting up to rank 4 with (p, β) = (1/4, 1/4). Considering low probability of such event we propose not to restrict the UE from reporting all zero in the bitmap for a polarization for each layer.

Proposal 4:

- The UE is not restricted from reporting all “zero” in the bitmap for a polarization for each layer

## 2.3. Supported values of codebook parameters

At the RAN1#96 meeting [4] the following agreement was made.

Agreement On subset selection for layer 0, agree on the following: - Unrestricted (polarization-independent) subset selection which requires a size-2LM bitmap in UCI part 2 - β∈14,12,34 - FFS: Further down selection of supported combinations of FD compression parameters

As it can be seen from the above agreement, further down selection of supported combinations of FD compression parameters can be considered. Currently there are 4 possible values of codebook parameter p (p = {1/4, 1/2} are supported, p = {1/8, 3/4} are FFS) and 3 supported values of codebook parameter β for L ≤ 4. In total there are 12 possible combinations of codebook parameters p and β. In order to downselect combinations of p and β system level evaluation were carried out for Dense Urban scenario with 16 Tx antennas at the gNB. Evaluation results are presented in figure 9 and 10 for rank 1-2 Type II CSI DFT-based compression with L = 4, R = 1, p = {1/8, 1/4, 1/2, 3/4} and β = {1/4, 1/2, 3/4}, evaluation results for Rel. 15 Type I and Type II are presented for reference. The detailed evaluation assumptions can be found in the Appendix.

Figure 9. Average packet throughput for different values of p and β

Figure 10. Cell-edge packet throughput for different values of p and β

As it can be observed based on the above evaluation results, some combinations of codebook parameters (p, β) have high overhead (similar or higher to Rel. 15 Type II CSI with 8-PSK phase and SB amplitude) and do not provide sufficient performance gains over other cases. Thus, in our view (p, β) = {(3/4, 3/4), (3/4, 1/2), (1/2, 3/4)} should not be supported due to large overhead and small performance gains comparing to Type II CSI. As it can be seen from the above evaluation results, Type II CSI DFT-based compression with (p, β) = {(1/8, 1/4), (1/8, 1/2)} provide considerable performance loss comparing to other cases, thus (p, β) = {(1/8, 1/4), (1/8, 1/2)} should not be supported. Codebook parameters (p, β) = {(1/8, 3/4), (1/4, 1/4), (1/4, 1/2), (1/4, 3/4), (1/2, 1/4), (1/2, 1/2), (3/4, 1/4)} provide reasonable performance/overhead tradeoff. However, it is desirable to reduce number of supported combinations of codebook parameters. Codebook parameters (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)} cover a broad range of overhead and performance, hence we propose to support the following combinations of codebook parameters (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)}.

Observation 2:

- (p, β) = {(3/4, 3/4), (3/4, 1/2), (1/2, 3/4)} provide large overhead and small performance gains comparing to Type II CSI

- (p, β) = {(1/8, 1/4), (1/8, 1/2)} provide considerable performance loss comparing to other cases

- (p, β) = {(1/8, 3/4), (1/4, 1/4), (1/4, 1/2), (1/4, 3/4), (1/2, 1/4), (1/2, 1/2), (3/4, 1/4)} provide reasonable performance/overhead tradeoff

  - (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)} cover a broad range of overhead and performance

Proposal 5:

- Support (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)}

## 2.4. UE capability signalling

The design of UE capability signalling for CSI measurements and reporting considers many aspects such as implementation of processing algorithms (e.g. PMI search with a particular codebook type and number of ports), memory size to store CSI, UE complexity for CSI processing considering multiple simultaneous calculations for different CSI reports, processing time constraints, etc. There are different UE capabilities for CSI which are defined per component carrier (CC), per band, per frequency range (FR1 or FR2) or per UE. The list of Rel. 15 UE capabilities for CSI is provided below [9].

- csi-RS-IM-ReceptionForFeedback – This parameter is configured per band; same parameter is configured 	per UE for the case of FR1-FR2 band combination. Values of UE 	capabilities in the parameter configured per band are further limited 	by the parameter configured per UE.

  - maxConfigNumberNZP-CSI-RS-PerCC

  - maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC

  - maxConfigNumberCSI-IM-PerCC

  - maxNumberSimultaneousNZP-CSI-RS-PerCC

  - totalNumberPortsSimultaneousNZP-CSI-RS-PerCC

- CSI-RS-IM-ReceptionForFeedbackPerBandComb – This parameter is configured per band combination.

  - maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC

  - totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC

- csi-ReportFramework – This parameter is configured per band; same parameter is configured per UE for  	the case of FR1-FR2 band combination. Values of UE capabilities in the 	parameter configured per band are further limited by the parameter configured 	per UE.

  - maxNumberPeriodicCSI-PerBWP- ForCSI-Report

  - maxNumberPeriodicCSI-PerBWP-ForBeamReport

  - maxNumberAperiodicCSI-PerBWP-ForCSI-Report

  - maxNumberAperiodicCSI-PerBWP-ForBeamReport

  - maxNumberAperiodicCSI-triggeringStatePerCC

  - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report

  - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport

  - simultaneousCSI-ReportsPerCC

- simultaneousCSI-ReportsAllCC – This parameter is configured per band combination.

- codebookParameters – This parameter is configured per band.

  - type1

    - singlePanel

      - supportedCSI-RS-ResourceList

      - modes

      - maxNumberCSI-RS-PerResourceSet

    - multiPanel

      - supportedCSI-RS-ResourceList

      - modes

      - nrofPanels

      - maxNumberCSI-RS-PerResourceSet

  - type2

    - supportedCSI-RS-ResourceList

    - parameterLx

    - amplitudeScalingType

    - amplitudeSubsetRestriction

  - type2-PortSelection

    - supportedCSI-RS-ResourceList

    - parameterLx

    - amplitudeScalingType

- SupportedCSI-RS-Resource – This parameter is configured per band as part of codebook parameters for 	each codebook type (list of SupportedCSI-RS-Resource corresponds to 	the supportedCSI-RS-ResourceList parameter for each codebook type).

  - maxNumberTxPortsPerResource

  - maxNumberResourcesPerBand

  - totalNumberTxPortsPerBand

Support of CSI reporting with a given codebook type is indicated by UE capability codebookParameters which is configured per band. For each codebook type UE indicates a list of supported combinations of the maximum number of CSI-RS ports per CSI-RS resource, the maximum number of resources across all CCs within a band simultaneously, the total number of Tx ports across all CCs within a band simultaneously (supportedCSI-RS-ResourceList parameter). There are also codebook-specific parameters such as mode for Type I codebooks (modes parameter), supported values of L for Type II codebooks (parameterLx parameter), etc. Codebook-specific UE capabilities are needed to indicate support of a codebook type, specific features for each supported codebook type and maximum number of ports/resources for each codebook to control UE computational complexity since UE computational complexity is different for different codebook types. Since new codebook type is introduced in Rel. 16, new codebook-specific parameter for UE capabilities is needed similar to Rel. 15 codebook types. Parameter for new codebook type should include at least supportedCSI-RS-ResourceList parameter and other parameters to indicate codebook-specific features.

Proposal 6:

- Support extension of Rel. 15 UE capabilities for new codebook type(s) introduced in Rel. 16

  - Higher layer parameter for indication of UE capabilities for new codebook type(s) introduced in Rel. 16 should include at least a list of supported combinations of the maximum number of CSI-RS ports per CSI-RS resource, the maximum number of resources across all CCs within a band simultaneously, the total number of Tx ports across all CCs within a band simultaneously

Since the same processing unit can be reused for PMI calculation with different codebook types, separate UE capability signalling for each codebook type is not enough to represent actual UE capabilities for CSI processing. There are many parameters for UE capability signalling which are indicated for all codebook types (codebook-common parameters) and can be used to control UE processing complexity (e.g. simultaneousCSI-ReportsAllCC, simultaneousCSI-ReportsPerCC, maxNumberSimultaneousNZP-CSI-RS-PerCC, maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC, etc.). However, it is hard to use codebook-specific and codebook-common parameters together since the granularity of UE capability reporting is different. For example, maxNumberResourcesPerBand parameter limits the maximum number of NZP CSI-RS resources per codebook type, maxNumberSimultaneousNZP-CSI-RS-PerCC parameter limits the maximum number of NZP CSI-RS resources for all codebook types, both parameters are indicated per band. However, it is hard to use those two parameters together since the number of NZP CSI-RS resources for maxNumberSimultaneousNZP-CSI-RS-PerCC is limited in each CC while for maxNumberResourcesPerBand it is limited per band.

Thus, UE capability signalling is not optimal and lead to underestimate of actual UE processing capabilities. For example, let’s assume that UE is capable to simultaneously process up to T CSI-RS resources for Type I and Type II codebook in total. Since UE processing unit is shared for different codebook types and UE capabilities are defined separately per each codebook type, UE should distribute it’s capabilities among Type I and Type II, e.g. up to T1 CSI-RS resources for Type I and up to T2 CSI-RS resources for Type II. In that example UE is actually capable to do simultaneous processing for T CSI-RS resources for Type I and Type II codebook, however, gNB assumes that UE is not capable to do it according to the UE capability signalling.

Considering that the complexity of CSI calculation for Type II with DFT-based compression is higher comparing to Rel. 15 Type I and Type II codebooks, efficiency of utilization of UE computational resources becomes more important. Hence, optimization of UE capability signaling is needed for Rel. 16.

Observation 3:

- UE capability signaling does not optimally reflect the actual UE CSI processing capabilities for the case where multiple codebook types are supported by the UE

Proposal 7:

- Consider optimization of UE capability signaling for the case where multiple codebook types are supported by the UE taking into account that the complexity of CSI calculation for Type II with DFT-based compression is higher comparing to Rel. 15 Type I and Type II codebook

One simple solution which can solve the above problem is introduction of additional UE capability signaling parameter which indicates the list of supported combinations of the maximum number of CSI-RS ports, the maximum number of resources, the total number of Tx ports across all CCs within a band simultaneously across all the supported codebook type.

Proposal 8:

- Support UE capability signaling parameter which indicates the list of supported combinations of the maximum number of CSI-RS ports, the maximum number of resources, the total number of Tx ports across all CCs within a band simultaneously across all the supported codebook types

## 2.4. Other remaining issues

In order to decrease inter-cell interference it may be needed to avoid DL transmission to the certain direction. To support valid CSI for such scenario codebook subset restriction (CBSR) feature was specified for NR in Rel. 15 for Type I and Type II CSI. CBSR allows to control the PMI and avoid situation when the UE selects precoding matrix corresponding to the DL transmission towards the restricted direction. CBSR for Rel. 15 Type II CSI also allows to control the maximum relative transmission power allocated for a transmission for different beamforming directions.

For beam-combining based Type II CSI feedback there is alternative approach how to avoid DL transmission to a certain direction or reduce its power without CBSR. Algorithms like MMSE or zero-forcing can be applied to the precoding matrix reported by the UE with correction of CQI at the gNB side. Such approach allows to use all the channel information provided by the UE and achieve full flexibility of control for transmission direction and corresponding power. Such approach, however, cannot be used for a case where all the beams reported by the UE are restricted for transmission. So, simple CBSR with beamforming direction restriction only may still be needed depending on the actual deployment scenario.

Observation 4:

- Control of the transmission direction and corresponding power can be done at the gNB side without codebook subset restriction

Another feature specified in the Rel. 15 for Type I and Type II CSI is CSI omission for aperiodic CSI reporting. Partial CSI omission allows to omit part of channel state information if container size (capacity of PUSCH) is lower than the number of bits required for the CSI report. Such case is possible if the gNB underestimate the number of bits required for CSI reporting in case of dynamically changing CSI size, for example in case of rank adaptation with Type II CSI. In the contribution [10] it was shown that the rank value of Type II CSI can be predicted based on the rank of Type I report. Hence, gNB can handle the dynamically changing number of bits for aperiodic Type II CSI reporting without significant losses of CSI reporting efficiency.

Observation 5:

- gNB can handle the dynamically changing number of bits for aperiodic Type II CSI reporting without significant losses of CSI reporting efficiency

Proposal 9:

- Strive to minimize specification effort for codebook subset restriction and CSI omission features

# Conclusions

In this contribution enhancements to CSI for MU-MIMO are discussed including overhead reduction for Type II CSI and the support of higher ranks for Type II CSI. The following proposals and observations were made.

Observation 1:

- Type II CSI DFT-based compression with padding of FD coefficients provides similar performance comparing to the case without padding (N3 = NSB × R)

- Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead

Proposal 1:

- Support padding of FD coefficients to smallest multiple of 2, 3, or 5 which is ≥ NSB×R for N3 > 13

Proposal 2

- Support  -bit SCI for RI = 1

- Support size-2LM-1 bitmap for coefficient downselection

Proposal 3

- For N3 > 19, support two-step FD basis subset selection with fixed

Proposal 4:

- The UE is not restricted from reporting all “zero” in the bitmap for a polarization for each layer

Observation 2:

- (p, β) = {(3/4, 3/4), (3/4, 1/2), (1/2, 3/4)} provide large overhead and small performance gains comparing to Type II CSI

- (p, β) = {(1/8, 1/4), (1/8, 1/2)} provide considerable performance loss comparing to other cases

- (p, β) = {(1/8, 3/4), (1/4, 1/4), (1/4, 1/2), (1/4, 3/4), (1/2, 1/4), (1/2, 1/2), (3/4, 1/4)} provide reasonable performance/overhead tradeoff

  - (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)} cover a broad range of overhead and performance

Proposal 5:

- Support (p, β) = {(1/4, 1/4), (1/4, 1/2), (1/2, 1/2)}

Proposal 6:

- Support extension of Rel. 15 UE capabilities for new codebook type(s) introduced in Rel. 16

  - Higher layer parameter for indication of UE capabilities for new codebook type(s) introduced in Rel. 16 should include at least a list of supported combinations of the maximum number of CSI-RS ports per CSI-RS resource, the maximum number of resources across all CCs within a band simultaneously, the total number of Tx ports across all CCs within a band simultaneously

Observation 3:

- UE capability signaling does not optimally reflect the actual UE CSI processing capabilities for the case where multiple codebook types are supported by the UE

Proposal 7:

- Consider optimization of UE capability signaling for the case where multiple codebook types are supported by the UE taking into account that the complexity of CSI calculation for Type II with DFT-based compression is higher comparing to Rel. 15 Type I and Type II codebook

Proposal 8:

- Support UE capability signaling parameter which indicates the list of supported combinations of the maximum number of CSI-RS ports, the maximum number of resources, the total number of Tx ports across all CCs within a band simultaneously across all the supported codebook types

Observation 4:

- Control of the transmission direction and corresponding power can be done at the gNB side without codebook subset restriction

Observation 5:

- gNB can handle the dynamically changing number of bits for aperiodic Type II CSI reporting without significant losses of CSI reporting efficiency

Proposal 9:

- Strive to minimize specification effort for codebook subset restriction and CSI omission features

# References

- RP-181453 WI Proposal on NR MIMO Enhancements, Samsung

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 12th – 16th, 2018

- R1-1813002 Summary of CSI enhancement for MU-MIMO support, Samsung

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting Ad-Hoc Meeting 1901, Taipei, Taiwan, 21st – 25th January, 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96bis, Xi’an, China, 8th – 12th April, 2019

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, 13th – 17th, May 2019

- R1-1907783 Feature lead summary on offline email discussion for N3 issue, Samsung

- 3GPP TS 38.306 V15.4.0 (2018-12) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities (Release 15)

- R1-1717367 Remaining issues on CSI reporting, Intel Corporation

# Appendix

Table. Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Layout | Hexagonal Grid with 2 tiers |
| ISD | 200 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 10 MHz with 15 kHz subcarrier spacing, 52 PRB |
| Tx power | 41 dBm |
| UE distribution | Uniform 20% outdoor (30 km/h), 80% indoor (3 km/h) |
| UE antenna configuration | 2 Rx for evaluations with max rank 2, 4 Rx for evaluations with max rank 4, X-pol slant 0/90 degrees, dH = 0.5 λ |
| BS antenna configuration | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Traffic model | FTP 1 with 0.5 Mbytes packet size. High traffic load with ~70% resource utilization for evaluations with max rank 2. Medium traffic load with ~50% resource utilization and low traffic load with ~20% resource utilization for evaluations with max rank 4. |
| TRP association | RSRP based Handover margin = 0 dB |
| Transmission mode | SU-MIMO for evaluations with max rank 4 and low traffic load; MU-MIMO with 8 BS layers maximum for other cases; Rank adaptation with max rank 2 or 4 |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| MU-MIMO precoding | MMSE |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| UE noise figure | 9 dB |
| Max number of HARQ transmissions | 4 |
| Coefficients quantization | Rel. 15 Type II CSI: - QPSK/8-PSK + WB/WB+SB amplitude Rel. 16 Type II CSI: - Agreed quantization scheme [4] with 8-PSK phase quantization |