3GPP TSG RAN WG1 Meeting #98																R1-1908659

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	DL Reference Signals for NR Positioning

Agenda item:	7.2.10.1

Document for:	Discussion and Decision

Introduction

Work item on NR Positioning support was approved in [1]. The WID document has the following objectives:

Define new DL positioning reference signals applicable at least for DL-TDOA, DL-AoD, RTT

In this document, we discuss design of DL PRS for NR Positioning based on agreements made by RAN1 WG at the previous meetings (see Annex-A). Our views on other NR Positioning aspects are provided in companion contributions [3]-[5].

Configuration of DL Reference Signals for NR Positioning

## DL PRS Configuration

The overall resource allocation for DL PRS may be structured in DL PRS Resource and DL PRS Resource Sets (see Figure 1). The illustrated structure provides flexibility to describe DL PRS transmission settings in various deployment scenarios including support of beamforming/-sweeping, support of neighboring cells, etc.

Figure 1: Structure for configuration of DL PRS transmissions

With respect to DL PRS Resource Set and DL PRS Resources, the following agreements were made by RAN1:

A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID. The DL PRS Resources in a DL PRS Resource Set are associated with the same TRP

A DL PRS Resource ID in a DL PRS Resource Set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams)

In addition, a term “DL PRS occasion” was defined as “one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted”

### DL PRS Resource Allocation

Before we discuss DL PRS Resource allocation, let us recall related agreements on DL PRS resources and transmissions:

NR DL PRS design for FR1 and FR2 supports: - Localized in time NR DL PRS transmissions with periodic and/or on-demand resource allocation - FFS signaling details - Dedicated NR DL PRS resources - time-frequency grid at resource block level - PRS transmitted in one cell may or may not collide with PRS transmissions in other cell - e.g. frequency vShift/comb-offset is the same or different for two different PRSs in the same RB - There is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources - FDM multiplexing with other signals at RE level inside of PRS time-frequency grid is precluded

Taking into account the above agreement as well as definition of DL PRS Resources/Resource Set and DL PRS occasions, the following options can be considered for DL PRS Resources and DL PRS Resource Set allocation in DL PRS occasions:

Option 1: DL PRS Resource Set allocates DL PRS Resources w/ the same DL TX spatial filter inside DL PRS occasion

  - Enables gNB TX sweeping across DL PRS Resource Sets and UE RX sweeping across DL PRS Resources of configured DL PRS Resource Set per single occasion

Option 2: DL PRS Resource Set allocates DL PRS Resources w/ different DL TX spatial filters inside DL PRS occasion

  - Enables gNB TX sweeping across DL PRS Resources of DL PRS Resource Set and UE RX sweeping across DL PRS occasions

Option 3: DL PRS Resource Set allocates DL PRS Resources w/ different DL TX spatial filters across multiple DL PRS occasions

  - Enables gNB TX sweeping across DL PRS Resources mapped to different DL PRS occasions and UE RX sweeping across DL PRS occasions cycles

Option 4: DL PRS Resource Set allocates DL PRS Resources w/ the same DL TX spatial filter across multiple DL PRS occasions

Figure 2: DL PRS Resource Allocation Option 1 – 
DL PRS Resource Set allocates DL PRS Resources with the same DL TX spatial filter inside DL PRS occasion

Figure 3: DL PRS Resource Allocation Option 2 - 
DL PRS Resource Set allocates DL PRS Resources with different DL TX spatial filter inside DL PRS occasion

Figure 4: DL PRS Resource Allocation Option 3 - 
DL PRS Resource Set allocates DL PRS Resources with different DL TX spatial filter across multiple DL PRS occasions

Figure 5: DL PRS Resource Allocation Option 4 - 
DL PRS Resource Set allocates DL PRS Resources with the same DL TX spatial filter across multiple DL PRS occasions

It is straightforward that described above options have tradeoff in terms of latency to complete full TX-RX sweep procedure, amount of allocated resources per DL PRS occasion and possibly transmission density. Assuming that transmission density per resource is the same analysis of latency vs number of resources is presented in Table 1.

Table 1: Analysis of DL PRS Resource Allocation Options

| DL PRS Resource Allocation Option | Number of DL PRS resources per DL PRS occasion per single TRP | Number of DL PRS occasions for spatial relation acquisition (Beam pairing latency) |
| --- | --- | --- |
| Option 1 | NTX_SWNRX_SW | DL PRS occasion period |
| Option 2 | NTX_SW | NRX_SW DL PRS occasions |
| Option 3 | 1 | NTX_SWNRX_SW DL PRS occasions |
| Option 4 | 1 | NTX_SWNRX_SW DL PRS occasions |

NTX_SW – number of DL TX beams, NRX-SW – number of DL RX beams

From NR DL Positioning performance and design perspective, Option 1 and Option 2 are the most attractive and thus prioritized in this document for NR Positioning design in Rel.-16.

  - DL PRS Resources of any DL PRS Resource Set are allocated inside one DL PRS occasion

  - NR Positioning supports configuration of DL PRS Resource Sets where all DL PRS Resources use either 1) the same DL TX spatial filter or 2) different DL TX spatial filters

  - DL PRS occasion may allocate either 1) DL PRS Resource Sets where all DL PRS Resources use the same DL TX spatial filter or 2) DL PRS Resource Sets where all DL PRS Resources use different DL TX spatial filters

### Resource Configuration

In this section, we continue discussion on DL PRS Resource configuration settings and focus on remaining opens based on the agreements below made by RAN1 WG last time.

- DL PRS Resource is described by at least the following parameters - DL PRS Resource ID (previously agreed) - Sequence ID (previously agreed) - Comb Size-N - RE Offset in frequency domain - FFS whether this offset is a single value or multiple values - Starting slot and symbol of DL PRS Resource - FFS whether it can be represented by time offset with respect to some reference - Number of symbols per DL PRS Resource (Duration of DL PRS Resource) - Quasi-colocation information (QCL with other DL reference signals) - FFS QCL type and source reference signals - FFS: Number of Tx Ports - FFS: Power offset b/w DL PRS and SSB - FFS: Transmission bandwidth and starting PRB with respect to Point A - FFS: Numerology - Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not. Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} DL PRS Resource comb-N value is configurable from the set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 8, 12} Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98: - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level - Common period is used for DL PRS resources within a DL PRS Resource Set - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level - Different periods can be used for DL PRS resources within a DL PRS Resource Set

Resource element offset in frequency domain

RAN1 agreed on configurable comb- size N and number of symbols M per DL PRS Resource. The RE Offset in frequency domain was also agreed, however its values were not fixed. In order to complete discussion on RE Offset pattern for DL PRS, in the next table we propose several RE Offset patterns that are recommended to be supported by NR Positioning (please also refer to Figure 6).

Table 2: Resource Element Patterns for DL PRS Transmission

|  |  | Comb Size – N |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2 | 4 | 6 |
| Number of symbols per DL PRS Resource M | 2 | {0, 1} | {0, 2} | {0, 3} |
|  | 4 | {0, 1, 0, 1} | {0, 2, 1, 3} | NA |
|  | 6 | {0, 1, 0, 1, 0, 1} | NA | {0, 3, 1, 4, 2, 5} |

Figure 6: RE Patterns for DL PRS Resources

  - Resource element offset in frequency domain depends on configured number of symbols and comb-size N configured per resource varies depending on symbol index of DL PRS Resources varies

  - For DL PRS adopt resource element patterns provided by Table 2 illustrated by Figure 6

Starting slot and symbol of DL PRS Resource

The starting slot and symbol of DL PRS Resource within a given DL PRS Resource Set should be indicated by time offset to the starting symbol of DL PRS Resource Set (please refer to Figure 7). Time offset of DL PRS Resource should be defined with respect to the time offset of DL PRS Resource Set where that time offset of DL PRS Resource Set is defined relative to SFN-0 timing of serving cell. The time offset of DL PRS Resource should point to slot and starting symbol within a slot and thus can be represented by slot and symbol offsets:

Starting symbol offset: 0 – 13 symbols

Starting slot offset: 0 – (Periodicity-1) slots, where “Periodicity” is configurable parameter

  - Starting slot and symbol of DL PRS Resource is defined relative to time offset of DL PRS Resource Set, which is determined with respect to SFN0 of serving cell

    - Starting symbol offset: 0 – 13 symbols

    - Starting slot offset: 0 – (Periodicity-1) slots, where “Periodicity” is configurable parameter

Quasi-colocation information (QCL with other DL reference signals)

The discussion on QCL information is provided in our companion contribution [5]. In summary, we propose to signal the following QCL information in FR1 (Table 3) and FR2 (Table 4).

Table 3: DL PRS QCL for FR1

| Target RS | DL RS – 1 | QCL type – 1 |
| --- | --- | --- |
| DL PRS | SSB | QCL type C |
|  | CSI-RS for Tracking | QCL type A |
|  | DL PRS | QCL type A |

Table 4: DL PRS QCL for FR2

| Target RS | DL RS – 1 | QCL type – 1 | DL RS – 2 | QCL type – 2 |
| --- | --- | --- | --- | --- |
| DL PRS | SSB | QCL type C | SSB | QCL type D |
|  |  |  | CSI-RS for BM | QCL type D |
|  | CSI-RS for Tracking | QCL type A | CSI-RS for Tracking | QCL type D |
|  | DL PRS | QCL type A |  |  |

- Note: QCL type A: Doppler shift, Doppler spread, average delay, delay spread; QCL type B: Doppler shift, Doppler spread; QCL type C: Average delay, Doppler shift.

Power offset b/w DL PRS and SSB

In CSI-RS framework similar threshold is used to derive CSI-RS EPRE and applied for radio-link monitoring and UL PC for PUSCH. The use of this power offset for NR Positioning needs further discussion.

Numerology (DL PRS Subcarrier Spacing and Cyclic Prefix)

It was agreed by RAN1 that “for intra-frequency measurements (when measurement gaps are configured), the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs”. Given that DL PRSs are configured independently of DL BWPs, the numerology settings for DL PRS Resources should be indicated separately, if measurement gaps are configured, otherwise the numerology of active DL BWP can be assumed. Therefore DL-PRS-SCS and DL-PRS-Cyclic Prefix need to be signaled.

  - DL PRS numerology (subcarrier spacing and cyclic prefix) are signaled to UEs

Transmission BW and starting PRB with respect to Point A

Similar to numerology, the DL PRS transmission bandwidth and offset to start PRB relative to reference Point A should be separately indicated. It was argued that these configuration settings are redundant and that all DL PRSs can use the whole system BW. If wideband transmissions are confirmed by RAN1 this particular signaling can be omitted.

  - DL PRS transmission BW and starting PRB with respect to Point A are signaled to UEs

Additional configuration settings for number of symbols

Number of symbols per DL PRS resource is agreed to be configured from the set {2, 4, 6}. Considering hearability issues are quite typical especially for deployments with large inter-site distance, we propose to add 12 symbols as a possible configuration per DL PRS resource.

  - Support configuration of 12 symbols per DL PRS Resource

Additional configuration settings for Comb-N values

The agreed set of Comb-N values is rather flexible and consistent with the number of symbols per DL PRS resource. The agreed Comb-N values may fit multiple coverage scenarios. Therefore, we do not see very strong motivation to introduce additional values to configure Comb-N size.

  - Do not introduce additional Comb-N values

Periodicity of DL PRS Resource Allocation

All DL PRS Resources that belong to DL PRS Resource Set should have the same common periodicity. Therefore we do not see much motivation to define periodicity at the DL PRS Resource level which will bring extra overhead to signaling w/o clear benefits. The following periodicities of DL PRS Resource allocation in slots are proposed:

15kHz SCS: {5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560} slots

30kHz SCS: {10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120} slots

60kHz SCS: {20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots

120kHz SCS: {40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480} slots

In order to be consistent with CSI-RS configuration settings, the additional periodicity values can be considered {4, 8, 16, 32, 64}.

The proposed above periodicities may be defined at either DL PRS Resource Set level or above. In our design we assume that DL PRS occasion periodicity is the same as the periodicity of DL PRS Resource Set allocation.

  - Support the following periodicity of DL PRS resource allocation

    - 15kHz SCS: {5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560} slots

    - 30kHz SCS: {10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120} slots

    - 60kHz SCS: {20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots

    - 120kHz SCS: {40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480} slots

  - All DL PRS Resources within DL PRS Resource Set have the same periodicity

  - Periodicity of DL PRS Resource Set is the same as periodicity of DL PRS occasion

Common Settings across DL PRS Resources of DL PRS Resource Sets

RAN1 still need to discuss whether certain DL PRS Resource parameters are to be defined at DL PRS Resource, DL PRS Resource Set level or even higher. Parameters defined at DL PRS Resource Set levels will have common values across all DL PRS Resources within considered DL PRS Resource Set or above.

The following proposal provides our view with respect to definition of DL PRR Resource parameters at the DL PRS Resource or DL PRS Resource set level:

  - The following DL PRS Resource parameters are defined at DL PRS Resource level:

    - DL PRS Resource ID

    - DL PRS Sequence ID

    - Starting slot and symbol of DL PRS Resource (with respect to start of DL PRS Resource Set)

  - The following parameters describing DL PRS Resource are defined at DL PRS Resource Set level or above:

    - Resource Element Offset in Frequency Domain

    - Bandwidth of DL PRS Resource

    - Start PRB of DL PRS Resource

    - Comb-N Size of DL PRS Resource

    - Number of symbols per DL PRS Resource

    - Periodicity of DL PRS Resource Allocation

    - Numerology of DL PRS Resource (Subcarrier Spacing and Cyclic Prefix)

### Resource Set Configuration

In this section, we define attributes to describe DL PRS Resource Set configuration settings:

Cell ID

  - Indicates to which cell the configured DL PRS Resource Set belongs to.

DL PRS Resource Set ID

  - Uniquely identifies configured DL PRS Resource Set

Time Offset of DL PRS Resource Set

  - Provides time offset to the start slot/symbol of a given DL PRS Resource Set relative to SFN0 of serving cell (please refer to Figure 7)

List of DL PRS Resource IDs

  - Uniquely identifies DL PRS Resources in a given DL PRS Resource Set (please refer to Figure 7)

Number of Repetitions

  - Indicates number of consecutive repetitions of DL PRS Resource Set allocation within single DL PRS occasion (please refer to Figure 7)

Spatial filter indicator (Repetition On/Off)

  - Indicates whether the same DL Tx spatial filter (antenna port) is applied across all DL PRS Resources within a DL PRS Resource Set and its repetitions. Needed for RX antenna training across DL PRS Resources

Muting Pattern(s)

  - Indicates DL PRS muting pattern acting across Repetitions of DL PRS Resource Set or DL PRS occasions

Figure 7: DL PRS Resource Allocation

Based on description of the above fields, we have following proposal on DL PRS Resource Set configuration settings:

  - DL PRS Resource Set is described at least by the following attributes

    - Cell ID

    - DL PRS Resource Set ID

    - Time Offset of DL PRS Resource Set

    - List of DL PRS Resource IDs

    - Number of DL PRS Resource Set Repetitions

    - Spatial Filter Indicator (Repetition On/Off)

    - Muting Pattern(s)

Common Settings across DL PRS Resource Sets

In general, many of parameters describing DL PRS Resource or DL PRS Resource Set can be defined even above the DL PRS Resource Set level and are likely to have common values across DL PRS Resource Sets in many practical configurations. The list of candidate parameters that are likely to have common settings even across DL PRS Resource Sets is given below:

DL PRS Resource Bandwidth

DL PRS Resource Start PRB

Comb-N Size of DL PRS Resource

Number of symbols per DL PRS Resource

Periodicity of DL PRS Resource Allocation

Numerology of DL PRS (Subcarrier Spacing and Cyclic Prefix)

Time Offset of DL PRS Resource Set

Number of DL PRS Resource Set Repetitions

Although above parameters may often have common values even across DL PRS Resource Sets, we believe it is worthwhile to keep them at the DL PRS Resource Set level to keep high flexibility of the NR system.

## DL PRS Coexistence w/ DL Transmissions

In Rel.15, mechanisms of forward compatibility were developed, e.g. RateMatchPattern for PDSCH, etc. This mechanism indicates resources (slots and symbol and PRBs), where UE is expected to do rate-matching. By configuring proper RateMatchPattern(s) to Rel.15 UEs it is possible to avoid conflict with DL PRS transmission. Another option is to allocate DL PRS transmissions in slots that are not supposed to have DL transmission due configuration settings and gNB scheduling.

  - The developed in Rel.15 forward compatibility mechanisms (e.g. RateMatchPattern) can be reused to avoid conflicts with Rel16 DL PRS allocation

DL PRS Transmission Schedule

The following agreements were made with respect to DL PRS transmission schedule and muting support.

DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements - The UE is not expected to perform any blind detection of DL PRS configurations DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted.

In this section, we provide more details on transmission schedule and muting principles for NR positioning. For NR positioning system, it is beneficial to support two modes for DL PRS transmission:

Predefined DL PRS Transmission Mode

  - In this mode, it is assumed that TRPs (identified by PRS IDs) are intelligently mapped to allocated orthogonal PRS resources for PRS transmission. The DL PRS transmission schedule is automatically determined by UE for each DL PRS occasion once PRS resources are configured. This mode of operation enables optimal planning of the deployment and PRS transmissions in dedicated positioning areas.

Randomized (or pseudo-randomized) DL PRS Transmission Mode

  - Randomized DL PRS transmission mode can be used in deployment scenarios not optimized for positioning, i.e. without special considerations on deployment planning to maximize positioning performance. In this mode PRS ID mapping to PRS resources may be suboptimal.

Evaluation results of two scheduling approaches are illustrated in the Figure 8, where Predefined and Randomized transmission modes are compared, evaluation assumptions can be found in Annex-B.

Figure 8: Comparison of predefined and randomized scheduling modes for DL positioning

In the subsequent, sections we assume that DL PRS occasion is a grid of dedicated orthogonal PRS resources and that either DL PRS Resource Set or DL PRS Resource is allocated to one or more orthogonal resources within this grid.

Figure 9: DL PRS Occasion - Logical View

## Predefined DL PRS Transmission Schedule (Mode)

The predefined PRS transmission mode distributes PRS transmissions from all stations across configured orthogonal PRS resources (i.e. maps unique PRS IDs / station IDs to allocated PRS resources or resource sets). In order to optimize performance, all transmitting stations (NPRS-ID) should be equally distributed across N orthogonal resources (e.g. N=NTNF orthogonal time frequency resources) allocated for transmission (note either DL PRS Resources or DL PRS Resource Sets can be mapped to orthogonal resource). In general, this problem is similar to finding K combinations out of NPRS-ID length sequence (i.e. combinations of K= floor(NPRS-ID/(N)) stations out of NPRS-ID stations). However simply finding arbitrary combinations is not sufficient. In order to optimize positioning performance, we need to ensure that each station has one transmission opportunity in N allocated resources. In addition, at the next transmission window opportunity (e.g. DL PRS occasion or repetitions of DL PRS Resource Set), the new unique combinations of transmitting stations should occupy allocated orthogonal resources.

If number of orthogonal resources is a prime number P, then at least P2 combinations (each of K stations/IDs) with no more than one intersecting element if K≤P   and nor more than ceil(K/P) intersecting elements if P<K can be analytically found. In fact, if K≤P it is possible to form P groups, each composed of P combinations, where combinations in each group do not have intersecting elements (i.e. unique combinations within a group). This is an optimal framework to build PRS transmission schedule and optimize positioning performance of NR system. If (P<K) then there is a maximum of floor(K/P)  intersecting elements among any two combinations of different groups. In order to avoid multiple intersections among combinations, each combination can be divided into floor(K/P)  sub-combinations intersecting by nor more than one element.

The corresponding notations and analytical framework are provided in the following section.

### Mapping of PRS IDs in Predefined DL PRS Transmission Schedule

Let us introduce the following notations to define mapping of PRS IDs to orthogonal PRS Resources across multiple occasions:

NPRS-ID  – maximum number of PRS IDs supported by specification;

IPRS-ID – set of all PRS IDs, IPRS-ID={0, 1,…,NPRS-ID-1};

P – is the number of orthogonal resources per occasion. In order to form P groups (each composed of P combinations of K-length sequences), the P should be the largest prime number which is  ≤floor(NPRS-ID/K);

K= floor(NPRS-ID/(NTNF)) – number of PRS IDs per orthogonal resource;

w – is the index of PRS transmission occasion within SFN cycle period 0≤w≤ wmax-1;

wmax is the maximum number of PRS transmission occasions per SFN cycle;

w – is the index that identifies group of P combinations to be used on occasion w; w=modw,P, 0≤w≤(P-1);

i – is the logical index of PRS resource within transmission occasion 0≤i≤(P-1);

Sw,i –combination of PRS IDs mapped to i-th PRS resource in occasion w;

Using notations above, the mapping of PRS IDs to orthogonal PRS resources can be generalized by the following equations:

ind(n,i,w) = n+K∙mod(n∙w+i,P)

Sw,i= IPRSindn,i,w ; n=0, 1, 2,…., (K-1)

w=0, 1, 2,…., (P-1)

i=0, 1, 2,…., (P-1)

The above equations generate P combinations of PRS IDs for P PRS occasions, where each combination is of length K. The alternative option is to use equivalent equation below that generates logical index i of PRS resource for each PRS ID IPRS-IDm and each occasion 0≤w≤(P-1):

iIPRS-IDm,w=modIPRS-IDm+w∙floorIPRS-IDmP∙P-1, P;

m=0, 1,…,NPRS-ID-1

w=0, 1, 2,…., (P-1)

Let’s consider a few illustrative examples with different configuration settings to show and analyze results produced by equations above.

Example # 1: PRS Transmission Schedule (P is prime, P=K=3, NPRS=9)

Let’s consider example of NPRS-ID =9, N=NTNF=3, IPRS-ID= {0, 1, 2, 3, 4, 5, 6, 7, 8}. In this case, number of orthogonal resources is equal to the number of stations transmitting on resources, i.e. P=K=3. Applying equations above, we can determine PRS IDs transmitting on each resource in three consecutive occasions (see also Figure 10):

Occasion #0 (Group 0, w = 0)

  - IPRS, i = 0 = {0, 3, 6}; IPRS, i = 1 = {1, 4, 7}; IPRS, i = 2  = {2, 5, 8}

Occasion #1 (Group 1, w = 1)

  - IPRS, i = 0 = {0, 4, 8}, IPRS, i = 1 = {1, 5, 6}, IPRS, i = 2 = {2, 3, 7}

Occasion #2 (Group 2, w = 2)

  - IPRS, i = 0 = {0, 5, 7}, IPRS, i = 1 = {1, 3, 8}, IPRS, i = 2  = {2, 4, 6}

Figure 10: Example of PRS Transmission Schedule (P is prime, P=K=3, NPRS=9)

As it can be seen from the provided example and Figure 10, combinations of stations transmitting on orthogonal resources within each single occasion are not intersecting, while any two combinations of stations transmitting in different occasions do not intersect on more than one entry.

Example # 2: PRS Transmission Schedule (P is prime, P=3<K=6, NPRS=18)

In this case, the set of PRS ID is extended IPRS-ID= {0, 1, 2, 3, 4, 5, …,16, 17}. Number of allocated orthogonal resources is two times less than the number of stations transmitting on resources. Applying equations above, we can determine PRS IDs transmitting on each resource in three consecutive occasions (see also Figure 11):

Occasion #0 (Group 0, w = 0)

  - IPRS, i = 0 = {0, 3, 6, 9, 12, 15}; IPRS, i = 1 = {1, 4, 7, 9, 13, 17}; IPRS, i = 2  = {2, 5, 8, 11, 14, 17}

Occasion #1 (Group 1, w = 1)

  - IPRS, i = 0 = {0, 4, 8, 10, 13, 16}, IPRS, i = 1 = {1, 5, 6, 10, 14, 15}, IPRS, i = 2 = {2, 3, 7, 11, 12, 16}

Occasion #2 (Group 2, w = 2)

  - IPRS, i = 0 = {0, 5, 7, 11, 14, 17}, IPRS, i = 1 = {1, 3, 8, 11, 12, 16}, IPRS, i = 2  = {2, 4, 6, 11, 13, 15}

Based on analysis it can be observed that combinations of PRS IDs within a single occasion are still unique combinations (non-intersecting). On the other hand, subset of combinations from different occasions have at most two intersecting entries. The number of intersecting elements may further increase if the ratio floor(K/P) increases.

Figure 11: Example of PRS Transmission Schedule (P is prime, P=3<K=6, NPRS=18)

The problem of intersecting entries can be resolved if each combination is divided into two sub-combinations (e.g. sub-combination #0 is composed from the first three entries (e.g. {0, 3, 6}) and sub-combination #1 is composed from the last three entries (e.g. {9, 12, 15}). It can be verified that sub-combinations are unique across occasions. Therefore in order to avoid intersections, the sub-combinations may occupy two different sets of orthogonal resources or muting based on sub-combinations can be applied. The alternative option is to simply increase the number of allocated resources so that P becomes equal or larger than K.

Example # 3: PRS Transmission Schedule (P is not prime, P=4<K=6, NPRS=24)

Finally, let’s consider example when P is not a prime number. In this case, we can still generate P groups each having a unique (non-intersecting) combinations of transmitting stations. The number of intersections among combinations that belong to different groups/occasions may exceed the value of floor(K/P) given that P is not a prime number (see Figure 12).

Figure 12: PRS Transmission Schedule (P is not prime, P=4<K=6, NPRS=24)

Similar to example 2 in order to cope with intergroup intersections either muting can be applied or amount of resources can be increased.

Finally, we propose to use the described above equations in order to map PRS IDs (or station IDs) to orthogonal PRS resources. Depending on amount of allocated resources and length of PRS ID set, different scenarios are possible.

- Number of PRS IDs is not multiple of the amount of configured orthogonal PRS resources

  - In this case, we can extend the set of PRS IDs so that it size is multiple of the allocated orthogonal PRS resources and then apply described equations. The extra PRS IDs can be simply ignored in mapping process.

- Number of allocated resources is not a prime number

  - RAN1 agreed that Comb-size can take one of the following values: 2, 4, 6. Among agreed numbers, only number 2 is a prime number. In order to support operation with prime numbers, the number of PRS repetitions or consecutive PRS occasions can be configured to be a prime number (e.g. 2, 3, 5, 7, 11, 13, 17, 19, 23, 29, 31, etc). In this scenario, amount of frequency resources can be used to split the whole set of PRS IDs on orthogonal subsets transmitting on orthogonal frequency resources (e.g. different frequency shifts). The subsets of PRS IDs can be further divided into combinations to be transmitted across prime number of repetitions or occasions.

  - NR DL PRS transmission schedule supports predefined hopping of TRP transmissions across orthogonal resources of DL PRS occasions, e.g. hopping of DL PRS Resource Sets across orthogonal resources of DL PRS occasions

  - Unique combinations of DL PRS Resource Sets are transmitted on orthogonal resources across predefined number of DL PRS occasions

## Randomized DL PRS Transmission Schedule (Mode)

Randomized DL PRS transmission mode can be useful in deployment scenarios not optimized for positioning, i.e. without special considerations given to deployment planning to maximize positioning performance. In this mode, the resource used for PRS transmission by a given PRS ID is an output of the pseudo-random generator. This mode of operation does not ensure nice properties that can be provided by the predefined transmission mode such as 1) uniform loading across orthogonal PRS resources and 2) transmission of unique sets of TRPs across different PRS occasions and thus may require more spectrum resources to achieve similar performance. On the other hand this mode may require less specification efforts while providing sufficient performance in many scenarios.

The simple way to implement randomized DL PRS transmission mode is to use uniform pseudo random generator with predefined initialization procedure to generate resource index to be used by given PRS ID in a given occasion.

The pseudo random selection procedure of DL PRS transmission resource can be based on the following formula, where notations defined to describe predefined transmission schedule are reused:

iIPRS-IDm,w=modfloorxwP, P;

m=0, 1,…,NPRS-ID-1

w=0, 1, 2,…., wmax

In above equations, values of x are obtained from the pseudo random generator. For instance, the following generator used to generate LTE search space, can be used here to derive x:

xn=moda∙x[n-1],b

x-1=moda∙(IPRS-IDm+1),b

where, a and b are predefined numbers (a=39827,  b=65537).

  - NR supports DL PRS transmission schedule based on pseudo random generator with uniform distribution

## Hopping of DL PRS Transmissions of DL PRS Resource Set

DL PRS Resource Set defines multiple DL PRS Resources representing spatial beams of a TRP. When TX beam sweeping is supported it is also desirable to randomize spatial interference from different TRPs across multiple PRS occasions or across DL PRS Resource Set repetitions.

Figure 13: Beam hopping across repetitions of DL PRS Resource Set

Assume, that initial assignment of DL TX spatial filters to DL PRS Resources of DL PRS Resource Set is defined as B0. Different DL TX spatial filters (Beam IDs) can be assigned to a PRS Resource in the subsequent DL PRS occasions or DL PRS Resource Set repetitions. Therefore, the new permuted set of Beam IDs can be a function of occasion ID, number of resources /beams per DL Resource Set and initial assignment of beams per resources (occasion ID = 0).

- NBF – number of spatial filters applied for DL PRS transmission. In order to determine a set of beam IDs for w PRS occasion, following deterministic or randomized procedures can be applied:

Deterministic procedure (cyclic shift)

- Bw = Bw-1modn+∆BF,NBF, n=0,1,…NBF-1, where

- Bw-1 – set of beam IDs for occasion (w-1);

- ∆BF – TRP specific beamforming index permutation parameter;

Pseudo random procedure

- Bwn =Bw-1rwmn,  n=0,1,…NBF-1, where

- Bw-1 – beam Id set for w-1 occasion;

- rwm – vector of randomly permuted indexes for occasion w for PRS ID =  (values: from 0 to NBF-1);

  - Support beam permutation/hopping before transmission on DL PRS Resources of DL PRS Resource Set

DL PRS Muting

This section describes DL PRS muting and its relationship w/ DL PRS transmission schedule. Muting mechanism is beneficial to control density of DL PRS transmissions on DL PRS Resources and spatial reuse in the system.

## Bitmap Muting Pattern

For muting, we assume that bitmap pattern (similar to LTE Positioning) is configured where each bit is associated with certain time instance (occasion or repetition). The one-to-one correspondence between DL PRS Resource/Resource Set and its muting on DL PRS occasion or DL PRS Resource Set repetition is assumed.

## Resource Level Muting

A muting pattern can be applied on a DL PRS Resource level, where the muting rule is capable to activate or deactivate subset of DL PRS Resources inside of DL PRS Resource Set. Figure 14 and Figure 15 illustrate two examples of DL PRS Resource based muting. The first example assumes that bitmap pattern is defined per DL PRS resource and periodically applied across occasions. The second example assumes that bitmap pattern is configured per DL PRS Resource Set and the rule is defined to derive the pattern from one occasion to another occasion.

Figure 14: Resource muting using bitmap muting pattern per DL PRS Resource

Figure 15: Resource muting using bitmap muting pattern per DL PRS Resource Set

Although the muting can be applied at the DL PRS Resource level (beam level), we do not see much motivation to support resource level muting. It complicates angle based positioning measurements and its performance benefits are questionable due to spatially directive transmissions, especially if beam permutation is applied across PRS occasions. Therefore, we believe that muting defined at DL PRS Resource Set level is sufficient for NR positioning.

## Resource Set Level Muting

Muting mechanism can be applied on a DL PRS Resource Set level as it is illustrated in Figure 16. Each DL PRS Resource Set is configured with bitmap pattern that controls transmission of TRP (or given DL PRS Resource Set) on each DL PRS occasion or repetition.

Figure 16: DL PRS Resource Set Muting

If muting is applied per DL PRS Resource Set then muting pattern configuration should be a part of DL PRS Resource Pool configuration.

  - NR supports DL PRS muting at the DL PRS Resource Set (i.e. TRP) level

  - DL PRS Resource Set is configured with bitmap pattern that indicates muting of DL PRS Resource Set across DL PRS occasions and DL PRS Resource Set repetitions

## Predefined DL PRS Muting

As it was discussed in Section 3, the muting can be considered as the transmission schedule. If N orthogonal resources are allocated for transmission by NPRS stations, and there is a high loading per resource (NPRS/N) then muting can reduce number of stations transmitting on the same resource by switching off transmissions for half (or subset) of stations. However, muted stations should have opportunity for transmission at the next occasion while another half (or subset) of stations will be switched off. This problem formulation is equivalent to the design of transmission schedule with doubled amount of resources (i.e. 2N orthogonal resources). Solution for optimized transmission schedule across occasions was proposed in Section 3.

The muting may be applied on top of the transmission schedule and further optimize it. For instance, it can be used to reduce number or avoid intersecting elements among combinations of stations transmitting on different occasions or any other purpose. Let’s again consider example 2 and mute PRS IDs to avoid intersections in combinations occupying the same resource across different occasions (see Figure 17).

Figure 17: Example 2 w/ PRS muting applied on top of PRS transmission schedule

In general case, the following options can be applied for PRS muting in NR system:

Intra-Resource Muting: Muting of PRS transmissions on subset of time-frequency resources inside of DL PRS Resource

Resource Level Muting: Muting of PRS transmissions on subset of DL PRS Resources inside of DL PRS Resource Set

Resource Set Level Muting: Muting of PRS transmissions on subset of DL PRS Resource Sets within DL PRS Resource occasion

TRP Level Muting: Muting of dedicated TRPs configured with a DL PRS Resource Sets

In the next sub-sections, we discuss DL PRS Resource and DL PRS Resource Set based muting mechanisms as the most promising approaches for NR positioning.

## Randomized DL PRS Muting

Similar to randomized DL PRS transmission schedule (resource allocation), the PRS muting can be also randomized across DL PRS occasions. As an example, the following mechanisms for muting pattern randomization can be used:

NPRS-ID  – maximum number of PRS IDs supported by specification;

IPRS-ID – set of all PRS IDs, IPRS-ID={0, 1,…,NPRS-ID-1};

w – is the index of transmission occasion 0≤w≤ wmax;

M – muting periodicity

K – number of transmission per muting period M

- The node with PRS-ID IPRS-ID(m) will mute the PRS Resource Set in occasion w, if parameter’s value j[w] exceeds the value of K.

- jw=floorxwM, xw=moda∙xw-1,b

- x-1= moda∙(2∙NPRS-ID+ IPRS-IDm+1),b,

a=39827,b=65537

  - NR supports random DL PRS Resource Set muting across sets of PRS occasions

DL PRS Design

In this section we continue discussion on remaining details of DL PRS design.

## Number of DL PRS Sequences

As it was agreed at the previous meeting the amount of DL PRS sequences is at least 4096. In our view, the 4096 sequences is sufficient number for NR Positioning purposes.

  - Keep 4096 as a number of sequences for NR Positioning

## DL PRS Sequence Initialization

RAN1 agreed to reuse the pseudo-random sequence generator from Section 5.2.1 in [2]. Discussion on initialization of generator for DL PRS Resource sequence generation requires more progress on DL PRS Resource allocation. For instance, it may be dependent on support of TX/RX sweep and how it is implemented across Resource Sets or across Resources within Set, etc. For RX Sweeping with unknown timing it may be desirable to generate repeated signal in order to enable sweep with reduced complexity at UE RX side.

In general the following equation may be considered as a starting point for discussion in cases where no RX sweeping is assumed:

cinit=212∙14(ns,fμ+1)+l+1∙NPRSID mod 256+1+NPRSID

where NPRS_ID  ϵ {0…4095} equals NPRS_ID (Resource or Resource Set ID), TRP specific parameter for positioning configuration, ns,fμ – slot number within a frame, l – symbol index inside of the slot.

In case of DL PRS transmission with different or similar spatial filters in different PRS Resources inside one PRS Resource Set, the initialization seed for each PRS resource can be same as the initialization seed for the first PRS Resource, as it is depicted in Figure 18.

Figure 18: Initialization seed assignment for resources with different spatial filters

  - Apply the same cinit values for DL PRS Resources with different or similar DL TX spatial filter within DL PRS Resource Set

  - For cinit initialization, apply equation provided in this section

DL PRS Multiplexing with DL Transmissions

## TDM

TDM transmission of DL PRS signals with other DL signals/channels is preferred in both FR1 and FR2. This will help to avoid dynamic range and in-channel selectivity issues at the UE side leading to more predictable performance and accurate measurements. RAN1 has already agreed on allocation of dedicated resources for DL PRS transmission. It should be clarified further the DL PRS transmissions are TDM with other DL signals/channels.

## FDM

FDM transmission of DL PRS with other DL signals/channels is not preferred in both FR1 and FR2, and should be precluded at least for FR2. The FDM of DL-PRS with other signals may negatively affect the UE dynamic range and lead to unpredictable results, considering wideband beam management.

  - DL PRS transmissions are multiplexed in time with other DL reference signals / channels.

# Conclusion

In this contribution, we have provided our views on DL reference signal design for NR positioning. In summary, we have following proposals:

  - DL PRS Resources of any DL PRS Resource Set are allocated inside one DL PRS occasion

  - NR Positioning supports configuration of DL PRS Resource Sets where all DL PRS Resources use either 1) the same DL TX spatial filter or 2) different DL TX spatial filters

  - DL PRS occasion may allocate either 1) DL PRS Resource Sets where all DL PRS Resources use the same DL TX spatial filter or 2) DL PRS Resource Sets where all DL PRS Resources use different DL TX spatial filters

  - Resource element offset in frequency domain depends on configured number of symbols and comb-size N configured per resource varies depending on symbol index of DL PRS Resources varies

  - For DL PRS adopt resource element patterns provided by Table 2 illustrated by Figure 6

  - Starting slot and symbol of DL PRS Resource is defined relative to time offset of DL PRS Resource Set, which is determined with respect to SFN0 of serving cell

    - Starting symbol offset: 0 – 13 symbols

    - Starting slot offset: 0 – (Periodicity-1) slots, where “Periodicity” is configurable parameter

  - DL PRS numerology (subcarrier spacing and cyclic prefix) are signaled to UEs

  - DL PRS transmission BW and starting PRB with respect to Point A are signaled to UEs

  - Support configuration of 12 symbols per DL PRS Resource

  - Do not introduce additional Comb-N values

  - Support the following periodicity of DL PRS resource allocation

    - 15kHz SCS: {5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560} slots

    - 30kHz SCS: {10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120} slots

    - 60kHz SCS: {20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots

    - 120kHz SCS: {40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480} slots

  - All DL PRS Resources within DL PRS Resource Set have the same periodicity

  - Periodicity of DL PRS Resource Set is the same as periodicity of DL PRS occasion

  - The following DL PRS Resource parameters are defined at DL PRS Resource level:

    - DL PRS Resource ID

    - DL PRS Sequence ID

    - Starting slot and symbol of DL PRS Resource (with respect to start of DL PRS Resource Set)

  - The following parameters describing DL PRS Resource are defined at DL PRS Resource Set level or above:

    - Resource Element Offset in Frequency Domain

    - Bandwidth of DL PRS Resource

    - Start PRB of DL PRS Resource

    - Comb-N Size of DL PRS Resource

    - Number of symbols per DL PRS Resource

    - Periodicity of DL PRS Resource Allocation

    - Numerology of DL PRS Resource (Subcarrier Spacing and Cyclic Prefix)

  - DL PRS Resource Set is described at least by the following attributes

    - Cell ID

    - DL PRS Resource Set ID

    - Time Offset of DL PRS Resource Set

    - List of DL PRS Resource IDs

    - Number of DL PRS Resource Set Repetitions

    - Spatial Filter Indicator (Repetition On/Off)

    - Muting Pattern(s)

  - NR DL PRS transmission schedule supports predefined hopping of TRP transmissions across orthogonal resources of DL PRS occasions, e.g. hopping of DL PRS Resource Sets across orthogonal resources of DL PRS occasions

  - Unique combinations of DL PRS Resource Sets are transmitted on orthogonal resources across predefined number of DL PRS occasions

  - NR supports DL PRS transmission schedule based on pseudo random generator with uniform distribution

  - Support beam permutation/hopping before transmission on DL PRS Resources of DL PRS Resource Set

  - NR supports DL PRS muting at the DL PRS Resource Set (i.e. TRP) level

  - DL PRS Resource Set is configured with bitmap pattern that indicates muting of DL PRS Resource Set across DL PRS occasions and DL PRS Resource Set repetitions

  - NR supports random DL PRS Resource Set muting across sets of PRS occasions

  - Keep 4096 as a number of sequences for NR Positioning

  - Apply the same cinit values for DL PRS Resources with different or similar DL TX spatial filter within DL PRS Resource Set

  - For cinit initialization, apply equation provided in this section

  - DL PRS transmissions are multiplexed in time with other DL reference signals / channels.

# References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson

- TS38.211, V15.4.0, “NR: Physical channels and modulation”

- R1-1908660, “UL reference signals for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908661, “Physical layer measurements for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- aR1-1908662, “NR positioning physical layer procedures”, Intel Corporation, Prague, Czech Republic, August, 2019

# Annex A – Summary of Agreements on DL PRS

In this section, we provide summary of agreements made by RAN1 with respect to DL PRS design at the previous meetings.

RAN1#96bis Agreements

A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID - The DL PRS Resources in a DL PRS Resource set are associated with the same TRP A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams) Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE. DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1 - FFS: cinit value for DL PRS QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM FFS: Whether a DL PRS sequence generated using a different mechanism is also specified Comb-N resource element pattern per DL PRS Resource is supported to map DL PRS sequence to resource elements in frequency domain - Comb-N pattern can shift in frequency domain across symbols within DL PRS Resource - FFS values for N. The potential values for down-selection are provided in the set {1,2,3,4,6,8,12} - FFS relationship between N and number of symbols per DL PRS Resource - FFS support of staggered and non-staggered patterns and exact definition of staggered pattern Number of DL PRS Sequence IDs is at least 4096 Support of numerologies (CP length and sub-carrier spacing) for the DL PRS is the same as for data transmissions in Rel-15. DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements - The UE is not expected to perform any blind detection of DL PRS configurations DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted.

RAN1#97 Agreements

- DL PRS Resource is described by at least the following parameters - DL PRS Resource ID (previously agreed) - Sequence ID (previously agreed) - Comb Size-N - RE Offset in frequency domain - FFS whether this offset is a single value or multiple values - Starting slot and symbol of DL PRS Resource - FFS whether it can be represented by time offset with respect to some reference - Number of symbols per DL PRS Resource (Duration of DL PRS Resource) - Quasi-colocation information (QCL with other DL reference signals) - FFS QCL type and source reference signals - FFS: Number of Tx Ports - FFS: Power offset b/w DL PRS and SSB - FFS: Transmission bandwidth and starting PRB with respect to Point A - FFS: Numerology - Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not. - Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} - DL PRS Resource comb-N value is configurable from the set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 8, 12} - Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters - RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98: - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level - Common period is used for DL PRS resources within a DL PRS Resource Set - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level - Different periods can be used for DL PRS resources within a DL PRS Resource Set DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted - FFS: If this definition is introduced for muting or DL PRS resource allocation or both

# Annex-B: Evaluation Assumptions

Table 5. Table of system level evaluation assumptions.

| Parameter | Value |
| --- | --- |
| Deployment scenario, Bandwidth | UMa, FR1, 5/100 MHz |
| Muting assumption | No muting |
| Number of PRS occasions | 2 |
| Number of orthogonal PRS resources per occasion | 6 |
| Number of used symbols for DL PRS | 1 |
| Number of PRS occasions for UE coordinate calculation | 2 |
| Measurement selection criteria between UE and each TRP | Best SINR |
| Frequency mapping pattern | Comb-1 |