3GPP TSG RAN WG1 Meeting #98																R1-1908662

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	NR Positioning Physical Layer Procedures

Agenda item:	7.2.10.4

Document for:	Discussion and Decision

Introduction

Work item on NR Positioning support was approved in [1]. The WID has objective to define (if necessary) physical layer procedures to support UE / gNB measurements:

If necessary, define physical-layer procedures to support UE/gNB measurements for NR positioning [RAN1]

- According to the agreement made at the last meeting the potential physical layer procedures to support UE/gNB measurements for NR positioning may include:

UE procedures for receiving DL PRS

UE procedures for transmitting UL PRS

gNB procedures to support NR positioning measurements

In this document, we discuss procedures for NR Positioning. Our views on other NR Positioning aspects are provided in our companion contribution [2]-[4].

UE Procedures for DL PRS Reception

## Relationship of PRS w/ DL BWPs

At the previous RAN1 WG meeting, the following agreements were made with respect to DL BWPs and UE behavior in terms of DL PRS processing.

For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis) - Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met

The UE behavior in terms DL PRS processing was clarified while the UE behavior in terms of other channel reception was not discussed. By measurement gap definition, if it is configured for DL PRS processing, then UE is not expected to process other DL channels and signals during measurement gap period. If UE is not configured with measurement gap and processes DL PRS within the active DL BWP then it needs to be discussed whether UE is supposed to monitor other DL control channel and signals. The overall answer may depend on multiplexing of DL PRS and other DL signals / channels and RAN4 analysis on scheduling availability, considering the following RAN1 agreement: “there is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources”.

  - UE is not expected to process other DL channels / signals when measurement gap for DL PRS processing is configured

  - Send LS to RAN4 to discuss scheduling availability when DL PRS Resources are processed in active BWP

## DL Beam Management/Alignment

Under proper configuration of DL PRS Resources and DL PRS Resource Sets, a UE may train Rx beams and apply it for DL PRS reception, or use fixed RX beam. Subject to DL PRS Resource Set / Resource configuration, UE may sweep RX beam for each DL PRS Resource. The following options were identified by RAN1 WG to assist UE to perform Rx beamforming:

For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported: - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS) - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts

Considerations on Option 1

Regarding Option 1 and quasi-collocation, we think it is beneficial to indicate it to UE. The following can be proposed for operation in FR1 and FR2:

Table 1: DL PRS QCL for FR1

| Target RS | DL RS – 1 | QCL type – 1 |
| --- | --- | --- |
| DL PRS | SSB | QCL type C |
|  | CSI-RS for Tracking | QCL type A |
|  | DL PRS | QCL type A |

Note: QCL type A: Doppler shift, Doppler spread, average delay, delay spread; QCL type B: Doppler shift, Doppler spread; 
QCL type C: Average delay, Doppler shift.

Table 2: DL PRS QCL for FR2

| Target RS | DL RS – 1 | QCL type – 1 | DL RS – 2 | QCL type – 2 |
| --- | --- | --- | --- | --- |
| DL PRS | SSB | QCL type C | SSB | QCL type D |
|  |  |  | CSI-RS for BM | QCL type D |
|  | CSI-RS for Tracking | QCL type A | CSI-RS for Tracking | QCL type D |
|  | DL PRS | QCL type A |  |  |

Note: QCL type A: Doppler shift, Doppler spread, average delay, delay spread; QCL type B: Doppler shift, Doppler spread; 
QCL type C: Average delay, Doppler shift; QCL type D: Spatial Rx parameters.

Based on analysis for tables above, we have following proposal:

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with each other using QCL Type-A, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with CSI-RS for tracking (TRS) using QCL Type-A and/or QCL Type-D, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with CSI-RS for beam management (BM) using QCL Type-D, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with SSB (SSB index) using QCL Type-C and/or QCL Type-D, if applicable

Considerations on Option 2

RAN1 agreed that UE may perform an Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. There are two possible options how to support it:

Alternative 1: UE performs RX beam sweeping across DL PRS Resources (which are transmitted with the same downlink spatial domain transmission filter) of DL PRS Resource Set mapped within DL PRS occasion

  - This alternative has advantage in terms of latency but may require more DL PRS Resources

Alternative 2: UE performs RX beam sweeping across DL PRS Resources (which are transmitted with the same downlink spatial domain transmission filter) of DL PRS Resource Set mapped to subsequent DL PRS occasions

  - This is a long term process and may not be always suitable for certain positioning applications, especially in cases of low latency requirements or high mobility requirements. The advantage of this approach is relatively low DL PRS overhead

- In order to address all scenarios, both alternatives can be supported by specification. For implementation of Alternative 1, the principle used for CSI-RS beam management can be reused, i.e. “Repetition On/Off” field can be defined per DL PRS Resource Set to indicate whether the same downlink spatial domain transmission filter is applied across resources of DL PRS Resource Set. If “Repetition” is On, it is assumed that all DL PRS Resources with the same downlink spatial filter are allocated within single DL PRS occasion instance.

  - DL PRS Resource Set configuration defines “Repetition” field to control whether the same downlink spatial domain transmission filter is used across all DL PRS Resources of DL PRS Resource Set

  - If “Repetition” is On, UE is expected to perform RX beam sweeping over DL PRS Resources that belong to DL PRS Resource Set

When “Repetition” is Off, it is expected that DL PRS Resource with the same DL PRS Resource ID use the same downlink spatial transmission filter across DL PRS occasions. In this case, UE may do RX beam sweeping across different DL PRS occasions. In this case, it is also assumed that all DL PRS Resources of DL PRS Resource Set are allocated within single DL PRS occasion instance.

  - When “Repetition” is Off, UE is expected to perform RX beam sweeping across different DL PRS occasions

  - UE is expected to keep track of RX beams used for DL Processing at different DL PRS occasions

- Considerations on Option 3

- RAN1 has agreed that UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. In our view, this case is described by the scenario when DL PRS Resource Set is configured with “Repetition = Off” and UE switches RX beam across DL PRS occasions.

- Finally, we would like to note that for all of the above options, the UE RX beam sweeping procedure should be left up to UE implementation (i.e. procedures whether and how to select RX beam are not specified). RAN1 assumes that for each DL PRS Resource, a UE may select different RX beam and establish association between DL PRS Resources and UE RX beams however these details are also left up to UE implementation.

  - UE RX beam-sweeping procedures during DL PRS reception are left up to UE implementation

    - i.e. procedures whether and how to select UE RX beam for DL PRS reception are not specified

  - RAN1 assumes that for each DL PRS Resource, a UE may select different RX beam and establish association between DL PRS Resources and UE RX beams

    - i.e. association of DL PRS Resources with UE RX beams is up to UE implementation including the case when single RX beam is associated with multiple DL PRS Resources

## DL PRS Configuration (UE- and Cell-Centric Resource Allocation)

For NR positioning (especially for operation in FR2), two types of DL PRS resource allocation may be desirable depending on use case and scenario.

Cell-Centric DL PRS Resource Allocation

The cell-centric DL PRS Resource Allocation targets support of all UEs interested in positioning services and is not optimized in terms of DL signaling and PRS allocation towards specific UE. Oppositely it targets to serve all UEs and is supposed to have all DL PRS Resources configured comparing to UE-Centric. For Cell-Centric DL PRS Resource Allocation UE is supposed to process at least subset of configured DL PRS Resources and provide feedback report aiming to construct UE-Centric DL PRS Resource Allocation.

UE-Centric DL PRS Resource Allocation

The UE-Centric DL PRS Resource Allocation assumes that UE has provided feedback to network based on processing of Cell-Centric DL PRS Resources or Rel.15 legacy signals such as SSB or CSI-RS for BM. The UE feedback may be used to construct UE-Centric DL PRS Resource and Resource Set configuration. This type of operation may be beneficial to reduce UE processing complexity and measurement delay given that only subset of Resources may be configured and monitored by the UE.

UE-Centric DL PRS Measurement Report

In case of Cell-Centric DL PRS Resource Allocation, the UE-Centric operation in terms of measurements and reporting can be controlled through configuration of UE-Centric DL PRS Measurement Report that will request UE to do measurements on subset of Cell-Centric DL PRS Resources.

If there is no Cell-Centric DL PRS Resource Allocation provided by network, UE can be configured with UE-Centric DL PRS Resource Allocation. The main benefit of this approach is to reduce overhead of DL PRS Resource allocation aiming to provide service to specific target UEs. The UE-Centric DL PRS resource allocation may be especially beneficial to support configuration of on-demand DL PRS transmissions.

  - NR Positioning supports configuration of Cell-Centric DL PRS Resource Sets and Resources

  - NR Positioning supports configuration of UE-Centric DL PRS Resource Sets and Resources

  - NR Positioning supports UE-Centric DL PRS Measurement Report to request measurements on subset of Cell-Centric DL PRS Resources or UE-Centric DL PRS Resources

UE Assistance Information for UE-Centric Resources or Measurement Report Configuration

For construction of UE-Centric DL PRS Resource Allocation or DL PRS Measurement Report (DPMR) UE may provide the following information to network / location server:

Cell IDs

SSB indexes (e.g. corresponding to max SS-RSRP value)

CSI-RS Resource IDs (e.g. corresponding to max L1-RSRP value)

UE location information (e.g. if UE has a-priori knowledge of it coordinates)

UE TX-RX time difference measurement based on Rel.15 reference signals

Recommended DL PRS Resources (Resource IDs) based on processing of Cell Centric DL PRS Resources

  - For construction of UE-Centric DL PRS Resource Sets (or configuration of UE-Centric DL PRS Measurement Report), UE may be requested to provide at least the following assistance information

    - Cell IDs

    - SSB indexes (e.g. corresponding to max SS-RSRP value)

    - CSI-RS Resource IDs (e.g. corresponding to max L1-RSRP value)

    - UE location information (e.g. if UE has a-priori knowledge of it coordinates)

    - UE Tx-Rx time difference based on Rel.15 reference signals

    - Recommended list of DL PRS Resources (IDs) based on processing of Cell-Centric DL PRS Resources

## DL PRS Measurement Reporting

The time and frequency resources that can be used by the UE to report measurements are controlled by the gNB. The UE is expected to be configured with DL PRS Measurement Report that will instruct UE to perform measurements on indicated DL PRS Resources. More discussion on details of UE measurements is provided in our companion contribution [4].

## DL PRS Multiplexing with Other DL Signals/Channels

During the study item phase on NR positioning, RAN1 made the following agreement with respect to DL PRS Resource allocation:

Dedicated NR DL PRS resources - time-frequency grid at resource block level - PRS transmitted in one cell may or may not collide with PRS transmissions in other cell - e.g. frequency vShift/comb-offset is the same or different for two different PRSs in the same RB - There is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources - FDM multiplexing with other signals at RE level inside of PRS time-frequency grid is precluded

Our understanding of the agreement above, is that DL PRS resources are allocated from system perspective and there is no other transmissions in dedicated DL PRS resources other than DL PRS transmissions.

CORESET and SIB1 Considerations

Similar to CSI-RS considerations, UE shall not expect to be configured with DL PRS over the symbols during which the UE is also configured to monitor the CORESET. In addition, UE is not expected to receive CSI-RS and SIB1 message in the overlapping PRBs in the OFDM symbols where SIB1 is transmitted.

  - UE shall not expect to be configured with DL PRS over the symbols during which the UE is also configured to monitor the CORESET

  - UE is not configured to receive DL PRS and SIB1 message in the overlapping PRBs in the OFDM symbols where SIB1 is transmitted

  - DL PRS Resource / Resource Set configuration does not affect UE rate matching behaviour for DL reception, which is indicated separately

UE Procedures for UL SRS Transmission

In this section, we continue discussion on remaining open aspects of UL SRS transmission for NR Positioning.

## Timing Advance for UL SRS (PRS)

The UE TA behavior for UL-TDOA and RTT is one of the open items for discussion. There are different options to be considered as it was agreed at the first meeting of WI:

For positioning purposes, with regard to the UL SRS transmission timing, at least the following options have been identified: - Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols). - Option 2: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request. - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement - Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE.

In our view, Option 1 is the simplest option and should be supported for NR positioning. It will simplify timing estimation procedure and have similarity with communication framework so that no additional UE behavior is needed. In our view, other options introduce unnecessary complexity at UE side without clear benefit in terms of positioning performance.

  - Adopt Option 1 as UE timing advance behaviour for UL SRS(PRS) transmission, where the same timing advance value based on serving cell is applied for transmission

## Power Control for UL SRS

The following agreement was made with respect to UE PC for UL SRS (PRS) at the previous meeting:

For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour: - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - Study further the UE configuration signalling and procedure. - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference. - FFS on number of measurements for pathloss.

Open loop and Closed Loop Power Control

The configuration of neighbor cell pathloss for UL PC was agreed by RAN1 as an optimization technique to reduce inter-cell interference. However it was not discussed whether UL PC should be supported for that case. Considering potential inter-cell communication delays CL PC can be problematic.

  - Do not support UL SRS CL PC for the case when neighbor cell pathloss is configured for OLPC

Reference Signals for Neighbor Cell Pathloss

In order to apply OLPC based on neighbor cell pathloss, UE need to estimate it. The pathloss can be derived based on RSRP measurement for SSB or DL PRS if knowledge of TX power is available.

  - UE is configured whether to use DL PRS or SSB of neighbor cells to derive DL pathloss of neighbor cells. Configuration signaling details are up to RAN2

UE Configuration Signaling and Procedure

UE need to know reference signal power of the DL PRS and SSB of neighbor cells. Therefore UE should be configured by network with reference signal power settings of the DL PRS or SSB for neighbor cells.

  - UE is configured with reference signal power of the DL PRS and SSB of neighbor cells. Configuration signaling details are up to RAN2

In order to configure neighbor cell pathloss as a reference, network should be aware that UE has reliable estimate of neighbor cell pathloss and monitor its value. It means that reliable DL measurements (that meet requirements) should be in place before configuration of UL SRS transmission and when neighbor cell pathloss is in use. In general network can apply proper signaling to enable this. At the same time, due to mobility UE may not be able to obtain reliable pathloss measurements. In this case, either UE need to be preconfigured with RSRP threshold that can be used as a trigger to inform serving cell/network that neighbor cell pathloss is below threshold and fallback to serving cell pathloss.

  - UE is preconfigured with RSRP threshold to trigger report that neighbor cell pathloss is below threshold and fallbacks default behavior (i.e. serving cell pathloss and PC settings). Configuration signaling details are up to RAN2

## UL Beam Management/Alignment w/ Serving/Neighbouring TRPs

The following options were identified by RAN1 WG with respect to UL beam management / alignment considering operation with serving and neighboring cells/TRPs:

For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping): - Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. - Reference DL RS that can be used include at least SSB. FFS on CSI-RS, DL-PRS. - A fixed Tx beam for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2. - FFS how the fixed beam is selected. - FFS on specification impacts. - Note: In Rel-16, UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol (Rel-15 behaviour)

Spatial relation with DL RS

RAN1 agreed on configuration of spatial relationship between SSB and UL SRS (PRS). Given that in general, SSB and DL PRS transmission strategies may be different and considering different options for multi-cell RTT support, the configuration of spatial relationship with DL PRS (independently of SSB), seems necessary.

Selection of the fixed beam

Considering that UE needs to transmit on multiple SRS Resources w/ fixed beam, the “Repetition On/Off” principle should be extended to SRS Resource Set. In general, the fixed beam can be selected by UE or provided by gNB

The following combinations are possible in this case:

Case 1: UE is configured with single SRS Resource Set w/ “Repetition On” and w/o spatial relation information

  - In this case, UE autonomously selects one TX beam (e.g. the one corresponding to quasi-omni transmission) and gNB can perform RX beam training and measurements for multiple RX beams

Case 2: UE is configured with multiple SRS Resource Sets w/ “Repetition On” and w/o spatial relation information

  - In this case, UE autonomously selects multiple TX beams (e.g. to cover the whole space) and gNBs can perform RX beam training  measurements and UE TX Beam Sweeping across SRS Resource Sets)

Case 3: UE is configured with single or multiple SRS Resource Sets w/ “Repetition On” and w/ spatial relation information

  - In this case, UE is expected to align TX beams with provided spatial relation information and gNBs can perform RX beam training and measurements across SRS Resource Sets

Case 4: UE is configured with single or multiple SRS Resource Sets w/ “Repetition On” and SRS Resource IDs are spatially associated to SRS Resources from pre-configured SRS Resource Set w/ “Repetition Off”

  - In this case, UE is expected to use TX beam corresponding to spatially associated SRS Resources from the SRS Resources Sets w/ “Repetition Off” used for UL TX beam sweeping

Case 1 and Case 2 assume UE selection of the fixed TX beam for transmission. Case 3 assumes that gNB provides information on beam pairing with DL RS (i.e. spatialRelationInfo). Therefore UE is expected to select TX beams matched with the “best” beams used for reception of DL RS (i.e. SSB and/or DL PRS). For positioning purposes, it needs to be discussed whether each SRS Resources should contain spatialRelationInfo with only one SSB/DL PRS Resource Index or multiple SSB/DL PRS Resources Indexes (IDs) can be configured including serving and neighboring cells. If configuration of multiple SSB/DL PRS Resource Indexes (IDs) is allowed then UE should be expected to select beam that best matches (e.g. with a majority) of indicated set of resources provided by spatialRelationInfo. Case 4 assumes that UE need to apply the beam spatially related to the beam used for transmission in other SRS Resource Sets.

In summary, UE can autonomously select fixed beam if no spatial information is configured for target SRS resource or based on beam pairing information if spatial relation information is provided. Finally, gNB can configure spatial relation to SRS Resource in another SRS Resource Set and thus UE selects resource based on gNB determination.

  - UE autonomously selects beam if spatial relation information is not configured for target SRS Resource

  - UE is expected to select beam based on spatial relation if spatial relation information is provided for target SRS Resource

Specification impact for UL beam management / alignment

We see the following potential impact on UL PRS configuration with respect to UL SRS configuration:

Repetition On/Off field is added to UL PRS Resource Set configuration

Configuration of spatial relationship of DL PRS Resource and SSB with target UL PRS resource

Configuration of spatial relationship with multiple DL PRS Resources and SSB (i.e. from serving and/or neighbor cells)

  - Repetition On/Off field is added to UL PRS Resource Set configuration

  - NR Positioning supports configuration of spatial relation of target UL PRS Resource with DL PRS Resource (and/or SSB index agreed at the previous meeting)

  - NR Positioning supports configuration of spatial relation with multiple DL PRS Resources and/or SSB indexes (i.e. from serving and/or neighbor cells)

    - When spatial relation with multiple DL PRS Resources and/or SSBs Indexes is configured UE is expected to select the best beam matching to configured spatial relation (e.g. that matches the majority of configured resources)

# Conclusion

In this contribution, we have provided initial views on radio-layer procedures to support NR positioning. In summary, we have following proposals:

  - UE is not expected to process other DL channels / signals when measurement gap for DL PRS processing is configured

  - Send LS to RAN4 to discuss scheduling availability when DL PRS Resources are processed in active BWP

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with each other using QCL Type-A, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with CSI-RS for tracking (TRS) using QCL Type-A and/or QCL Type-D, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with CSI-RS for beam management (BM) using QCL Type-D, if applicable

  - DL PRS Resource(s) within DL PRS Resource Set can be configured as quasi-collocated with SSB (SSB index) using QCL Type-C and/or QCL Type-D, if applicable

  - DL PRS Resource Set configuration defines “Repetition” field to control whether the same downlink spatial domain transmission filter is used across all DL PRS Resources of DL PRS Resource Set

  - If “Repetition” is On, UE is expected to perform RX beam sweeping over DL PRS Resources that belong to DL PRS Resource Set

  - When “Repetition” is Off, UE is expected to perform RX beam sweeping across different DL PRS occasions

  - UE is expected to keep track of RX beams used for DL Processing at different DL PRS occasions

  - UE RX beam-sweeping procedures during DL PRS reception are left up to UE implementation

    - i.e. procedures whether and how to select UE RX beam for DL PRS reception are not specified

  - RAN1 assumes that for each DL PRS Resource, a UE may select different RX beam and establish association between DL PRS Resources and UE RX beams

    - i.e. association of DL PRS Resources with UE RX beams is up to UE implementation including the case when single RX beam is associated with multiple DL PRS Resources

  - NR Positioning supports configuration of Cell-Centric DL PRS Resource Sets and Resources

  - NR Positioning supports configuration of UE-Centric DL PRS Resource Sets and Resources

  - NR Positioning supports UE-Centric DL PRS Measurement Report to request measurements on subset of Cell-Centric DL PRS Resources or UE-Centric DL PRS Resources

  - For construction of UE-Centric DL PRS Resource Sets (or configuration of UE-Centric DL PRS Measurement Report), UE may be requested to provide at least the following assistance information

    - Cell IDs

    - SSB indexes (e.g. corresponding to max SS-RSRP value)

    - CSI-RS Resource IDs (e.g. corresponding to max L1-RSRP value)

    - UE location information (e.g. if UE has a-priori knowledge of it coordinates)

    - UE Tx-Rx time difference based on Rel.15 reference signals

    - Recommended list of DL PRS Resources (IDs) based on processing of Cell-Centric DL PRS Resources

  - UE shall not expect to be configured with DL PRS over the symbols during which the UE is also configured to monitor the CORESET

  - UE is not configured to receive DL PRS and SIB1 message in the overlapping PRBs in the OFDM symbols where SIB1 is transmitted

  - DL PRS Resource / Resource Set configuration does not affect UE rate matching behaviour for DL reception, which is indicated separately

  - Adopt Option 1 as UE timing advance behaviour for UL SRS(PRS) transmission, where the same timing advance value based on serving cell is applied for transmission

  - Do not support UL SRS CL PC for the case when neighbor cell pathloss is configured for OLPC

  - UE is configured whether to use DL PRS or SSB of neighbor cells to derive DL pathloss of neighbor cells. Configuration signaling details are up to RAN2

  - UE is configured with reference signal power of the DL PRS and SSB of neighbor cells. Configuration signaling details are up to RAN2

  - UE is preconfigured with RSRP threshold to trigger report that neighbor cell pathloss is below threshold and fallbacks default behavior (i.e. serving cell pathloss and PC settings). Configuration signaling details are up to RAN2

  - UE autonomously selects beam if spatial relation information is not configured for target SRS Resource

  - UE is expected to select beam based on spatial relation if spatial relation information is provided for target SRS Resource

  - Repetition On/Off field is added to UL PRS Resource Set configuration

  - NR Positioning supports configuration of spatial relation of target UL PRS Resource with DL PRS Resource (and/or SSB index agreed at the previous meeting)

  - NR Positioning supports configuration of spatial relation with multiple DL PRS Resources and/or SSB indexes (i.e. from serving and/or neighbor cells)

    - When spatial relation with multiple DL PRS Resources and/or SSBs Indexes is configured UE is expected to select the best beam matching to configured spatial relation (e.g. that matches the majority of configured resources)

# References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson

- R1-1908659, “DL reference signals for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908660, “UL reference signals for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019

- R1-1908661, “Physical layer measurements for NR positioning”, Intel Corporation, Prague, Czech Republic, August, 2019