3GPP TSG RAN WG1 #98	R1-1908672

Prague, CZ, August 26th – 30th, 2019

Source:	OPPO

Title:	Configured grant enhancements for URLLC

Agenda Item:	7.2.6.6

Document for:	Discussion and Decision

Introduction

In RAN1 97, some agreements are achieved on configured grant transmission and SPS transmission:

Agreements:

- For the maximum number of UL CG configurations per BWP of a serving cell:

  - 12

Agreements:

- Regarding Q1 in the LS in R1-1905940:

  - Although RAN1 has not completely analysed the potential impact of supporting up to 16 SPS configurations for a given BWP of a serving cell, RAN1 has the understanding that 8 SPS configurations for a given BWP of a serving cell is sufficient in Rel-16

Agreements:

- Support joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell if the bit-length for indication which configurations released is no more than 4 bits and DCI size is not impacted by adopting joint release.

  - FFS details.

In this contribution, we shall focus on multiple configured grant transmission.

Discussion

  - Multiple configured grant transmission

HARQ Process ID

In Rel-15, HARQ Process ID is mainly determined by time resource for the first transmission occasion and number of HARQ Process for configured grant. For multiple configured grants, independent HARQ Process ID can be configured and multiple HARQ Process ID will be reserved, e.g. 12 HARQ Process ID are reserved for 12 configured grants, then only 4 HARQ process ID can be used for dynamic transmission, which will leads schedule restriction due to HARQ Process ID is deficient.

For multiple configured grants for different service, Independent HARQ Process ID for multiple configured grants is benefit to support multiple services in the meantime, as shown in Figure 1(a). However, for multiple configured grants for the same service, independent HARQ Process ID is wasteful, due to only one configured grant can be used at one time, as shown in Figure 1(b).

Figure 1 HARQ Process allocation for multiple configured grants

Therefore, HARQ Process ID offset needs to be considered to avoid redundant HARQ Process ID allocation for configured grants. Then HARQ Process ID can be calculated as:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes+HARQ Process ID Offset

Proposal 1 HARQ Process ID offset needs to be added to avoid redundant HARQ Process ID allocation.

Signaling design for joint release

In 97, joint release for multiple configured grant transmission has been agreed. Signaling design is FFS. One simple solution is to define ConfiguredGrantGroupConfig, which is configured by RRC. One example is shown in the following. For Type 2 configured grant, ConfiguredGrantGroupConfigId for release is indicated in DCI. This solution does not modify ConfiguredGrantConfig.

ConfiguredGrantGroupConfig ::=                SEQUENCE {

configuredgrantgroupConfigId                          ConfiguredGrantGroupConfigId,

configuredgrantList   SEQUENCE (SIZE (1..maxNrofConfiguredGrant)) OF ConfiguredGrantConfigId

}

Proposal 2: ConfiguredGrantGroupConfig is defined to support joint release.

DCI for one configured grant release in Rel-15 can be baseline. Redundancy version, Modulation and coding scheme and Frequency domain resource assignment can still be set the same values as Rel-15. The field for HARQ process number can indicate ConfiguredGrantGroupConfigId. The field for HARQ process number is reused due to the bit-length for HARQ process number is enough and not much more to indicate ConfiguredGrantGroupConfigId.

Table 1: Special fields for UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 |
| --- | --- |
| HARQ process number | ConfiguredGrantGroupConfigId |
| Redundancy version | set to '00' |
| Modulation and coding scheme | set to all '1's |
| Frequency domain resource assignment | set to all '1's |

Proposal 3: DCI for configured grant release in Rel-15 can be baseline and the field for HARQ process number can indicate ConfiguredGrantGroupConfigId.

In addition, Maximum number of configured grant group needs to be defined. On the one hand, the bit-length for configured grant group release is no more than 4 bits, so upper bond of configured grant group number is 16. On the other hand, maximum number of configured grant is 12 and lower bond of configured grant group number is 12 to support independent release for multiple configured grants. To support both independent and joint release, maximum number of configured grant group can be 16. For example, 12 of the configured grant groups have one configured grant per configured grant group, and then the residual configured grant groups have more than one configured grants per configured grant group.

Proposal 4: Maximum number of configured grant group is 16.

Joint activation

Multiple configured grants can be configured to support different services with different requirement and packet size or enhance reliability and reducing latency. For different services with different requirement and packet size, configuration parameters in ConfiguredGrantConfig and UL grant type2 scheduling activation PDCCH differentiate significantly. So signaling for configured grants for different services should be independent, including ConfiguredGrantConfig and UL grant type2 scheduling activation/release PDCCH.

Proposal 5: Signaling for configured grants for different services should be independent, including ConfiguredGrantConfig and UL grant type2 scheduling activation PDCCH.

When multiple configured grants are configured to enhance reliability and reduce latency, most configuration parameters are similar. So multiple configured grants for the same service with different starting points can share one DCI for activation to reduce signaling overhead and avoid schedule latency due to PDCCH congestion.

Proposal 6: Multiple configured grants for the same service can share one UL grant type2 scheduling activation PDCCH to reduce signaling overhead.

Although most parameters keep the same values, small number of parameters needs to be different, as listed in the following:

- Time domain resource

The time domain resource of multiple configured grants for the same service is usually different to support data transmission which arrives at different time.

- DMRS port

Although multiple configured grants are configured, only one of the multiple configured grants can be used at one time. The same time-frequency resource can be shared by multiple configured grants to reduce reserved resource. To distinguish multiple configured grants, DMRS port needs to be different.

- HARQ Process ID offset (If exist)

For multiple configured grants, it is benefit that independent HARQ Process ID offsets are configured for multiple configured grants to share HARQ Process ID among multiple configured grants.

Proposal 7: The parameters, e.g. time domain resource, DMRS port and HARQ Process ID offset need to be different among multiple configured grants.

HARQ Process ID offset can be added in ConfiguredGrantConfig directly and then it can be configured per configured grants.

However, Time domain resource and DMRS port are configured by DCI. If multiple values for time domain resource and DMRS ports are added in DCI, then DCI size will get larger, which leads some issues on PDCCH monitoring. If multiple values for time domain resource and DMRS ports are added in ConfiguredGrantConfig directly, it will reduce schedule flexibility.

One solution is that time domain resource and DMRS port are still used and offsets to start point indicated by DCI can be configured by RRC to distinguish multiple configured grants. This solution keeps schedule flexibility and supports different configurations among multiple configured grants.  For example, TimeDomain ResourceOffset and DMRSOffset are added in ConfiguredGrantConfig or ConfiguredGrantGroupConfig.

Proposal 8: TimeDomain ResourceOffset and DMRSOffset are added in ConfiguredGrantConfig or ConfiguredGrantGroupConfig.

- Time domain resource is determined by Time domain resource assignment in DCI plus TimeDomain ResourceOffset in ConfiguredGrantConfig or ConfiguredGrantGroupConfig

- DMRS port is determined by Antenna ports in DCI (if exist) plus DMRSOffset in ConfiguredGrantConfig or ConfiguredGrantGroupConfig.

Conclusions

In this contribution, we show our views on multiple configured grants with following proposals:

Proposal 1 HARQ Process ID offset needs to be added to avoid redundant HARQ Process ID allocation.

Proposal 2: ConfiguredGrantGroupConfig is defined to support joint release.

Proposal 3: DCI for configured grant release in Rel-15 can be baseline and the field for HARQ process number can indicate ConfiguredGrantGroupConfigId.

Proposal 4: Maximum number of configured grant group is 16.

Proposal 5: Signaling for configured grants for different services should be independent, including ConfiguredGrantConfig and UL grant type2 scheduling activation PDCCH.

Proposal 6: Multiple configured grants for the same service can share one UL grant type2 scheduling activation PDCCH to reduce signaling overhead.

Proposal 7: The parameters, e.g. time domain resource, DMRS port and HARQ Process ID offset need to be different among multiple configured grants.

Proposal 8: TimeDomain ResourceOffset and DMRSOffset are added in ConfiguredGrantConfig or ConfiguredGrantGroupConfig.

- Time domain resource is determined by Time domain resource assignment in DCI plus TimeDomain ResourceOffset in ConfiguredGrantConfig or ConfiguredGrantGroupConfig

- DMRS port is determined by Antenna ports in DCI (if exist) plus DMRSOffset in ConfiguredGrantConfig or ConfiguredGrantGroupConfig.

References

- RAN1 97 Chairman notes