3GPP TSG RAN WG1 #98	                                                            	R1- 1908679

Prague, CZ, August 26th – 30th, 2019

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Resource Allocation for Mode 2 NR V2X

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

# Introduction

The work item for NR V2X was approved in RAN#83, and revised in RAN#84 [1], and the following objectives were identified in relation to resource allocation:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - Resource allocation [RAN1, RAN2] - Mode 1 - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome - Mode 2 - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome - Support for simultaneous configuration of Mode 1 and Mode 2 for a UE - Transmitter UE operation in this configuration is to be discussed after the design of mode 1 only and mode 2 only. - Receiver UE can receive the transmissions without knowing the resource allocation mode used by the transmitter UE. - UE relaying resource pool configuration or resource configuration is not supported in this work in Rel-16.

In the previous meeting in RAN1#97, the following agreements and conclusions were made [2]:

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

- RAN1 to further select between the following options of sidelink resource reservation for blind retransmissions:

  - Option 1: A transmission can reserve resources for none, one, or more than one blind retransmission

  - Option 2: A transmission can reserve resource for none or one blind retransmission

- Resource selection window is defined as a time interval where a UE selects sidelink resources for transmission

  - The resource selection window starts T1 ≥ 0 after a resource (re-)selection trigger and is bounded by at least a remaining packet delay budget

  - FFS T1 value, whether it is measured in slots, symbols, ms, etc.

  - FFS other conditions

- Support a sub-channel as the minimum granularity in frequency domain for the sensing for PSSCH resource selection

  - No additional sensing for other channels

Conclusion:

- RAN1 to discuss further the following

  - Maximum number of blind retransmissions supported for one TB

  - Maximum number of reserved blind retransmission

  - Maximum number of HARQ feedback-based retransmissions supported for one TB

  - Maximum number of reserved HARQ feedback-based retransmission

In this contribution, we propose the different resource allocation schemes which are to be employed in Mode 2.

# Sidelink Resource (Re-) Selection and Exclusion Procedure

The existing selection scheme in LTE for a UE operating in an autonomous resource selection mode dictates that within a defined sensing window, the UE will carry out sensing based on the measured SL-RSRP to decide whether a given resource is occupied or not. Resources are selected by comparing the SL-RSRP to a pre-defined SL-RSRP threshold in order to decide its occupancy status. The pre-defined threshold is an SL-RSRP value deduced from a list [3] ranging from -∞ dBm, -128 dBm to 0 dBm, +∞ dBm by considering the priority of the SCI to be transmitted (TX priority) as well as the priority stated in the decoded SCI (RX priority) in a given resource [4]. Based on the table defined in [5], the reporting range of RSRP by a UE is defined from -156 dBm to -44 dBm with 1 dB resolution, where -44 dBm signifies a resource with high occupancy, and -156 dBm signifies a resource with low occupancy.

If the TX priority and RX priority are taken to be the lowest value, the threshold against which the SL-RSRP is compared to is -2 dBm, which would mean that the UE is capable of selecting resources which are completely unoccupied as well as resources which are completely occupied. The issue with this approach is 2-fold:

- If a UE intending to carry out a low priority transmission selects a resource that is completely unoccupied, but does not really need the level of quality afforded by the selected resource, it deprives another UE of using the unoccupied resource for a high priority transmission.

- Although the priority of the transmission is low, a UE might select a resource that is already occupied and cause a resource collision with another UE having an ongoing high priority transmission.

Observation 1: In the LTE V2X selection procedure, a UE intending to transmit low priority transmissions is provided with a SL-RSRP threshold to select resources that can either deprive other high priority transmissions of an unoccupied resource, or even cause collisions to a UE with an ongoing high priority transmissions.

In the case where a UE cannot determine the required amount of resources within the defined threshold, the threshold value is increased by 3dB and the selection process is repeated. The increase of the threshold and selection process is repeated until the UE secures the required number of resources.

The drawback of this scheme is that the UE does not segregate the resources after increasing the threshold, even though it is aware that other UEs are occupying the resources and are of a lower quality that the initial ones with the defined threshold. All the resources that are selected, are eventually put into the same set of available resources, immaterial of whether the received power measured on these resources was higher in one and lower in another.

Observation 2: The increase of the SL RSRP threshold results in the aggregation of resources with varying occupancy levels, of which some resources may be of a lower quality that what the transmitting UE requires.

In order to overcome the issues listed above, as well as to cater to not only the priority of a transmission, but also the reliability and latency requirements, it would be advantageous to bring in the new PQI parameters to determine the SL-RSRP threshold for the selection of resources. Also, instead of using a single threshold, a range or a set of thresholds catering to different QoS levels can also be introduced, primarily in order to restrict UEs requiring resources for low QoS transmissions from hogging unused resources. If a UE is forced to carry out reselection and increase the threshold, the reselected resources with a higher occupancy should be categorized appropriately so that the UE can select resources based on the QoS of the transmission accordingly.

Proposal 1: We propose to carry out (re-) selection and exclusion of resources by incorporating the following procedures:

- Introduce QoS parameters in the selection of SL-RSRP thresholds,

- A range or a set of thresholds are to be used to cater to different QoS requirements,

- Classification of resources selected based on the measured SL-RSRP values.

# Assistance Information for UE Resource (Re-) Selection

Assistance information for resource selection can be provided to UEs in the form of reports being sent from one UE to another. When UEs are operating in Mode 2, they are expected to select their resources autonomously and hence will carry out sensing and selection procedures. The results of these sensing procedures can be sent to other UEs in the form of occupancy or measurement reports.

Since these reports are relevant only to the UEs in the same vicinity, or the same geographical area as described in LTE [6], a UE is expected to send the report only to other UEs in the same or neighbouring areas. Using these reports, the other UEs can form a zonal area resource usage map, where reports from different UEs will help understand the congestion/occupancy status of resource pools in the vicinity. This will enable them to decide the best available resources across geographical areas as well.

The content of the reports should include the following:

- Congestion or occupancy information – A UE reports sensing results of the previous slots, stating which resources could be used or which resources should be avoided by other UEs. This information can further be classified based on the communications type so as to give the other UEs a better picture of which resources to select.

- Feedback information – A UE reports the link characteristics between the UE and other UEs, providing information regarding the condition of the link, and whether it is feasible to use for certain priority based transmissions. This can be particularly advantageous among members of a group using groupcast communications.

Observation 3: UEs carrying out sensing can share their sensing results to neighbouring UEs to enable better resource allocation.

Proposal 2: We propose that assistance information in the form of reports containing the congestion/occupancy status of resources as well as information regarding link characteristics can be shared between UEs.

# Reservation of Resources

In order to cater to the stringent reliability and latency requirements for UEs operating in Mode 2, it is imperative for UEs to reserve resources in advance to meet the high reliability requirements. Resources can be reserved for HARQ retransmissions, blind retransmissions as well as for pre-emptive transmissions. Since the reservation of resources for HARQ retransmissions was already discussed in the previous meeting, this section focuses on the remaining 2 aspects of resource reservation.

## Reservation of Resources for Blind Retransmissions

Every time a UE sends control information pertaining to a data transmission, it can reserve resources in advance for a possible retransmission of the data. It can also reserve multiple resources in a periodic manner for a certain duration of time, given that the UE knows it would require these resources for data transmission. This would provide support for the periodic transmission of data as well as ensure that other UEs listening to the control information would be aware of the resource usage of a given UE.

At the same time, support for one-shot transmissions are also important, especially when the relevance of a message decreases with the UE moving away from the event that triggered the transmission of the message. This deems the retransmission as unnecessary since any retransmission would essentially carry outdated information.

Proposal 3: For the resource reservation for blind retransmissions, we support option 1, where the initial transmission can reserve resources for none, one or more than one blind retransmission.

## Pre-emptive Reservation of Resources

Pre-emption of a resource defines the act of temporarily interrupting the use of a given resource by a UE to allow data traffic of higher QoS from another UE to use this resource without asking for cooperation. The UE that pre-empts its resource would have to resume its transmission by deferring to another resource by following the resource re-selection procedure.

Proposal 4: We propose to support the pre-emptive reservation of resources for high priority transmissions.

When a UE reserves resources in a future time slot, it essentially requires another UE intending to use the resources in the future time slot to refrain from its transmission. However, this can cause conflicts in transmissions, as described below:

- A UE intending to use a scheduled resource within a future time slot in an SPS manner to transmit high priority transmissions, clashes with another UE that has pre-emptively reserved the same resource for a high priority transmission as well.

- More than one UE reserves the same resources within the same future time slot for high priority transmissions.

It is possible, however, for the TX UE carrying out the pre-emptive reservation of resources to reserve more than one resource at the same intended time slot, but in different sub-channels, where it is required to transmit. This would resolve the conflict where there is another TX UE that is already scheduled to use one of the reserved resources with a high priority transmission. The initial TX UE would simply carry out short term sensing [3, 4], e.g. listen before talk with a random back-off counter, within the time slot in order to determine which resource it can use from among the resources reserved across the different sub-channels.

The use of short-term sensing to determine the exact resource to be used will also resolve the conflict where more than one TX UE reserves the same resources. Using a random back-off counter, both TX UEs will have different back-off counters, enabling them to listen and check whether the resource in question is available or not.

Proposal 5: We propose to define the following procedures for the implementation of the pre-emptive reservation of resources:

- A UE will reserve more than one resource in order to resolve any resource conflict that may arise.

- A UE will carry out short term sensing on the given time slot in order to select the available resource.

# Mode 2 Resource Configuration Mechanisms for Groupcast Communications

In order to cater to the high reliability and low latency of groupcast communications, even when operating in Mode 2, we propose that different procedures have to be followed if the group was initialized when in-coverage of a gNB and when out-of-coverage. This is described in more detail in the following subsections.

## Group Initialization Occurs In-Coverage

If the group was formed and initialized when the UEs were in the coverage of a gNB, the gNB would provide resources for the member UEs of the group based on the requests sent by these UEs. The gNB could either allocate resources individually for each member, or could also allocate a set of resources common to all the members, following which the members can carry out sensing within this set.

When the group moves out of the coverage of the gNB and into Mode 2, we propose that the gNB provides a (pre-) configuration of a set of resources to the members based on the amount of resources that were used by the group while in-coverage. The member UEs can then utilize this set of resources and carry out sensing within them in order to communicate with each other. The advantage of such a (pre-) configuration is that the member UEs will have a dedicated set of resources which will not collide with other UEs carrying out groupcast or any other type of transmissions. This will ensure that the reliability and latency requirements of groupcast communications are met even when operating in Mode 2.

The (pre-) configuration of resources for the group can be updated when it moves back in-coverage with the gNB. This will enable the gNB to incorporate the updated demands of the group, for example, in the case where more UEs have joined the group.

Proposal 6: We propose to support that the gNB provides a (pre-) configuration of resources, exclusive to a group, to the member UEs while in-coverage to be utilized when the group moves into Mode 2.

## Group Initialization Occurs Out-of-Coverage

If the group was formed and initialized out-of-coverage of a gNB and when operating in Mode 2, the member UEs would have to carry out sensing for each of their individual communications within the group.

However, when the group moves in-coverage of a gNB, the members can receive resource configurations from the gNB based on the requests sent by the UEs. Once the gNB is aware of the requirements of the group, it can also provide a (pre-) configuration of resources for the group to communicate in when it moves out-of-coverage of the gNB.

Proposal 7: We propose to support that the gNB provides a (pre-) configuration of resources for a group when it moves in-coverage of a gNB, even if it was formed out-of-coverage of the gNB.

# Conclusion

Based on our analysis carried out in this contribution, we have the following observations:

Observation 1: In the LTE V2X selection procedure, a UE intending to transmit low priority transmissions is provided with a SL-RSRP threshold to select resources that can either deprive other high priority transmissions of an unoccupied resource, or even cause collisions to a UE with an ongoing high priority transmissions.

Observation 2: The increase of the SL RSRP threshold results in the aggregation of resources with varying occupancy levels, of which some resources may be of a lower quality that what the transmitting UE requires.

Observation 3: UEs carrying out sensing can share their sensing results to neighbouring UEs to enable better resource allocation.

Based on these observations, we propose the following:

Proposal 1: We propose to carry out (re-) selection and exclusion of resources by incorporating the following procedures:

- Introduce QoS parameters in the selection of SL-RSRP thresholds.

- A range or a set of thresholds are to be used to cater to different QoS requirements.

- Classification of resources selected based on the measured SL-RSRP values.

Proposal 2: We propose that assistance information in the form of reports containing the congestion/occupancy status of resources as well as information regarding link characteristics can be shared between UEs.

Proposal 3: For the resource reservation for blind retransmissions, we support option 1, where the initial transmission can reserve resources for none, one or more than one blind retransmission.

Proposal 4: We propose to support the pre-emptive reservation of resources for high priority transmissions.

Proposal 5: We propose to define the following procedures for the implementation of the pre-emptive reservation of resources:

- A UE will reserve more than one resource in order to resolve any resource conflict that may arise.

- A UE will carry out short term sensing on the given time slot in order to select the available resource.

Proposal 6: We propose to support that the gNB provides a (pre-) configuration of resources, exclusive to a group, to the member UEs while in-coverage to be utilized when the group moves into Mode 2.

Proposal 7: We propose to support that the gNB provides a (pre-) configuration of resources for a group when it moves in-coverage of a gNB, even if it was formed out-of-coverage of the gNB.

# References

- RP-190984, “Revised WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#84, June 2019.

- Chairman’s Notes, TSG RAN WG1 #97, May 2019.

- 3GPP TS 36.331 V15.5.1, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 15)”, April 2019.

- 3GPP TS 36.213 V15.5.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Physical layer procedures (Release 15)”, March 2019.

- 3GPP TS 36.133 V16.1.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Requirements for support of radio resource management (Release 16)”, March 2019.

- 3GPP TS 36.300 V14.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”, September 2017.