3GPP TSG RAN WG1 #98				  	          R1-1908698

Prague, Czech, 26th – 30th August, 2019

Agenda item: 7.2.8.1.

Source: LG Electronics

Title: Discussion on overhead reduction for Type II codebook

Document for: Discussion and Decision

- Introduction

In RAN1#97 meeting, the following agreements are captured in the chairman’s note [1].

Agreement

Alt3C (illustrated in the table below) is supported where the parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0, v0)(y0,v0) take value from   12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

- FFS: Possible down-selection on the FD combination parameters in RAN1#98

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

  - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

- For RI=3-4, bitmaps, each with size-2LMi -1(i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2

- The following FD basis subset selection scheme is supported:

  - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

  - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where  is higher-layer configured from two possible values

    - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Agreement

For RI=3-4, given the value of K0, the number of non-zero coefficients per layer shall be less than or equal to K0

- No further restriction on the maximum number of non-zero coefficients per layer

In this contribution, we discuss remaining issues on the overhead reduction for Type II CSI codebook and provide evaluation results in order to determine the codebook parameters and frameworks to support RI=3-4. To be specific, based on above agreements in RAN1#97 and the offline e-mail discussions, we suggest our view on the following issues as:

- Finalize the issues on UCI parameters

  - Values of

  - Values of Minitial

  - Restriction on bitmap to RI=3-4

- Identify alternatives for supported parameter combination

  - Supported parameter combinations for (L, p, β)

- Identify alternatives for UCI omission

- Identify alternatives for CBSR

- Discuss layer-/layer-group-specific quantization method and configuration to RI=3-4

- Discussions on Type II CSI extension to RI=3 and 4

In order to reduce the overhead for Rel-15 Type II CSI, DFT-based compression framework has been agreed for Rel-16 Type II CSI. In this codebook, the precoder for a layer is given by size-P×N3 matrix

W=W1W2WfH

where P=2N1N2, N3 is the size of frequency domain, W2 is size-2L×M matrix, L is the number of combining beams and M is the number of columns in Wf. For RI=1 and 2, independent FD basis selection and quantization has been agreed in RAN1#96. In this section, based on above agreements in RAN1#97, we further discuss on Type II CSI extension to RI=3 and 4 in the consideration that the amount of total overhead should be at least comparable to that of RI=2.

  - Issues on UCI parameters

- Values of α and Minitial

Agreement In RAN1#98, finalize the values of based on the following aspects - Candidate values for to be down selected/evaluated: at least {1.5, 2, 2.5} - The set of values is to be finalized via offline email discussion prior to RAN1#98 - FFS: =3 as an additional candidate - Configuration of : - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R - Whether is rank-specific or rank-common - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) Agreement On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table. - FFS: details on bitwidth and possible values for Minitial Minitialreporting in UCI part 2 - FFS: whether the possible value(s) for Minitial Minitialcan depend on configured FD compression parametersL,p,β,α

In the last meeting, FD basis subset selection scheme was determined by adopting a two-step report of the FD basis. So, the first step is to report an intermediate FD basis subset, which stands for the union of the N3' FD bases. Then, in the second step, FD bases for each layer are reported from the intermediate set by a combinatorial indicator which leads a quite reduction of reporting payload with less performance loss compared to FD basis subset reporting based on N3.

As described above, the intermediate FD basis subset is obtained among N3 FD basis set by a window-based operation and fully parameterized by  and Minitial shown in Figure 1. Then, we investigate the candidate values for the parameters in order to achieve the good performance-overhead trade off.

Figure 1. Window-based intermediate FD basis subset selection parameterized by  and Minitial

First, to specify the values of Minitial, it is important to consider the characteristics of delay profile. It means that the strong FD components comprising the FD basis are likely to be located within the window starting index = 0 or index = N3-N3'2 of N3 in a statistical manner.

Proposal 1: On Minitial values for FD basis subset selection for N3>19,

  - Support at least two Minitial values in consideration of delay profile

- 0

- N3-N3'2

Based on above Minitial values, the FD basis subset is comprised by adopting  values. Figure 2 presents the average UPT with the Type II codebook parameter setting. It is assumed that 16-port CSI-RS and medium traffic load. Also, each UE is equipped with 4 Rx antenna ports and maximum rank 4 transmission is considered. The parameter p is chosen in {y0=1/2, v0=1/4} and N3 is set to 20, respectively. Also, beam indicating method such as independent DFT beam selection in FD for layer-specific manner is taken into account. Other simulation assumptions are listed in Annex.

Figure 2. Average UPT vs. overhead trade-off for various parameter α

Observation 1. For RI-group-specific and layer-common p setting, poor performance and overhead trade-off is observed when alpha is set to 3, i.e., the large amount of window-size is quite redundant.

Proposal 2: On  values for FD basis subset selection for N3>19,

- Support  = {1.5, 2} as candidate values for N3' where  is higher layer configured from two values

- Support rank-common configuration of  from above two values

- Restriction on RI=3-4 bitmap

Agreement In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer.

Bitmap design is to indicate the indices of the reported NZ coefficients per layer. For RI=1-2, the bitmap has a 2LM size for each layer 0 and 1. For RI=3-4, it has been decided to have 2LMi-1 bitmap size for each layer in the last meeting.

From the agreement above, the bitmap design can be varied according to this additional polarization indicator. In this method, if some polarization for certain layer are turned off which will be indicated in UCI Part 1, the payload will be reduced since the bitmap can be expressed in half. However, such situation may be rarely happened, and large performance degradation is expected due to the lack of channel information corresponding to turn-off polarization. Also, as mentioned, such restriction may require additional payload in UCI Part 1. Thus, we do not see any strong reason for such restriction.

Proposal 3: Do not support the restriction on RI=3-4 bitmap for a polarization in each layer.

  - Supported parameter combinations (L, p, β)

Offline agreement - Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Type II - the UPT lower than Rel.15 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead

Here, the parameters L and p affect the number of SD and FD basis, respectively, and β determines the max number of non-zero coefficients per layer. Therefore, the parameters (L, p, β) are the main factors for determining the payload for CSI feedback. To further investigate the impact on SD/FD basis parameters, we only consider the value of β as 1/2 and 1/4.

The main point is that the parameter setting with RI-group-specific and layer-common allows a small amount of implementation impact and leads the performance enhancement in terms of performance-overhead trade off, which is depicted in Figure 3 as:

Figure 3. Average UPT vs. overhead trade-off for various parameter setting (L, p)

Figure 3 presents the average UPT with various parameter setting schemes. It is assumed that 16-port CSI-RS and medium traffic load. Also, each UE is equipped with 4 Rx antenna ports and maximum rank 4 transmission is considered. Other simulation assumptions are listed in Annex.

Observation 2: For p=12 (M=7) and β=14,12 provides good performance-overhead trade off, and for p=14 (M=4) and β=12,34 provides good performance-overhead trade off. Since total overhead depends on both p and β values, ranges of both parameters should be jointly selected.

Proposal 4: For L=4, support p,β=12, 14,12and p,β=14, 12,34 for parameter combination in Rel-16 Type II CSI.

Then, we provide various simulation results to verify the benefit of supporting L=6. For the evaluation, it is assumed that 32-port CSI-RS and medium traffic load. Also, UE is equipped with 2 or 4 Rx antenna ports, so that maximum rank can be 2 or 4, respectively. For payload comparison, we assume parameter β,p=12,14 in the simulation in Figure 4. To be payload comparable with RI=2 case, simulation in Figure 5 assumed βi =14 ∀i=1,…4 for RI=4. Other simulation assumptions are listed in Annex.

Figure 4. Performance comparison with various quantization schemes in RI=2 (L=6)

Figure 5. Performance comparison with various quantization schemes in RI=4 (L=6)

Observation 3: In rank 2 transmission with 32 Tx antennas, L=6 provides 7% performance gain over L=4 in terms of average UPT.

Observation 4: In rank 4 transmission with 32 Tx antennas and p=1/4, L=6 provides marginal performance gain over L=4 with p=1/2 in terms of average UPT. Also, the payload for L=6 with p=1/2 becomes more than 1000 bits which is not comparable with RI=2 payload.

Proposal 5: For L=6, support p,β=14, 14 and p,β=18, 14 for parameter combination in Rel-16 Type II CSI.

| L | p=y0 (rank 1-2) | p=v0 (rank 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- |
| 4 | 1/2 |  | {1/4, 1/2} | 16 Tx antennas |
|  |  | 1/4 | {1/2, 3/4} |  |
| 6 | 1/4 |  | 1/4 | 32 Tx antennas |
|  |  | 1/8 | 1/4 |  |

Table 1. Combination of parameter setting (L, p, β)

- Discussions on UCI omission

In Rel-15, a CSI omission procedure was introduced for the UE to report some part of CSI when the allocated PUSCH resource was not enough to contain full CSI. This is achieved by partitioning the Part 2 CSI into different priority levels and dropping CSI from the lowest priority level, until actual UCI coderate falls below a threshold, i.e., the CSI payload will fit on the PUSCH allocation.

However, in Rel-16, Type II CSI codebook has been designed based on the FD compression. Since the LC coefficients related to SD/FD basis are quite different from those of Rel-15, the Rel-15 CSI omission procedure cannot be directly reused for the Rel-16 codebook.

To address this issue, we investigate the components of UCI part 2. The largest impact on payload size is LC coefficients for amplitude/phase and the corresponding NZC bitmap per layer. For those components, the CSI omission scheme can be specified based on SCI per layer. This is due to the fact that the LC coefficients corresponding SD/FD basis on SCI may have greater impact on CSI accuracy than other LC coefficients.

As an example of CSI omission in Figure 9, we depict the LC coefficient omission with SCI. As described in agreements, the SCI only indicates the index of SD basis since FD basis index is remapped via modulo operation. Then, the LC coefficients which has higher priority to report are in row and column of W2 based on SCI. Then, the number of coefficients can be reduced from 56 to 14 and the size of the bitmap can be shorten as a same manner.

Figure 9. Example of UCI omission scheme for LC coefficients based on SCI

Proposal 6: Support LC coefficient omission for Rel-16 Type II codebook, where the FD/SD bases corresponding to NZC values are ordered according to a given priority rule and LC coefficients for the FD/SD bases are omitted from lower priority to higher priority. The FD/SD basis indicated by SCI have the highest priority for the omission.

- Discussions on CBSR

In RAN1#95 meeting, following agreement regarding codebook subset restriction (CBSR) has been made as:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

The CBSR plays important functionality for interference management by properly restricting reported PMIs which cause strong interference to other UEs. For this reason, in Rel-15, two different CBSR is adopted. One is per-SD DFT beam based restriction which is applied for Type I CSI, and the other one which is applied for Type II CSI is based on the combination of SD-DFT beam and its amplitude.

Unlike the Rel-15 Type II CSI, Rel-16 Type II CSI introduces additional FD basis in order to reduce the payload. In our view, restricting such FD basis (interpreted as delay tap) is not a reasonable solution, since gNB cannot accurately estimate which delay-taps cause the interference. Thus, our preference is re-using Rel-15 CBSR. However, due to the compression functionality, restricting amplitude may not directly control beam power, so the benefit form such CBSR needs to be further investigated.

Therefore, we propose to consider only beam-based CBSR for Rel-16. For simplicity, we can reuse the Rel-15 Type I CBSR, which can be directly applied to the Rel-16 Type II codebook. Also, as already supported in the previous release in both LTE and NR, RI restriction should be supported in Rel-16 in order to efficiently manage the interference.

Proposal 7: For Rel-16 Type II CSI, DFT beam-based CBSR is supported.

Proposal 8: For Rel-16 Type II CSI, RI restriction is supported.

- Discussions on LCC quantization to RI=3-4

In RAN1#96 and RAN1#96b meeting, the following agreement regarding quantization scheme of linear combining coefficient (LCC) for Rel-16 Type II CSI is captured in the chairman’s note as:

Agreement

UE reports the following for the quantization of the non-zero coefficients in W2

- A log2KNZ-bit indicator for the strongest coefficient index l*,m*

  - Strongest coefficient cl*,m*=1 (hence its amplitude/phase are not reported)

- Two polarization-specific reference amplitudes:

  - For the polarization associated with the strongest coefficient cl*,m*=1, since the reference amplitude = 1, it is not reported

  - For the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is {1,1214,1414,1814,…,121414 , 'reserved'} (-1.5dB step size)

- For cl,m,  l,m≠(l*,m*):

  - For each polarization, differential amplitudes of the coefficients calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

    - Note: The final quantized amplitude pl,m is prefl,m×pdiffl,m

  - Each phase is quantized to either 8PSK (3-bit) or 16PSK (4-bit) (configurable)

Agreement

On “zero” in the reference amplitude value set, “zero” is removed and the associated code point is designated as “reserved”.

- Note: “Reserved” implies that the associated code point is not used in reference amplitude reporting or, at least in Rel-16, any other purpose(s)

In this section, based on above agreements in RAN1#96b, we further discuss on a quantization method for higher rank extension RI=3 and 4 in the consideration that the amount of total overhead should be at least comparable to that of RI=2.

In Rel-16 Type II CSI, the total payload is affected by the number of combining beams, quantization level for combining coefficients, and the size of compression unit, etc. To ensure the performance, it is agreed that W2 should be applied with a layer-independent manner, hence most of CSI feedback payload comes from reporting it. Then, the number of non-zero coefficients (i.e., KNZ,i, i∈{1, 2, …, RI}) per layer has been adopted to reduce the payload related to W2 on RI=3-4 extension.

  - Adjustment of different quantization level to RI=3-4

Note that, based on the NNZC indication, controlling a quantization level among layer-/layer-group has a potential advantage in a perspective of average UPT-overhead trade-off. The motivation is that the channel quality among layers is much affected by eigenvalues of the channel which are different in general, and the channel can be expressed as a linear combination of eigenvectors and their corresponding eigenvalues. It can give the criterion for determining dominant channel directions, and the loss of the channel accuracy may not be so large when the combining coefficients corresponding to the dominant layer (e.g. layer 0) adopt higher quantization level compared to those of other layers.

This tendency can be shown in the following figure. Note that Table # presents the quantization level and the corresponding alphabet subset selection for each scheme. Here, it is assumed that for phase-bit set to {2, 3, 4}, the quantization is mapped with {4, 8, 16}-PSK. Also, when the number of amplitude-bit is less than 3 for differential amplitude, the amplitude subset can be organized with the following methods:

- ‘Seq.’ method: it selects the alphabets sequentially corresponding bits, e.g.,

  - For 2-bit (4 alphabets): {1,12,12,122,14,142,18,182}

  - For 1-bit (2 alphabets): {1,12,12,122,14,142,18,182}

- ‘Uni.’ method: it selects the alphabets uniformly corresponding bits, e.g.,

  - For 2-bit (4 alphabets): {1,12,12,122,14,142,18,182}

  - For 1-bit (2 alphabets): {1,12,12,122,14,142,18,182}

| Scheme | Subset method | # of amplitude bits |  |  |  | # of phase bits |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Layer 0 | Layer 1 | Layer 2 | Layer 3 | Layer 0 | Layer 1 | Layer 2 | Layer 3 |
| Conventional | - | 3 |  |  |  | 4 |  |  |  |
| Phase 1 | - | 3 |  |  |  | 4 |  | 3 | 2 |
| Phase 2 | - | 3 |  |  |  | 3 |  |  |  |
| Amp 1 | ‘Seq.’ | 3 |  | 2 | 1 | 4 |  |  |  |
| Amp 2 | ‘Uni.’ | 3 |  | 2 | 1 | 4 |  |  |  |
| Amp 3 | ‘Uni.’ | 3 |  | 2 |  | 4 |  |  |  |
| Combined | ‘Uni.’ | 3 |  | 2 |  | 3 |  |  |  |

Table 2. List of quantization level and amplitude alphabet subset for each quantization scheme

  - Simulation results

Figure 10 provides the average UPT performance comparison for max layer 4 per UE utilizing different quantization level for each layer as listed in Table #. In this plot, we observe that layer-specific quantization for amplitude has a lot of impact on the performance compared to that for phase (Amp1 vs. Phase1). Regarding to the amplitude subset, it is preferred to select the alphabets corresponding to the quantization bits uniformly to guarantee the UPT performance (Amp1 vs. Amp2). Most of all, compared to the conventional approach, proposed quantization scheme ‘Combined’ to RI=3-4 shows significant overhead reduction with 16%p with less performance loss. Note that this scheme adopts different quantization level for amplitude and phase by exploiting the benefits on both Amp3 and Phase2, respectively.

Figure 10. Average UPT vs. overhead trade-off for various quantization level on amplitude and phase

Observation 5: Layer-specific quantization for amplitude is more sensitive on the performance compared to that for phase.

Observation 6: For designing amplitude subset to guarantee the UPT performance, alphabets in differential amplitude should be selected uniformly.

Observation 7: For average performance-overhead trade-off, proposed quantization scheme shows significant overhead reduction (16%) with less performance loss (5%) by exploiting different quantization level for amplitude and phase at the same time.

Proposal 9: Support layer-/layer-group-specific quantization setting to RI=3-4 at least for the amplitude quantization level in an explicit or implicit manner.

- Conclusion

In this contribution, we discuss the overhead reduction for Type II CSI. Based on the discussion above, we have following observations and proposals as:

Observation 1. For RI-specific to RI=3-4 and layer-common p setting, the performance is degraded when alpha is set to 3, i.e., the amount of window-size is quite redundant

Observation 2: For p=12 (M=7) and β=14,12 provides good performance-overhead trade off, and for p=14 (M=4) and β=12,34 provides good performance-overhead trade off. Since total overhead depends on both p and β values, ranges of both parameters should be jointly selected.

Observation 3: In rank 2 transmission with 32 Tx antennas, L=6 provides 7% performance gain over L=4 in terms of average UPT.

Observation 4: In rank 4 transmission with 32 Tx antennas and p=1/4, L=6 provides marginal performance gain over L=4 with p=1/2 in terms of average UPT. Also, the payload for L=6 with p=1/2 becomes more than 1000 bits which is not comparable with RI=2 payload.

Observation 5: Layer-specific quantization for amplitude is more sensitive on the performance compared to that for phase.

Observation 6: For designing amplitude subset to guarantee the UPT performance, alphabets in differential amplitude should be selected uniformly.

Observation 7: For average performance-overhead trade-off, proposed quantization scheme shows significant overhead reduction (16%) with less performance loss (5%) by exploiting different quantization level for amplitude and phase at the same time.

Proposal 1: On Minitial values for FD basis subset selection for N3>19,

  - Support at least two Minitial values in consideration of delay profile

- 0

- N3-N3'2

Proposal 2: On  values for FD basis subset selection for N3>19,

- Support  = {1.5, 2} as candidate values for N3' where  is higher layer configured from two values

- Support rank-common configuration of  from above two values

Proposal 3: Do not support the restriction on RI=3-4 bitmap for a polarization in each layer.

Proposal 4: For L=4, support p,β=12, 14,12and p,β=14, 12,34 for parameter combination in Rel-16 Type II CSI.

Proposal 5: For L=6, support p,β=14, 14 and p,β=18, 14 for parameter combination in Rel-16 Type II CSI.

Proposal 6: Support LC coefficient omission for Rel-16 Type II codebook, where the FD/SD bases corresponding to NZC values are ordered according to a given priority rule and LC coefficients for the FD/SD bases are omitted from lower priority to higher priority. The FD/SD basis indicated by SCI have the highest priority for the omission.

Proposal 7: For Rel-16 Type II CSI, DFT beam-based CBSR is supported.

Proposal 8: For Rel-16 Type II CSI, RI restriction is supported.

Proposal 9: Support layer-/layer-group-specific quantization setting to RI=3-4 at least for the amplitude quantization level in an explicit or implicit manner.

Reference

[1] 3GPP RAN1, RAN1#97, Chairman Notes

Annex

Table A-1. Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz with 15kHz SCS for 10MHz BW, 30kHz SCS for 20MHz BW), ISD=200m |
| BS Tx Power | 41 dBm |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Dense Urban: 16ports=(8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| MS antenna configurations | 2 Rx/4Rx X-pol (0/+90) |
| Etilt angle | 102 degree |
| System bandwidth | 10MHz (52RBs), SB size = 4RBs |
| UE attachment | Based on RSRP |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for outdoor |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (medium ~50% RU) |
| Receiver | Non-ideal channel estimation and interference modeling LMMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Feedback delay is 5 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput vs. feedback overhead |
| Overhead | PDCCH (2 symbols), TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |