3GPP TSG RAN WG1 Meeting #98	R1-1908712

Prague, Czech Republic, August 26 – 30, 2019

Agenda Item:	6.2.4.2

Source:	Huawei, HiSilicon

Title:	Potential enhancement for CAS

Document for:	Discussion and Decision

Introduction

In RAN #83 meeting, a new work item, i.e., LTE-based 5G terrestrial broadcast was approved [1]. One of its objectives is as follows:

- Specify, if found necessary, enhancements to the physical channels and signals in the CAS [RAN1, RAN4]

  - This objective includes determining a realistic modelling for the time variation of the desired and interfering signals (e.g. a model between the 50%/50% and 50%/1%), and identifying based on the modelling what channels and signals (if any) need to be enhanced.

  - For the specified enhancements (if any), specify related RRM core requirements (if needed) [RAN4]

This contribution discuss the potential enhancement for physical channel in CAS.

Discussion of enhancements for PDCCH in CAS

In the companion contribution [2], the system level and link level simulation results show that the PBCH and PDSCH channel in CAS do not need enhancements, and the PDCCH needs to be enhanced.

In this section, two potential PDCCH enhancement schemes are discussed.

CFI assignment enhancement

The evaluation results in 3GPP TS 36.776 [3] and the companion contribution [2] test PDCCH and PCFICH jointly, i.e., a miss detection of PCFICH implies a miss detection of PDCCH. If the CFI is assigned dynamically by PCFICH, UE needs to detect PCFICH successfully before detecting PDCCH which reduces the success rate of PDCCH detection.

As one potential enhancement, the CFI can be configured in MBMS-dedicated MIB. If the serving UE of MBMS-dedicated cell is configured with this higher layer parameter, then the UE will acquire the CFI after the MBMS-dedicated MIB is received successfully, which could reduce the performance loss of additional detection of PCFICH.

Table 1 lists the comparison of the simulation results of required SNR for 1% BLER for PCFICH/PDCCH joint detection and PDCCH independent detection. And the comparison shows that the gain of PDCCH independent detection compared to PCFICH/PDCCH joint detection are 1.4dB for car mounted reception and 2.6dB for fixed rooftop reception, respectively.

Table 1: Link level simulation results of required SNR for 1% BLER for PCIFICH+PDCCH and PDCCH (dB)

| Reception | PCFICH+PDCCH | PDCCH |
| --- | --- | --- |
| Car Mounted | -3.8 | -5.2 |
| Fixed Rooftop | -4.0 | -6.6 |

Observation 1: The gain of PDCCH independent detection compared to PCFICH/PDCCH joint detection are 1.4dB~2.6dB.

Based on the above discussions and observation, the following is proposed:

Proposal 1: The CFI can be assigned by MBMS-dedicated MIB for CAS.

PDCCH enhancement

Extending maximum CCE AL (aggregation level), i.e., from AL8 to AL16 may reduce the performance requirements for PDCCH reception in CAS, but the legacy UE cannot detect the PDDCH of AL16.

We propose an enhancement scheme that PDCCH with AL8 in common search space can be transmitted repeatedly twice, as illustrated in Figure 1. Then the required SNR for 1% BLER for PDCCH repetition of new UE will be decreased by nearly 3dB theoretically, and the legacy UE can detect the two PDCCH candidates with AL8 independently.

Figure 1: Example for AL8 jointly detection

The simulation results of required SNR for 1% BLER for PDCCH with AL8, AL16 and proposed PDCCH repetition with two AL8 are summarized in Table 2.

Table 2: Link level simulation results of required SNR for 1% BLER for PDCCH with different AL (dB)

| Reception | AL8 | AL16 | AL8+AL8 |
| --- | --- | --- | --- |
| Car Mounted | -5.2 | -7.3 | -7.2 |
| Fixed Rooftop | -6.6 | -9.0 | -8.9 |

It can be seen from Table 2:

- The PDCCH link level performance for PDCCH repetition with two AL8 is almost equivalent to AL16.

- The SNR requirements of PDCCH repetition with two AL8 compared to AL8 PDCCH are decreased by 2.0dB for car mounted reception and 2.3dB for rooftop reception, respectively.

Observation 2: PDCCH repetition with two AL8 reduces the SNR requirements for 1% BLER by 2.0~2.3dB.

Thus, we propose

Proposal 2: PDCCH repetition with two AL8 needs to be supported for PDCCH enhancement in CAS.

Conclusions

In this contribution, we discussed the potential enhancements for PDCCH in CAS, and the following observations and proposals are made:

Observation 1: The gain of PDCCH independent detection compared to PCFICH/PDCCH joint detection are 1.4dB~2.6dB.

Observation 2: PDCCH repetition with two AL8 reduces the SNR requirements for 1% BLER by 2.0~2.3dB.

Proposal 1: The CFI can be assigned by MBMS-dedicated MIB for CAS.

Proposal 2: PDCCH repetition with two AL8 needs to be supported for PDCCH enhancement in CAS.

- References

RP-190732, “LTE-based 5G terrestrial broadcast,” Qualcomm Incorporated, RAN#83, Shenzhen, China, March 18-21, 2019.

R1-1908094, “Simulation results for CAS evaluation,” Huawei, HiSilicon, RAN1#98, Prague, Czech, August 26-30, 2019.

TR 36.776, “Study on LTE-based 5G terrestrial broadcast,” v2.0.0 (2019-03).