3GPP TSG RAN WG1 Meeting #98                                                                      	     R1-1908726

Prague, CZ, 26th – 30th August, 2019

Agenda Item:	6.2.1.2

Source:	Lenovo, Motorola Mobility

Title:	UL transmission in preconfigured resources for MTC

Document for:	Discussion and decision

Introduction

The contribution is the update from R1-1906278.

In the RAN#80 plenary meeting, a new work item for Rel.16 eMTC is approved. One of the objectives is to specify the uplink transmission based on preconfigured resource for machine-type communications for BL/CE UEs.

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

Note: This is limited to orthogonal (multi) access schemes

In the last RAN1 meeting, the following agreements are achieved for the design of uplink transmission based on preconfigured resource for UE power saving:

Agreement

For a given UE, for dedicated PUR in idle mode and for a given CE mode, the same size DCI, the same PUR M-PDCCH candidates, and the same RNTI is used for all DCI messages for unicast transmissions.

Agreement

For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

FFS: Value of x, and if x is fixed or signaled

FFS: FD-FDD UEs, TDD UEs

FFS: Support for monitoring of PUR SS Window before PUR transmission

Note: The PUR SS Window is the time period where the UE monitors the MPDCCH for at least a time period after a PUR transmission

Agreement

For dedicated PUR in idle mode,

The maximum mPDDCH repetitions, rmax-mPDCCH-PUR, is included in the PUR configuration

Agreement

For dedicated PUR in idle mode, the duration of the PUR SS window is configured by eNB

How the duration is configured, and the possible values, will be decided by RAN2.

Agreement

For dedicated PUR in idle mode, the CE mode is

Option 1: explicitly configured in the PUR configuration.

Option2: based on CE mode of last connection

Down select in RAN1#98

Agreement

Select one of the following in RAN1#98

- Alt1: In idle mode, the PUR search space PRB pairs is configured between {2, 2+4, 4} PRBs

- Alt2: In idle mode, the PUR search space PRB pairs is fixed to 2+4 PRBs

Agreement

For dedicated PUR in idle mode, if a UE skips a PUR transmission, it is not mandated to monitor the associated PUR SS window.

In this contribution, detailed considerations of transmissions in pre-configured UL resources, especially TA issues and HARQ related issue based on dedicated PUR in IDLE mode for Rel.16 MTC are presented.

Discussion

Dedicated PUR Issues

TA Update Mechanisms

When TA is validated and found to be invalid, and UE has data to send, the legacy EDT and RA procedure can be adopted. Some new procedures of simplified TA update mechanism are discussed last meeting. With simplified 2-step RACH on dedicated RACH resource or PUR resource, UE updates TA in optimized RAR or PDCCH order. However, there is possibility that TA is updated in L1 signaling. If even so, when TA is validated and found to be still invalid, only TA update is not sufficient that is because UE has moved to a different coverage level or it has changed cell. Therefore, we propose to follow legacy TA update mechanism via EDT and RA procedure when TA is found to be invalid.

Proposal 1: When TA is validated and found to be invalid, TA is updated via mechanism of legacy EDT and RA procedure.

CE mode

Since the DCI format (e.g., DCI format 6-0A and DCI format 6-0B) is different for CE mode A and B, the UE needs to monitor the corresponding DCI format even in the PUR search space in idle mode. In connected mode, the CE mode is configured in RRC signaling, so CE mode adopted in idle mode can be based on CE mode of last connection, and there is no need to configure the CE mode in PUR configuration.

Proposal 2: For dedicated PUR in idle mode, the CE mode is based on CE mode of last connection.

HARQ

- In the RAN1 #95 meeting, it was agreed that if multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and a single HARQ process. However, the detail of the scheduling multiple TBs in Rel.16 is not finalized at this stage, especially the HARQ, DCI design, which is even another story for transmission in IDLE mode. So we propose to move supporting multiple HARQ process number with multi-TB grant on PUR transmission to later release.

- Proposal 3: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- When UE is being assigned with PUR resource, e.g., time/frequency resource, it is also necessary to assign a dedicate RNTI for UE to monitor HARQ feedback. After uplink transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB.

- Proposal 4: After transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB.

- In response to “ACK”, UE would stop MPDCCH monitoring for power saving and go directly to idle mode or continue new transmission in the next PUR resource if needed.  Furthermore, it is agreed that in idle mode, updating PUR configurations and/or PUR parameters via L1 signaling after a PUR transmission is supported. Along with ACK indication in corresponding DCI, the following PUR reconfiguration/parameters can be also included:

- Timing advance adjustment

- UE TX power adjustment

- PUSCH Repetition adjustment

- Proposal 5: In response to “ACK”, UE would go directly to idle mode or continue new transmission in the next PUR resource if needed.

- Proposal 6: Along with ACK indication in corresponding DCI, the following PUR reconfiguration/parameters can be also included:

- Timing advance adjustment

- UE TX power adjustment

- PUSCH Repetition adjustment

- In response to “NACK” (e.g., the UL retransmission GRANT), UE would 1) retransmit in the configured new grant resource or 2) retransmitted in the next PUR transmission or 3) fallback to legacy RACH/EDT or 4) release the PUR resource, which is determined by the DCI indication.

- Proposal 7: In response to “NACK”, UE behaviour depends on the DCI indication.

- In response to “NOTHING”, if UE receive nothing in the PUR search space after the PUR transmission, UE may move to the edge of the cell or fail to validate TA, UE would initialize legacy EDT or RA procedure for insurance purposes.

- Proposal 8: In response to “NOTHING”, UE would initialize legacy EDT or RA procedure for insurance purposes.

PUR Search Space

For connected mode UEs, UE-specific MPDCCH can be used for monitoring HARQ feedback after uplink transmission. However, for IDLE mode UEs the necessity of introduce new UE-specific search space for dedicated PUR is widely discussed.

Considering the scheduling flexibility and HARQ feedback delay, a modified type 2-like common search space can be a kind of new UE-specific search space for UE. In order to reduce power consumption, only search space after the PUR transmission until the end of the short PUR search window will be monitored by UE.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3, 4) [subframes] and has length of M for the corresponding coverage level. In order to offload the MPDCCH and reduce the PUR MPDCCH blind detection, M and N can be configured in PUR configuration and M should be configured much shorter than PUR search window with unit of MPDCCH period. Within PUR search window, if there is no response received by UE, UE will fall back to legacy RACH or EDT.

- The PUR search space is associated with a particular PUR resource. In order to align the search space to different PUR resources for different UEs; the starting position of PUR search space can be configured as an absolute subframe similar as type 2 MPDCCH common search space. If the PUR resource itself is configured as an absolute subframe, the starting position of PUR search space can also be relative to the end of the PUR resource (PUR transmission). For the PUR search space configuration, it further includes maximal repetition Rmax, period of PUR MPDDCH G, and absolute offset αT, starting subframe  for each MPDCCH period is a subframe satisfying the condition , where .

Figure 1: UE-specific search space for PUR

Proposal 9: UE would monitor new UE-specific search space within PUR search window length after uplink transmission based on PUR.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3,4) [subframes] and has length of M for the corresponding coverage level, M and N is  configured by RRC signaling

- The starting position of PUR search space can be absolute subframe similar configured as type 2 MPDCCH common search space.

Fallback

For transmission in PUR, fallback mechanism to RACH/EDT procedure was agreed in the last meeting, but the details is still FFS. The fallback mechanism can be categorized to two cases.  Firstly, the fallback to RACH/EDT is triggered by UE and no new signalling is required. For example, TA validation failure; transmission in PUR unsuccessfully decoded, e.g., ACK is not received during PUR search window.  In this case, UE can transmit preamble in the next RACH occasion if fallback decision made.

Secondly, the fallback to RACH/EDT is triggered by eNB and an indication signalling needs to be introduced, e.g., multiple PUR transmission failure, system load balance requirement, PUR resource unavailable, etc. Fallback indicator can be sent via dedicated RRC signalling or DCI signalling when eNB finds UE’s condition is no longer applicable for PUR.

Proposal 10: Fallback to RACH/EDT can be triggered by eNB or UE.

PUR Resource Configuration

Dedicated PUR is defined as PUSCH resource used by a single UE. For periodic resource configuration, the resource can be configured as periodic starting subframe and resource duration for each resource occasion as shown in Figure 2. For the periodic starting subframe of resource occasion, resource configuration of LTE SPS and NR type 1 configured grant uplink transmission can be the starting point as shown in Figure 3. For aperiodic resource configuration, multiple PUR resources can be configured with different periods to one particular UE.

The resource occasion number of PUR can be configured as a predefined value, the predefined value can be 1 to deal with traffic model of one shot uplink transmission and can also be zeros or infinite to indicate the PUR resource is always valid or reserved for UE.

Proposal 11:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

Figure 2 Periodic PUR resource configuration illustration

Figure 3 LTE SPS resource configuration illustration

Release PUR

Similar to LTE SPS release mechanism, it is agreed that release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS.  There are several potential PUR release mechanisms: 1) PUR resource released after the transmission until to preconfigured number. 2) Implicit PUR release mechanism after consecutive multiple missed transmissions to avoid the waste of allocated PUR. 3) eNB trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

- Proposal 12: PUR resource is implicitly released by 1) transmission until to the preconfigured number; 2) consecutive multiple missed transmissions.

- Proposal 13: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

Shared PUR Issues

In RAN1 #94bis, it was agreed that “In idle mode, dedicated PUR is supported”, while the “Support for CFS PUR” and the “Support for CBS PUR” remained FFS.

For CFS PUR, multiple UEs share the same configured time-frequency resource with orthogonal multiple access, e.g., MU-MIMO and CDM. For MU-MIMO, it can be transparently to configured multiple UE same time-frequency resource with UE-specific DMRS via dedicated PUR. Therefore, CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

For CBS PUR, contention based PUR is required to ensure the reliability of uplink transmission and solve collision problems. Furthermore, CBS PUR may cause more specification effort, waste more UE power consumption, and even bring more challenges for eNB receiver, so we propose that CBS PUR is not supported in Rel.16 eMTC.

- Proposal 14: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

- Proposal 15: CBS PUR is not supported in Rel.16 eMTC.

Conclusions

In this contribution, considerations of multiple transport block transmission with DCI are provided. The following proposals are given.

Proposal 1: When TA is validated and found to be invalid, TA is updated via mechanism of legacy EDT and RA procedure.

Proposal 2: For dedicated PUR in idle mode, the CE mode is based on CE mode of last connection.

- Proposal 3: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- Proposal 4: After transmission based PUR, UE start to monitor the (M)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB.

- Proposal 5: In response to “ACK”, UE would go directly to idle mode or continue new transmission in the next PUR resource if needed.

- Proposal 6: Along with ACK indication in corresponding DCI, the following PUR reconfiguration/parameters can be also included:

- Timing advance adjustment

- UE TX power adjustment

- PUSCH Repetition adjustment

- Proposal 7: In response to “NACK”, UE behaviour depends on the DCI indication.

- Proposal 8: In response to “NOTHING”, UE would initialize legacy EDT or RA procedure for insurance purposes.

Proposal 9: UE would monitor new UE-specific search space within PUR search window length after uplink transmission based on PUR.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3,4) subframes and has length of M for the corresponding coverage level, M and N is  configured by RRC signaling

- The starting position of PUR search space can be absolute subframe similar configured as type 2 MPDCCH common search space.

Proposal 10: Fallback to RACH/EDT can be triggered by eNB or UE.

Proposal 11:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

- Proposal 12: PUR resource is implicitly released by 1) transmission until to the preconfigured number; 2) consecutive multiple missed transmissions.

- Proposal 13: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

- Proposal 14: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

- Proposal 15: CBS PUR is not supported in Rel.16 eMTC.

- References

RP-181420, “New WID on Rel-16 MTC enhancements for LTE”, Ericsson, RAN#80

R1-1907583, “LTE-M Preconfigured UL Resources Summary RAN1 #97”, Sierra Wireless, RAN1#97

R1-1905957, “Support for transmission in preconfigured UL resources in LTE-MTC”, Ericsson, RAN1#97

R1-1905980, “UL transmission in preconfigured resource”, Huawei, HiSilicon, RAN1#97

R1-1906460, “LTE-M Pre-configured UL Resources Design Considerations”, Sierra Wireless, S.A. , RAN1#97

R1-1906893, “Discussion on transmission in preconfigured UL resources for MTC”, Samsung, RAN1#97

R1-1906278, “UL transmission in preconfigured resources for eMTC”, Lenovo, Motorola Mobility, RAN1#97