3GPP TSG RAN WG1 #98	                                                 R1-1908730

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	7.2.4.1

Source:	Lenovo, Motorola Mobility

Title:	Sidelink Physical layer structures in NR V2X

Document for:	Discussion

Introduction

In RAN#83 Plenary, a new WID on 5G V2X with NR sidelink was approved [1]. Regarding physical layer structures in NR V2X, the related objectives are listed as follows:

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

In this contribution, we provide our views on physical layer structures.

Discussion

SL and UL BWP Numerology Mismatch

Agreements: - Configuration for SL BWP is separated from Uu BWP configuration signalling. - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. - FFS the time scale - FFS relation to DL BWP including initial Uu BWP - FFS relation in terms of frequency location and bandwidth

In NR V2X, based on previous agreements, for UEs in RRC_CONNECTED mode, there is only one active SL BWP in a carrier and there is no signalling exchange over SL for the activation or deactivation of a SL BWP. Hence the SL BWP can be considered always active, and there is no switch to another SL BWP for aligning with the numerology of current active UL BWP.

If the configured UL BWP has a different numerology than the active SL BWP’s, then we have several possibilities:

- UE considers this an invalid configuration – triggers Reconfiguration failure procedure as described in chapter 5.3.5.8 of [6]

- The configured SL BWP is considered deactivated and needs to be activated e.g. by sending another RRC reconfiguration when a matching UL BWP is active, or by L1/ L2 signaling

- The UL BWP can be switched to another UL BWP autonomously that has the same numerology as that of configured SL BWP

- Network can explicitly switch the UL BWP (with the right numerology) to align with the SL BWP being configure

Also, the implications of L1/L2 based UL BWP activation/deactivation needs to be considered. For example for the UE autonomous change of UL BWP for RA, BWP changes due to inactivity timer, DCI based switch, etc. In all these cases, the SL BWP should be considered deactivated, or the numerology switching time when transitioning between UL and SL slots needs to be considered. We propose that RAN1 should further inform RAN2 about the possible implications and check if they have any preference. A companion contribution is submitted in RAN2 [7].

Proposal 1: RAN1 should inform RAN2 about the possible implications of L1/L2 based UL BWP activation/deactivation and check if they have any preference of the possible implications and check if they have a different understanding of the implications or a possible solution.

Sidelink Control Information

Agreements:

- Layer-1 destination ID can be explicitly included in SCI

  - FFS how to determine Layer-1 destination ID

  - FFS size of Layer-1 destination ID

- The following additional information can be included in SCI

  - Layer-1 source ID

    - FFS how to determine Layer-1 source ID

    - FFS size of Layer-1 source ID

  - HARQ process ID

  - NDI

  - RV

FFS whether some of the above information may not be present etc. in some operations

(e.g., depending on whether they are used for unicast, groupcast, broadcast)

From RAN1 #97, Reno meeting

Conclusion:

- If two-stage SCI is supported, the following details are used.

  - Information related to channel sensing is carried on 1st-stage.

  - 2nd-stage is decoded by using PSSCH DMRS.

  - Polar coding used for PDCCH is applied to 2nd-stage

  - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool.

  - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage.

  - FFS other details

From the SCI candidate content during offline email discussion during RAN-1 #96bis meeting as shown in the below table, the size of SCI for unicast varies from 80 to 120 bits, whereas groupcast varies from 90 to 130 bits and broadcast could be from 65 to 100 bits. Most of the SCI content overhead is due to the size of the source id and destination id. In order to reduce the payload size of the SCI, the Layer-1 destination ID can be derived from Layer-2 destination ID where the size of the Layer-1 destination ID can be same as Rel-12 D2D, i.e., 8 bits and Layer-1 source ID size can also be optimized.

Proposal 2: Consider deriving Layer-1 destination id from that of Layer 2 id, with reduced size for destination id and source id in the SCI

In LTE V2X the sidelink control and data channels, all UEs receive and decode all SCIs and associated data. Since NR V2X supports sidelink unicast and groupcast transmission beside broadcast, and some content of SCIs related to the data part of unicast and groupcast data need not necessarily needs to be decoded by other UEs. Hence remote UEs that are not part of the unicast or groupcast needs to receive and decode only those content of SCIs related to sensing like resource reservation and selection.

The content of each SCI stage was discussed offline in RAN1 #96b is shown in the below table, wherein the size of the first stage could vary from 62 to 98 bits and the second stage could vary from 70 to 100 bits. Since each part of the SCI carries equal number of bits and also considering the forward compatibility advantage, the 2 stage SCI design is beneficial for release 16. Due to overhead in transmitting the source id and destination id, these Ids can be either part of the first or second stage but not needed to be transmitted in both stages.

Proposal 3: Beneficial to consider 2 stage SCI for forward compatibility, new fields can be added to the second stage SCI without affecting the content and design of the first stage SCI

Proposal 4: Consider the overhead in transmitting source id and destination id in either first SCI stage or second SCI stage but not needed to be transmitted in both stages

The first stage includes information related to minimum parameter set of the SCI that need to be signaled each time a PSSCH is scheduled, including occupied resource block for data, occupied resource block for second PSCCH, NDI, RV, HARQ process number etc. The second stage includes other information that may be signaled less often, including MCS for transport blocks or delta MCS, S-CSI request, CBG related information, feedback related information, etc. SCI content for the group-cast transmission might be varying depending on HARQ feedback is enabled or disabled. The transmission periodicity/occasion of the second part of PSCCH can be configurable and its presence or absence can be implicitly signaled with valid/invalid RB assignment in the first part. This method is beneficial considering certain cast type and its enabled configuration. The SCI size of the broadcast and groupcast without HARQ should be same or not should be further discussed in RAN1 meetings. In some cases the second stage PSCCH may not be necessary to be transmitted all the time or may not be necessary to be transmitted in every time slot together with first stage PSCCH.

Proposal 5: Consider 2-stage PSCCH transmission for NR V2X with configurable second stage PSCCH, the transmission of second stage can be implicitly signaled with valid/invalid RB assignment in the first stage SCI content

Conclusion

In this contribution, we focus on sidelink physical layer structures in NR V2X and present our views. We have the following observation and proposals:

Proposal 1: RAN1 should inform RAN2 about the possible implications of L1/L2 based UL BWP activation/deactivation and check if they have any preference of the possible implications and check if they have a different understanding of the implications or a possible solution.

Proposal 2: Consider deriving Layer-1 destination id from that of Layer 2 id, with reduced size for destination id and source id in the SCI

Proposal 3: Beneficial to consider 2 stage SCI for forward compatibility, new fields can be added to the second stage SCI without affecting the content and design of the first stage SCI

Proposal 4: Consider the overhead in transmitting source id and destination id in either first SCI stage or second SCI stage but not needed to be transmitted in both stages

Proposal 5: Consider 2-stage PSCCH transmission for NR V2X with configurable second stage PSCCH, the transmission of second stage can be implicitly signaled with valid/invalid RB assignment in the first stage SCI content

- References

- RP-190766, New WID: 5G V2X with NR sidelink, LG Electronics, Huawei, Shenzhen, China, March 18-21, 2019

- 3GPP TR 38.885 V2.0 (2019-03), Study on Vehicle-to-Everything (Release 16)

- 3GPP TR 22.886 V15.1.0 (2017-03), Study on enhancement of 3GPP Support for 5G V2X Services

- Chairman's Notes RAN1 AdHoc 1901

- Chairman's Notes RAN1 96, Athens

- 3GPP TS 38.331-f40, NR; Radio Resource Control (RRC); Protocol specification

- R2-1907152, SL and UL BWP Numerology Mismatch, Lenovo, Motorola Mobility, Reno, USA, 13th May - 17th May 2019

Annexure

From the email discussion on the PSCCH bit field for purpose of PSCCH design, the following bit fields can be considered.

|  |  |  |  |  | only for 2-stage SCI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit field | candidate size | unicast | groupcast | Broadcast | 1st SCI | 2nd SCI | Note for Need to agree |
| HARQ process number | [1-4] | O | O |  |  | O | agreed in RAN1-#AH1901 |
| NDI | 1 | O | O |  |  | O | agreed in RAN1-#AH1901 |
| RV | 2 | O | O |  |  | O | agreed in RAN1-#AH1901 |
| Layer-1 source ID | [8-24] | O | O | [O] | [O] | [O] | agreed in RAN1-#AH1901 |
| Layer-1 destination ID | [8-24] | O | O | [O] | [O] | [O] | agreed in RAN1-#AH1901 |
| MCS | [5] | O | O | O | O |  | needed |
| CRC | 24 | O | O | O | O | O | needed |
| Priority (QoS) | 3 | O | O | O | O |  | seems needed |
| CSI request | [1] | O |  |  |  | O | use case agreed only for unicast |
| Antenna port(s) | [2] | O | [O] | [O] | [O] | [O] | related to #layer/DMRS ports |
| DMRS sequence initialization | [1] | O | [O] | [O] | [O] | [O] | whether to support dynamic changes of DRMS sequence initialization - may be not needed for V2X |
| PTRS-DMRS association | [1-2] | O | [O] | [O] | [O] | [O] | whether to support PTRS-DMRS association - may be not needed for V2X |
| CBGTI | [1-4] | O | O |  |  | O | whether to support CBG or not |
| resource reservation | [4] | O | O | O | O |  | in AI of resource allocation |
| Time gap between initial transmission and retransmission | [4] | O | O | O | O |  | in AI of resource allocation |
| Retransmission index | 1 | O | O | O | O |  | in AI of resource allocation |
| Transmission format /cast type indicator | [1-2] | O | O | O | O |  | need to discuss |
| Zone ID | [10] |  | O |  |  | O | in AI of procedure |
| NACK distance | [5] |  | O |  |  | O | in AI of procedure |
| HARQ feedback indication for enabling | [1] | O | O |  |  | O | in AI of procedure |
| time-domain resource assignment for scheduling PSSCH | [3] | O | O |  |  | O | whether to support multi-slot scheduling/slot aggregation |
| second SCI indication | [2-4] |  |  |  | O |  | only for 2-stage SCI |
| RS pattern | [2] | O | O | O | O |  | whether to support dynamic indication of RS pattern |

Table 1: SCI Content discussion