3GPP TSG RAN WG1 Meeting #98	R1-1908735

Prague, CZ, 26th August – 30th August 2019

Agenda Item:	7.2.11.3

Source:	CEA-LETI

Title:	Dense Multipath Components in IIOT channels

Document for:	Discussion and decision

# Introduction

In the framework of the new study item (SI) on channel model for indoor industrial scenarios. Here we provide an overview on DMC results for these environments.  This contribution is based on previous analysis on the R1-1905209 that presented channel characterization in the 2-6 GHz band for an industrial environment with high clutter density.  The results are proposed for the study item on channel modelling for indoor industrial scenarios.

# Dense Multipath Components (DMCs)

In this section the modeling of the Dense Multipath Components (DMCs) of the measured data will be presented. Our proposed models are parametrized based on extensive measurement campaigns, where individual propagation paths are extracted with high- resolution algorithm (SAGE). When extracting individual propagation paths, part of the power is not captured.

Therefore, DMCs describes the distribution of the radio channel emanating from diffuse scattering, which is difficult and computationally exhausting to capture using a superposition of deterministic propagation paths.  They are considered as the remaining complex impulse responses after removing the contribution of the reliable estimated specular components (SCs) and measurement.

Omitting the DMC has been commonly recognized as being one of the most fundamental problems in all measurement-based channel models. The problem in omitting the DMC from the models is that, in addition to an underestimation of the received power, the multipath richness becomes underestimated, and, subsequently, the impact on the system level is seen as an underestimated channel capacity.

In the first sub-section, we will use the proposed approach in [2] to model the DMCs in the time-delay domain. In the second sub-section, the room electromagnetics’ approach developed in [4] will be applied to our use-cases to model the diffuse components of the measured PDPs.

### DMC in delay domain

Undoubtedly, the most significant factor in the characterization and modeling of the DMC is to quantify the proportion of the power carried by the DMCs over the total measured power in the different scenarios. The mean percentage of the DMC power in the LOS conditions is 11.84 %. As expected, the proportion of DMC over the total measured power in the OLOS and NLOS conditions are higher with respectively a mean percentage of the DMC of 23.89 % and 24.70 %.

The scope of this study is limited to the temporal modeling of the DMC contribution.  The measured power delay profile of the first measured point in the LOS scenario and the reconstructed PDP from the SAGE algorithm are plotted in Figure 2-1. Furthermore, the residual part representing the contribution of the DMCs is also plotted in Figure 2-2.  It is important to note that the figures are not plotted to the same scale.

| Figure 2-1 : Measured PDP and the the SAGE estimated PDP | Figure 2-2 : Residual PDP |
| --- | --- |

Different works tend to model the PDP of the dense components through an exponentially decaying function [2][3].

The exponential power profile ψdτ can thus be described by the parameters peak power αd, base delay τd and slope Bd:

|  | ψdτ=0, τ< τdαd2 τ= τdαdexp-Bdτ- τd τ< τd | (1) |
| --- | --- | --- |

Bd is the coherence bandwidth of the dense components, τd is the base delay of the diffuse components normalized to the total length of the observed impulse-response. αd is the power of the diffuse components at τ= τd. Typically αd represents the highest value of the power-delay profile.

Observing the PDP of the DMCs (Figure 2-2), one can notice two exponentially decaying functions. Indeed, there is a significant contribution of late DMCs arriving around 470 ns. Therefore, we considered two models for the DMCs in these measurements. In the first model, the whole PDP is considered at once while in the second one, we modeled the DMCs as two individual exponential decaying functions.

The first model is plotted in Figure 2-3 while the second model is depicted in Figure 2-4. The accuracy of the models can be judged on the evaluation of the RMS delay spread. Based on the residual part of the measured PDP, we obtained a delay spread of 110.5 ns. The RMS delay spreads retrieved from model 1 and model 2 are 134.4 ns and 194.4 ns respectively. The percentage error of the model 1 is estimated at 25 % while that of model 2 is larger with 75 %. Therefore, it seems better to consider the whole PDP at once when modeling the contribution of the DMCs in the delay domain.

Figure 2-3 : DMC  model 1

Figure 2-4 : DMC model 2

### The room electromagnetics’ approach

In this section, the electromagnetic reverberation time model is proposed. It is based on the theory of room electromagnetics which has been developed in [4] in analogy of the well-established theory of room acoustics. Basically, the room electromagnetics describes the PDP in indoor environments as a superposition of two parts: a LOS component (if present) plus the rest, which is composed of the diffuse multipath components as shown in Fig. 1 in [6]. Thus supposes that the weak specular contribution are neglected in the dense multipath components, hence yielding the diffuse multipath components.

The basic model simply considers the first arriving LOS signal if present and the other multiple components arising from reflections, diffractions and scattering which give rise to a tail with an exponential decay in the received power as a function if the arrival delay [5]. The exponential decay in the delay domain has a time constant denoted as the reverberation time.

Estimation of the reverberation time

If the exponential decaying PDP is plotted into logarithmic scale, a linear tail with a negative slope is observed in the decay process. The reverberation time is related to the slope of the decaying tail through the expression given in :

|  | τ= -10∙log10eslope | (2) |
| --- | --- | --- |

where τ is the reverberation time, e is the Euler’s number (= 2.718) and slope is defined the coefficient of the exponentially decaying tail of the PDP. A large value of τ shows the slow decay of the diffuse fields in the considered environments. In other words, it indicates how fast the electromagnetic energy is absorbed by the materials and objects in the considered environment.   In Figure 2-5, the measured PDP at the first measuring point M0 is plotted in logarithmic scale. As seen in the previous section, the PDP can be split in two parts as there is presence of late power contributors. In the first part, we can observe a linear decaying function which is not the case in the second part of the PDP (as from approximately 450 ns).

Figure 2-5 : The measured PDP at M0 in the LOS scenario

We consider, at first, two cases to determine the reverberation time using the room electromagnetics’ approach. Firstly, the whole PDP after the LOS contribution is used to evaluate the reverberation time as shown in Figure 2-6. In the second place, only the first part of the PDP leading to a linear regressive function will be kept (Figure 2-7) to evaluate the reverberation time of the environment. Thus contributions of MPCs with very late time of arrival are discarded.

In the first model, we obtained a reverberation time of 105.29 ns while a value of 47.67 ns upon consideration of the first part. The latter is closer to the reverberation time of 36 ns obtained in a mechanical room [6].  Higher value in our environment was expected as there were presence of larger metallic objects. Values in classical indoor environments range from 21.6 ns [4] to 27 ns [7].

Figure 2-6 : Room electromagnetic model 1

Figure 2-7 : Room electromagnetic model 2

Furthermore, the reverberation time for each scenario at every measuring points will be tabulated below. Let us recall that only the first part of the PDP is considered here. Table 2-1, Table 2-2 and Table 2-3 resume the evaluated reverberation times in the LOS, OLOS and NLOS scenario respectively. We can notice that the values are close to each other albeit the change in measuring points or even the scenario. This is in accordance with the general diffuse theory [8], which implies that the tail of the diffuse component is approximately the same regardless of the measured position.

Table 2-1 : Reverberation time in the LOS scenario

|  | LOS scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured points | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 |
| DTx-Rx(m) | 20. 5 | 19.5 | 18.5 | 17.5 | 16.5 | 15.5 | 14.5 | 13.5 | 12.5 | 11.5 |
| τ (ns) | 47.67 | 49.36 | 51.27 | 49.86 | 48.5 | 49.49 | 50.49 | 50.22 | 49.25 | 19.52 |

Table 2-2 : Reverberation time in the OLOS scenario

|  | OLOS scenario |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured points | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 |
| DTx-Rx(m) | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 |
| τ (ns) | 51.17 | 48.65 | 49.08 | 49.4 | 47.68 | 47.28 | 27.50 | 45.81 | 46.50 |

Table 2-3 : Reverberation time in the NLOS scenario

|  | NLOS scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured points | M0 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 |
| DTx-Rx(m) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 17.5 | 16.5 |
| τ (ns) | 50.09 | 49.67 | 48.78 | 47.92 | 52.36 | 51.21 | 52.46 | 52.26 | 52.56 | 51.24 |
|  | NLOS scenario |  |  |  |  |  |
| Measured points | M10 | M11 | M12 | M13 | M14 | M15 |
| DTx-Rx(m) | 15.5 | 14.5 | 13.5 | 12.5 | 11.5 | 10.5 |
| τ (ns) | 52.30 | 50.43 | 48.24 | 47.25 | 46.63 | 47.91 |

# CONCLUSIONS

Dense Multi Path components (DMCs) can have a strong impact on channel characteristics especially in those scenarios with high clutter density. Obviously the definition of DMCs and the way are extracted from measurements can have an impact on modeling. Here we consider the DMCs as the remaining part of the channel after SAGE algorithm specular components extraction. The DMCs can be modeled with a simple exponential decay of the residual PDP or with the room electromagnetics’ approach

Proposal

Include DMCs as additional feature in IIOT channel model, by modeling the residual PDP with an exponential decay or using room electromagnetics’ approach.

The values can be used for 3GPP model parametrization in scenario with high density clutter and clutter-embedded antennas/

# References

R1-1905209, “Channel characterization in industrial environment with high clutter density” RAN1#96 bis Xi an, China, April 9th – 13th, 2019

N. Czink, A. Richter, E. Bonek, J. Nuutinen and J. Ylitalo, "Including Diffuse Multipath Parameters in MIMO Channel Models," 2007 IEEE 66th Vehicular Technology Conference, Baltimore, MD, 2007, pp. 874-878.

F. Quitin, C. Oestges, F. Horlin and P. De Doncker, "Diffuse multipath component characterization for indoor MIMO channels," Proceedings of the Fourth European Conference on Antennas and Propagation, Barcelona, 2010, pp. 1-5.

J. B. Andersen, J. O. Nielsen, G. F. Pedersen, et al. Room electromagnetics. IEEE Antennas and Propagation Magazine, 2007, vol. 49, no. 2, p. 27–33. DOI: 10.1109/MAP.2007.376642.

A. Bamba, “Channel modeling for the investigation of human absorption in realistic indoor environments,” Ph. D. dissertation, Ghent University, Belgium and Université de Lille 1, France, 2015.

Y. Ai, J. B. Andersen and M. Cheffena, "Path-Loss Prediction for an Industrial Indoor Environment Based on Room Electromagnetics," in IEEE Transactions on Antennas and Propagation, vol. 65, no. 7, pp. 3664-3674, July 2017.

She, J & Yu, Y & Cui, Pengfei & Lu, W & Zhu, H.-B. Reverberation Time and Power Model in Indoor Wireless Scenarios. Radioengineering. 27. 485-493, 2018.

O. Franek, J. B. Andersen, and G. F. Pedersen, “Diffuse scattering model of indoor wideband propagation,” IEEE Transactions on Antennas and Propagation, vol. 59, no. 8, pp. 3006–3012, 2011.

This work has been funded by the French National Agency for Research (ANR) under the project named MOREOVER.