3GPP TSG RAN WG1 Meeting #98                                   R1-1908736

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:		7.2.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Enhancements to Initial Access Procedures for NR-U

Document for:		Discussion and Decision

# 1. Introduction

During RAN Plenary #82 meeting the TR related to the Study on NR-Based Access to Unlicensed Spectrum [1] has been approved. In the same meeting a new WID related to NR-Based Access to Unlicensed Spectrum has been approved (latest update [2]), starting the Specifications phase for Release 16.

In this contribution, we discuss details of initial access and mobility related physical layer procedures for NR-U. We consider the following procedures:

- DRS Transmission

- Random Access

- RRM and RLM

- Paging

Note: this contribution follows the RAN Plenary #84 meeting guidance for NR-U prioritization [10], i.e. unless indicated (and justified) otherwise functionalities listed as “optimization” – or not listed but considered as “non-essential” - are not addressed for this meeting.

# 2. DRS Transmission

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

2.1 DRS Transmission Window

In RAN1#96bis the following agreement was made related to the duration of DRS transmission window:

Agreement: The maximum DRS transmission window duration is 5 ms. - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS. - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions. - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

Related to the DRS transmission window configuration:

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

We think that the duration of the DRS transmission window should be fixed and repeated once at least every 20ms.

Proposal 1: The duration of the DRS transmission window is fixed, periodicity is ≥ 20ms.

The potential impacts of the ETSI harmonized standards (COT duration ≤ 1ms when CAT2 LBT is used, duty cycle currently ≤ 5%) are discussed in our companion contribution [4].

2.2 Serving cell timing and QCL

In RAN1#97 the following agreements were made related to both serving cell timing and QCL determination:

Agreement: The mechanism to determine serving cell timing is as follows: - The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index - 10-bits SFN and half-frame indicator are indicated as in Rel-15 - PBCH payload size is not increased compared to Rel-15 - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination - If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE Agreement: For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE - FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index - FFS: How Q is indicated or determined - FFS: Restriction on the range of Q Note: Neighbor cell RRM measurements will be addressed separately

First, and although this was not explicitly stated within the above agreements, we understand from the corresponding discussions that for a given serving cell the number of DMRS sequences used is kept as for NR Rel-15, i.e. up to 8 different DMRS sequences; otherwise, if the number of DMRS sequences used would be increased to cover 10/20 candidate positions, this would have a potential impact on performance as compared to baseline NR Rel-15 as well as a significant specification effort. Based on the above discussion, we have the following proposal:

Proposal 2: For a given serving cell, the number of DMRS sequences used is kept as for NR Rel-15, i.e. up to 8 different DMRS sequences.
Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame.

Then with respect to the last FFS:

- FFS: Restriction on the range of Q.

We note first that neither the NR-U TR [1] nor the NR-U WID [2] indicate any restriction related to the number of beams to be transmitted (Q) and therefore assume NR Rel-15 to be taken as a baseline (i.e. up to 8 beams to be transmitted). Secondly, although we acknowledge that a restriction on the range of Q may simplify some proposals, no justification in terms of NR-U use cases has been ever provided to our knowledge by the corresponding proponents. Based on the above, we have the following proposal:

Proposal 3: the range of Q should be kept as for the NR Rel-15 baseline, i.e. 1 ≤ Q ≤. 8.

If number of transmitted beams Q is different than number of used DMRS sequences Stx, then it may be difficult to determine, without payload in PBCH, for which DMRS sequences a UE may assume QCL to be the same. In fact, it was previously proposed online that if number of transmitted DMRS sequences is reduced to be equal to the number of transmitted beams Q, then the proposal would work out of box. However, as a consequence of such proposal the PBCH payload might be unnecessary increased. This is illustrated on an example in Table 1, a typical scenario with Q = 1 is considered, and a single DMRS sequence s=0  is transmitted. To recover timing t, index c in PBCH of 5bits [ceil(log2(20))] will be needed.

Table 1 Increased PBCH overhead in case the same DMRS sequence index implies the same beam

|  | 5 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | 00000 | 00001 | 00010 | 00011 | 00100 | 00101 | 00110 | 00111 | 01000 | 01001 | 01010 | 01011 | 01100 | 01101 | 01110 | 01111 | 10000 | 10001 | 10010 | 10011 |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Note: both tables 1 and 3 are based upon SSBs cycling over half slot, i.e. 2 SSBs/slot, but the same principles apply for SSBs cycling over full slot, i.e. a single SSB/slot.

Following the above RAN1#97 agreement, it has been acknowledged that 5bits for c is unnecessary. If the number of used sequences Stx is an integer multiple of Q, then:

- the beam index can be uniquely identified by mod(s,Q)

- with current set of DMRS sequences Stot=8  maximum value ceilY=20/Stx=5-1=3 is needed for c, requiring 2 bits.

The number of used DMRS sequences Stx given Q could be based on Table 2.

Table 2 Number of used sequences Stx assuming Stot=8

| Q | Stx | Maximum c for Y=20 |
| --- | --- | --- |
| 1,2,4,8 | 8 | 2 |
| 3,6 | 6 | 3 |
| 5 | 5 | 3 |
| 7 | 7 | 2 |

With this proposal the Table 1 would be updated as follows, showing the benefit related to PBCH overhead:

Table 3 Reduced PBCH overhead

|  | 5 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 01 | 01 | 01 | 01 | 01 | 01 | 01 | 01 | 10 | 10 | 10 | 10 |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |

Hence with respect to the first FFS:

  - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination

In our proposal the knowledge of Q is necessary to allow the UE to determine for serving cells both the timing and QCL. As it has been commented that the knowledge of the timing is beneficial to acquire the RMSI we propose to have Q carried through the PBCH MIB by reusing other available bits, more details are provided below. With this change the PBCH MIB payload will remain unchanged over a 80ms TTI, as for NR Rel-15.

Observation 1: the PBCH MIB payload will remain unchanged over a 80ms TTI if Q is added.

Proposal 4: For the serving cell Q is provided to the UE via the PBCH MIB by reusing other available bits.

With respect to the second FFS:

- FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index

Based on above discussion and assuming a single SSB/slot is configured, we propose that A is the PBCH DMRS sequence index according to the following proposals:

Proposal 5: To recover the beam index of the SSB, the UE performs an operation mod(s,Q) where Q denotes the number of transmitted beams and s denotes the DMRS sequence index.

Proposal 6: The UE determines the timing t=2(Stx*c+s) , where c is the cycle index indicated in the PBCH, Stx is the number of used DMRS sequences from the total number of sequences Stot and given as the maximum integer multiple of the number of transmitted beams per cell Q*i, subject to Q*i≤S_tot, and s denotes the DMRS sequence index.

Note: in the case two SSBs/slot are configured the timing becomes t=Stx*c+s.

Note: Although “SSB repetition” is not explicitly listed within [10] we consider it as “non-essential”, hence for the sake of NR-U progress we do not discuss it in this contribution.

Related to the third FFS:

- FFS: How Q is indicated or determined

For the serving cell and as stated above, we think that Q shall be provided to the UE by reusing available bits within the PBCH MIB payload; our proposal is to reuse the pdcch-ConfigSIB1 field, with corresponds to 2 sub-fields of 4 bits each (controlResourceSetZero and searchSpaceZero) [9].

Let’s discuss first the controlResourceSetZero case. According to [6], section 13 and the two agreements below, the following tables shall be taken as a baseline for NR-U:

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block (RAN1#AH-2019).

- The SCS for all SSBs and CORESET #0 on a carrier is always the same for operation of NR in unlicensed spectrum (RAN1#96).

Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

Table 13-4: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and control resource set multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 1 |
| 2 | 1 | 24 | 2 | 2 |
| 3 | 1 | 24 | 2 | 3 |
| 4 | 1 | 24 | 2 | 4 |
| 5 | 1 | 24 | 3 | 0 |
| 6 | 1 | 24 | 3 | 1 |
| 7 | 1 | 24 | 3 | 2 |
| 8 | 1 | 24 | 3 | 3 |
| 9 | 1 | 24 | 3 | 4 |
| 10 | 1 | 48 | 1 | 12 |
| 11 | 1 | 48 | 1 | 14 |
| 12 | 1 | 48 | 1 | 16 |
| 13 | 1 | 48 | 2 | 12 |
| 14 | 1 | 48 | 2 | 14 |
| 15 | 1 | 48 | 2 | 16 |

Taking into account the two additional agreements:

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS (RAN1#96).

- Only CORESET #0 lengths of 1 and 2 symbols are supported for NR-U (RAN1#96bis).

We have the following observation:

Observation 2: related to controlResourceSetZero MIB sub-field:

- At most 2 configurations are relevant for the SCS = 15 kHz case (96 RBs).

- At most 6 configurations are relevant for the SCS = 30 kHz case (48 RBs).

Observation 3: for SCS = 30 kHz and assuming same  and , the 3 corresponding configurations only differ by their “Offset (RBs)” value.

We think that such flexibility is not needed for NR-U, we have therefore the following proposal:

Proposal 7: controlResourceSetZero is reduced to 2 bits for NR-U, with 2 indexes corresponding to 1 and 2 CORESET symbols, the two remaining indexes being reserved for future use.

Then about the searchSpaceZero case, according to [6], section 13, the following table shall be taken as a baseline for NR-U:

Table 13-11: Parameters for PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 1 and frequency range 1

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 2 | 2 | 1 | 1 | 0 |
| 3 | 2 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 6 | 7 | 1 | 1 | 0 |
| 7 | 7 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 8 | 0 | 1 | 2 | 0 |
| 9 | 5 | 1 | 2 | 0 |
| 10 | 0 | 1 | 1 | 1 |
| 11 | 0 | 1 | 1 | 2 |
| 12 | 2 | 1 | 1 | 1 |
| 13 | 2 | 1 | 1 | 2 |
| 14 | 5 | 1 | 1 | 1 |
| 15 | 5 | 1 | 1 | 2 |

In RAN1#97 the following agreement was made related to SSB positions and type-0 PDCCH monitoring in a slot:

Agreement: For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further. Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97 R1-1907883 Feature lead summary on initial access signals and channels for NR-U Qualcomm Incorporated Conclusion: There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

Our understanding of the above conclusion is as follows: unless this topic is revisited there will be a single type-0 PDCCH monitoring occasion per slot, starting at symbol zero. Furthermore we think that there no benefit to allow to be different from zero for NR-U.

We have the following observation:

Observation 4: related to searchSpaceZero MIB sub-field, a single configuration is relevant (index 0); in the case the above conclusion would be revisited the configuration corresponding to index 1 could be added.

We have therefore the following proposal:

Proposal 8: searchSpaceZero is reduced to 2 bits for NR-U, with one index corresponding to NR Rel-15 index 0, the three remaining indexes being reserved for future use.

According to proposals 7 and 8 we observe that 4 remaining bits are available within the PBCH MIB; our proposal is to use 3 of them to carry Q, the remaining bit being reserved for future use.

Proposal 9: Q is carried via the PBCH MIB using 3 bits, the remaining bit being reserved for future use.

2.3 Neighbour cell QCL determination

We observe first that SSB-based RRM measurements with report of SSB index are already supported by NR Rel-15 and should be useful e.g. for the two following use cases, both related to the handover procedure:

- It should help the network to prepare proper random access resources by allocating the UE to a specific SSB beam during the random access procedure.

- It should help the network to take the proper decision for which target cell the handover should actually take place. For instance for two given target cells with similar cell-level measurements results but with different beam-level results the network may prefer to select a target cell for which the beam-level results distribution is the most homogeneous one, or for which the number of strongest beams is the highest.

Additionally, SSB-based RRM measurements with report of SSB index could be also used by the network for network optimizations; e.g. it might be useful for the network to be aware that UEs that are currently served by beam #X actually measure beam #Y as the strongest one.

In our view all these use cases are relevant for NR-U as well, meaning that beam-level neighbour cells RRM measurements should be supported for NR-U.

We have therefore the following proposal:

Proposal 10: SSB-based RRM measurements with report of SSB index should be supported for NR-U.

We have then some observations related to the following agreement from the RAN1#97 meeting:

- If the UE is required to perform PBCH decoding of neighbour cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE

This agreement is related to the TS 38.133, which states the following:

- For intra frequency measurements without/with measurement gaps ([11], sections 9.2.5 and 9.2.6): The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

- For inter frequency measurements without/with measurements gaps ([11], section 9.3.4): When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

From these TS 38.133 statements we have the following observations (related to NR Rel-15) and proposals (for NR-U Rel-16):

Observation 5: The UE is not required to perform PBCH decoding for a new detectable intra frequency neighbour cell if the UE is not indicated to report SSB index or the neighbour cell is synchronous with the serving cell.

On the other hand for intra frequency RRM measurements and since synchronous serving and neighbour cells may be assumed synchronous, as for the licensed case deriveSSB-IndexFromCell is assumed to be always enabled for FR1 TDD, hence the observation 5 leads to the following proposal for NR-U Rel-16:

Proposal 11: The UE should not be required to perform PBCH decoding for a new detectable intra frequency neighbour cell in any case.

For inter frequency neighbour cells the principle is similar, except that the “serving and neighbour cells are always synchronous” assumption is not applicable, hence:

Observation 6: The UE is not required to perform PBCH decoding for a new detectable inter frequency neighbour cell when the UE is not indicated to report SSB index.

This observation is actually applicable for the FR2 case (i.e. L=64), for which the UE needs to read the PBCH, and so does the RAN#97 agreement recalled on top of this section ; for the FR1 case (i.e. L=4 or L=8) the SSB index shall be derived from the DMRS channel estimation only ([6], section 4.1). We think that such principle should be kept for NR-U (FR1 case), i.e. for RRM measurements the SSB index shall be derived from the DMRS channel estimation only, preventing in any case the UE from reading the PBCH. Hence the observation 6 leads to the following proposal for NR-U:

Proposal 12: The UE should not be required to perform PBCH decoding for a new detectable inter frequency neighbour cell in any case.

As a consequence of the proposals 11 and 12 the PBCH MIB cannot be used to provide the UE with Q for neighbour cells RRM measurements.

Proposal 13: For neighbour cells RRM measurements Q is provided to the UE via the following signalling:
- For UEs in idle/inactive mode, SIB3/IntraFreqNeighCellInfo IE (for intra-frequency measurements) and SIB4/ InterFreqNeighCellInfo IE (for inter-frequency measurements).
- For UEs in connected mode, dedicated RRC signalling (MeasObjectNR IE).

In any case the beam index is determined as for the serving cell (proposal 5), i.e. from both s and Q.

2.4 Comparison between various proposals

Assuming our proposal 3 is agreed our understanding of the last RAN1#97 meeting outcomes is that two alternatives remain on the table:

- Alt-1a, as described within [12] and [13] (with no restriction on Q range).

- Alt-1b, as described within this contribution.

We found useful to provide a high-level comparison between these two proposals, bearing in mind that our understanding of Alt-1a is based upon RAN1#97 contributions which may be therefore updated for the current meeting.

Table 4 Comparison between Alt-1a and Alt-1b proposals

|  | Alt-1a | Alt-1b |
| --- | --- | --- |
| Common characteristics | 1 ≤ Q ≤ 8, 0 ≤ s ≤ 7 and 1 ≤ Stx ≤ 8 |  |
| Stx | Always equal to 8 | Stx = f(Q) |
| Serving cell |  |  |
| Timing index (assuming a single SSB/slot) | t = 2(8*c + s) | t = 2(Stx*c + s) |
| SSB/Beam index | b = Mod[t,Q] | b = Mod[s,Q] |
| How the UE is aware of Q | PBCH MIB or RMSI | PBCH MIB |
| Intra/inter frequency neighbour cells (new detectable cell) |  |  |
| QCLed SSBs: - Sync cells, no SSB index report - Sync cells, SSB index report - Async cells, no SSB index report - Async cells, SSB index report | - From serving cell timing - From serving cell timing - From Δt ([12]) - From Δt ([12]) | From beam index (see next row) |
| SSB/Beam index: - Sync cells, no SSB index report - Sync cells, SSB index report - Async cells, no SSB index report - Async cells, SSB index report | - Not needed - From serving cell timing - Not needed - b = Mod[t,Q] (see next row) | b = Mod[s,Q] |
| Timing index (assuming a single SSB/slot): - Sync cells, no SSB index report - Sync cells, SSB index report - Async cells, no SSB index report - Async cells, SSB index report | - Not needed - Not needed - Not needed - From PBCH reading [t = 2(8*c + s)] | Not needed |
| How the UE is aware of Q | TBD | SIB3/SIB4 or RRC signalling |

Notes:

- As per the proposal 11 we assume for intra frequency RRM measurements that the serving and neighbour cells are always synchronous, as it is the case for NR licensed FR1 TDD. However this may be revisited for further NR-U Releases, e.g. because UL-DL interference issues with asynchronous TDD deployments in the licensed spectrum may become less critical for NR-U due to much lower UL/DL power imbalance.

- As per proposal 16 and for the sake of NR-U Rel-16 timely completion we are fine for inter frequency RRM measurements with postponing the support of asynchronous networks to a further Release.

Observation 7: From the above notes we may support intra/inter-frequency RRM measurements for synchronous networks only for NR-U Release 16; on the other hand RAN1 should strive for a future-proof solution able to further support asynchronous networks in the most simple and efficient way.

From the above table we have then the following additional observations:

Observation 8: From a UE complexity perspective, one drawback of Alt-1a is the multiple methods used for QCL determination for both serving and neighbour cells: i.e.:

- QCLed SSBs are determined in a different way for synchronous and asynchronous networks.

- For asynchronous networks and SSB index report PBCH reading is needed.

whereas Alt-1b always uses the same method for both serving and intra/inter frequency neighbour cells (SSB/Beam index is always derived from the PBCH DMRS sequence index and provides directly QCLed SSBs).

Observation 9: From a UE complexity perspective, one drawback of Alt-1b according to [12] is due to the fact that Stx = f(Q), meaning that the PBCH DMRS detection hypotheses increase compared to Alt-1a. However, such statement assumes that Q is unknown up to the end of the SSBs accumulation process, which should not be the case at least for neighbour cells (Q is known via SIB3/SIB4 or RRC signalling before processing SSBs).

Observation 10: From a UE battery consumption perspective Alt-1a has another drawback, i.e. the need to read for asynchronous networks and SSB index report the PBCH, whereas this is not needed for Alt-1b.

## 2.5 On DRS Transmission in Wideband

Wideband operation in Rel-15 is illustrated on an example in Figure 5 where there are multiple SSBs (both cell defining CD-SSBs and non-cell defining NCD-SSBs) in wideband network carrier. In the figure there are two cell-defining SSBs which identify two serving PCells that may have overlapping DL BWPs in frequency domain. From the UE perspective, each Pcell is associated to at most a single SSB. Further, there are two NCD-SSBs also on the same carrier, e.g. for two serving Scells of some UE. These NCD-SSBs indicate (for initial access of a UE) where the UE may find the cell defining SSB. RRM measurements based on both CD and NCD-SSBs can be configured, i.e. on all SSB1, SSB2, SSB3 and SSB4.

Observation 11: For synch purposes, a serving cell is associated with one and only SSB (in frequency domain).

Figure 1 Rel-15 wideband operation with multiple SSBs (both cell defining and non-cell defining SSBs).

In NR-U, operating single SSB (in frequency domain) for a DRS of a serving cell, it may quite frequently occur that periodic DRS transmission is due on sub-band #X while gNB has acquired a COT for other sub-band #Y few slots ahead. Typically, a gNB cannot perform LBT on one sub-band while transmitting on other sub-bands. Stopping DL transmission to perform LBT for sub-band #X is not desirable, because there is no guarantee that LBT on sub-band #X will be successful, and at the same time the gNB may lose the right to transmit on sub-band #Y

Observation 12: When operating WB carrier with multiple sub-bands, the DRS of a serving cell may be due on the sub-band #X while gNB transmits DL COT on sub-bands other than sub-band #X.

We think RAN1 should discuss what is the behavior in this situation. Does UE drop the reception of DRS, and instead continue monitoring on sub-bands of the acquired DL burst, or should the DRS be transmitted on sub-band of the ongoing DL COT instead?

Proposal 14: RAN1 to discuss behavior for the situation when the DRS of the cell/BWP may due on the sub-band which is not part of the gNBs ongoing DL COT.

# 3. Random Access

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1)

In this contribution we discuss two possible ways to alleviate the impact of LBT failures upon the Random Access procedure:
- Increase of Msg1/MsgA transmission opportunities
- Reduction of contention failures

3.1 Increase of Msg1/MsgA transmission opportunities

Following the approval during RAN Plenary #82 of a new WID related to 2-step RACH in NR (latest update [3]) and except for the topics mentioned on top of section 3, the 2-step RACH design is out of the scope of the NR-U WID. Hence, the proposals/observations in this section assume 4-step RACH as a baseline but are in principle also applicable to 2-step RACH as far as MsgA transmission is concerned.

To increase Msg1/MsgA transmission opportunities, several options were identified in [1], as listed within subclause 7.2.1.3.2. As such topic is listed as an “optimization” within [10], hence for the sake of NR-U progress we do not discuss it in this contribution.

3.2 Reduction of contention failures

Although this functionality is not explicitly listed within [10] we consider it as “non-essential”, hence for the sake of NR-U progress we do not discuss it in this contribution.

# 4. RRM and RLM

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

## Although RRM/RLM extension for NR-U operations may raise similar issues – e.g. which signals are used for measurements - they do not mandatorily lead to the same solutions; on the other hand, some issues may be specific to either RRM or RLM, e.g. the configuration of the SMTC for RRM. In the following we will therefore address RRM and RLM topics separately.

4.1 RRM

In RAN1 #96 the following agreement was made related to RRM:

Agreement: - At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported - FFS: - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario - Reporting of a new medium contention/load metric other than channel occupancy - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

4.1.1 SSB/CSI-RS transmission and SMTC configuration

As for a UE in idle mode CSI-RS measurements are not applicable, the main use of CSI-RS RRM measurements will be related to Connected mode mobility. As such performing CSI-RS transmission outside of the DRS transmission window may lead to additional complexity and may not be compatible with the strict timing requirements for neighboring cells monitoring.

Another issue is related to CSI-RS transmission power, as the network shall adjust CSI-RS transmission power because of e.g. different PSD requirements depending on the transmission bandwidth. To ensure reliable RRM measurements – and e.g. a reliable evaluation of the pathloss - it has been agreed for LTE-LAA that the network should guarantee a constant CRS power transmission inside the DMTC window, such constraint being not valid outside of the DMTC window; hence for LTE-LAA no RRM measurements are performed outside of the DMTC window.

It is therefore proposed to support only SSB/CSI-RS-based RRM measurements inside the DRS transmission window.

Proposal 15: Only SSB/CSI-RS-based RRM measurements inside the DRS transmission window are supported.

The way the NR Rel-15 SMTC configuration needs to be updated for NR-U has to be further discussed, but as per the above proposal it seems clear that the SMTC has to be configured based upon the DRS transmission window characteristics for both serving and neighbor cells.

The following aspects should to be investigated:

- The DRS transmission windows distribution between serving and neighbor cells for both synchronized and unsynchronized networks and the consequence upon SMTC configuration are to be discussed. In particular and for unsynchronized networks the longer DRS transmission window periodicity compared to the NR Rel-15 baseline – mandatory to cope with the impacts of LBT – will require specific SMTC configurations, given that the gNB is not aware of the neighbor cells timing. Several tracks to be further investigated could solve this issue, such as the use of the maximum SMTC duration or the use of a sliding SMTC window, but we think that such improvements are not suitable to ensure reliable mobility, as the intervals between measurements are too large.

As a consequence and for the sake of NR-U Rel-16 timely completion:

Proposal 16: Only synchronous networks are supported for NR-U Rel-16, asynchronous networks being supported for a further NR-U Release.

- Different SMTC configurations for intra and inter-frequency measurements.

- SMTC duration: should be based upon the DRS Tx window duration with the possible use of SSB-ToMeasure to restrict the set of subframes to be actually measured by the UE.

- SMTC periodicity: should be based upon DRS Tx window periodicity.

Proposal 17: The SMTC configuration for NR-U should assume NR Rel-15 as a baseline. The way it needs to be updated for NR-U is to be further discussed, including the aspects above mentioned.

### 4.1.2 RSSI and other metrics

As agreed during RAN1 #96 meeting at least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported.

The RSSI - measured by the UE over a given bandwidth – corresponds to the signal strength received from the nodes belonging to the same operator plus the interferences coming from other nodes, including nodes from other operators and/or other RATs, which contribute to increase the interference level. For LTE-LAA a single RSSI measurement has been defined. As the signal strength received from the nodes belonging to the same operator can be directly measured by the UE from all the neighbour cells it monitors, the level of interferences can be derived from both these measurements and the RSSI.

Although we see benefits for supporting RSSI measurements on a per sub-band basis, as for LTE-LAA, following the guidance from [10] for which such feature is listed as an “optimization” and for the sake of NR-U progress we do not discuss such proposal for this meeting. Our proposal is therefore as follows (see also our RAN4 contribution [14]):

Proposal 18: To support wideband RSSI measurements for NR-U Rel-16.

Let us discuss now the corresponding measurements bandwidth and how it relates with the active DL BWP bandwidth.

For intra frequency RSSI measurements and given the above RSSI definition it may be found beneficial for the UE to measure with a bandwidth different from its active DL BWP bandwidth, we have therefore two cases to be considered:

- RSSI BW fully overlaps with the active DL BWP BW. This case should be supported; given the above statement related to sub-band support we propose that in such a case the RSSI BW always equals to the active DL BWP BW.

- RSSI BW partially overlaps with the active DL BWP BW, or does not overlap at all. As said above we believe that such case would be beneficial to be supported, but on the other hand:
- UEs not supporting a bandwidth > 20 MHz may not support such a case, depending upon the RSSI BW.
- Regardless of whether the RSSI BW partially overlaps or does not overlap at all with the active DL BWP our thinking is that to perform such measurements the UE would have at least to perform a RF retuning or possibly mandate measurement gaps; another possibility is that such a case would not be supported for NR-U Rel-16 (to be discussed).

We have therefore the following proposal:

Proposal 19: Intra frequency RSSI measurements for which the RSSI BW equals the active DL BWP BW should be supported. Support of cases for which the RSSI BW partially overlaps (or does not overlap at all) with the active DL BWP BW has to be further discussed, including the need for the UE to perform RF retuning or use measurement gaps; in such cases the RSSI BW has to be provided by the gNB in the measurement object configuration.

For inter frequency RSSI measurements our proposal is to follow LTE-LAA baseline, i.e. the gNB provides the RSSI BW in the measurement object configuration, and the UE uses measurement gaps to perform the measurement.

Proposal 20: As for LTE-LAA, inter frequency RSSI measurements are supported using measurement gaps; the RSSI BW has to be provided by the gNB in the measurement object configuration.

In our view reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U; note that such statement is in line with [10] guidance, for which channel occupancy enhancements – i.e. introduction of new metrics - is listed as an “optimization”.

Proposal 21: Reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U.

## 4.1.3 Impacts of LBT upon RRM measurements

4.1.3.1 Handling of missing L1 samples

Once initial access is performed, the UE is expected to be implicitly aware of the DRS transmission characteristics in the serving and neighboring cell, including DRS timing. On the other hand, providing the UE with an explicit indication of missing L1 samples is not needed in our view, as such indication will be provided “a posteriori” and does not bring any benefit compared to the L3 filter input rate adaptation we propose below.

Observation 13: It is not needed to provide the UE with explicit indication of DRS transmission.

As the UE is aware of SSB/CSI-RS configuration within the DRS transmission window we believe it is reasonable to assume the UE can detect the presence of the SSB/CSI-RS. Thus, missing L1 samples due to e.g. LBT can be identified and removed. Of course, this means there may be fewer samples in L1 measurement period. However, in agreement with what discussed and concluded in RAN2, we think L3 measurement filter should be able to accommodate this - as it was also concluded for RRM measurements in LTE-LAA.

Proposal 22: Missing L1 samples due to LBT are handled in UE implementation specific manner by adapting the input rate to the L3 filter. No changes to the RAN1/RAN2 Specifications are needed to address impacts of LBT on RRM measurements based on SSB and/or CSI-RS.

4.1.3.2 Cell-quality indicator

Note: this topic is not listed within [10]. On the other hand as our proposal is to stick to the NR Rel-15 approach we have kept the discussion.

In Rel-15 NR the cell-quality indicator is calculated as the linear average of up to N best beams (at a given moment) that are above a certain absolute threshold - before L3 filtering. If one of the best N beams fails because of e.g. LBT, the corresponding L1 measurement may be somehow corrupted. However, this only becomes a relevant issue in case multiple SSB-beams are supported in NR-U. If only single-SSB beam is supported by the gNB, then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

Observation 14: If only single-SSB beam is supported by the gNB then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

On the other hand, to support multi-SSB beams there are two possible ways to handle the cell-quality indicator (see figure 2):

- Option A: use the Rel-15 NR approach, i.e. the cell-quality indicator is periodically computed as the average of the N best beams above a threshold, the final cell-quality indicator being built from these periodic computations.

- Option B: perform for each beam measurement over a given period (including L3 filtering as by Proposal 22), the final cell-quality indicator being built by averaging the N best L3 filtered beam measurements.

Figure 2: Two possible ways to handle the cell-quality indicator

Both approaches guarantee that L3 filtering compensates for the missing SSB/CSI-RS. The main advantage of Option B could be that as individual beam measurements are computed over a given period, the impact of missing SSB/CSI-RS for a given beam is less sensitive than for the Rel-15 NR approach (Option A), leading to a more robust cell-quality indicator. On the other hand, assuming a missing SSB/CSI-RS pseudo-random distribution both approaches are expected to be equivalent.

Given the above and as from a specifications complexity perspective the benefits are obvious, we propose to use the Rel-15 NR approach as a baseline for NR-U (i.e. Option A in Figure 2).

Proposal 23: To use the Rel-15 NR approach as a baseline for cell-quality indicator evaluation:
- When a single-SSB beam is supported by the gNB, as for RRM measurements based on SSB and/or configured CSI-RS.
- When multiple-SSB beams are supported by the gNB, as the average of N best beams above threshold before L3 filtering (Rel-15 NR approach).

4.2 RLM

In RAN1 #96 the following agreement was made related to RLM:

Agreement: - An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations. - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations. - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window. - FFS: Mechanism to handle missing RLM-RS due to LBT failure

In RAN1 #97 the following agreement was made related to RLM:

Agreement: For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window. - Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window - FFS: Whether and how DRS transmission window is configured to the UE

## 4.2.1 RLM measurements inside the DRS transmission window

About the current RLM framework in NR, UEs are provided RLM reference signal configurations by implicit determination or explicit configuration from the gNB, implicit configuration meaning same monitoring occasions for both RLM-RS and RMSI; hence related to the implicit RLM-RS configuration we propose to use as RLM-RS both SSB and CSI-RS within the DRS transmission window, the RLM measurement window being equal to the DRS transmission window. The UE shall monitor the downlink quality based on the SSB and/or CSI-RS and assess the downlink radio link quality per every evaluation period.

Proposal 24: SSB/CSI-RS-based RLM measurements inside the DRS transmission window are supported, the RLM measurement window being equal to the DRS transmission window.

However, in NR unlicensed spectrum, the periodical transmission of RLM-RS may be interrupted due to LBT failure as shown in Fig.3.

Figure 3: Periodical RLM-RS transmission in NR-U

Once the gNB misses the RLM-RS transmission due to the failed LBT, it must wait until the next transmission opportunity. In this case, the UE cannot distinguish whether such lack of transmission is due to poor link quality or because the RLM-RS transmission is blocked by LBT; therefore, missing RLM-RS have to be taken into account for both IS and OOS evaluations. If the UE always reports OOS indication to higher layers even if the channel condition is good, the inaccurate measurement result may lead to unnecessary RLF. On the other hand, if the UE always doesn’t report OOS indications when it cannot detect any of its configured RLM-RS resources, this will result in unacceptable delays in declaring RLF. Therefore, the RLM/RLF mechanism may be impacted by the possible lack of reference signals due to LBT.

Hence, we propose that the event of the UE not detecting any RLM-RS transmission within the DRS transmission window should be counted in both the in-sync and out-of-sync evaluations. How to count the missing RLM-RS samples within the DRS transmission window in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

Proposal 25: RLM-RS samples within the DRS transmission window are taken into account for both in-sync and out-of-sync evaluations. How to count within the DRS transmission window both the detected and missing RLM-RS samples in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

## 4.2.2 RLM measurements outside of the DRS transmission window

As by proposal 25 LBT blocking within the DRS transmission window may result in unnecessary declaration of RLF and also slow down the RLM recovery. Though as mentioned above for the RRM topic RLM-RS transmission outside the DRS transmission window may be affected by a fluctuant transmission power, they can still be used for RLM measurements purpose (in-sync evaluations only). To allow for faster RLM recovery in case RLM-RS transmission within the DRS transmission is blocked, we propose that detected L1 samples outside the DRS transmission window in correspondence of configured RLM-RS resources can also be used for RLM measurements for in-sync evaluations only.

Another reason to use RLM-RS signals outside of the DRS transmission window for RLM measurements is linked with the RLM-RS density. It seems that the number of detected RLM-RS signals, which relies on the RLM-RS periodicity and the success probability of LBT, may impact the in-sync evaluation performance. The RLM-RS signals used for in-sync evaluations might not have sufficient density when the periodical RLM-RS signal could not always be transmitted periodically at the configured time resource due to the unsuccessful channel access.

Observation 15: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are blocked by failed LBT.

Proposal 26: RLM measurements outside of the DRS transmission window are supported for in-sync evaluations only. How to count outside of the DRS transmission window the detected RLM-RS samples in the in-sync evaluations is for RAN4 to be discussed.

Hence, further enhancements for RLM-RS signal outside of the DRS transmission window need to be investigated, especially for long periodicity of RLM-RS resources. For example, when the gNB could not send the periodical RLM-RS signals for pre-defined time duration (e.g., K=2 consecutive periods), an additional opportunity window could be triggered as shown in Fig.4, where some additional occasions of RLM-RS signal transmission should be introduced.

Proposal 27: The RLM measurement should consider the additional opportunistic RLM-RS transmission in one gNB triggered window in the case no RLM-RS sample within the DRS transmission window has been detected after a pre-defined time duration.

Figure 4: Opportunistic RLM-RS transmission in one triggered window in NR-U

Related to opportunistic RLM-RS, our proposal is to rely upon DMRS carried by the GC-PDCCH channel, bearing in mind that the GC-PDCCH channel is always present within a gNB acquired COT, as it is used to provide the UEs with the COT structure. The presence of the GC-PDCCH (i.e. the gNB acquired COT) can be confirmed once the GC-PDCCH DMRS has been detected by the UE.

Hence the presence of DMRS transmission can be determined based on COT structure indication in GC-PDCCH; otherwise no additional indication to the UE is expected to be needed.

Observation 16: No explicit indication to the UE of opportunistic transmission other than COT structure indication in GC-PDCCH is expected to be needed.

As a consequence and following the RAN1#97 agreement (no SSB-based RLM-RS outside of the DRS transmission window) we propose to use the GC-PDCCH DMRS for in-sync evaluations, the GC-PDCCH DMRS being used to confirm the presence of the GC-PDCCH.

Proposal 28: The UE should use GC-PDCCH DMRS for in-sync evaluations, the GC-PDCCH DMRS being used to confirm the presence of the GC-PDCCH.

# 5. Paging

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

During RAN2#106 meeting the following has been agreed:

The UE should also stop monitoring paging for the PO even if it does not decode a P-RNTI if it can detect that the gNB had access to the channel at the pdcch monitoring occasion. FFS if there are additional detection methods to detection of PRNTI and what those are.

As stated within our RAN2 contribution [15]:

So, the UE is allowed to stop monitoring the POs of the DRX cycle if

- the UE detects that the gNB has gained access to the channel (i.e. the gNB is transmitting on PDCCH), and

- the UE does not detect DCI scrambled with P-RNTI

The RAN2#106 agreement assumes that the gNB attempts to gain access to the channel and send the paging message at each PO during the paging DRX, at least until the gNB can successfully access the channel. If the gNB gains access to the channel but does not transmit any paging message (i.e. no P-RNTI is detected), the UE assumes the gNB has no page to transmit and can therefore stop monitoring for paging messages in other POs within the DRX cycle, thus reducing power consumption.

The RAN2#106 agreement also assumes that, once gained access to the channel, the gNB does always transmit some reference signal (e.g. PDCCH DMRS) in correspondence of the monitoring occasions of the of the paging search-space of the UE, and that the UE can detect DL transmission from the gNB based on that. Other mechanisms for the UE to detect DL transmission from the gNB during a PO are not precluded but are for RAN1 to discuss.

Our proposal is to use the same mechanism as described within section 4.2.2 for RLM, i.e.:

Proposal 29: The UE should use GC-PDCCH DMRS for DL transmission from the gNB detection, the GC-PDCCH DMRS being used to confirm the presence of the GC-PDCCH.

# 6. Conclusions

In this contribution, we have discussed potential changes and enhancements needed for physical layer procedures for NR unlicensed scenarios. Based on the discussion, we make the following proposals and observations:

DRS Transmission

Proposal 1: The duration of the DRS transmission window is fixed, periodicity is ≥ 20ms.

Proposal 2: For a given serving cell, the number of DMRS sequences used is kept as for NR Rel-15, i.e. up to 8 different DMRS sequences.
Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame.

Proposal 3: the range of Q should be kept as for the NR Rel-15 baseline, i.e. 1 ≤ Q ≤. 8.

Observation 1: the PBCH MIB payload will remain unchanged over a 80ms TTI if Q is added.

Proposal 4: For the serving cell Q is provided to the UE via the PBCH MIB by reusing other available bits.

Proposal 5: To recover the beam index of the SSB, the UE performs an operation mod(s,Q) where Q denotes the number of transmitted beams and s denotes the DMRS sequence index.

Proposal 6: The UE determines the timing t=2(Stx*c+s) , where c is the cycle index indicated in the PBCH, Stx is the number of used DMRS sequences from the total number of sequences Stot and given as the maximum integer multiple of the number of transmitted beams per cell Q*i, subject to Q*i≤S_tot, and s denotes the DMRS sequence index.

Note: in the case two SSBs/slot are configured the timing becomes t=Stx*c+s.

Observation 2: related to controlResourceSetZero MIB sub-field:

- At most 2 configurations are relevant for the SCS = 15 kHz case (96 RBs).

- At most 6 configurations are relevant for the SCS = 30 kHz case (48 RBs).

Observation 3: for SCS = 30 kHz and assuming same  and , the 3 corresponding configurations only differ by their “Offset (RBs)” value.

Proposal 7: controlResourceSetZero is reduced to 2 bits for NR-U, with 2 indexes corresponding to 1 and 2 CORESET symbols, the two remaining indexes being reserved for future use.

Observation 4: related to searchSpaceZero MIB sub-field, a single configuration is relevant (index 0); in the case the above conclusion would be revisited the configuration corresponding to index 1 could be added.

Proposal 8: searchSpaceZero is reduced to 2 bits for NR-U, with one index corresponding to NR Rel-15 index 0, the three remaining indexes being reserved for future use.

Proposal 9: Q is carried via the PBCH MIB using 3 bits, the remaining bit being reserved for future use.

Proposal 10: SSB-based RRM measurements with report of SSB index should be supported for NR-U.

Observation 5: The UE is not required to perform PBCH decoding for a new detectable intra frequency neighbour cell if the UE is not indicated to report SSB index or the neighbour cell is synchronous with the serving cell.

Proposal 11: The UE should not be required to perform PBCH decoding for a new detectable intra frequency neighbour cell in any case.

Observation 6: The UE is not required to perform PBCH decoding for a new detectable inter frequency neighbour cell when the UE is not indicated to report SSB index.

Proposal 12: The UE should not be required to perform PBCH decoding for a new detectable inter frequency neighbour cell in any case.

Proposal 13: For neighbour cells RRM measurements Q is provided to the UE via the following signalling:
- For UEs in idle/inactive mode, SIB3/IntraFreqNeighCellInfo IE (for intra-frequency measurements) and SIB4/ InterFreqNeighCellInfo IE (for inter-frequency measurements).
- For UEs in connected mode, dedicated RRC signalling (MeasObjectNR IE).

Observation 7: From the above notes we may support intra/inter-frequency RRM measurements for synchronous networks only for NR-U Release 16; on the other hand RAN1 should strive for a future-proof solution able to further support asynchronous networks in the most simple and efficient way.

Observation 8: From a UE complexity perspective, one drawback of Alt-1a is the multiple methods used for QCL determination for both serving and neighbour cells: i.e.:

- QCLed SSBs are determined in a different way for synchronous and asynchronous networks.

- For asynchronous networks and SSB index report PBCH reading is needed.

whereas Alt-1b always uses the same method for both serving and intra/inter frequency neighbour cells (SSB/Beam index is always derived from the PBCH DMRS sequence index and provides directly QCLed SSBs).

Observation 9: From a UE complexity perspective, one drawback of Alt-1b according to [12] is due to the fact that Stx = f(Q), meaning that the PBCH DMRS detection hypotheses increase compared to Alt-1a. However, such statement assumes that Q is unknown up to the end of the SSBs accumulation process, which should not be the case at least for neighbour cells (Q is known via SIB3/SIB4 or RRC signalling before processing SSBs).

Observation 10: From a UE battery consumption perspective Alt-1a has another drawback, i.e. the need to read for asynchronous networks and SSB index report the PBCH, whereas this is not needed for Alt-1b.

Observation 11: For synch purposes, a serving cell is associated with one and only SSB (in frequency domain).

Observation 12: When operating WB carrier with multiple sub-bands, the DRS of a serving cell may be due on the sub-band #X while gNB transmits DL COT on sub-bands other than sub-band #X.

Proposal 14: RAN1 to discuss behavior for the situation when the DRS of the cell/BWP may due on the sub-band which is not part of the gNBs ongoing DL COT.

Random Access

Not discussed in this contribution (proposals listed as an “optimization” within [10] or not listed but considered as “non-essential”)

RRM

Proposal 15: Only SSB/CSI-RS-based RRM measurements inside the DRS transmission window are supported.

Proposal 16: Only synchronous networks are supported for NR-U Rel-16, asynchronous networks being supported for a

Proposal 17: The SMTC configuration for NR-U should assume NR Rel-15 as a baseline. The way it needs to be updated for NR-U is to be further discussed, including the aspects above mentioned.

Proposal 18: To support wideband RSSI measurements for NR-U Rel-16.

Proposal 19: Intra frequency RSSI measurements for which the RSSI BW equals the active DL BWP BW should be supported. Support of cases for which the RSSI BW partially overlaps (or does not overlap at all) with the active DL BWP BW has to be further discussed, including the need for the UE to perform RF retuning or use measurement gaps; in such cases the RSSI BW has to be provided by the gNB in the measurement object configuration.

Proposal 20: As for LTE-LAA, inter frequency RSSI measurements are supported using measurement gaps; the RSSI BW has to be provided by the gNB in the measurement object configuration.

Proposal 21: Reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U.

Observation 13: It is not needed to provide the UE with explicit indication of DRS transmission.

Proposal 22: Missing L1 samples due to LBT are handled in UE implementation specific manner by adapting the input rate to the L3 filter. No changes to the RAN1/RAN2 Specifications are needed to address impacts of LBT on RRM measurements based on SSB and/or CSI-RS.

Observation 14: If only single-SSB beam is supported by the gNB then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

Proposal 23: To use the Rel-15 NR approach as a baseline for cell-quality indicator evaluation:
- When a single-SSB beam is supported by the gNB, as for RRM measurements based on SSB and/or configured CSI-RS.
- When multiple-SSB beams are supported by the gNB, as the average of N best beams above threshold before L3 filtering (Rel-15 NR approach).

RLM

Proposal 24: SSB/CSI-RS-based RLM measurements inside the DRS transmission window are supported, the RLM measurement window being equal to the DRS transmission window.

Proposal 25: RLM-RS samples within the DRS transmission window are taken into account for both in-sync and out-of-sync evaluations. How to count within the DRS transmission window both the detected and missing RLM-RS samples in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

Observation 15: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are blocked by failed LBT.

Proposal 26: RLM measurements outside of the DRS transmission window are supported for in-sync evaluations only. How to count outside of the DRS transmission window the detected RLM-RS samples in the in-sync evaluations is for RAN4 to be discussed.

Proposal 27: The RLM measurement should consider the additional opportunistic RLM-RS transmission in one gNB triggered window in the case no RLM-RS sample within the DRS transmission window has been detected after a pre-defined time duration.

Observation 16: No explicit indication to the UE of opportunistic transmission other than COT structure indication in GC-PDCCH is expected to be needed.

Proposal 28: The UE should use GC-PDCCH DMRS for in-sync evaluations, the GC-PDCCH DMRS being used to confirm the presence of the GC-PDCCH.

Paging

Proposal 29: The UE should use GC-PDCCH DMRS for DL transmission from the gNB detection, the GC-PDCCH DMRS being used to confirm the presence of the GC-PDCCH.

# References

- 3GPP TR 38.889 v16.0.0, “Study on NR-Based Access to Unlicensed Spectrum (Release 16)”

- RP-190706, “Revised WID on NR-Based Access to Unlicensed Spectrum (Release 16)”, Qualcomm

- RP-190711, “Revised WID: 2-step RACH for NR”, ZTE, Sanechips

- R1-1908686, “Initial Access Signals and Channels for NR-U”, Nokia, Nokia Shanghai Bell

- R1-1908684, “NR-U enhancements for uplink signals and channels”, Nokia, Nokia Shanghai Bell

- 3GPP TS 38.213, “Physical layer procedures for control”

- R1-1907857, “Feature lead summary #2 of Enhancements to initial access procedure”, Charter Communications

- 3GPP TS 38.212, “Multiplexing and channel coding”

- 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”

- RP-191581, “Guidance on essential functionality for NR-U”, Qualcomm

- 3GPP TS 38.133, “Requirements for support of radio resource management”

- R1-1906786, “Enhancements to initial access and mobility for NR-unlicensed”, Intel Corporation

- R1-1907455, “Enhancements to initial access procedures”, Ericsson

- R4-1909220, “RSSI and Channel Occupancy Measurements in NR-U”, Nokia, Nokia Shanghai Bell

- R2-1909243, “Paging in NR-U”, Nokia, Nokia Shanghai Bell