3GPP TSG RAN WG1 #98		R1-1908744

Prague, CZ, August 26th – 30th, 2019

Title:	[DRAFT] Reply LS on NR V2X resource pool configuration and selection

Response to:

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	Futurewei [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

#### Name: Philippe Sartori

E-mail Address:	philippe.sartori@futurewei.com

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for their reply LS on sidelink HARQ feedback for groupcast and will take RAN2’s feedback into account during future discussions.

In addition, RAN1 would like to provide the following feedback and answer regarding the question asked by RAN2:

Question: Does RAN1 assume the UE needs to be aware of each of the other UEs in the group for support of Option 2?

RAN1 assumes that for option 2, a UE would be aware of each of the other UEs in the group and that separate PSFCH resources are allocated to UEs in the group needing to send HARQ feedback. RAN1 is aware of the additional signaling overhead but sees benefits for option 2 for some scenarios. For instance, a UE not receiving the PSCCH, would not receive the associated PSSCH. With option 1, no HARQ feedback is sent, thus the source UE would not be aware of the transmission failure. With option 2, no HARQ feedback would be sent, thus the source UE would be aware of the transmission failure.

RAN1 has not yet completed the PSFCH design for either option 1 or option 2, and will liaise with RAN2 once the design is available.

2. Actions:

RAN1 respectfully ask RAN2 to take into account RAN1’s feedback.

3. Date of Next TSG-RAN2 Meetings:

3GPP RAN1#98b 		14 - 18 Oct 2019			Chongqing, CN

3GPP RAN1#99		18 - 22 Nov 2019			Reno, US