3GPP TSG RAN WG1 Meeting #98 	R1-1908748

Prague, Czech Republic, 26th – 30th August, 2019

Agenda Item: 6.2.2.5

Source: MediaTek Inc.

Title: DL Quality Report on non-anchor carrier in NB-IoT

Document for: Discussion and Decision

# Introduction

A new RAN1-led Working Item on Rel-16 enhancements for NB-IoT was approved at RAN Plenary #80 [1].  In RAN#82, the scope of the WID was extended to include Connection to 5GC. In RAN#83, a further WID extension was approved to include  support for quality reporting in connected mode for anchor and non-anchor carriers.

In RAN1#97, the following agreements and aspects requiring further study were made:

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1.

For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported.

- FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI)

For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4.

- number of candidate values for channel quality reports

- mapping value of reported channel quality

- report in RRC message or MAC CE

For further discussion

For channel quality report in connected mode other than Msg3, RAN1 further discuss whether physical layer triggering mechanism is needed.

RAN1 further discuss the followings for channel quality report in Msg3 on non-anchor access in connected mode

- The support of channel quality report in Msg3 in connected mode

- If supported, which carrier can be measured for the channel quality report in Msg3

  - RAR carrier

  - unicast carrier

- If supported, how to trigger the report in Msg3

In RAN2#106, the following agreements and aspects requiring further study were made

Channel Quality Reporting:

- The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE.

- Use the same LCID as agreed for eMTC.

- We need to wait for RAN4 before deciding when to report.

- FFS whether broadcast or dedicated signalling is used for NW to enable the reporting.

This contribution aims to discuss support of Msg3 quality reporting for non-anchor carrier.

# Msg3 quality reporting for non-anchor carrier

Explicit signaling in SI to report Msg3 on non-anchor access in idle mode

After receiving paging or if the UE has some data to transmit, the UE initiates Contention-Based Radom Access procedure. RAN1#97 made agreement that UE measures the RAR carrier for the DL quality report. It is for further study how the network indicate which carrier to use to report Msg3 (e.g. explicit signalling in SI). After Msg2 in Contention-Based Random Access (CBRA), the idle UE has not yet moved to connected. Hence, it is not yet configured with a non-anchor unicast carrier it can use for data transfer in connected mode.

RRC parameters Rel-15 dl-ConfigListMixed in SIB22 list up to 15 non-anchor carriers and associated configurations for paging or  random access. SystemInformationBlockType22-NB contains radio resource configuration for paging and random access procedure on non-anchor carriers. SystemInformationBlockType23-NB contains radio resource configuration for NPRACH resources using preamble format 2 on non-anchor carriers. RRC parameters PhysicalConfigDedicated-NB in RadioResourceConfigDedicated-NB in RRCConnectionSetup-NB message configures a unicast carrier for connected UE.

The simplest way for the carrier used for the reporting of DL quality report in Msg3 is to use the RAR carrier. The idle UE assumes the RAR carrier is used for the DL quality report in msg3. It is not possible to use the unicast carrier as the UE has not yet been configured one. A default unicast carrier could be indicated via explicit SI, but it would require a specification change and new behaviour in UE. In case the RAR carrier is also the configured unicast carrier, it is not helpful if the UE transmit on a default unicast carrier which is different from configured uncast carrier. In case the default unicast is the same as the configured carrier, the usefulness seems corner case assuming RAR carrier cannot have sufficient resources for Msg3 transmission. For these reasons, we have preference for DL quality report to be transmitted in Msg3 in RAR carrier without explicit signalling in SI.

Proposal 1: DL quality report is transmitted in Msg3 in RAR carrier without explicit signalling in SI

Trigger to report in Msg3 channel quality report on non-anchor access in connected mode.

In the specifications, the connected UE re-select cell follows RLF trigger. There is no mobility during data transmission in NB-IoT. The specification does not allow a UE mobility trigger other than RLF in connected mode based on UE measurements on other Carrier(s) configured by CarrierConfigDedicated-NB. For stationary devices in smart meters, the DL quality report in Msg3 sent in CBRA may not change significantly for a long period of time. For moving devices in bikes or wearables, the DL quality report in Msg3 sent in CBRA can change significantly. For the latter case, support of the channel quality report in Msg3 on non-anchor access in connected mode would allow to schedule the devices with an MCS that reflects the changing propagation conditions due to mobility.

The trigger for DL quality report depends on how quickly the network adapts MCS.

- In case of fast fading, a faster trigger mechanism allowing MSC to be adapted within tens of ms would be needed. This level of optimization of link adaptation would require new CQI mechanism in NB-IoT and frequent measurements by devices on serving cell that would increase battery consumption.

- In case of shadow fading, a slower trigger mechanism allowing MSC to be adapted within several seconds could be sufficient.

Since the trigger mechanism depends on the mobility of the device, the simplest way from specification is that the connected UE transmits the DL quality report in the Contention-Free Random Access (CFRA) for UL timing alignment procedure. After the device has moved to connected state, the network may trigger CFRA for UL timing alignment procedure. Since the UE is in connected state, it seems unnecessary to measure the RAR carrier and report DL quality of RAR carrier in Msg3 during CFRA. UL grant on unicast carrier in Msg3 would better use DL quality report measured on configured unicast carrier. The agreement in RAN2#107 “the DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE” can be extended to CFRA. It is up to the network to configure the dedicated timeAlignmentTimerDedicated on MAC-MainConfig IE since eNB last gave TA command. The TA Timer has value indicated in IE TimeAlignmentTimer = {sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240, infinity}. How often the CFRA is triggered by NPDCCH order will depend on the link adaptation and UL timing alignment requirements. This can be left to the eNB scheduler implementation without any further specification needed.

Proposal 2: Channel quality report in Msg3 on non-anchor access is measured by connected UE in unicast carrier and reported in Msg3 in Contention-Free Random Access (CFRA) during UL timing alignment procedure.

Proposal 3:  Channel quality report in Msg3 on non-anchor access by connected UE is triggered by NPDCCH order in Contention-Free Random Access (CFRA) during UL timing alignment procedure.

In case UL timing alignment is not needed, it would seem wasteful to transmit Msg1 and Msg2 in CFRA. These messages could be dropped, with NPDCCH trigger only for UE to transmit MAC CE on indicated UL resources with DCI. This way would allow faster DL quality report but has the drawback of increasing NPDCCH overhead and requiring more frequent measurements and reporting by the connected UE. It would require new specification for the DCI contents. The use case and usefulness could be further studied.

Proposal 4: Whether NPDCCH trigger for DL channel quality report in Msg3 on non-anchor access in connected mode can be transmitted without Msg1 and Msg2 can be further studied.

# Conclusion

In this contribution, we discussed support of Msg3 quality reporting for non-anchor carrier.

Proposal 1: DL quality report is transmitted in Msg3 in RAR carrier without explicit signalling in SI

Proposal 2: Channel quality report in Msg3 on non-anchor access is measured by connected UE in unicast carrier and reported in Msg3 in Contention-Free Random Access (CFRA) during UL timing alignment procedure.

Proposal 3:  Channel quality report in Msg3 on non-anchor access by connected UE is triggered by NPDCCH order in Contention-Free Random Access (CFRA) during UL timing alignment procedure.

Proposal 4: Whether NPDCCH trigger for DL channel quality report in Msg3 on non-anchor access in connected mode can be supported without Msg1 and Msg2 can be further studied.

# References

- RP-181451, New WID on Rel-16 enhancements for NB-IoT, Ericsson, TSG RAN#80, June 2018

- RP-182292, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#82, December 2018

- RP-190337, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#83, March 2019