3GPP TSG RAN WG1 Meeting #98	R1-1908756

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	6.2.1.1

Source: 	Sony

Title:	UE-group wake-up signal for MTC

Document for:	Discussion and Decision

# Introduction

One of the objective in eMTC release 16 is to support UE-group wake-up signal (WUS) mechanism in order to improve DL transmission efficiency and/or UE power consumption [1]. In RAN1 meeting #97 [2], the following related agreements on UE-group wake up signal for MTC were made:

Agreement

- Support the following options for WUS resource configuration

  - Up to 2 orthogonal resources may be configured in time domain.

  - Up to 2 orthogonal resources may be configured in frequency domain

  - FFS: both options can be configured simultaneously to have up to 4 orthogonal WUS resources

- Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource

- Each UE monitors up to X WUS sequences.

  - Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

Working Assumption

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

In this contribution, we discuss some aspects on the operation of UE-group wake-up signal for MTC (MWUS), particularly related to the above recent agreements.

# Discussion

Grouping mechanism of UEs within a MWUS will allow only a group of UEs to be woken up by the MWUS for a paging occasion (PO). The UE grouping operation will reduce the number of UEs having to wake up unnecessarily to detect MPDCCH and decode PDSCH. Thus, it reduces UE power consumption for such UEs.

# Multiplexing of MWUS

The multiplexing of MWUS for the grouping mechanism should consider both UE-group MWUS in Rel-16 and UE MWUS in Rel-15 (legacy). One of the easiest solutions is to allocate different MWUS physical resources for UE-group MWUS and legacy MWUS. Moreover, MWUS occupies 2 PRBs and theoretically an eNB can configure 3 MWUS resources within a narrowband (i.e. FDM operation). Another solution is to allocate different WUS physical resources in TDM manner. A MWUS physical resource can be assigned to a multiple UE-group. However, the eNB only transmit a single-sequence CDM MWUS signal in that resource. A group that has been assigned to that single-seq CDM will wake-up and the remaining UE group(s) continues in idle mode. In RAN1#97 meeting, we agreed to limit the number of resources up to 2 orthogonal MWUS resources in time domain and 2 resources in frequency domain. Furthermore, Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource (i.e for UE rel-15).

Apart from specific UE-group(s) MWUS for rel-16 and a UE-group MWUS rel-15, it has been agreed to consider common-group MWUS signal. A common-group MWUS signal is a MWUS signal targeting to all UEs that has been assigned to monitor a MWUS resource. There is a risk that legacy UE rel.15 is not aware or cannot decode the common-group WUS signal. One can consider legacy MWUS rel-15 signal as a common-group MWUS signal. However, this can increase false wake-up to the UE rel-16 in case the eNB intends to page UE-rel.15 only. Considering the above we propose to introduce separate MWUS resource for UE-group MWUS (rel.16) and legacy UE MWUS (rel.15). Subsequently, the common-group MWUS signal is applicable to MTC UE release 16. In case the eNB intends to page all UEs then the eNB needs to send MWUS signal in all MWUS resources.

Proposal 1: UE-group MWUS (rel.16) and legacy UE MWUS (rel.15) are using separate MWUS resource, and preferably multiplex in FDD.

Proposal 2: Common-group MWUS signal is applicable to MTC UE release 16.

In a legacy LTE system, the probability of UEs having the same paging frame as a function of the number of paging frames is given in Table 1. While a reduction of the occasions of many UEs being paged at the same time can be achieved by allocating many PFs, this solution might not be the optimal one as more resources are being used to carry paging information.

Table 1: Number of PF versus Probability UEs with same PF

| N | UE_ID mod N | % UEs have same paging frame |
| --- | --- | --- |
| 1 | 0 | 100% |
| 2 | 0,1 | 50% |
| 4 | 0,1,2,3 | 25% |
| 8 | 0,1,2,…,7 | 12.5% |
| 16 | 0,1,2,…,15 | 6.75% |

In scenarios where the number of paging frames is limited or can be configured to be a small number, introducing large number of UE-groups would be clearly beneficial. The advantage is that the network can have more flexibility and the number of UEs being paged unnecessarily can be further reduced. In terms of the maximum supported number of UE-groups, we propose to maximize the benefit of the UE-group mechanism. In order to limit the probability of overhearing to below 15% we propose the maximum number of UE groups supported is at least 6.

Proposal 3: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate).

# UE MWUS Detection Operation

The common MWUS is supported in Rel-16. In such case, basically there is a dedicated MWUS to wake up all UEs assigned to the corresponding paging occasion. Common MWUS is in addition to the individual UE group MWUS. The benefit of having common MWUS is that the eNB only needs to transmit the common MWUS when eNB needs to wake up all UEs. However, from the UE point of view, this could increase UE power consumption and complexity as the UE needs to detect both common MWUS and group specific MWUS when listening to the channel for potential WUS. The design of MWUS signal in Rel-16 should consider the UE power consumption and complexity aspects. A design constraint is, for example, the UE does not have to detect twice even when no MWUS exist on the channel.

Observation 1: On the supporting of both common MWUS and group MWUS, the MWUS sequence design shall consider UE power consumption and complexity.

MWUS grouping may support the operation where the eNB pages only one group, several groups (sub-set groups), or all groups. In RAN1 meeting #96, we concluded to further study the additionally support the feature of waking up a subset of all WUS UE groups. The information on a subset of WUS groups should be carried in group ID indication embedded in MWUS for the case of MWUS with single-seq CDM. MWUS with single-seq CDM also indicate that a MWUS resource can be assigned for multiple groups. The UE needs to identify the group ID information in order to decide whether to continue with paging reception or go to sleep. The WUS-group ID can be included in form of bitmap operation as illustrated in Table 2. Here, an example is given when the supported maximum number (Nmax) of groups is 4. It can be observed that the table size is significantly bigger in proportional to the maximum number of WUS groups (i.e. 2^Nmax – 1).

Table 2: UE-group ID with bitmap operation

| No. | Bitmap | WUS Group ID being paged |
| --- | --- | --- |
| 1 | 1111 | 1,2,3,4 |
| 2 | 1110 | 2,3,4 |
| 3 | 1101 | 1,3,4 |
| 4 | 1100 | 1,2,4 |
| 5 | 1011 | 1,2,3 |
| 6 | 1010 | 3,4 |
| 7 | 1001 | 2,4 |
| 8 | 1000 | 2,3 |
| 9 | 0111 | 1,4 |
| 10 | 0110 | 1,3 |
| 11 | 0101 | 1,2 |
| 12 | 0100 | 4 |
| 13 | 0011 | 3 |
| 14 | 0010 | 2 |
| 15 | 0001 | 1 |

It can also be observed that most of the possible operation/combination is to page subset of all WUS UE groups. The number of operations/combinations will be significantly less in case MWUS with single-seq CDM for some of the groups is not supported. Furthermore, the number of supported groups can be higher if it does not require to waking up a subset of all WUS UE groups. On the other hand, it may have some potential issues if group MWUS is only designed to page a group and all groups. In RAN1 #97, it has been agreed that each UE monitors up to X WUS sequences and X is either 2 or 3. Here, basically the UE needs to monitor a MWUS sequence for its own group, common group, and a service-based group. Having one sequence represents subset of all WUS UE groups may not be sub-optimal. In order to keep the power consumption low, we propose X=2.

Proposal 4: Each UE monitors up to 2 WUS sequences.

In RAN1 #97, we discussed the transmit power and duration of rel.16 WUS signal. We consider it is important to keep the same parameters on transmit power and maximum WUS duration for both WUS rel-15 and WUS rel-16.

Proposal 5: Confirm these working assumptions:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Regarding on how the UEs are grouped, configurable weighting factor for UE distribution per time/freq WUS resource was discussed in RAN1 #96bis meeting. The intention was to reduce false wake-up rate by allowing dynamic UE distribution per WUS resource. This would be beneficial for reducing the UE power consumption. We consider this as a way to assign/re-assign the UE group based on paging. According to the progress in the other working group, it is feasible for the MME to provide assistance information including paging probability information to the eNB [3]. Further detailed on defining the grouping is described in our companion contribution [4]. In our view, the method to allocate the UE in certain group is beyond RAN1 scope.

# 3	Conclusions

In this contribution, the following proposals and observation are made:

Observation 1: On the supporting of both common MWUS and group MWUS, the MWUS sequence design shall consider UE power consumption and complexity.

Proposal 1: UE-group MWUS (rel.16) and legacy UE MWUS (rel.15) are using separate MWUS resource, and preferably multiplex in FDD.

Proposal 2: Common-group MWUS signal is applicable to MTC UE release 16.

Proposal 3: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate).

Proposal 4: Each UE monitors up to 2 WUS sequences.

Proposal 5: Confirm these working assumptions:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

# References

- RP-190770 Revised WID: Additional MTC enhancements for LTE, March 2019

- RAN1 #97 Chairman notes, Reno, USA, May 2019

- S2-1908147, Reply LS on assistance indication for WUS grouping

- R2-1909910 On supporting UE-group wake-up signal for MTC, Sony, RAN2#107, Prague, August 2019.