3GPP TSG RAN WG1 Meeting #98	R1-1908762

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.1.1

Source: 	SONY

Title:	Channel Structure for Two-Step RACH

Document for:	Discussion and Decision

# Introduction

The work item on 2-step RACH for NR aims to specify 2-step RACH as general MAC procedure covering both physical layer and higher layer aspects [1]. In RAN1#97 meeting the following agreements were made [2]:

- The following parameters are defined per msgA PUSCH configuration:

- Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include:

  - MCS and/or TBS (to be further decided)

  - Number of FDMed POs

    - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

  - Number of PRBs per PO

  - Number of DMRS symbols/ports/sequences (if support) per PO

  - FFS whether or not support repetitions for msgA PUSCH

  - FFS bandwidth of PRB-level guard band or duration of guard time

  - FFS PUSCH mapping type

- Parameters specific to option 1, at least include:

  - Periodicity (msgA PUSCH configuration period)

    - FFS value range

  - Offset(s) (e.g., symbol, slot, subframe, etc.)

  - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc.

  - Frequency starting point

- Parameters specific to option 2, at least include:

  - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point

    - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc.

  - Number of symbols per PO

    - FFS explicit or implicit indication

  - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency)

- FFS: Number of TDMed POs

- Support multiple msgA PUSCH configurations for a UE

- FFS the maximum number of configurations

- FFS which parameters, if any, are common for all configurations

- FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

- FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be  cell ID or configurable, to be FFS).

- FFS details of the RNTI

- FFS the inclusion of DMRS index.

In this contribution, we provide our views on the 2-step RACH procedure particularly on the channel structure for the PUSCH in msgA of two-step RACH.

# Discussion

# 2-step RACH Procedures

The 2-step RACH operation in general is illustrated in Figure 1. MsgA contains both preamble and information/data. Preamble is transmitted in PRACH and the information/data is transmitted in PUSCH. The information in MsgA can be the legacy Msg.3 of 4-step RACH. The size can be in the range of less than 100 bits (e.g. 72 bits). Furthermore, it may also contain small data transmission from upper layer (e.g. application layer). It may result in larger size, such as around 1000 bits. In response to MsgA transmission, the gNB transmits MsgB which may contain RAR, TA information, and RRC response. This can essentially be similar to the combination of legacy Msg2 and Msg4.

Figure 1: 2 step Random Access Procedure

According to the WID [1], 2-step RACH is applied for RRC_INACTIVE, RRC_CONNECTED and RRC_IDLE state. It implies that the 2-step RACH shall be designed to be operated in all RRC states. The UE must know the 2-step RACH configuration, particularly the MsgA configuration, prior to the 2-step RACH operation. The configuration of msgA may include information regarding data size, resource allocation, supported modulation and coding scheme (MCS), and UEs data multiplexing configuration.  The data size depends on the payload/information that can be carried in MsgA which is mostly defined in RAN2. For 2-step RACH at the initial access stage the msgA configuration can be sent to the UE in system information. This would particularly be beneficial if all UEs in a cell are expected to use the same configuration. In case the UE has been configured from RRC active to RRC inactive mode, the configuration can be sent via dedicated RRC signaling.

Proposal 1: The msgA configuration in 2-Step RACH is broadcasted by the gNB to the UEs (e.g. via system information) or using a dedicated signaling (i.e. RRC signaling).

  - MsgA PUSCH Configuration

MsgA PUSCH configuration has been discussed in previous meetings and two options related to the PUSCH allocation have been identified:

- Option 1 with separate PUSCH configuration. MsgA PUSCH configuration period may or may not be the same as PRACH configuration period

- Option 2 PUSCH configuration with relative location. MsgA PUSCH configuration period is the PRACH configuration period

We consider 2-step RACH can be beneficial for legacy NR operation in some scenarios and also many specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC. Option 2 (relative location) provides less signaling overhead and also possible low-latency of PUSCH occasion (PO) after the RACH occasion (RO). In addition, as Option 1 is configured separately, the gNB has more freedom and flexibility in allocating PO. We prefer the gNB to have MsgA configuration with Option 2. After the UE transmits the preamble, the UE can immediately transmit PUSCH with a relative timing from the preamble transmission. We expect the UE uses a fix PUSCH transport block size (TBS) for such operation. In addition, some UEs may require to transmit larger transport block sizes or PUSCH transmission with repetition. In such cases, we consider a large PUSCH allocation is required. Furthermore, MsgA PUSCH configuration with Option 1 is applicable in this case and the gNB can provide the allocation regardless of the RACH configuration.

Observation 1: 2-step RACH should aim to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC.

Proposal 2: Support the gNB to have a flexible MsgA PUSCH configuration that supports both options, such as Option 2 (relative location) is configured as the default mode and Option 1 (separate PUSCH configuration) can be configured to complement Option 2 and/or to be used for specific use-cases.

Proposal 3: Option 2 is used for the small payload size (e.g. small MCS/TBS) and Option 1 can be used for the larger payload size (e.g. large MCS/TBS) and/or PUSCH with repetition.

The WID objectives describe the preamble and MsgA are TDM’ed [1]. We consider there should be a timing relationship (i.e. Option 2 as described above) or a time-gap between preamble and MsgA. The time-gap can be used for example as the guard time period after preamble transmissions, time-alignment to support various SCS, etc. The possibility and benefit of 0 (zero) time-gap between preamble and PUSCH is for further study. The essential part is the gNB can configure the  time-gap between preamble and MsgA PUSCH.

Proposal 4: On option 2: the relative time-offset between preamble and msgA PUSCH can be configured by the gNB (e.g. to consider guard time, time alignment to support various SCS, RACH configuration).

We consider the UE to use MCS for msgA based on the received msgA configuration. In order to simplify the operation, a fixed MCS can be assigned to a UE. It can be a lower MCS to provide robustness and minimize failure of msgA detection. Retransmission of msgA can be relatively costly as the UE has to transmit both preamble and msgA. This type of retransmission was agreed to be supported. However, the occurrence of such retransmission should be minimized/anticipated. Another option is a UE is configured with a limited set of MCS. The UE can dynamically select the MCS, for example, based on the payload size or pathloss. This may require the gNB to blindly-detect the MCS of the received data. In order to avoid blind-detection mechanism, the MCS could also be associated to the preamble. However, this may cause a limitation for the set of preamble index/resource which the UE can choose, which tends to increase PRACH preamble collision. Another option could be a UE to dynamically signal the MCS on reserved resources within the PUSCH (i.e. signaled with msgA).

Proposal 5: Further study whether the MCS is statically configured (e.g. a fixed MCS) or UE can dynamically select from a limited set of MCS and signal via piggy back on msgA.

  - PUSCH resource unit (PRU) definition and Mapping between Preamble and PRU

In RAN1#96bis meeting [3], PUSCH resource unit (PRU) was defined as the PUSCH occasion and DMRS port / DMRS sequence used for a msgA payload transmission and it is further discussed for supporting only one or both of DMRS port / DMRS sequence. The following candidates of the PRU definition would be considered.

- PUSCH occasion and DMRS port

- PUSCH occasion, DMRS port, and DMRS sequence

For PUSCH occasion and DMRS port, this definition could maintain channel estimation accuracy even if PUSCH is collided because DMRS between collided PUSCHs are orthogonal by using different DMRS ports, while it results in increasing overhead of reserved physical resource. On the other hand, for both DMRS ports and DMRS sequences, although reserved physical resource overhead could be reduced due to increasing the number of PUSCH multiplexing within the same PUSCH occasion would be increased, this definition causes performance degradation due to non-orthogonal DMRS multiplexing.

Therefore, we evaluated performance degradation due to non-orthogonal DMRS multiplexing using LLS. Figure 2 shows the LLS performance comparison among different DMRS port & different DMRS sequence, different DMRS port & same DMRS sequence, and same DMRS port & different DMRS sequence. In this simulation we evaluate the collision case of 2 PUSCHs transmitted from different 2 UEs. Left figure shows SNR vs PUSCH BLER in the case of 2 Rx, while right figure shows that in the case of 4 Rx. Simulation assumptions for this evaluation is shown in Table A of Appendix A. In the case of 2 Rx (as shown in left figure), all schemes have the same performance gain in the condition that target BLER is 10% and performance of different DMRS sequence schemes are about 1 dB lower than that of different DMRS port & same DMRS sequence scheme in the condition that target BLER is 1%. In the case of 4 Rx (as shown in right figure), since gain of diversity is more dominant than loss caused by channel estimation inaccuracy, all schemes achieve the same performance gain in all conditions.

(A) 1Tx 2 Rx					         (B) 1 Tx 4 Rx

Figure 2: PUSCH BLER performance with orthogonal/non-orthogonal DMRS multiplexing

Observation 2: Compared to orthogonal DMRS multiplexing, performance degradation due to non-orthogonal DMRS multiplexing is relatively small.

Based on the above simulation results, we conclude that there is no impact on performance for non-orthogonal DMRS multiplexing. Considering the benefit of non-orthogonal DMRS multiplexing, both DMRS ports and DMRS sequences should be supported for the definition of PRU.

Proposal 6: Support both DMRS ports and DMRS sequences for the PRU definition

Furthermore,  a working assumption on “supporting one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit” was made in RAN1#96bis meeting [3]. This discussion point was whether multiple-to-one mapping is supported or not. If multiple-to-one mapping is used, it would be further beneficial to reduce overhead of reserved physical resource for PUSCH transmission, while DMRS is completely collided due to the same PRU mapping. As [4] is shown, by using PRACH for channel estimation, impact on DMRS collision could be minimized and PUSCH BLER performance could be kept in the case that PRACH SCS is the same as PUSCH SCS. Moreover, even if PUSCH is failed to decode due to PUSCH as well as DMRS collision, fallback to transmit msg3 of 4-step RACH could be operated so that the occurrence of random access failure would be minimized. Therefore, we proposed to confirm this working assumption.

Proposal 7: Confirm the working assumption on supporting one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - msgA PUSCH Scrambling

In RAN1#97 meeting [2], msgA PUSCH scrambling was discussed and it is further discussed for the detail of RNTI and the inclusion of DMRS index. For the detail of RNTI, for a UE with RRC_CONNECTED, C-RNTI should be simply used because C-RNTI has been already configured to connected UEs. For a UE with RRC_IDLE/RRC_INACTIVE state, since TC-RNTI/C-RNTI is not known, another scramble ID for PUSCH instead of TC-RNTI/C-RNTI has to be considered.

Since 2-step RACH UE anyway generate RA-RNTI, a simple way is to reuse the RA-RNTI for msgA PUSCH scrambling. For the inclusion of DMRS index, given that multiple PUSCH could occupy one PUSCH occasion, it would be desirable that each of the PUSCHs in any PUSCH occasion have different scramble IDs. If one-to-multiple mapping between preamble and PRU is supported, DMRS index has to be included into the c_init to distinguish multiple PUSCHs in the same PUSCH occasion, since PUSCH could not have different scramble IDs only using RNTI, preamble index, and n_ID.

Proposal 8: For a UE in RRC_IDLE or RRC_INACTIVE state, RNTI used for msgA PUSCH scrambling should be RA-RNTI. For a UE in RRC_CONNECTED state, RNTI used for msgA PUSCH scrambling should be C-RNTI.

Proposal 9: DMRS index should be included into the c_init for msgA PUSCH scrambling if one-to-multiple mapping between the preamble and PRUs is supported.

# Conclusion

In this contribution, we discussed the msgA channel structure of 2-step RACH procedure and conclude on the following observations and proposals:

Observation 1: 2-step RACH should aim to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC.

Observation 2: Compared to orthogonal DMRS multiplexing, performance degradation due to non-orthogonal DMRS multiplexing is relatively small.

Proposal 1: The msgA configuration in 2-Step RACH is broadcasted by the gNB to the UEs (e.g. via system information) or using a dedicated signaling (i.e. RRC signaling).

Proposal 2: Support the gNB to have a flexible MsgA PUSCH configuration that supports both options, such as Option 2 (relative location) is configured as the default mode and Option 1 (separate PUSCH configuration) can be configured to complement Option 2 and/or to be used for specific use-cases.

Proposal 3: Option 2 is used for the small payload size (e.g. small MCS/TBS) and Option 1 can be used for the larger payload size (e.g. large MCS/TBS) and/or PUSCH with repetition.

Proposal 4: On option 2: the relative time-offset between preamble and msgA PUSCH can be configured by the gNB (e.g. to consider guard time, time alignment to support various SCS, RACH configuration).

Proposal 5: Further study whether the MCS is statically configured (e.g. a fixed MCS) or UE can dynamically select from a limited set of MCS and signal via piggy back on msgA.

Proposal 6: Support both DMRS ports and DMRS sequences for the PRU definition

Proposal 7: Confirm the working assumption on supporting one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

Proposal 8: For a UE in RRC_IDLE or RRC_INACTIVE state, RNTI used for msgA PUSCH scrambling should be RA-RNTI. For a UE in RRC_CONNECTED state, RNTI used for msgA PUSCH scrambling should be C-RNTI.

Proposal 9: DMRS index should be included into the c_init for msgA PUSCH scrambling if one-to-multiple mapping between the preamble and PRUs is supported.

# References

- RP-190711, “Revised work item: 2-step RACH for NR”, ZTE, Sanechips, Shenzhen, China, March, 2019.

- Chairman notes on RAN1#97, ETSI MCC

- Chairman notes on RAN1#96bis, ETSI MCC

- R1-1906779, “Discussion on channel structure for 2-step RACH”, Intel Corporation, Reno, USA, May, 2019.

# Appendix A: Link Level Simulation Assumptions

The following table summarizes the LLS assumptions:

Table A: Link Level Simulation Assumptions

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz |
| Number of symbols in PUSCH slot | 14 symbols |
| DMRS | Type1, dual-symbol |
| TBS | 72 bits |
| Number of PRBs | 6 PRBs |
| Number of UEs | 2 UEs |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx, 4Rx |
| Propagation channel & UE velocity | TDL-C 300ns, 3km/h |
| Receiver | MMSE-IRC |
| Channel Estimation | Realistic |
| Timing and frequency offset | Ideal |