3GPP TSG RAN WG1 Meeting #98	R1-1908788

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.10.1

Source: 	Sony

Title:	DL reference signals for NR positioning

Document for:	Discussion / Decision

# Introduction

In RAN plenary #83, a new work item “NR positioning support” was approved [1] . In RAN1 #97 [2] , the following agreements related to DL positioning reference signals were made:

- DL PRS Resource is described by at least the following parameters

  - DL PRS Resource ID (previously agreed)

  - Sequence ID (previously agreed)

  - Comb Size-N

  - RE Offset in frequency domain

    - FFS whether this offset is a single value or multiple values

  - Starting slot and symbol of DL PRS Resource

    - FFS whether it can be represented by time offset with respect to some reference

  - Number of symbols per DL PRS Resource (Duration of DL PRS Resource)

  - Quasi-colocation information (QCL with other DL reference signals)

    - FFS QCL type and source reference signals

  - FFS: Number of Tx Ports

  - FFS: Power offset b/w DL PRS and SSB

  - FFS: Transmission bandwidth and starting PRB with respect to Point A

  - FFS: Numerology

Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not.

- Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6}

  - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12}

- DL PRS Resource comb-N value is configurable from the set {2, 4, 6}

  - FFS: Inclusion of other values in the set including values in {1, 8, 12}

Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters

- RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98:

  - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level

    - Common period is used for DL PRS resources within a DL PRS Resource Set

  - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level

    - Different periods can be used for DL PRS resources within a DL PRS Resource Set

- DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted

  - FFS: If this definition is introduced for muting or DL PRS resource allocation or both

# In this contribution, we discuss our view on downlink reference signals to support NR positioning, particularly on downlink PRS configuration.

# Downlink PRS Configuration

In legacy LTE, DL PRS transmission is essentially a cell-specific type of signal. The DL PRS is configured based on cell ID and periodically transmitted with certain duration and periodicity. Furthermore, the positioning requirement in LTE is only to fulfil the regulatory requirement. It has been identified that NR positioning requires to fulfil both regulatory and commercial requirements. The requirements can be differed in term of positioning accuracy in vertical / horizontal and also the latency requirements. Various UEs in a cell may have different positioning requirements. Considering these aspects, it would be beneficial to have two operating NR positioning modes.

The first operation mode is a cell-specific positioning where the gNB transmits DL PRS with certain duration and periodicity. This is almost similar as DL PRS in LTE, except DL PRS in NR needs to at least consider the transmit/receive beam operation. We can expect all the UEs in at least a cell know the DL PRS configuration. Furthermore, we can consider the cell-specific DL PRS is designed to fulfil at least the regulatory requirements.

The second operation mode is a UE-specific positioning where the gNB may transmits an aperiodic UE-specific DL PRS. This is particularly for the case in supporting stringent positioning commercial requirement. We can expect a UE that requires higher positioning accuracy can be allocated with additional or more DL-PRS allocation. The scheduling information of the DL PRS is only informed to a designated UE or group of UEs.

Proposal 1: Support the operation of cell-specific and UE-specific DL PRS signals.

In RAN1 meeting 96bis, it was agreed to support DL PRS with the same numerologies (CP length and sub-carrier spacing) as for data transmissions in Rel-15. It implies a possibility of multiplexing DL PRS with other channels (e.g. PDSCH). Furthermore, it also exposes the case where different gNB may use different numerology (SCS, CP). Those possible situations do not exist in LTE positioning. In order to facilitate an efficient DL PRS reception at the UE side, the design of DL-PRS shall consider the possibility of a UE to receive DL-PRS from multiple gNBs within the certain time/frequency resource. It can be called as within a positioning occasion. Furthermore, in order to facilitate an efficient DL-PRS multiplexing, the DL-PRS from multiple gNBs can be grouped together within a positioning occasion with the same numerology. This can be supported by introducing NR positioning bandwidth part. Within an NR positioning bandwidth part, it may contain one or multiple DL PRS resource set.

Proposal 2: Support NR positioning bandwidth part, in which it may contain multiple DL PRS resource set with the same numerology.

NR is designed to support lean carrier operation. The transmission of periodic signals, such as CRS in LTE has been avoided in NR. The only periodic and broadcast transmission in NR is the SSB transmission. SSB has an important role such as time/frequency measurement, cell detection, etc. The SSB is also only transmitted with a short burst within 5 MHz bandwidth. In contrast, DL-PRS has a specific purpose for UE positioning measurement. DL-PRS may be transmitted with a wider bandwidth for improving positioning measurement accuracy. We consider, the gNB has a flexibility to enable / disable DL-PRS transmission in order to support lean carrier operation, particularly in the case where there is no demand of positioning service in a specific time/location. The operation of NR positioning bandwidth part can facilitate this operation. The DL-PRS in an entire NR bandwidth part can be disabled/enabled by the gNB. The gNB can broadcast this information to the UE.

Proposal 3: NR positioning bandwidth part can be activated / deactivated by the gNB

In RAN1 meeting #97 [2] , the periodicity of DL PRS resource is discussed and results in several options. We support the periodicity of DL PRS resource that is configured at DL PRS resource set level. It implies a common period is used for DL PRS resources within a DL PRS resource set. A UE is expected to receive DL PRS from more than one gNB. Configuring the DL PRS resource periodicity in this manner makes the DL PRS multiplexing in a gNB and also the possible multiplexing with DL PRS from other gNB can be easier.

- Proposal 4: Support Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level

  - Common period is used for DL PRS resources within a DL PRS Resource Set

Muting operation in NR was discussed in RAN1 meeting #97 [2] . We consider to support the muting operation per DL PRS resource level. This will enable the gNB to mute the DL-PRS in certain DL PRS resource, particularly to avoid DL-PRS transmission in certain undesired direction/beam.

Proposal 5: Support the muting operation per DL PRS resource

In legacy NR release 15, it is well-known that the gNB transmits synchronization signal and reference signals (SSB, CSI-RS, TRS). These signals have some specific purposes. SSB is designed to assist the UE for synchronization and cell detection purposes. CSI-RS is to support channel state measurement. TRS as a kind of CSI-RS is also designed for the time/frequency tracking purpose. SSB transmission bandwidth is also relatively limited (5 MHz). Thus, those signals are not designed for positioning measurement purpose. The UE should have a freedom to utilize those signals for positioning purpose. It may provide some benefits, such as improving the accuracy. It may also give an extra burden for the UE, such as increasing UE power consumption, in case the UE uses those legacy signals for positioning purpose in addition to the DL-PRS. We consider it is up to UE implementation whether to use the legacy signals for positioning measurement purpose. However, those legacy signal can be used to assist positioning measurement using DL-PRS, such as QCL-assumption of DL-PRS to the legacy reference signals.

Proposal 6: It is up to the UE to use the legacy signals (SSB, CSI-RS, TRS) for positioning measurement purpose

NR positioning support the operation of DL-PRS transmission with one antenna port. Considering the UE needs to receive DL-PRS from multiple gNBs and in a need to simplify the multiplexing of DL-PRS, the cell-specific DL-PRS (as proposed earlier) shall be operated with one antenna port. The UE-specific DL-PRS can be transmitted in order to obtain a better UE positioning accuracy. It may be transmitted from some / selected gNBs. Thus, we consider two antenna ports can be supported for UE-specific DL PRS transmission.

Proposal 7: Two antenna ports can be supported for UE-specific DL PRS transmission

# Conclusion

In this contribution, we discussed our view on DL reference signals for NR positioning. We have made the following proposals:

Proposal 1: Support the operation of cell-specific and UE-specific DL PRS signals.

Proposal 2: Support NR positioning bandwidth part, in which it may contain multiple DL PRS resource set with the same numerology.

Proposal 3: NR positioning bandwidth part can be activated / deactivated by the gNB

- Proposal 4: Support Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level

  - Common period is used for DL PRS resources within a DL PRS Resource Set

Proposal 5: Support the muting operation per DL PRS resource

Proposal 6: It is up to the UE to use the legacy signals (SSB, CSI-RS, TRS) for positioning purpose

Proposal 7: Two antenna ports can be supported for UE-specific DL PRS transmission

# References

- RP-190752, “New WID: NR Positioning Support”

- RAN1 Meeting #97 Chairman Notes