3GPP TSG RAN WG1 Meeting #98	R1-1908789

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.10.2

Source: 	Sony

Title:	UL reference signals for NR positioning

Document for:	Discussion / Decision

# Introduction

In RAN plenary #83, a new work item “NR positioning support” was approved [1] . In RAN1 meeting #97, the following agreements were made [2] :

Agreement:

- SRS transmissions for positioning are realized with staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource

- FFS: construction of the pattern inside the SRS resource structure

- For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12}

- FFS: Other values including 3,6,14

- Note: Values of 1, 2 and 4 within an SRS resource can already be configured in Rel-15

- For positioning, starting positions in the time domain for the SRS resource can be anywhere in the slot, i.e. an offset loffset  range of {0,1,…,13}.

- For positioning, with regard to UL Beam management/alignment towards serving and neighbouring cells, at least the following (option 2 from prior agreement) can be used:

- UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources

- FFS: other options

- For positioning, the SRS comb size set is extended from {2,4} to {2,4,8}

- FFS: Additional comb sizes: 1, 6, 12

  - Note: For the comb sizes of 6 and 12, the number of PRBs may be restricted if currently defined sequences are to be used

- FFS: Maximum number of cyclic shifts for the different comb sizes (cyclic shifts for comb sizes of 2 and 4 already exist in Rel-15)

# In this contribution, we discuss our view on uplink reference signals to support NR positioning, particularly on SRS enhancements for uplink-based positioning.

# Uplink SRS Enhancements

It is well-known that in 5G NR, especially in FR2, a directional (narrow) beam is typically needed for both gNB and UE in order to compensate the required pathloss. In legacy NR, the UE and serving gNB are designed to maintain a proper beam direction during transmission. In case of uplink-based positioning in NR, the multi-lateration is required for UE positioning estimation. In this case, the uplink positioning signal from UE needs to be received by multiple gNBs, such as a serving cell gNB and the neighbor cell gNBs. The gNBs can performs timing-based measurement (RTOA) or received power-based measurement (UL-RSRP). The angular information related to the uplink positioning reference signal (UL-PRS) can be used to improve the positioning accuracy. The angular information that has been discussed is the Angle of Arrival (AoA) of the UL-PRS. In this context and based on the WID progress, we can call it UL-SRS.

The existing 5G NR rel.15 only specify serving gNB to receive the UL-SRS transmission. The specification has not defined the controlling of neighbor gNB Rx beam for SRS reception of a selected UE. Thus, the neighbor gNBs may not be able to perform positioning measurement. Furthermore, UTDOA with multi-lateration cannot be performed as the LS may only receive the positioning measurement from the serving gNB. In FR1, the UE may be equipped with an omnidirectional antenna and thus the beam-alignment requirement may not be as stringent as in FR2. However, the gNB may still be equipped with directional antenna. Thus, the beam coordination in both FR1 and FR2 are required.

Observation 1: Beam coordination between UE transmit beam and one or more gNB(s) receive beam is essential in uplink-based NR positioning.

The beam coordination for uplink-based positioning can be facilitated based on the downlink measurement operation at the UE. NR downlink measurement mechanism has been established/defined in NR rel.15. It is typically performed by the UE to measure mainly the channel quality (e.g. RSRP) of serving gNB. The RSRP measurement of neighbor cell is only performed if the RSRP measurement of serving cell is deteriorated. For positioning purpose, first the UE needs to perform measurement based on the received downlink reference signals from multiple gNBs. Secondly, this measurement may affect the uplink SRS configuration, such as SRS transmit power. In case the UE needs to reach at least 3 gNBs, the maximum transmit power can be based on the selected gNB where the measured RSRP is as the third-largest according to the list of measured RSRP values from the neighbor gNBs.

Proposal 1: The transmit power of UL SRS for positioning is determined based on the downlink measurement information of serving cell and neighbor cell gNBs.

The UE measures RSRP from each surrounding gNBs within a predefined interval and duration. Moreover, this measurement assists the UE to collect information on the best gNB transmit beam. Based on the downlink measurement in NR rel.15, the UE is basically has collected information related to the surrounding cells, such as channel quality and beam direction of each gNBs. Furthermore, this information can be used by the UE with minimum effort for the uplink-based positioning, particularly for beam coordination purpose. The only additional effort is the UE transmits the measurement report in preparation for configuring the UE SRS transmission for positioning purpose. The UE can transmit the measurement report either to LS or the serving cell that has been configured to support a similar LS function.

In practice, the UE can be allocated to transmit SRS without the UE downlink measurement reports. However, the SRS may not be optimally transmitted by the UE. The UL-SRS may not be received by sufficient number of gNBs or best gNBs. The UE can perform blind beam-sweeping operation with the objective the UL-SRS can be received by multiple gNBs. However, this may also increase the interference to other UEs.

Proposal 2: The UL SRS for positioning purpose (e.g. beam sweeping) can be determined based on the UE downlink measurement reports.

In legacy NR rel.15, UL SRS allocation is scheduled and allocated by the serving cell gNB. The main purpose is to support or maintain a good connectivity between the UE and serving gNB. As the UL SRS for positioning can serve different purpose and targeting to serving and neighbor cells, some modifications to the legacy operation is needed. We consider a LS may have obtained a knowledge (e.g. based on the UE downlink measurement report) the possible gNBs around the target UE. The LS can trigger those gNBs to be prepared in reception of possible UL SRS transmission. The LS also trigger the serving gNB to allocate UL-SRS for UE positioning purpose in which the UL-SRS is targeting for both serving and neighbor gNBs.

Proposal 3: Serving gNB allocates UL SRS for UE positioning purpose.

The UE uplink transmission (including UL SRS) is often based on the information of downlink measurement. For example, the UE TX beam direction is typically the same as UE Rx beam and this is known as beam correspondence. Beam correspondence in NR is mandatory and the respective capability signaling definition in NR UE feature 2-20:

- UE that fulfills the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1

- UE that fulfills the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0

It can be observed that some UE may need to perform uplink beam sweeping as it has a loose beam correspondence requirement. In this case, the uplink beam configuration may not completely rely on the received beam configuration. In such condition, the transmission of SRS with beam sweeping operation can be supported.

Proposal 4: UL positioning with beam-sweeping shall be supported particularly for the UE that need to fulfills the beam correspondence requirement with the uplink beam sweeping

Based on the agreements in RAN1 meeting 96bis, one of the supported SRS configurations for positioning is the aperiodic configuration. Here, we consider the aperiodic SRS transmission can be triggered in conjunction with periodic SRS transmission or periodic DL PRS transmission. This is basically to support on-demand positioning where it may require UE positioning estimation with higher accuracy.

Proposal 5: Aperiodic UL SRS transmission can be used to improve positioning measurement accuracy and to support DL & UL positioning.

# Conclusion

In this contribution, we discussed our view on UL SRS enhancements for NR positioning. Our proposals are listed below:

Observation 1: Beam coordination between UE transmit beam and one or more gNB(s) receive beam is essential in uplink-based NR positioning.

Proposal 1: The transmit power of UL SRS for positioning is determined based on the downlink measurement information of one or more gNBs.

Proposal 2: The UL SRS for positioning purpose can be determined based on the UE downlink measurement reports.

Proposal 3: Serving gNB allocates UL SRS for UE positioning purpose.

Proposal 4: UL positioning with beam-sweeping shall be supported particularly for the UE that need to fulfills the beam correspondence requirement with the uplink beam sweeping

Proposal 5: Aperiodic UL SRS transmission can be used to improve positioning measurement accuracy and to support DL & UL positioning.

# References

- RP-190752, “New WID: NR Positioning Support”

- Chairman notes RAN1 Meeting 97.