3GPP TSG RAN WG1 #98		R1-1908816

Prague, Czech Republic, 26th – 30th August 2019

Source:	Panasonic

Title: 	System level evaluation and link budget for NTN

Agenda Item:		7.2.5.1

Document for:	Discussion

Introduction

A study item on Non Terrestrial Network (NTN) has been started. According to the SID[1], the following is to be studied in RAN1.

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In RAN1#97, system level evaluation assumptions were agreed. In this document, our evaluation results for calibration are provided. In addition, link budget analysis based on the system level evaluation results are shown.

Evaluation results for calibration

System level simulations for calibration were conducted based on the propagation model in [2] and simulation assumptions agreed in RAN1#97 [3]. GEO, LEO 600km and LEO 1200km with parameter set 1 (smaller cell size) and set 2 (larger cell size) are evaluated. Three types of UE defined in [3] (i.e. VSAT, Handheld, “other”) are evaluated. Atmospheric absorption [2] are assumed for both S-band and Ka-band. No scintillation loss is assumed. 19-beam layout (i.e. 2 tiers) are used to obtain the geometry distribution. For each observed beam, interference from beams within 2 tiers (i.e. 18 beams) are assumed. (In total 61 beams including interfering beams are deployed in the simulation)

Frequency reuse option 1 (reuse 1), option 2 (reuse 3), and option 3 (reuse 2 + polarization reuse) as shown in Figure 1 are evaluated. Note that frequency reuse option 3 is only applied to circular polarization terminal (i.e. VSAT) as mentioned in [3].

Option 1                                           Option 2                                               Option 3

Figure 1  frequency reuse options

Simulation results for the coupling loss and geometry distribution for each scenario are shown from Figure 2 to Figure 10 for Set 1 and from Figure 11 to Figure 19 for Set 2. Rural channel model in TR38.901 is used including inter-polarization interference. Large scale (only pathloss and shadowing considered) evaluation results for frequency reuse option 1 (i.e. reuse 1) are also shown as a reference.

Table 1 to Table 4 summarizes 5%-tile, 50%-tile and 90%-tile SINR for each scenario.

Considerations

- Large scale and full scale evaluation results are well aligned.

- In GEO scenarios, 5%tile SINR is extremely low. For GEO, elevation angle as low as 10 degrees is considered based on the assumptions. This increases NLOS UE ratio defined in [2]. In addition to large propagation loss due to NLOS, path loss for UE with low elevation angle is large because such UE is located basically far from satellite. Therefore, extremely low SINR is observed. This causes low 5%-tile SINR.

- Frequency reuse option 2 and 3 shows similar results. Reuse factor of option 2 is three while reuse factor including polarization of option 3 is four. On the other hand, polarization signals are combined in option 2, and inter-polarization interference degrades SINR in option 3. Therefore, the resulting SINR for option 2 and option 3 would become similar.

- SINR for Set 2 (larger cell size) is basically smaller than Set 1 (smaller cell size). Set 1 is more interference limited condition. SINR improvement from frequency reuse for Set 1 is larger than Set 2.

- SINR for “other” type of terminal for Ka-band is very low. Receiver and transmitter model defined for “other” seems too poor for Ka-band (i.e. large pathloss). Comparison of G/T (as receiver performance) and EIRP (as transmitter performance) for VSAT and “other” defined in [3] are shown below. G/T and EIRP for “other” is derived from antenna configuration and equations in [4]. As a reference, a commercial antenna system for Ka-band is also shown. Because “other” is intended for moving platform (e.g. aircraft, vessels, trains) as described in [3], reconsideration of the parameters would be required.

|  | G/T | EIRP |
| --- | --- | --- |
| VSAT | 15.9 dB/K | 46.2 dBW |
| “Other” | -10.3 dB/K | 36.1 dBW |
| Aircraft antenna (*1) | 15 dB/K | 48.5 dBW |

(*1) Thinkom, https://www.thinkom.com/wp-content/uploads/2018/09/ka2517-datasheet_9_18_web.pdf

Proposal 1: Reconsider the transmitter and receiver model for terminal type “other” because G/T and EIRP are too low compared to a commercial antenna system.

Evaluation results

Set 1

< Large scale >

< Full scale >

Figure 2  GEO S-band (Handheld) Set 1

< Large scale >

< Full scale >

Figure 3  GEO Ka-band (VSAT) Set 1

< Large scale >

Not available

< Full scale >

Figure 4  GEO Ka-band (other) Set 1

< Large scale >

< Full scale >

Figure 5  LEO 1200km S-band (handheld) Set 1

< Large scale >

< Full scale >

Figure 6  LEO 1200km Ka-band (VSAT) Set 1

< Large scale >

Not available

< Full scale >

Figure 7  LEO 1200km Ka-band (other) Set 1

< Large scale >

< Full scale >

Figure 8  LEO 600km S-band (handheld) Set 1

< Large scale >

< Full scale >

Figure 9  LEO 600km Ka-band (VSAT) Set 1

< Large scale >

Not available

< Full scale >

Figure 10 LEO 600km Ka-band (other) Set 1

Set 2

< Large scale >

< Full scale >

Figure 11  GEO S-band (Handheld) Set 2

< Large scale >

< Full scale >

Figure 12  GEO Ka-band (VSAT) Set 2

< Large scale >

< Full scale >

Figure 13  GEO Ka-band (other) Set 2

< Large scale >

< Full scale >

Figure 14  LEO 1200km S-band (handheld) Set 2

< Large scale >

< Full scale >

Figure 15  LEO 1200km Ka-band (VSAT) Set 2

< Large scale >

< Full scale >

Figure 16  LEO 1200km Ka-band (other) Set 2

< Large scale >

< Full scale >

Figure 17  LEO 600km S-band (handheld) Set 2

< Large scale >

< Full scale >

Figure 18  LEO 600km Ka-band (VSAT) Set 2

< Large scale >

< Full scale >

Figure 19 LEO 600km Ka-band (other) Set 2

Table 1  Summary of Set 1, Frequency reuse option 1 (SINR in dB)

| Set 1 Option 1 |  | 5%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- | --- |
| GEO | S-band (handheld) | -10 | -2.7 | -1.2 |
|  | Ka-band (VSAT) | -15 | -1.5 | 1.0 |
|  | Ka-band (other) | -30 | -17 | -13.5 |
| LEO 1200km | S-band (handheld) | -3.0 | -1.2 | 1.0 |
|  | Ka-band (VSAT) | -3.0 | -1.2 | 1.0 |
|  | Ka-band (other) | -18 | -17 | -15.5 |
| LEO 600km | S-band (handheld) | -2.8 | -1.2 | 1.0 |
|  | Ka-band (VSAT) | -2.7 | -1.2 | 1.0 |
|  | Ka-band (other) | -18 | -17 | -15.5 |

Table 2  Summary of Set 1, Frequency reuse option 2/3 (SINR in dB)

| Set 1 Option 2/3 |  | 5%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- | --- |
| GEO | S-band (handheld) | -10 | 1.2 | 2.6 |
|  | Ka-band (VSAT) | -15 (-15) | 7.7 (7.7) | 9 (9) |
|  | Ka-band (other) | -30 | -17 | -13.5 |
| LEO 1200km | S-band (handheld) | 7.5 | 8.2 | 8.8 |
|  | Ka-band (VSAT) | 7 (6.7) | 7.8 (7.6) | 8.5 (8.5) |
|  | Ka-band (other) | -18 | -17 | -15.5 |
| LEO 600km | S-band (handheld) | 7.5 | 8.2 | 8.8 |
|  | Ka-band (VSAT) | 7 (6.5) | 7.8 (7.6) | 8.2 (8.2) |
|  | Ka-band (other) | -18 | -17 | -15.5 |

Note: values in () is values of option 3 for VSAT

Table 3  Summary of Set 2, Frequency reuse option 1 (SINR in dB)

| Set 2 Option 1 |  | 5%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- | --- |
| GEO | S-band (handheld) | -15 | -5.8 | -4 |
|  | Ka-band (VSAT) | < -15 | -2.5 | -0.5 |
|  | Ka-band (other) | < -35 | -24.8 | -22 |
| LEO 1200km | S-band (handheld) | -3.5 | -1.8 | 0.3 |
|  | Ka-band (VSAT) | -7 | -2.5 | -1 |
|  | Ka-band (other) | -33 | -24.9 | -21.3 |
| LEO 600km | S-band (handheld) | -3.7 | -2 | 0.4 |
|  | Ka-band (VSAT) | -7 | -2.5 | -0.7 |
|  | Ka-band (other) | -32 | -24.8 | -21.8 |

Table 4  Summary of Set 2, Frequency reuse option 2/3 (SINR in dB)

| Set 2 Option 2/3 |  | 5%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- | --- |
| GEO | S-band (handheld) | -15 | -4 | -2.4 |
|  | Ka-band (VSAT) | < -15 | 2.5 | 5.6 |
|  | Ka-band (other) | < -35 | -24.8 | -22 |
| LEO 1200km | S-band (handheld) | 3 | 5.1 | 6 |
|  | Ka-band (VSAT) | -5 | 2.5 | 5.8 |
|  | Ka-band (other) | -33 | -24.9 | -21.3 |
| LEO 600km | S-band (handheld) | 3 | 5.1 | 6.2 |
|  | Ka-band (VSAT) | -4 | 2.5 | 5.6 |
|  | Ka-band (other) | -32 | -24.8 | -21.8 |

Note: values for Option 2 and 3 are the same.

Link budget

Link budget is calculated based on the agreed condition [4] and CIR evaluation results by system level evaluation.

Table 5 Link budget for DL Set 1, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka) |  |  | handheld(S) |  |  | other(Ka) |  |  |
| Satellite altitude | km | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Frequency | GHz | 20 | 20 | 20 | 2 | 2 | 2 | 20 | 20 | 20 |
| Bandwidth | MHz | 400 | 400 | 400 | 30 | 30 | 30 | 400 | 400 | 400 |
| Elevation angle | degree | 10 | 30 | 30 | 10 | 30 | 30 | 10 | 30 | 30 |
| Maximum distance | km | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 |
| Propagation |  |  |  |  |  |  |  |  |  |  |
| Free space path loss | dB | 210.6 | 184.5 | 179.1 | 190.6 | 164.5 | 159.1 | 210.6 | 184.5 | 179.1 |
| atmospheric path loss | dB | 1.44 | 0.5 | 0.5 | 0.2 | 0 | 0 | 1.44 | 0.5 | 0.5 |
| rain fades | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |  |  |  |
| Tx power | dBW | - | - | - | - | - | - | - | - | - |
| Cable loss | dB | - | - | - | - | - | - | - | - | - |
| antenna gain | dBi | 58.5 | 38.5 | 38.5 | 51 | 30 | 30 | 58.5 | 38.5 | 38.5 |
| EIRP density | dBW/MHz | 40 | 10 | 4 | 59 | 40 | 34 | 40 | 10 | 4 |
| EIRP | dBW | 66.0 | 36.0 | 30.0 | 73.8 | 54.8 | 48.8 | 66.0 | 36.0 | 30.0 |
| Receiver |  |  |  |  |  |  |  |  |  |  |
| antenna gain | dBi | 39.7 | 39.7 | 39.7 | 0 | 0 | 0 | 8 | 8 | 8 |
| number of element |  | 1 | 1 | 1 | 2 | 2 | 2 | 64 | 64 | 64 |
| polarization loss | dB | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |
| Noise Figure | dB | 1.2 | 1.2 | 1.2 | 7 | 7 | 7 | 9 | 9 | 9 |
| ambient temprature(T0) | K | 290 | 290 | 290 | 290 | 290 | 290 | 290 | 290 | 290 |
| Antenna temperature(Ta) | K | 150 | 150 | 150 | 290 | 290 | 290 | 290 | 290 | 290 |
| G/T | dB/K | 15.9 | 15.9 | 15.9 | -31.6 | -31.6 | -31.6 | -10.6 | -10.6 | -10.6 |
| Output |  |  |  |  |  |  |  |  |  |  |
| CNR | dB | 12.4 | 9.5 | 8.9 | 2.1 | 9.5 | 8.9 | -17.0 | -19.9 | -20.6 |
| Frequency reuse 1 (option 1) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | -0.7 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.9 | -0.9 | -0.9 |
| CIR @5% | dB | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.9 | -2.8 | -2.8 |
| CNIR @50% | dB | -0.9 | -1.2 | -1.2 | -2.6 | -1.2 | -1.2 | -17.1 | -20.0 | -20.6 |
| CNIR @5% | dB | -2.8 | -3.0 | -3.0 | -3.9 | -3.0 | -3.0 | -17.2 | -20.0 | -20.6 |
| Frequency reuse 3 (option 2) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.5 | 10.7 | 10.7 |
| CIR @5% | dB | 10.1 | 10.2 | 10.2 | 10.2 | 10.2 | 10.2 | 9.4 | 10.1 | 10.1 |
| CNIR @50% | dB | 8.5 | 7.1 | 6.7 | 1.6 | 7.1 | 6.7 | -17.0 | -20.0 | -20.6 |
| CNIR @5% | dB | 8.1 | 6.8 | 6.5 | 1.5 | 6.8 | 6.5 | -17.1 | -20.0 | -20.6 |
| Frequency reuse 2 + polarization 2 (option 3) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.5 | 10.5 | - | - | - | - | - | - |
| CIR @5% | dB | 10.1 | 9.2 | 9.2 | - | - | - | - | - | - |
| CNIR @50% | dB | 8.5 | 6.9 | 6.6 | - | - | - | - | - | - |
| CNIR @5% | dB | 8.1 | 6.3 | 6.0 | - | - | - | - | - | - |

Table 6 Link budget for UL Set 1, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka) |  |  | handheld(S) |  |  | other(Ka) |  |  |
| Satellite altitude | km | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Frequency | GHz | 30 | 30 | 30 | 2 | 2 | 2 | 30 | 30 | 30 |
| Bandwidth | MHz | 100 | 100 | 100 | 1 | 1 | 1 | 1 | 1 | 1 |
| Elevation angle | degree | 10 | 30 | 30 | 10 | 30 | 30 | 10 | 30 | 30 |
| Maximum distance | km | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 |
| Propagation |  |  |  |  |  |  |  |  |  |  |
| Free space path loss | dB | 214.2 | 188.0 | 182.6 | 190.6 | 164.5 | 159.1 | 214.2 | 188.0 | 182.6 |
| atmospheric path loss | dB | 1.44 | 0.5 | 0.5 | 0.2 | 0 | 0 | 1.44 | 0.5 | 0.5 |
| rain fades | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |  |  |  |
| Tx power | dBm | 33 | 33 | 33 | 23 | 23 | 23 | 43 | 43 | 43 |
| Cable loss | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| antenna gain | dBi | 43.2 | 43.2 | 43.2 | 0 | 0 | 0 | 8 | 8 | 8 |
| number of element |  | 1 | 1 | 1 | 1 | 1 | 1 | 32 | 32 | 32 |
| EIRP density | dBW/MHz | - | - | - | - | - | - | - | - | - |
| EIRP | dBW | 46.2 | 46.2 | 46.2 | -7.0 | -7.0 | -7.0 | 36.1 | 36.1 | 36.1 |
| Receiver |  |  |  |  |  |  |  |  |  |  |
| antenna gain | dBi | 51 | 30 | 30 | 51 | 30 | 30 | 51 | 30 | 30 |
| polarization loss | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Noise Figure | dB | - | - | - | - | - | - | - | - | - |
| ambient temprature(T0) | K | - | - | - | - | - | - | - | - | - |
| Antenna temperature(Ta) | K | - | - | - | - | - | - | - | - | - |
| G/T | dB/K | 28.0 | 13.0 | 13.0 | 19.0 | 1.1 | 1.1 | 28.0 | 13.0 | 13.0 |
| Output |  |  |  |  |  |  |  |  |  |  |
| CNR | dB | 7.2 | 19.3 | 24.7 | -13.2 | -4.8 | 0.6 | 14.1 | 26.1 | 31.5 |
| Frequency reuse 1 (option 1) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | -0.7 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.9 | -0.9 | -0.9 |
| CIR @5% | dB | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.9 | -2.8 | -2.8 |
| CNIR @50% | dB | -1.4 | -0.8 | -0.8 | -13.5 | -6.2 | -3.2 | -1.0 | -0.9 | -0.9 |
| CNIR @5% | dB | -3.1 | -2.7 | -2.7 | -13.6 | -6.9 | -4.4 | -3.0 | -2.8 | -2.8 |
| Frequency reuse 3 (option 2) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.5 | 10.7 | 10.7 |
| CIR@5% | dB | 10.1 | 10.2 | 10.2 | 10.2 | 10.2 | 10.2 | 9.4 | 10.1 | 10.1 |
| CNIR @50% | dB | 5.6 | 10.2 | 10.6 | -13.3 | -4.9 | 0.2 | 8.9 | 10.6 | 10.7 |
| CNIR @5% | dB | 5.4 | 9.7 | 10.0 | -13.3 | -4.9 | 0.1 | 8.1 | 10.0 | 10.1 |
| Frequency reuse 2 + polarization 2 (option 3) |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.5 | 10.5 | - | - | - | - | - | - |
| CIR @5% | dB | 10.1 | 9.2 | 9.2 | - | - | - | - | - | - |
| CNIR @50% | dB | 5.6 | 10.0 | 10.3 | - | - | - | - | - | - |
| CNIR @5% | dB | 5.4 | 8.8 | 9.1 | - | - | - | - | - | - |

Table 7 Link budget for DL Set 2, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit | Assumptions / Values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  | VSAT(Ka) |  |  | handheld(S) |  |  | other(Ka) |  |  |  |
| Satellite altitude | km | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Frequency | GHz | 20 | 20 | 20 | 2 | 2 | 2 | 20 | 20 | 20 |
| Bandwidth | MHz | 400 | 400 | 400 | 30 | 30 | 30 | 400 | 400 | 400 |
| Elevation angle | degree | 10 | 30 | 30 | 10 | 30 | 30 | 10 | 30 | 30 |
| Maximum distance | km | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 |
| Propagation |  |  |  |  |  |  |  |  |  |  |  |
| Free space path loss | dB | 210.6 | 184.5 | 179.1 | 190.6 | 164.5 | 159.1 | 210.6 | 184.5 | 179.1 |  |
| atmospheric path loss | dB | 1.44 | 0.5 | 0.5 | 0.2 | 0 | 0 | 1.44 | 0.5 | 0.5 |  |
| rain fades | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| shadowing margin | dB | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| additional loss | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Transmitter |  |  |  |  |  |  |  |  |  |  |  |
| Tx power | dBW | - | - | - | - | - | - | - | - | - |  |
| Cable loss | dB | - | - | - | - | - | - | - | - | - |  |
| antenna gain | dBi | 50.5 | 30.5 | 30.5 | 45.5 | 24 | 24 | 50.5 | 30.5 | 30.5 |  |
| EIRP density | dBW/MHz | 32 | 2 | -4 | 53.5 | 34 | 28 | 32 | 2 | -4 |  |
| EIRP | dBW | 58.0 | 28.0 | 22.0 | 68.3 | 48.8 | 42.8 | 58.0 | 28.0 | 22.0 |  |
| Receiver |  |  |  |  |  |  |  |  |  |  |  |
| antenna gain | dBi | 39.7 | 39.7 | 39.7 | 0 | 0 | 0 | 8 | 8 | 8 |  |
| number of element |  | 1 | 1 | 1 | 2 | 2 | 2 | 64 | 64 | 64 |  |
| polarization loss | dB | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| Noise Figure | dB | 1.2 | 1.2 | 1.2 | 7 | 7 | 7 | 9 | 9 | 9 |  |
| ambient temprature(T0) | K | 290 | 290 | 290 | 290 | 290 | 290 | 290 | 290 | 290 |  |
| Antenna temperature(Ta) | K | 150 | 150 | 150 | 290 | 290 | 290 | 290 | 290 | 290 |  |
| G/T | dB/K | 15.9 | 15.9 | 15.9 | -31.6 | -31.6 | -31.6 | -10.6 | -10.6 | -10.6 |  |
| Output |  |  |  |  |  |  |  |  |  |  |  |
| CNR | dB | 4.4 | 1.5 | 0.9 | -3.4 | 3.5 | 2.9 | -25.0 | -27.9 | -28.6 |  |
| Frequency reuse 1 (option 1) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | -0.8 | -0.8 | -0.8 | -0.8 | -0.7 | -0.7 | -0.8 | -0.8 | -0.8 |  |
| CIR @5% | dB | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 |  |
| CNIR @50% | dB | -2.0 | -2.8 | -3.1 | -5.3 | -2.1 | -2.3 | -25.1 | -28.0 | -28.6 |  |
| CNIR @5% | dB | -3.5 | -4.1 | -4.3 | -6.0 | -3.6 | -3.8 | -25.1 | -28.0 | -28.6 |  |
| Frequency reuse 3 (option 2) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.8 | 10.8 | 10.8 | 10.7 | 10.7 | 10.8 | 10.8 | 10.8 |  |
| CIR @5% | dB | 10.2 | 10.2 | 10.2 | 10.2 | 10 | 10 | 10.1 | 10.2 | 10.2 |  |
| CNIR @50% | dB | 3.5 | 1.0 | 0.4 | -3.5 | 2.7 | 2.2 | -25.0 | -27.9 | -28.6 |  |
| CNIR @5% | dB | 3.4 | 0.9 | 0.4 | -3.5 | 2.6 | 2.1 | -25.0 | -27.9 | -28.6 |  |
| Frequency reuse 2 + polarization 2 (option 3) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB | 10.8 | 10.8 | 10.8 | - | - | - | - | - | - |  |
| CIR @5% | dB | 5.9 | 9.6 | 9.6 | - | - | - | - | - | - |  |
| CNIR @50% | dB | 3.5 | 1.0 | 0.4 | - | - | - | - | - | - |  |
| CNIR @5% | dB | 2.1 | 0.8 | 0.3 | - | - | - | - | - | - |  |

Table 8 Link budget for UL Set 2, GEO, LEO1200km, LEO600km, frequency reuse option 1, 2 and 3

| Parameters | Unit |  | Assumptions / Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal |  |  | VSAT(Ka) |  |  | handheld(S) |  |  | other(Ka) |  |  |
| Satellite altitude | km |  | 35786 | 1200 | 600 | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Frequency | GHz |  | 30 | 30 | 30 | 2 | 2 | 2 | 30 | 30 | 30 |
| Bandwidth | MHz |  | 100 | 100 | 100 | 1 | 1 | 1 | 1 | 1 | 1 |
| Elevation angle | degree |  | 10 | 30 | 30 | 10 | 30 | 30 | 10 | 30 | 30 |
| Maximum distance | km |  | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 | 40586 | 1998.9 | 1075.1 |
| Propagation |  |  |  |  |  |  |  |  |  |  |  |
| Free space path loss | dB |  | 214.2 | 188.0 | 182.6 | 190.6 | 164.5 | 159.1 | 214.2 | 188.0 | 182.6 |
| atmospheric path loss | dB |  | 1.44 | 0.5 | 0.5 | 0.2 | 0 | 0 | 1.44 | 0.5 | 0.5 |
| rain fades | dB |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| shadowing margin | dB |  | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 |
| additional loss | dB |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Transmitter |  |  |  |  |  |  |  |  |  |  |  |
| Tx power | dBm |  | 33 | 33 | 33 | 23 | 23 | 23 | 43 | 43 | 43 |
| Cable loss | dB |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| antenna gain | dBi |  | 43.2 | 43.2 | 43.2 | 0 | 0 | 0 | 8 | 8 | 8 |
| number of element |  |  | 1 | 1 | 1 | 1 | 1 | 1 | 32 | 32 | 32 |
| EIRP density | dBW/MHz |  | - | - | - | - | - | - | - | - | - |
| EIRP | dBW |  | 46.2 | 46.2 | 46.2 | -7.0 | -7.0 | -7.0 | 36.1 | 36.1 | 36.1 |
| Receiver |  |  |  |  |  |  |  |  |  |  |  |
| antenna gain |  | dBi | 54 | 34 | 34 | 45.5 | 24 | 24 | 51 | 30 | 30 |
| polarization loss |  | dB | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Noise Figure |  | dB | - | - | - | - | - | - | - | - | - |
| ambient temprature(T0) |  | K | - | - | - | - | - | - | - | - | - |
| Antenna temperature(Ta) |  | K | - | - | - | - | - | - | - | - | - |
| G/T |  | dB/K | 20.0 | 5.0 | 5.0 | 14.0 | -4.9 | -4.9 | 20.0 | 5.0 | 5.0 |
| Output |  |  |  |  |  |  |  |  |  |  |  |
| CNR | dB |  | -0.8 | 11.3 | 16.7 | -18.2 | -10.8 | -5.4 | 6.1 | 18.1 | 23.5 |
| Frequency reuse 1 (option 1) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB |  | -0.7 | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.9 | -0.9 | -0.9 |
| CIR @5% | dB |  | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.7 | -2.9 | -2.8 | -2.8 |
| CNIR @50% | dB |  | -3.8 | -1.1 | -0.9 | -18.3 | -11.2 | -6.7 | -1.7 | -1.0 | -0.9 |
| CNIR @5% | dB |  | -4.9 | -2.9 | -2.7 | -18.4 | -11.4 | -7.3 | -3.4 | -2.8 | -2.8 |
| Frequency reuse 3 (option 2) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB |  | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.8 | 10.5 | 10.7 | 10.7 |
| CIR@5% | dB |  | 10.1 | 10.2 | 10.2 | 10.2 | 10.2 | 10.2 | 9.4 | 10.1 | 10.1 |
| CNIR @50% | dB |  | -1.1 | 8.0 | 9.8 | -18.2 | -10.8 | -5.5 | 4.7 | 10.0 | 10.5 |
| CNIR @5% | dB |  | -1.1 | 7.7 | 9.3 | -18.2 | -10.8 | -5.5 | 4.4 | 9.5 | 9.9 |
| Frequency reuse 2 + polarization 2 (option 3) |  |  |  |  |  |  |  |  |  |  |  |
| CIR @50% | dB |  | 10.8 | 10.5 | 10.5 | - | - | - | - | - | - |
| CIR @5% | dB |  | 10.1 | 9.2 | 9.2 | - | - | - | - | - | - |
| CNIR @50% | dB |  | -1.1 | 7.9 | 9.6 | - | - | - | - | - | - |
| CNIR @5% | dB |  | -1.1 | 7.1 | 8.5 | - | - | - | - | - | - |

Conclusion

System level evaluation results for calibration are shown in this contribution. Link budget calculation results are also shown. It is proposed to use evaluation results in section 2 and 3 for calibration.

It is found that SINR distribution for terminal type “other” is very low because G/T and EIRP are too low compared with a commercial antenna system for moving platform (aircraft, vessel, train). Therefore, we propose the following.

Proposal: Reconsider the transmitter and receiver model for terminal type “other” because G/T and EIRP are too low compared to a commercial antenna system.

- Reference

- RP-190710, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.811 V15.0.0, “Study on New Radio (NR) to support non terrestrial networks”

- R1-1907847, “Chairman's notes of AI 7.2.5 Study on solutions for NR to support Non-Terrestrial Network (NTN)”

- R1-1907836, “TP for Link Budget Analysis in TR 38.821”, Nokia, Nokia Shanghai Bell, Thales, MediaTek, Ericsson, ZTE