3GPP TSG-RAN WG1 Meeting #98	R1-1908822

# Prague, CZ, August 26th – 30th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | DRAFT | rev |  | Current version: | 14.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of larger UL TBS without explicit configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-14, a maximum TBS of 2984 bits was introduced for BL/CE UEs with maximum bandwidth of 1.4MHz. The following agreement was reached: RAN1#86bis agreement on TBS determination: - TBS for 1.4-MHz UE in CE mode A is indicated by using the existing TBS table - Introduce UE capability signaling for the larger TBS - The use of larger TBS is RRC configured by eNB - The tables in R1-1610861 are agreed. <Table omitted for brevity> Current specification, however, does not specify the UE behavior when the UE is not configured with ce-PUSCH-NB-MaxTBS and receives a DCI that indicates more than 1000 bit TBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify that a UE not configured with ce-PUSCH-NB-MaxTBS is not expected to be scheduled with a TBS greater than 1000 bits. 2) Correct RRC parameter name (ce-PUSCH-NB-MaxTBS instead of ce-pusch-nb-maxTbs-config) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent with agreements. Incorrect RRC parameter name. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1, 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.6.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For a BL/CE UE

-	if the UE is configured with CEModeA, and higher layer parameter ce-pusch-nb-maxTbs configured with value 'On', and if the MPDCCH corresponding to the PUSCH transmission is located in UE-specific search space, the modulation order is determined according to table 8.6.1-2A.

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On', and if the Modulation order override field in the DCI is set to 1, the modulation order is set to

-	otherwise, the modulation order is determined according to table 8.6.1-2.

A BL/CE UE configured with CEModeB is not expected to receive a DCI format 6-0B indicating .

<Unchanged parts are omitted>

### 8.6.2	Transport block size determination

For a BL/CE UE configured with CEModeA and a PUSCH transmission not scheduled by the Random Access Response Grant,

-	if the UE is configured with higher layer parameter ce-pusch-nb-maxTbs with value 'On', and if the MPDCCH corresponding to the PUSCH transmission is located in UE-specific search space, the UE shall first determine the TBS index () usingand Table 8.6.1-2A;

-	otherwise, the UE shall first determine the TBS index () usingand Table 8.6.1-2.

For a BL/CE UE configured with CEModeA,

-	if the UE is configured with higher layer parameter ce-pusch-maxBandwidth-config with value 5MHz, the TBS is determined by the procedure in Subclause 7.1.7.2.1, for

-	otherwise, the TBS is determined by the procedure in Subclause 7.1.7.2.1.

For a BL/CE UE configured with CEModeA and a PUSCH transmission scheduled by the Random Access Response Grant, the UE shall determine the TBS index by the procedure in Subclause 6.2.

A BL/CE UE of UL category M1 not configured with ce-pusch-nb-maxTbs is not expected to receive a DCI indicating a TBS of more than 1000 bits.

For a BL/CE UE configured with CEModeB, the TBS is determined according to the procedure in Subclause 7.1.7.2.1 for , and =6 when resource allocation field is '110' or '111' otherwise= 3.

<Unchanged parts are omitted>