3GPP TSG RAN WG1 #98			R1-1908833

Prague, CZ, August 26th – 30th, 2019

Agenda item:	6.2.1.9

Source: 	Qualcomm Incorporated

Title: 	Measurements based on RSS

Document for:	Discussion/Decision

# Introduction

The Rel 16 WID on using RSS for cell measurements has the following objective:

Mobility Enhancement:

•	Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2]

To this end, RAN2 #104 sent RAN1 an LS asking whether and how the size of the RSS parameters for measuring RSRP of neighbour cells can be minimised. In this contribution, we address this issue raised in the RAN2 LS.

In RAN1#97, the following agreements and working assumptions were made:

Working Assumption

Restrict the RSS locations by reducing the number of possible RSS locations

Agreement

Downselect and/or combine from the following in RAN1#98 for overhead reduction of the RSS time offset & RSS frequency location:

- Method 1: Reduce the number of possible RSS time offsets and RSS frequency locations

- Method 2: The RSS time offset & RSS frequency location is a function of Cell ID, i.e. no signaling bits required

- Method 3: The restricted RSS locations are dependent upon the Cell ID

- Method 4: The RSS time offset & RSS frequency location is carrier specific

- Method 5: The RSS frequency location of neighbor cells are within a frequency block relative to the RSS frequency location of the serving cell, e.g. the frequency block can be a narrowband

Other methods are not precluded

# Neighbour cell RSS information

From a signalling perspective, the two most significant contributors to the RSS neighbour cell information are the time offset (ce-rss-timeOffset-config)—requiring 5 bits, and the frequency location (ce-rss-freqPos-config)—requiring 7 bits.

In [1], it is proposed to reduce the granularity of timing and frequency information that is conveyed about the neighbour cell RSSs. Arguments made in favour of this approach stem from the fact that a UE looking for neighbour cell RSS information is already synchronized to its own cell, thereby reducing the number of hypotheses that need to be tested w.r.t an out-of-sync scenario. Keeping this rationale in mind, [1] proposes a coarsely quantized signalling of the time and frequency information of the neighbour cell RSSs, thereby requiring the UE to search across (purportedly limited) multiple hypotheses across time, frequency and cells to acquire neighbour cell RSS information.

While [1] disregards the possibility of an out-of-sync UE attempting to simultaneously read multiple RSSs, we believe there may be a case for such settings in future—as UE processing capabilities improve over time, an MTC UE (with limited but non-zero mobility) may indeed run a simultaneous search across multiple RSSs from a near-cold, out-of-sync start, if it has a rough estimate of its location in proximity to its previous serving cell. In effect, the RSS neighbour cell information may, over time, aid in primary synchronization in these settings. However, this future flexibility is essentially rendered unviable if an out-of-sync UE does not have access to the time and frequency locations of the RSSs in its neighbour cell list from its last active connection with its serving cell.

Observation 1: Reducing parameter resolution for neighbour cell RSS time-offset and frequency-location signalling annuls the possibility of simultaneous RSS detection across multiple cells in out-of-sync settings.

With this in mind, and to generally enable feasible simultaneous decoding of multiple RSSs, we propose to make both the RSS time offset (ce-rss-timeOffset-config) and RSS frequency location (ce-rss-freqPos-config) cell ID specific, i.e., to introduce an equation that determines the time offset and frequency location based on cell ID. This eliminates the need for explicit signalling in the neighbour cell RSS information, while also ensuring simultaneous RSS detection across multiple cells under sync and out-of-sync settings with reasonable UE complexity.

Proposal 1:  The RSS time-offset (ce-rss-timeOffset-config) and RSS frequency-location (ce-rss-freqPos-config) are determined based on Cell ID specific and may not be signalled as part of neighbour cell RSS information.

- For backwards compatibility with Rel15 UEs, these parameters for the serving cell shall continue to be signalled explicitly.

- Adopt method 3

# Summary

Observation 1: Reducing parameter resolution for neighbour cell RSS time-offset and frequency-location signalling annuls the possibility of simultaneous RSS detection across multiple cells in out-of-sync settings.

Proposal 1:  The RSS time-offset (ce-rss-timeOffset-config) and RSS frequency-location (ce-rss-freqPos-config) are determined based on Cell ID specific and may not be signalled as part of neighbour cell RSS information.

- For backwards compatibility with Rel15 UEs, these parameters for the serving cell shall continue to be signalled explicitly.

- Adopt method 3

# References

[1] R1-1901745, “Use of RSS for measurement improvements in LTE-MTC”, by Ericsson, RAN1#96, Athens, Greece.