3GPP TSG RAN WG1 Meeting #98                                         R1-1908836

Prague, CZ, 26th – 30th August 2019

Agenda item:	6.2.2.3

Source: 	Qualcomm Incorporated

Title: 	Scheduling of multiple DL/UL transport blocks

Document for:	Discussion and Decision

# 1	Introduction

In RAN#80, a new work item on NB-IoT enhancements was approved (RP-181451) with the following objective:

Scheduling enhancement:

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2]

  - Enhancement of SPS can be discussed.

In RAN1#97, the following agreements and conclusion were made:

Agreement

Confirm the working assumption last meeting:

3 bits are used to indicate scheduled TB number, HARQ process index, NDI for HARQ operation

  - FFS: Details coding scheme of these 3 bits

Agreement

The coding scheme of the three bits (scheduled TB number, HARQ process index, NDI for HARQ operation) is as follows:

| Codepoint | Description |
| --- | --- |
| 000 | Single TB scheduling, HARQ ID=0, NDI=0 |
| 001 | Single TB scheduling, HARQ ID=0, NDI=1 |
| 010 | Single TB scheduling, HARQ ID=1, NDI=0 |
| 011 | Single TB scheduling, HARQ ID=1, NDI=1 |
| 100 | Multi-TB scheduling, NDI=00 |
| 101 | Multi-TB scheduling, NDI=01 |
| 110 | Multi-TB scheduling, NDI=10 |
| 111 | Multi-TB scheduling, NDI=11 |

- Only one additional bit is added in the DCI format to support the above indication of 8 states

Note: How to capture the above agreement is up to the editor

Agreement

In case of non-interleaved transmission, for individual feedback of 2 TBs case, continuous uplink feedback starts, after the end of  DL subframe for FDD, where n is the ending subframe of last scheduled TB and  k0' is down-selected from the following two choices:

- The same value as the one for legacy one TB case (i.e. reuse the existing specification without change)

- New values are introduced which depends on the length of last TB and ACK/NACK resources

  - Existing values can also be used

Agreement

HARQ-ACK resource field is common across the HARQ-ACK feedback for all the DL TBs scheduled.

- ACK/NACK subcarriers are the same across all the TBs

In this contribution, we continue to discuss the remaining issues for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast.

# 2	TB interlacing

We note that consecutive transmission of TBs may not be optimal in terms of performance (e.g. due to lack of time diversity). In Figure 1 we show the difference between consecutive and interlaced PUSCH transmission.

Figure 1 Consecutive (Alt1) vs interlaced (Alt2) NPUSCH transmission.

In our companion contribution [1] we evaluated the gains of interlacing multiple transport blocks in eMTC. Similar gains should be observed in NB-IoT case (despite having a reduced number of HARQ processes).

Observation 1: Interlacing multiple transport blocks (in DL or UL) with multiple HARQ processes provides gain due to time diversity.

Proposal 1: Support the interlacing of TBs to achieve time diversity.

# 3 	HARQ Timeline

For unicast transmission, HARQ timeline can be described in two cases as show in Figure 2 below.

Figure 2 HARQ timeline for DL multi-TB scheduling (from [2])

For case 1, the value of X is dependent on the length of the second TB and the HARQ-ACK time domain resource. The determination of X value shall satisfy Rel-14 HARQ timing constraints, i.e. minimum 12 ms separation between the end of TB and the start of the HARQ-ACK transmission. For example, X can be indicated as where k0 is the scheduling delay indicated by the DCI and m is the length of the second TB. Alternatively, X can be also given by k0'=k0-L where L is the length of HARQ-ACK transmission.

For case 2, X is indicated by the scheduling delay field in the DCI same as in the current specification. Case 2 is simpler than case 1, however, it may not achieve maximum throughput increase since the transmission of HARQ-ACK is delayed. The throughput performance loss is clearly observed for packet with a small transmission duration. For example, we consider 256 bits TB to be repeated 4 times and HARQ-ACK is repeated by 8 times. Figure 3 depicts the HARQ timeline comparison of this example. For case 2, we can achieve ~5.1 kbps and for case 1, the gap for HARQ-ACK transmission is reduced from 12ms to 1ms and we can achieve ~5.8 kbps, which is 15% increase in throughput.

Figure 3 Example of HARQ timeline for DL multi-TB scheduling

As discussed in [2], the potential issue with case 1 is the increase of scheduling complexity for eNB and decoding complexity for UE. To address the concern, we can select a value from 1 to k0 under Rel-14 HARQ delay constraints as  k0'.  That is, the smallest value in the candidate set satisfying with Rel-14 HARQ delay constraint is indicated as k0'. In such case, we can achieve the benefits of maximizing throughput and reduce the complexity to find k0'.

Proposal 2: In case of non-interleaved transmission, for individual feedback of 2 TBs case, continuous uplink feedback starts after the end of  DL subframe for FDD where k0' is the smallest value in the range from 1 to k0 satisfying Rel-14 HARQ delay constraints.

# 4	Gap for multiple TBs in SC-PTM

Agreement

For SC-MTCH multiple TBs scheduling, down-select from the following options:

- Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field)

- Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers.

- Support both a) and b)

For decoding of DL data and control, the NB-IoT UE requires processing time. If DL multiple TBs are triggered by one DCI, the UE may need time gap, e.g., Gap1, to finish processing of the n-th NPDSCH and start reception of the (n+1)-th NPDSCH. Also, the UE requires processing time, e.g., Gap2, between last NPDSCH and next NPDCCH.

There are two different modes of UE processing. As illustrated in Figure 4, NPDCCH triggers NDPSCH0 and NPDSCH1 together. If UE is performing ‘real time demodulation’ with no buffering of next data, as illustrated in Figure 4(a), it is necessary to finish the processing of NPDSCH0 before starting the decoding of NPDSCH1. When the NPDSCH transmission duration of N subframes is shorter than the required decoding time N0, additional gap is needed to finish the NPDSCH0 before starting decoding NPDSCH1. On the other hand, if the duration of N subframes is longer than N0, the time N is sufficient already and NPDSCH0 and NPDSCH1 can be sent back to back, i.e., Gap1=0. Accordingly, the gap between two NPDSCHs is set as Gap1=max(N0-N,0). Similarly, similar principle can be applied to the separation between the last NPDSCH and the next NPDCCH. At least, the distance between the end of last NPDSCH and the end of the first NPDCCH candidate (e.g., 1st subframe assuming early termination for NPDCCH detection) of a search space should be no less than Gap2=N0. Similar gap configuration can be extended to the case of more than 2 TBs, with Gap1=max(N0-N,0) in between NPDSCHs and Gap2= N0 between the last NPDSCH and next NPDCCH.

Another case is shown in Figure 4(b), where the UE is able to do parallel processing of decoding NPDSCH0 and buffering NPDSCH1 at the same time, i.e. for UE capable of two HARQ processes. The buffer size should be sufficient for max 2 packets of max TB size since UE support 2 HARQ processes already. In this case, the UE does not need to finish the decoding of NPDSCH0 so that the eNB could transmit NPDSCH0 and NPDSCH1 back to back, even if the transmission duration N is smaller than N0. If there is a new NPDCCH following the last NPDSCH, the Gap2 should finish the processing of all the TBs. As shown in Figure 4(b), taking into account the NPDSCH duration, the distance between the end of last NPDSCH and the end of the first NPDCCH candidate (e.g., 1st subframe assuming early termination for NPDCCH detection) of a search space should be no less than Gap2=max{2N0-N,N0}.

The UE may have limited buffer and only can do batch processing of max two TBs. Further consideration is needed if more than 2 TBs are triggered together, on how to select the processing mode for efficient gap configuration.

- Processing without buffering

- Batch processing with buffering

Figure 4 UE processing of multi-TBs NPDSCH.

Proposal 3: For scheduling of multiple TBs for SC-PTM, consider the following processing modes for the UE:

- Option 1: “Real time processing” (UE receiving multiple NPDSCH with gap in between)

- Option 2: Batch processing (UE receiving multiple NPDSCH back to back + additional gap between last NDPSCH and next NPDCCH)

Proposal 4: The gap NPDSCH-NPDSCH and NPDSCH-NPDCCH depends on the required UE processing time and NPDSCH transmission duration.

# 5	SC-PTM and legacy UEs

One of the main discussions on the support of multi-TB scheduling for SC-PTM is whether the SC-MTCH is targeting both legacy and Rel-16 UEs. The differences in design are the following:

- For the case where legacy UEs are decoding the service, the eNB has to send DCI to schedule the NPDSCH. The new signalling (e.g. on SC-MTCH) would just enable the Rel-16 UE to skip the DCI and thus gain some degree of power savings, but there is no throughput increase.

- For the case where only new UEs are decoding the service, there is no need to send single DCI per NPDSCH. In this case, there is a power saving (no DCI monitored), resource saving from eNB perspective (no DCI transmitted) and throughput increase (NPDSCH can be placed closer together).

In view of the above reasoning, we make the following proposal:

Proposal 5: For SC-PTM, allow the following modes of operation:

- Mode 1: The SC-PTM service targets legacy and new UEs.

- Mode 2: The SC-PTM service targets only new UEs.

# 6	Summary

In this contribution we presented our views on scheduling of multiple UL-DL transport blocks. The following observations and proposals are made:

Observation 1: Interlacing multiple transport blocks (in DL or UL) with multiple HARQ processes provides gain due to time diversity.

Proposal 1: Support the interlacing of TBs to achieve time diversity.

Proposal 2: In case of non-interleaved transmission, for individual feedback of 2 TBs case, continuous uplink feedback starts after the end of  DL subframe for FDD where k0' is the smallest value in the range from 1 to k0 satisfying Rel-14 HARQ delay constraints.

Proposal 3: For scheduling of multiple TBs for SC-PTM, consider the following processing modes for the UE:

- Option 1: “Real time processing” (UE receiving multiple NPDSCH with gap in between)

- Option 2: Batch processing (UE receiving multiple NPDSCH back to back + additional gap between last NDPSCH and next NPDCCH)

Proposal 4: The gap NPDSCH-NPDSCH and NPDSCH-NPDCCH depends on the required UE processing time and NPDSCH transmission duration.

Proposal 5: For SC-PTM, allow the following modes of operation:

- Mode 1: The SC-PTM service targets legacy and new UEs.

- Mode 2: The SC-PTM service targets only new UEs.

# 7	References

[1] R1-1902368 Scheduling of multiple DL/UL transport blocks, Qualcomm Incorporated

[2] R1-1906507, Consideration on scheduling enhancement for NB-IoT, ZTE

[3] R1-1810084, “Scheduling multiple DL/UL transport blocks for SC-PTM and unicast”, Huawei, HiSilicon