3GPP TSG-RAN WG1 Meeting #98								R1-1908846

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:	7.2.2.2.3

Source:	NEC

Title:              	HARQ enhancement for NR-U

Document for:	Discussion and Decision

- Introduction

In previous RAN1 meeting [1], the following agreement were reached:

Agreement:

For operation with dynamic HARQ codebook (type-2 codebook):

- PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH

  - For any PDSCH scheduled with numerical or non-numerical value of K1

- The number of HARQ-ACK bits for one PDSCH group can change between successive requests for HARQ-ACK feedback for the same PDSCH group

- HARQ-ACK feedback for all PDSCHs in the same group is carried in the same PUCCH

- One DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH

- C-DAI/T-DAI is accumulated only within each PDSCH group

  - FFS: Choose between the following options:

    - T-DAI is included only for the scheduled group

    - T-DAI is included for each group

- New ACK-Feedback Group Indicator for each PDSCH Group operates as a toggle bit

- Maximum number of PDSCH groups: 2 (FFS: maximum number of groups 4 and maximum number of groups for which feedback is requested in the same PUCCH)

- A UE can signal support of this feature as part of capability signalling.

And some topics are also under discussion in [2]. In this contribution, we discuss the HARQ enhancement for NRU.

- Discussion on HARQ enhancement for NR-U

  - Dynamic HARQ codebook

As agreement in RAN1#97 meetings, one DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH. Meanwhile, C-DAI/T-DAI is accumulated only within each PDSCH group. In Rel-15 NR, dynamic HARQ codebook is generated based on C-DAI/T-DAI. For dynamic HARQ codebook of NR-U, as more PDSCH groups in the same PUCCH are supported which implies more C-DAI/T-DAI in one dynamic HARQ codebook, how to generate dynamic HARQ codebook regarding more C-DAI/T-DAI should be studied.

Proposal 1: As more PDSCH groups in the same PUCCH are supported which implies more C-DAI/T-DAI in one dynamic HARQ codebook, how to generate dynamic HARQ codebook regarding more C-DAI/T-DAI should be studied.

In rel-15 NR, RM code can be adopted for UCI encoding with bit length less than 11. For dynamic HARQ codebook aspect, related observation are listed:

Observation 1: T-DAI can assist for miss detection of last PDCCH within a group, however there is no T-DAI for single cell.

Observation 2: RM encoder cannot distinguish the miss of the last PDCCH or the NACK of the last PDSCH, i.e. the encoder bits are the same between UE miss detected the last PDCCH and the incorrect decoding of the last PDSCH. Hence, for Rel-15 NR, miss detection of the last PDCCH for single cell is not serious due to the nature of RM encoder.

For single cell without T-DAI, when there are more PDSCH groups in one PUCCH and for a nature encoding order (HARQ-ACK bits of the second group follow the first ones), the miss detection of the last PDCCH of the first group will cause HARQ-ACK bit shift at encoder, as figure 1 shows. In figure 1, there are two groups, A and B. Group A has n PDCCHs transmitted by gNB and Group B has p PDCCHs transmitted by gNB. UE miss detects the last PDCCH of group A, i.e. A(n-1). gNB cannot be aware of this shifted HARQ-ACK bit and NACK to ACK issue could happen which is more serious than single PDSCH group.

Figure 1. UE miss detects the last PDCCH of group A

Observation 3: Shifted HARQ-ACK bits may happen when more PDSCH groups in the same PUCCH with single cell operation.

Proposal 2: Avoidance of shifted HARQ-ACK bits when generating dynamic HARQ codebook should be studied.

When the maximum number of PDSCH groups is 2, there is a simple way to avoid that shift, i.e. two directional ordering, as figure 2 shows. The HARQ-ACK bit of the PDSCH group A is mapped from the beginning to the ending of RM encoder, i.e. from 0 to 10. The HARQ-ACK bit of the PDSCH group B is mapped from the ending to the beginning of RM encoder, i.e. from 10 to 0. The miss detection of the last PDCCH of either group A or group B at UE can be treated as NACK at gNB under such encoding method.

Figure 2. Two directional ordering of RM encoding

When HARQ-ACK is transmitted on PUSCH, the total number of HARQ-ACK bits is indicated by PUSCH grant DCI. For more PDSCH groups in one PUCCH meanwhile PUSCH is transmitted for HARQ-ACK feedback, more DAI field in uplink grant corresponding to each PDSCH group can be adopted to solve the ambiguous of miss detection of the last PDCCH of which PDSCH groups.

Proposal 3: For more PDSCH groups in one PUCCH meanwhile PUSCH is transmitted for HARQ-ACK feedback, more DAI field in uplink grant corresponding to each PDSCH group can be adopted.

- Summary

In this contribution, we have the following proposals and observations:

Proposal 1: As more PDSCH groups in the same PUCCH are supported which implies more C-DAI/T-DAI in one dynamic HARQ codebook, how to generate dynamic HARQ codebook regarding more C-DAI/T-DAI should be studied.

Observation 1: T-DAI can assist for miss detection of last PDCCH within a group, however there is no T-DAI for single cell.

Observation 2: RM encoder cannot distinguish the miss of the last PDCCH or the NACK of the last PDSCH, i.e. the encoder bits are the same between UE miss detected the last PDCCH and the incorrect decoding of the last PDSCH. Hence, for Rel-15 NR, miss detection of the last PDCCH for single cell is not serious due to the nature of RM encoder.

Observation 3: Shifted HARQ-ACK bits may happen when more PDSCH groups in the same PUCCH with single cell operation.

Proposal 2: Avoidance of shifted HARQ-ACK bits when generating dynamic HARQ codebook should be studied.

Proposal 3: For more PDSCH groups in one PUCCH meanwhile PUSCH is transmitted for HARQ-ACK feedback, more DAI field in uplink grant corresponding to each PDSCH group can be adopted.

- References

[1] R1-1907846	Chairman's notes of AI 7.2.2	 Study on NR-based Access to Unlicensed Spectrum	Ad-hoc Chair (Ericsson)

[2] R1-1907652	Feature lead summary of HARQ enhancements for NR-U	Huawei