3GPP TSG RAN WG1 #98                                                                                    R1-1908866

Prague, CZ, August 26th – 30th, 2019

Source:	CMCC

Title:	Considerations on HARQ for Non-Terrestrial Networks

Agenda Item:	7.2.5.4

Document for:	Discussion and Decision

- Introduction

3GPP RAN1 #97 meeting made the following agreement[1]:

Agreement:

Network disabling of HARQ via RRC configuration should be supported.

- FFS: Dynamic disabling of HARQ by gNB.

Agreement:

Evaluate impact of Satellite RTT when HARQ is enabled and potential solutions if needed

- At least the following aspects should be considered if the number of HARQ processes is > 16:

  - DCI size

  - HARQ soft buffer size

In this contribution, we mainly share our considerations on HARQ disabling and propose some potential HARQ enhancement mechanisms.

- Disabling of HARQ

HARQ is a combination of high-rate forward error-correcting coding and ARQ error-control, which can ensure the reliability of data transmission. In NTN, it is required to study the impact on NR HARQ operation due to the long RTT. According to the TR 38.811[2], as shown in table1 the minimum required number of the HARQ processes is different for different scenarios due to the height of the satellite.

Table 1: The minimum required number of the HARQ processes, , assuming a 1ms slot duration for 15 kHz* reference sub carrier spacing

| constellation | Max. | processes for 1 ms slot operation |
| --- | --- | --- |
| Terrestrial | 16ms | 16 |
| LEO | 50ms | 50 |
| MEO | 180ms | 180 |
| GEO/HEO | 600ms | 600 |

In NR, for the terrestrial networks, for downlink a maximum of 16 HARQ process per cell is supported by the UE. However, the long RTT in NTN will significantly increase the HARQ re-transmission latency and the HARQ process number. For example, assuming 15kHz reference sub carrier spacing, the minimum required number of the HARQ processes for GEO and LEO will be 600 and 50, respectively. In addition, the minimum required number of the HARQ processes will be scaled with the larger sub carrier spacing. Too many HARQ processes would significantly increase the soft buffer at the UE side.

In order to solve above problem, the HARQ disabling mechanism has been considered in RAN1. And the network disabling of HARQ via RRC configuration has been agreed. In terms of the dynamic signaling enabling/disabling of HARQ, from our perspective, we propose that it is also supported. Dynamic configuration with DCI allows gNB to configure HARQ function on/off flexibly depending on different traffic & use case.

From above discussion, we have the following proposal:

Proposal 1: Dynamic HARQ disabling via DCI is supported.

- The mechanism to guarantee the transmission reliability

According to RAN 2 #105 meeting discussion, the network can be able to configure the UE whether the HARQ is “turned off”[3]. This means there is no UL feedback for DL transmission if HARQ is turned off. It is obvious that, disabling HARQ feedback will increase the residual block error rate because HARQ feedback based re-transmissions are not allowed anymore. In order to guarantee the data transmission reliability some mechanisms should be considered. One way is to allow blind multiple transmission of the same transport block (TB) without waiting for the HARQ feedback. This mechanism can be configured through network configuration.

Note that, in Rel-15, multiple transmission of the same TB can be allowed in consecutive slots only. For NTN, considering the long propagation time, multiple transmissions in in-consecutive slots can be considered for achieving time diversity gain. For example, as shown in figure 1, the network can identify the number of repetitions of a TB based on the budget analysis, and select several slot to transmit the repetitive TB. Furthermore, the network can configure the slot interval to inform the multiple in-consecutive slots. Compared with repetition in consecutive slots, time diversity gain can be obtained, and the power consumption at the receiver can also be reduced as additional detection can be saved if the TB is detected successfully using the first transmission only.

Figure 1 The HARQ re-transmission mechanism

Hence, for the HARQ enhancement mechanism we have the following proposal:

Proposal 2: The multiple transmissions of same TBs in multiple in-consecutive slots can be considered when HARQ is disabled. And at least the following aspects should be included for the configuration:

- Repetition number

- The slot interval of repetition slots

The configuration of this scheme can be signaled via RRC signaling in a semi-static way. Besides, reusing HARQ related fields in DCI to indicate the repetition number and repetition slots can be considered also.

- Conclusion

In this contribution, we have presented some considerations on HARQ for NTN. The following proposal is given:

Proposal 1: Dynamic HARQ disabling via DCI is supported.

Proposal 2: The multiple transmissions of same TBs in multiple in-consecutive slots can be considered when HARQ is disabled. And at least the following aspects should be included for the configuration:

- Repetition number

- The slot interval of repetition slots

References

- 3GPP Final_Minutes_report_RAN1#97_v100.

- TR 38.811, Study on New Radio (NR) to support non terrestrial networks

- 3GPP Final_Minutes_report_RAN2#105_v100.