3GPP TSG RAN WG1 #98	R1-1908888

Prague, CZ, August 26th – 30th, 2019

Agenda item:    7.2.4.2.2

Source:			ITRI

Title:			Discussion on NR Sidelink Mode 2 Resource Allocation Mechanism

Document for:	Discussion and Decision

- Agreements from the RAN1 #97

At RAN 1 #97 [1], the resource allocation for NR sidelink mode 2 related agreements are shown as follows,

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

Agreements:

- RAN1 to further select between the following options of sidelink resource reservation for blind retransmissions:

  - Option 1: A transmission can reserve resources for none, one, or more than one blind retransmission

  - Option 2: A transmission can reserve resource for none or one blind retransmission

Agreements:

- Resource selection window is defined as a time interval where a UE selects sidelink resources for transmission

  - The resource selection window starts T1 ≥ 0 after a resource (re-)selection trigger and is bounded by at least a remaining packet delay budget

  - FFS T1 value, whether it is measured in slots, symbols, ms, etc.

  - FFS other conditions

Agreements:

- Support a sub-channel as the minimum granularity in frequency domain for the sensing for PSSCH resource selection

  - No additional sensing for other channels

- In this contribution, we provide our views on resource allocation for NR sidelink mode 2, including the issues left open

- in the preceding agreements.

- NR Sidelink Resource Allocation for Mode-2 operations

As mentioned in the RAN 1 #94 agreement [2], there are four types of NR sidelink resource allocation for mode-2 operation, it shows as follows:

    - UE autonomously selects sidelink resource for transmission

    - UE assists sidelink resource selection for other UE(s)

    - UE is configured with NR configured grant (type-1 like) for sidelink transmission

    - UE schedules sidelink transmissions of other UEs

For mode-2, UE determines the sidelink transmission resources within sidelink without BS or (pre-) configured sidelink resources by base station/network. Moreover, traffic patterns (periodic/aperiodic), BS (gNB and eNB) density, UE density, coverage, mobility, latency, reliability and Quality of Service (QoS) requirements need to be considered. The legacy sidelink resource allocation schemes may not be suitable to be reused for NR V2X mode 2.

In NR V2X, sidelink will support different requirements and different characteristics of traffic.

- Packets may vary in size

- Packet arrivals processes include periodic arrivals and aperiodic arrivals

Observation 1: In NR V2X, sidelink will support different requirements and different characteristics of traffic. For example, packet may vary in size and packet arrivals processes include periodic and aperiodic traffic arrivals.

LTE V2X resource sensing may not suitable to solve the aperiodic traffic. It may take 1000 (ms) for resource sensing and do not match the low latency for NR V2X. In addition, if all of the UE does the resource sensing and selection without any information from the base station or other UEs, it will cause a serious collision problem. Therefore, we need to redefine and redesign sidelink resource granularity and sidelink measurement for NR V2X mode 2.

  - Sidelink Resource Granularity for NR V2X Mode 2

NR V2X mode 2 supports the sensing and resource (re)-selection procedures according to the previously agreed definitions [3]. The resource granularity for sensing & resource (re)-selection is not decided yet. In the following section, we will distinguish the resource granularity into three parts and propose our views.

Agreements: (RAN1 Ad hoc 1901)

- Mode-2 supports the sensing and resource (re)-selection procedures according to the previously agreed definitions.

  - FFS resource granularity for sensing & resource (re)-selection, e.g., PRB(s), slots, resource patterns (when applicable), etc.

  - FFS detailed conditions when these procedures can apply

For NR sidelink, packet sizes vary dramatically even for the same application. The sidelink resource granularity for resource sensing and selection should be considered with different packet sizes.

- Physical resource block (PRB) may be related to different subcarrier spacing (SCS) and dynamic resource allocate the PRBs with different packet sizes.

- Slot based resource sensing and selection can also be considered. Slot can be allocated for all downlink, all uplink or mix downlink and uplink. Data transmission can be scheduled to span one or multiple slots. Moreover, slot aggregation may also be considered to support packet arrivals processes include periodic and aperiodic arrivals.

- Time-frequency resource pattern (TFRP) can be used for NR V2X resource sensing and selection. TFRP resource selection procedure will be based on the traffic types (periodic and aperiodic). In addition, TFRP ensures to mitigate the half-duplex impact and fulfill the stringent requirements if TFRP resources can be properly selected by UEs. For NR V2X mode 1 scenario, pattern selection can be fully controlled by gNB. For NR V2X mode 2, UE needs to get some assistance information from BS/Network or other UEs to enhance the pattern selection. The assistance information may be sidelink Reference Signal Receiving Power (SL-RSRP), sidelink Received Signal Strength Indication (S-RSSI), sidelink Reference Signal Received Quality (SL-RSRQ), geo-location information, and so on.

Proposal 1: For our point of view, the sidelink resource granularity for resource sensing and selection should be considered with different packet sizes.

  - Sidelink Measurement Content for NR V2X

- Agreements: (RAN 1 #96)

- Mode-2 sensing procedure utilizes the following sidelink measurement

  - L1 SL-RSRP based on sidelink DMRS when the corresponding SCI is decoded

    - FFS whether/which measurement is used if the corresponding SCI is not decoded e.g. SL-RSRP after blind DMRS detection, SL-RSSI

In RAN 1 #96 meeting [4], it was agreed to support the use of L1 SL-RSRP measurement of sidelink DMRS when the corresponding SCI is decoded. However, if the corresponding SCI is not decoded, whether or which measurement is not discussed yet. In our view, there are two alternatives when SCI is not decoded:

- Option 1: SL-RSRP after blind DMRS detection

- Option 2: SL-RSSI for sidelink measurement.

For option 1, it can be reused SL-RSRP after blind DMRS detection the same as the SCI is decoded. For option 2, it can easy to use SL-RSSI for sidelink measurement, especially in the interference-limit or noise-limit scenario. Therefore, Both SL-RSRP and SL-RSSI can be used for sidelink measurement.

Proposal 2: In our view, there are two alternatives when the SCI is not decoded. Option 1 is SL-RSRP after blind DMRS detection and option 2 is SL-RSSI for sidelink measurement.  For our point of view, we support to use both SL-RSRP and SL-RSSI for sidelink measurement.

Regarding the resource sensing criteria, we assume that sidelink measurement would be the one of baselines. Resource sensing could depend on sidelink measurements and link quality assessments such as S-RSRP, S-RSSI, S-RSRQ, and geographical location information.

- SL-RSRP: The average power is measurement of the power received from a single reference signal.  The sidelink resources of SL-RSRP would be DM-RS in either physical sidelink discovery channel or DMRS in physical broadcast channel.

- SL-RSRQ: RSRP is the average power of resource elements that carry cell specific reference signals over the entire bandwidth, so RSRP is the linear average of the downlink reference signals across the channel bandwidth. Similar to the legacy approach for RSRQ calculation using RSSI and RSRQ. The S-RSRQ can also be used.

- Geo-location information: Geo-location can be used to calculate the distance between Tx UE and Rx UE. The distance can also be used in measurement report.

In figure 1, each UE (periodic and aperiodic UE) will transmit the UE information to the gNB. The information includes the geo-location, traffic specific configuration (i.e. periodic and aperiodic traffic) or else. Furthermore, the measurement report contains the location indicator of the occupied resource, the overall resources bit mapping table or the overall resources measurement result. After the gNB gather the information from UEs, the gNB or road side unit (RSU) will broadcast the combination table to all of the UEs. Finally, the UE will select the idle resource based on the combination table.

Figure 1: Illustration of Sidelink measurement report

Proposal 3: Sidelink measurement for NR V2X mode 2 resource sensing and selection procedure may be important. The measurement report may receive from the base station (i.e. gNB) or the other UEs.

Proposal 4: The measurement report may include the UE location, traffic specific configuration (i.e. periodic and aperiodic traffic) or else.

2.3 Resource Sensing Procedure

Regarding the duration of resource sensing procedure, SCI decoding applied during sensing procedure is agreed in RAN1 #95 meeting [5] and RAN 1 # Ad hoc 1901 meeting [3]. In order to increase the resource utilization and reduce resource collision for NR V2X, long-term sensing may not be the only procedure for resource sensing.

Agreements: (RAN 1 # 95)

- Sensing procedure is defined as SCI decoding from other UEs and/or sidelink measurements

Agreement: (RAN 1 # Ad hoc 1901)

- SCI decoding applied during sensing procedure provides at least information on sidelink resources indicated by the UE transmitting the SCI

2.3.1 Hybrid Resource Sensing Procedure for Periodic and Aperiodic Traffic

In general, sidelink measurements and sidelink decoded information are used for identifying occupied or idle resources. The information for sidelink measurements include S-RSRP, S-RSSI, S-RSRQ, and geographical location information. The information discussed in detail in the previous section. Sidelink Information obtains from SCI decoding should include time/frequency resource allocation, resource reservation, and priority of the packet. As shown in figure 2, SCIs can be decoded by UE in each period time. UE have whole resource allocation information and resource reservation information of future period time. However, it depends on UE capability. Long-term sensing is mainly designed for periodic traffic and nearly perfect to solve resource allocation collision problem for LTE V2X.

The main problem is whether long term sensing can also nearly perfect to handle the periodic and aperiodic traffic resource allocation or not. The answer is no. There are many kinds of traffic types for NR V2X mode 2 and the aperiodic traffic comes instant and urgent. The resource allocation collision for NR V2X mode 2 will be more serious challenge than the LTE V2X mode 4. Therefore, in order to handle the collision problem for NR V2X mode 2, we support the hybrid resource sensing procedure for periodic and aperiodic traffic, which include the long-term sensing and short-term sensing procedure.

Proposal 5: For our point of view, we support the hybrid resource sensing procedure for periodic and aperiodic traffic, which include the long-term sensing and short-term sensing procedure.

There are several advantages of short-term sensing. One of the important advantages is short term sensing may reinforce the long term sensing to solve resource allocation collision problem. Moreover, short-term sensing is expected to provide degraded performance to handle aperiodic traffic. Therefore, we suppose short-term sensing can be merged with the long term sensing for sidelink resource sensing procedure.

Figure 2: Hybrid resource sensing type 1 scheme

Hybrid resource sensing type 1 scheme is shown in figure 2, long-term sensing is adopted similar with LTE V2X. The sensing period is 1000 (ms). After the trigger time, the periodic traffic UE will start to do the resource selection in the selection window. While the resource selection is in progress, the short term sensing can be held on at the same time. UE can decode the SCI or measure the energy to find out the idle resources. Besides, short term sensing can be based on the results of long term sensing. However, type 1 scheme may not suitable to apply the frequent occurring aperiodic traffic and it has less standard impact. Therefore, we propose a hybrid resource sensing type 2 scheme. As shown in figure 3, short term sensing can be done during the long term sensing. The aperiodic data comes when SCI decoding is completed or in progress, UE will do the SCI decoding at the next short term sensing. In conclusion, type 2 scheme can be more flexible to solve the periodic and aperiodic traffic coexistence and it can also achieve the goal of reducing resource collisions. However, the type 2 scheme will increase the complexity of the resource sensing procedure than the type 1 scheme, but it is a more efficient and low latency method. From our point of view, we support both hybrid resource sensing type 1 and type 2 scheme.

Figure 3: Hybrid resource sensing type 2 scheme

Proposal 6: From our point of view, we support both hybrid resource sensing type 1 and type 2 scheme.

2.4 Discussion on Resource Selection Window

Agreements:

- Resource selection window is defined as a time interval where a UE selects sidelink resources for transmission

  - The resource selection window starts T1 ≥ 0 after a resource (re-)selection trigger and is bounded by at least a remaining packet delay budget

  - FFS T1 value, whether it is measured in slots, symbols, ms, etc.

  - FFS other conditions

The use cases of latency requirements are summarized in the following table in 3GPP TR 22.886 [6], the latency requirements are varied from 3ms to 100ms. It is not feasible to realize the low latency requirement with Rel-14 V2X numerology and current UE’s capability.

Table I. The use cases of V2X in TR 22.886

|  | Latency | Reliability | Data rate | Communication range |
| --- | --- | --- | --- | --- |
| Platooning | 10 ~ 25 ms | 90 ~ 99.99% | 12 kbps ~ [65] Mbps | [5] ~ [10} sec * relative speed |
| Advanced driving | [3] ~ [100] ms | [99.99] ~ [99.999]% | 65 kbps ~ [50] Mbps ( DL [0.5] UL [50] Mbps) | [5] ~ [10} sec * relative speed |
| Extended sensors | 3 ~ 100 ms | 90 ~ 99.999% | 25 ~ 1000 Mbps | 50 ~ 1000 meters |
| Remote driving | 5 ~ [20] ms | [99.999]% | UL 25, DL 1 Mbps |  |

In the specification of Rel-14 V2X, UEs select resources in the selection window [n+T1, n+T2], where n is the packet arrival time variable at Layer 1 and T2 is between 20 ms and 100 ms and show in figure 4. For the minimum value of T2, the value lower than 20 ms is already included and can be used for shorter latency requirements in Rel-15. T1 value is set smaller or equal than 4 (ms) in Rel-14 and Rel-15 V2X. However, the latency requirement for NR V2X is stricter than the LTE-V2X and enhanced V2X. Therefore, we need figure out some solutions to adjust the T1 and T2 values.

Figure 4: LTE Rel-14 resource sensing and (re)-selection in sidelink transmission mode 4

In NR V2X, subcarrier spacing (SCS) can be one of the solutions. Compared to LTE-V2X, NR V2X can use multiple types of SCS. As shown in Table II, different SCS correspond to different slot duration. For example, when SCS is 15 KHz, the slot duration is 1ms; when SCS is 30 kHz, the slot duration is 0.5 ms. Therefore, T1 value can be reduced when adopting different SCS. It can be measured in slots with different slot duration.

Table II. Sub carrier spacing in TS 38.211 [7]

| SCS | PRB bandwidth | Frame duration | Subframe duration | Slots per frame | Slots per subframe | Slot duration | Symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 kHz | 180 kHz | 10 ms | 1 ms | 10 | 1 | 1 ms | 14 |
| 30 kHz | 360 kHz | 10 ms | 1 ms | 20 | 2 | 500 μs | 14 |
| 60 kHz | 720 kHz | 10 ms | 1 ms | 40 | 4 | 250μs | 14 |
| 120 kHz | 1.44 MHz | 10 ms | 1 ms | 80 | 8 | 125μs | 14 |
| 240 kHz | 2.88 MHz | 10 ms | 1 ms | 160 | 16 | 62.5μs | 14 |

There are many discussions in the past meeting for latency reduction with T2 value. Reduced T2 may lead to resource collision issue with smaller resource (re-)selection window size. Therefore, it may lead to resource collision issue more significant. For example, if T2 is set as 10ms, the candidate resource is halved compared to the case of T2 is set as 20ms. In order to mitigate the collision issue, UE with small T2 value also needs to have more candidate resources and/or candidate carriers. Therefore, under the cases of high ProSe Per-Packet Priority (PPPP) and/or small T2 value, more candidate resources and/or candidate carriers should be provided to mitigate collision issue in the Rel-15 meeting [8]-[11]. From our point of view, T2 value reduction can also consider adopting different SCS.

Proposal 7: T1 value can be reduced when adopting multiple types of SCS. It can be measured in slots with different slot duration. Furthermore, T2 value reduction can also consider adopting different SCS.

  - Reservation of Resources for Feedback-based HARQ Retransmission

Agreements:

- NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB

  - FFS impact on subsequent sensing and resource selection procedures

  - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported

    - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

    - FFS the behavior of the receiver UE(s) of this TB and other UEs

In the previous meeting, a consensus was reached that a transmission can be either reserved or not reserved, since supporting reservation helps resolve collisions for the initial transmission. It is also mentioned that an SL resource of an initial transmission of a TB can be reserved at least by an SCI associated with a different TB. Although the type of feedback is unclear for HARQ retransmission, it is foreseen that feedback-based HARQ retransmission is required to ensure the latency and reliability constraints. If resources for HARQ retransmissions can be reserved, it is more reliable for the reception of HARQ retransmissions. Therefore, we propose that an SCI reserves resources of multiple TBs and their corresponding HARQ retransmissions.

Proposal 8: An SCI can reserve resources of multiple TBs and their corresponding HARQ retransmissions

2.6.	Reservation of Resources for Blind Retransmission

Agreements:

- RAN1 to further select between the following options of sidelink resource reservation for blind retransmissions:

  - Option 1: A transmission can reserve resources for none, one, or more than one blind retransmission

  - Option 2: A transmission can reserve resource for none or one blind retransmission

There are two resource reservation methods, look-ahead reservation and chain reservation. Look-ahead transmission supports that an initial transmission can reserve multiple blind retransmissions; whereas chain reservation suggests that each transmission may reserve resource for one blind retransmission. Herein, we identify that look-ahead reservation is more reliable than chain reservation since more accurate sensing result is achieved for look-ahead reservation. However, the signaling overhead for look-ahead reservation could be large. Therefore, we suggest that a hybrid mechanism be adopted, in which the initial transmission reserves the resources for N blind retransmissions and chain reservation can be adopted right after the last blind retransmission reserved by look-ahead retransmission. From our viewpoint, we support option 1 for the sidelink resource reservation for blind retransmissions.

Observation 2: Look-ahead reservation and chain reservation pose a tradeoff between reliability and signaling overhead.

Proposal 9: A hybrid reservation approach is adopted, in which the chain reservation follows right after the last blind retransmission reserved by look-ahead retransmission. From our viewpoint, we support option 1 for the sidelink resource reservation for blind retransmissions.

- Geo-Based Resource Management for NR V2X

The mechanism of geo-based resource management techniques is similar to the radio resource sharing and reusing, i.e. reuse of radio resources at different geographical locations. For the NR V2X use cases, the geo-based resource management techniques may improve packet reception performance by reducing the impact from in-band emission and co-channel interference and reducing the latency and increasing the reliability. The in-band emission effect is one of the main limiting factors in vehicular deployments and thus it is desirable to utilize system-level approaches to reduce its negative impact. In addition, the geo-based resource management techniques can be utilized to substantially reduce the co-channel collision problem so that vehicles transmit on orthogonal resources improving interference conditions in certain range. The major idea of geo-based resource management scheme is that managing target vehicles to different geo-zones. Each geo-zone uses difference subset resource to reduce the interference impact between vehicles of different geo-zone. As a result, the interference of geo-zones apart from the others can be low. And it makes the benefit from the resource-reuse scheme possible.

From our observation, resource-reuse can improve the packet receive rate but it also increases the interference. Therefore, apropos dividing vehicles into the different groups to form a geo-zone can efficiently improve the location-based resource allocation. The major impacts of location-based resource allocation can be discussed in two parts, the intra-geo-zone interference and inter-geo-zone interference. The intra-geo-zone interference means that vehicles in the same geo-zone which use the same resource to transmit. The inter-geo-zone interference is that vehicles in different zone impact each other.

Observation 3: The geo-based resource management techniques can be utilized to substantially reduce the co-channel collision problem so that vehicles transmit on orthogonal resources improving interference conditions in certain range.

For out-of-coverage V2X service, the density of vehicle in a geo-zone will directly effect the intra-geo-zone interference. Higher density indicates that vehicles could have a higher probability in competing for the same resource at the same time, so that the intra-geo-zone interference will be increased. For in-coverage/part coverage V2X service, it can be seen that the intra-geo-zone interference does not exist because gNB can properly assign the resource. However, in the high-density scenarios, the resource may not be sufficient to support all the urgent transmission with a low latency requirement.

Observation 4: Geo-based resource management techniques require cross-layer interaction, hence that proper radio resources for vehicle transmissions are selected based on vehicle geo-position.

Proposal 10:  The mechanism to report geographical information to gNB should be FFS.

4.	Conclusion

In this contribution, we discussed resource allocation for NR V2X mode 2. The discussion and analysis lead to the following observations and proposals:

Observation 1: In NR V2X, sidelink will support different requirements and different characteristics of traffic. For example, packet may vary in size and packet arrivals processes include periodic and aperiodic traffic arrivals.

Proposal 1: For our point of view, the sidelink resource granularity for resource sensing and selection should be considered with different packet sizes.

Proposal 2: In our view, there are two alternatives when the SCI is not decoded. Option 1 is SL-RSRP after blind DMRS detection and option 2 is SL-RSSI for sidelink measurement.  For our point of view, we support to use both SL-RSRP and SL-RSSI for sidelink measurement.

Proposal 3: Sidelink measurement for NR V2X mode 2 resource sensing and selection procedure may be important. The measurement report may receive from the base station (i.e. gNB) or the other UEs.

Proposal 4: The measurement report may include the UE location, traffic specific configuration (i.e. periodic and aperiodic traffic) or else.

Proposal 5: For our point of view, we support the hybrid resource sensing procedure for periodic and aperiodic traffic, which include the long-term sensing and short-term sensing procedure.

Proposal 6: From our point of view, we support both hybrid resource sensing type 1 and type 2 scheme.

Proposal 7: T1 value can be reduced when adopting multiple types of SCS. It can be measured in slots with different slot duration. Furthermore, T2 value reduction can also consider adopting different SCS.

Proposal 8: An SCI can reserve resources of multiple TBs and their corresponding HARQ retransmissions

Observation 2: Look-ahead reservation and chain reservation pose a tradeoff between reliability and signaling overhead.

Proposal 9: A hybrid reservation approach is adopted, in which the chain reservation follows right after the last blind retransmission reserved by look-ahead retransmission. From our viewpoint, we support option 1 for the sidelink resource reservation for blind retransmissions.

Observation 3: The geo-based resource management techniques can be utilized to substantially reduce the co-channel collision problem so that vehicles transmit on orthogonal resources improving interference conditions in certain range.

Observation 4: Geo-based resource management techniques require cross-layer interaction, hence that proper radio resources for vehicle transmissions are selected based on vehicle geo-position.

Proposal 10:  The mechanism to report geographical information to gNB should be FFS.

5.	Reference

- [1] Chairman notes, 3GPP RAN1 #97, May 13th– May 17th, 2019.

- [2] Chairman notes, 3GPP RAN1 #94, August. 20th – 24th, 2018.

- [3] Chairman notes, 3GPP RAN1 Ad hoc meeting 1901, January. 21st – 25th, 2019.

- [4] Chairman notes, 3GPP RAN1 #96, February. 25th– March 1st, 2019.

- [5] Chairman notes, 3GPP RAN1 #95, November. 12th – 16th, 2018.

- [6] TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services (Release 16),” V16.2.0, 2018.

- [7] TR 38.211, “NR Physical channels and modulation,” V15.6.0, 2019.

- [8] R1-1719682, “Discussion on Latency Reduction for V2X Phase 2,” ITRI, Nov, 2017.

- [9] R1-1802080, “Considerations of Latency Reduction for V2X Phase 2,” ITRI, Feb, 2018.

- [10] R1-1804021, “Considerations of Latency Reduction between Packet Arrival and Resource Selection,” ITRI, April, 2018.

- [11] R1-1806819, “Remaining Issues on Latency Reduction for V2X,” ITRI, May, 2018.