3GPP TSG RAN WG1 #98		R1-1908969

Prague, USCZA, 26th – 30th August, 2019

Source:	Panasonic

Title: 	On inter UE Tx prioritization/multiplexing enhancements for NR URLLC

Agenda Item:   7.2.6.5

Document for:	Discussion

# Introduction

In 3GPP RAN1#97, following agreements related to inter UE UL cancellation are captured in [1]:

Agreements:

- Support at least group common DCI for cancelation indication

  - FFS whether or not to additionally support UE-specific DCI for cancelation indication

In RAN1#96bis, some other agreements with further open issues are captured in [2]:

Agreements:

- Upon detecting an UL cancelation indication, at least stop without resuming is supported

  - FFS whether and how to support stop with resume

Agreements:

- Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including

  - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS

  - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS

  - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS

  - PRACH

In this document, we discuss details related to the open issues for inter UE Tx prioritization/ multiplexing.

# UL cancellation scheme

UL cancellation mechanism

In RAN1#96bis, it has been already agreed to support stop without resume, meaning that once the previously scheduled is cancelled, it will not continue after the pre-empted resources and if needed, will be re-scheduled, as shown in Figure 1 below.

Figure 1.	Example of UL cancellation and stop

Now it for further study, whether additionally it is needed to support cancel and resume where the cancelled UL transmission resumes the remainder of the transmission after the pre-empted resources, as shown in Figure 2 below. In this case, the cancelled UL transmission is not received by gNB only on the pre-empted resources, the rest of the transmission is exactly the same. However, one of the key issues here is that the DMRS symbols of the cancelled UL transmission can be lost and there could be phase dis-continuity with the resumed part. Furthermore, a part of the transmission is lost.

Figure 2.	Example of eMBB UL cancellation and continue

Therefore, cancel and stop is rather straightforward operation and a new grant could be used to schedule remaining part of the cancelled UL transmission. We don’t see a strong motivation to additionally support cancel and resume.

Proposal 1: For NR URLLC UL in Rel. 16, cancel and resume scheme should not be supported.

UL cancellation indication and its reliability

Two potential high level solutions are possible for the indication of UL cancellation. First, PDCCH is used to indicate the UL cancellation and second, a sequence is used to indicate the UL cancellation. For PDCCH related indication, there are further two possibilities: UE-specific signalling or group-common signalling.

In RAN1#97, it has been agreed to support PDCCH based UL cancellation. In our thinking, for sequence based indication, more specification effort might be needed. It will be a new signal that would require additional resources, separate configuration and possibly additional monitoring in addition to PDCCH monitoring. In addition, because of the lack of CRC protection, to manage false-detection required level with good miss-detection performance is very difficult and sensitive to the variation of the interference level for the sequence based indication as discussed following section on the reliability. Therefore, sequence based indication shouldn’t be considered.

Proposal 2: In NR URLLC UL in Rel. 16, UE specific signalling for UL cancelation indication is not supported.

Another discussion point is to ensure that the UL cancellation indication is reliable enough to be received and successfully decoded by the UE. If the UL cancellation indication has insufficient reliability, then that particular UE continues with the transmission and possibly degrade the URLLC or higher priority transmission of other UE.

Observation 1: In NR URLLC UL in Rel. 16, in order to ensure the end-to-end reliability of 10E-6, the reliability of the UL cancellation indication should also satisfy similar reliability criteria.

However, as it is agreed to send UL cancellation indication with PDCCH, then the discussion/enhancements related to PDCCH in NR URLLC would be sufficient to ensure ultra-reliability for UL cancellation indication.

UE monitoring behaviour for UL cancellation indication

It has been agreed that the monitoring periodicity for the UL cancelation indication should be configurable by the gNB and UE supporting UL cancelation indication should be able to support more than one monitoring occasions for the UL cancelation indication in a slot. If PDCCH is used, whether the UE PDCCH monitoring capability (number of CCEs/BDs per slot) should be increased is ongoing discussion. As the monitoring periodicity can be configured by the gNB and indicated to the UE, therefore our thinking is that the increased monitoring is only configured when needed. Furthermore, when increased monitoring is configured so that UL cancellation indication can be detected by the UE, it might be reasonable to assume that other PDCCH might not be transmitted and possible the number of CCEs/BDs per slot wouldn’t need to be increased.

Observation 2: In NR URLLC  UL in Rel. 16, when PDCCH is used for UL cancellation indication, then it could be assumed that the gNB doesn’t schedule other PDCCHs in the monitoring occasion for the UE and therefore, increased number of CCEs/BDs per slot would not be necessary for the purpose of UL cancellation only.

Prioritization of signals/channels for selective cancellation

Based on the current agreements and status of discussion, the current cancellation mechanism can be described as follows:

- A certain group of UEs are configured to monitor PDCCH carrying GC DCI that contains information related to cancellation of the on-going transmission

- A high priority UE (possibly with URLLC traffic) is scheduled that may partially or completely overlap with the already scheduled or on-going UL transmission from other UEs

- gNB sends PDCCH with GC DCI for cancellation of the on-going UL transmissions – Only UEs that are configured to monitor GC DCI will respond and cancel any on-going transmissions, if they overlap either partially or completely with the time-frequency region indicated by the GC DCI, where the transmission is cancelled and not resumed

Figure 3 below shows an example of how the cancellation is done based on the current status.

Figure 3: Example of UL cancellation for inter-UE multiplexing

Based on the current agreements, it was agreed to simply cancel the on-going UL transmission upon detection of PDCCH with GC DCI with cancellation indication. Although, the intention is to make way for URLLC UL transmission and cancel all the low priority UL transmissions from the other UEs, but the current agreements/discussion doesn’t provide the mechanism on handling of on-going URLLC or high priority transmissions. Therefore, the issue is that even the URLLC transmissions or any other high priority transmissions would be cancelled. Some adjustment is needed to ensure that the URLLC (or any high priority traffic) for a UE that is configured to monitor GC-DCI for cancellation indication is not cancelled, only the low priority traffic is cancelled as shown in Figure 4.

Observation 3: In NR URLLC UL in Rel. 16, based on the current agreement, UEs that are configured to monitor GC-DCI for cancellation will always cancel any on-going transmission regardless of their priority level.

Figure 4: Example of selective UL cancellation (only lower priority cancelled) for inter-UE multiplexing

One possible approach could be that the priority level of the UL transmissions is known to at the PHY layer (via MAC) and this knowledge is used to selectively cancel the on-going UL transmissions in order to accommodate a higher priority UL transmission on the overlapping time-frequency region. For selective cancellation of only low-priority traffic (with respect to the new scheduled transmission of another UE), the GC-DCI should signal a priority indication. So all the UEs monitoring GC-DCI will check the indicated priority and compare with the priority of their currently scheduled or on-going transmission and it will be cancelled only if lower than indicated priority level.

Proposal 3: In NR URLLC UL in Rel. 16, GC-DCI for cancellation indication should also signal a priority indication to the UEs (that are configured to monitor GC-DCI for cancellation):

- to compare that with the priority of their scheduled or on-going transmission and only cancel it if this priority is lower than the indicated priority by GC-DCI.

Furthermore, in RAN1#96bis, it has been discussed that which channels could be cancelled by UL cancellation indication. Based on the information about the priority from MAC to PHY, each UL channel/signal such as PUSCH, PUCCH and SRS can be associated with URLLC or eMBB. If the priority of the already scheduled channels is lower than the later scheduled channels, then simply it should be allowed to cancel those channels. Therefore, the priority of channel shall prevail to decide whether cancellation is done or not. However, we think that PRACH should has the absolute priority as it is crucial to maintain the communication link and therefore, it should not be allowed to cancel PRACH from this UL cancellation indication.

Proposal 4: For NR URLLC UL in Rel. 16, channels with lower priority should be cancelled to allow the transmission of high priority channels

- Cancellation of PUSCH, PUCCH and SRS should be supported

- Cancellation of PRACH should not be supported

# Conclusion

Here we summarize the observations and proposals that have been presented in the sections above:

Observation 1: In NR URLLC UL in Rel. 16, in order to ensure the end-to-end reliability of 10E-6, the reliability of the UL cancellation indication should also satisfy similar reliability criteria.

Observation 2: In NR URLLC  UL in Rel. 16, when PDCCH is used for UL cancellation indication, then it could be assumed that the gNB doesn’t schedule other PDCCHs in the monitoring occasion for the UE and therefore, increased number of CCEs/BDs per slot would not be necessary for the purpose of UL cancellation only.

Observation 3: In NR URLLC UL in Rel. 16, based on the current agreement, UEs that are configured to monitor GC-DCI for cancellation will always cancel any on-going transmission regardless of their priority level.

Proposal 1: For NR URLLC UL in Rel. 16, cancel and resume scheme should not be supported.

Proposal 2: In NR URLLC UL in Rel. 16, UE specific signalling for UL cancelation indication is not supported.

Proposal 3: In NR URLLC UL in Rel. 16, GC-DCI for cancellation indication should also signal a priority indication to the UEs (that are configured to monitor GC-DCI for cancellation):

- to compare that with the priority of their scheduled or on-going transmission and only cancel it if this priority is lower than the indicated priority by GC-DCI.

Proposal 4: For NR URLLC UL in Rel. 16, channels with lower priority should be cancelled to allow the transmission of high priority channels

- Cancellation of PUSCH, PUCCH and SRS should be supported

- Cancellation of PRACH should not be supported

# Reference

[1]	3GPP RAN1#97, “Chairman Notes”

[2]	3GPP RAN1#96bis, “Chairman Notes”