3GPP TSG-RAN WG1 Meeting #98	R1-1908993

Prague, CZ, August 26th – 30th, 2019

Title:	Preliminary Calibration Results for NTN System-Level Evaluations

Source:		Nomor Research GmbH

Type:	Discussion

Document for:	Decision

Agenda Item: 	7.2.5.1 Link-Level and System-Level Evaluations

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

At RAN1#97 meeting in Reno, two parameter sets for NTN system-level calibration have been agreed and captured in [1].

The first part of this document captures further evaluation assumptions for Nomor’s system-level calibration. The second part of this document provides Nomor’s preliminary results for system-level calibration.

# Evaluation Assumptions on NTN System-Level Calibration

The parameter values or assumptions captured in Section 6.1.1 of [1] are applied for the following NTN system-level calibration results. Table 1 presents additional applied simulation assumptions.

Table 1 Simulation assumptions for NTN system-level calibration

| Attributes | Values or assumptions |
| --- | --- |
| Parameter set | Set-1 (Table 6.1.1-1 in TR 38.821) |
| Satellite orbit | GEO, LEO600 |
| Frequency band | 2GHz, 20GHz |
| Bandwidth per beam | 30MHz for f_c = 2GHz, 400MHz for f_c = 20GHz |
| Satellite constellation | Single satellite simulation |
| Wrap around method | Geographical distance based wrapping |
| UT attachment | based on RSRP |
| Ionospheric scintillation loss | Neglected (assume UE latitude between ±20° and ±60°) |
| Terrestrial scenario | Rural |

All three different options for frequency / polarization reuse listed in Table 6.1.1-5 of [1] are considered. In case of a frequency reuse factor larger than one, interference between two beams allocated to two distinct subbands is not modelled. In case of a polarization reuse factor of two, crosstalk between LHCP and RHCP signals is modelled.

# Preliminary Calibration Results

Table 2 lists the subset of study cases for which preliminary calibration results are provided. All calibration results are obtained for downlink transmission. The excel sheet attached captures the calibration results.

Table 2 List of study cases for which calibration results are provided

| Study Case | Satellite Orbit | Parameter Set | Central Beam Elevation [deg] | Terminal | Frequency Band | Frequency / Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 | VSAT | Ka-band | Option 1 |
| 2 | GEO | Set 1 | 45 | VSAT | Ka-band | Option 2 |
| 3 | GEO | Set 1 | 45 | VSAT | Ka-band | Option 3 |
| 4 | LEO-600 | Set 1 | 90 | VSAT | Ka-band | Option 1 |
| 5 | LEO-600 | Set 1 | 90 | VSAT | Ka-band | Option 2 |
| 6 | LEO-600 | Set 1 | 90 | VSAT | Ka-band | Option 3 |
| 10 | GEO | Set 1 | 45 | Handheld | S-Band | Option 1 |
| 11 | GEO | Set 1 | 45 | Handheld | S-Band | Option 2 |
| 12 | GEO | Set 1 | 45 | Handheld | S-Band | Option 3 |
| 7 | LEO-600 | Set 1 | 90 | Handheld | S-Band | Option 1 |
| 8 | LEO-600 | Set 1 | 90 | Handheld | S-Band | Option 2 |
| 9 | LEO-600 | Set 1 | 90 | Handheld | S-Band | Option 3 |

# References

- 3GPP TR 38.821 V0.7.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”