3GPP TSG RAN WG1 #98	                                                   	R1-1908997

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Wide-band operation for NR-U

Agenda item:	7.2.2.2.5

Document for:	Discussion and Decision

# Introduction

In RAN1#97 following agreements have been made [1]:

Agreement: When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst. - FFS: Signalling details of the indication, including e.g., the time domain validity of the indication - FFS: Whether and how to support the mechanism at the beginning of DL transmission burst - FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE Conclusion: A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16.

# Wide-band CORESETs

A majority of companies favor to have CORESETs which are confined within a LBT subband.

However, the actual mapping is up to gNB implementation. Nevertheless, increasing the number of CORESETs per BWP is discussed to enhance the reliability against LBT failures. This would enable the gNB to configure a UE with multiple CORESETs across different LBT subbands.

Even, if the LBT fails on a certain subband including a CORESET, the gNB can compensate by using the CORESETs in other subbands for scheduling. Although, the total number of blind decodes per slot is fixed and should not be increased, more CORESETs over time will lead to a higher overall blind decoding effort of the UE.

Hence, procedures to reduce the blind decoding effort should be considered. For example, skipping the blind decoding of CORESETs in subbands where LBT has failed would reduce the number of blind decodes on the UE at least in the case that some subbands are not used by the gNB due to LBT failures.

Proposal 1: Deactivate the blind decoding for CORESETs in subbands in which LBT has failed.

# PDSCH Enhancements for Wide-band Operation

Three options how to handle PDSCH transmission in wide-band operation have been identified:

- Option 1: Each PDSCH is mapped within a LBT sub-band or contiguous LBT sub-bands and the part of PDSCH is not punctured due to LBT outcome at gNB.

- Option 2: PDSCH can be spanned over multiple LBT sub-bands and the part of PDSCH is punctured if some of LBT sub-bands including PDSCH are not transmitted due to LBT failure at gNB.

- Option 3: gNB prepares for multiple sets of PDSCH mappings and selects one of them depending on LBT outcome.

Here, Option 1 cannot use the full potential of wide-band operation since it either does not allow to transmit a PDSCH due to a CCA failure on a subset of the LBT sub-bands or schedules small PDSCHs within the LBT sub-bands which increases the control overhead at least in the initial slot. Additionally, in the case CCA succeeds in all LBT sub-bands Option 1 takes a loss in data rate due to the small TBs into account. Hence, Option 2 and 3 should be preferred over Option 1. However, Option 3 requires more computational power and memory compared to Option 2. No specification work is required for Option 3. Additionally, Option 2 comes with the ease of less standardization efforts. CBG-based feedback can recover the punctured parts of PDSCH in an efficient manner. Nevertheless, Option 3 saves in the most cases a HARQ retransmission which is beneficial in terms of average latency and unnecessary HARQ operation, and should also be considered in addition to Option 2.

Proposal 2: Support Option 2 for PDSCH transmission.

# PUSCH enhancements for wide-band operation

In the last meeting, two alternatives for performing a PUSCH transmission in wide-band operation have been agreed:

- Alt1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

In the case of CAT1 LBT, obviously there is no issue to transmit in all LBT sub-bands scheduled by the PUSCH. Here, we assume that a UE is only scheduled in LBT sub-bands which are also actually used by the gNB COT. However, in the case of CAT2 LBT within the gNB COT or a CAT4 LBT outside the gNB COT, it is a valid question whether transmission of PUSCH has to be restricted to the case that CCA is successful in all LBT sub-bands of the scheduled PUSCH. Assuming a moderate amount of interference on a single LBT sub-band, Alt1 will cause a significant loss of efficiency. To make this more clear, even an interference on a single LBT sub-band, e.g. caused by IEEE802.11 a/h/n, will cause the NR-U UE to not use the spectrum at all. Hence, Alt2 should be preferred over Alt1, if supported by RAN4. Here, further restrictions, such as frequency contiguous allocations, also have to be provided by RAN4.

Proposal 3: Support Alt2 for wide-band PUSCH transmissions with CAT2 and CAT4 LBT, if supported by RAN4.

# Conclusion

In this contribution, we discussed enhancements to the NR-U wide-band operation.

Proposal 1: Deactivate the blind decoding for CORESETs in subbands in which LBT has failed.

Proposal 2: Support Option 2 for PDSCH transmission.

Proposal 3: Support Alt2 for wide-band PUSCH transmissions with CAT2 and CAT4 LBT, if supported by RAN4.

# References

- Chairman’s Notes, TSG RAN WG1 #97, May 2019.