3GPP TSG RAN WG1 Meeting #98	R1-1908998

Prague, CZ, August 26th – 30th, 2019

Source:	Ericsson

Title:	FL summary: agreements on Low PAPR RS

Agenda Item:	7.2.8

Document for:	Information

# Introduction

In this summary, the agreements made on Rel.16 NR_eMIMO Low PAPR RS to-date are compiled and systematically arranged, as requested by the WI Rapporteur. The main purpose is to identify missing parts, if any, as well as to aid specification editors. Therefore:

- For a given topic/component, the captured agreement represents the most mature form.

- Redundancies (including FFS items) are removed.

- The feature lead has inserted clarifying comments in red font, to increase understanding

# PDSCH and PUSCH using CP-OFDM waveform

- For PDSCH DMRS and PUSCH DMRS for CP-OFDM, DMRS enhancements are specified in Rel.16 to reduce the PAPR to the same level as for data symbols for all port combinations given by 38.212

  - For both DMRS type 1 and 2, the following cinitcinit for CDM group λ is used for Rel-16 DMRS sequence generation

  - nSCIDnSCID is provided by DCI (as in Rel.15)

  - λ is absolute CDM group index (as defined in Rel.15 TS 38.211)

  - The two N^(ID) are the same as in a Rel.15 DM-RS configuration

  - Note: The λ=0 CDM group have same cinit as in Rel.15

- The RRC parameters DMRS-DownlinkConfig and DMRS-UplinkConfig are extended to configure the alternative use of Rel. 16 sequence selection and behavior for the 2nd and 3rd CDM group for PDSCH and PUSCH respectively and for CP-OFDM only

- For CP-OFDM, if Rel.16 DMRS is configured by RRC, it only applies when scheduled by DCI formats 0_1 and 1_1 or with a PUSCH transmission with configured grant.

  - Note: It is up to the editor whether/how to capture this agreement

- Rel.16 DMRS is not supported for MSG3 transmissions, Rel.15 behavior is used for MSG3

- When scheduled by DCI format 0_0 with CSS, the new Rel-16 DMRS is not supported.

- For DCI format 1_0, SPS use port 0 and Rel-15 sequence applies

  - No specification impact necessary

# PUSCH and PUCCH using DFT-S-OFDM waveform

- For PUSCH/PUCCH DMRS for pi/2 modulation, new DMRS sequences are specified in Rel.16 to reduce the PAPR to the same level as for data symbols

- For sequences with allocation length 6,12,18 and 24 CGS is used for DMRS for π/2 BPSK modulation in case of PUSCH and PUCCH

- For length 12,18, and 24 respectively, NR Rel-16 supports the binary CGS in the Table C1, C2, and C3 respectively followed by pi/2 BPSK modulation followed by DFT as DMRS sequence for π π/2 BPSK modulation for both PUSCH and PUCCH.

  - Table C1, C2, and C3 can be found in R1-1901362

  - For the case of CGS sequence in case of two adjacent symbol DMRS, the same CGS sets are used for both single and two symbol DMRS for pi/2 BPSK modulation.

- When scheduled by DCI format 0_0 with CSS, the new Rel-16 DMRS is not supported.

## Agreements specific of DMRS for PUCCH

- Support PUCCH DMRS enhancements for PUCCH format 3 and 4

- When Rel-16 DMRS sequence for pi/2 BPSK PUCCH Format 3 and Format 4 is configured, the Rel-16 sequence is used for all sequence lengths.

- Rel-16 DMRS sequence for pi/2 BPSK PUCCH Format 3 for length >30 is determined according to the following: the Gold sequence shall be initialized in the same way as for the DMRS for PUCCH format 2 in NR Rel-15, i.e.,

  - If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, scramblingID0 is obtained from dmrs-UplinkForPUSCH-MappingTypeB.

  - For sequences with length 30 or larger, DMRS for π/2 BPSK modulation for PUCCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding.

- The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

- The group and sequence hopping procedures in section 6.3.2.2.1 of TS38.211 in Release 15 are applied to Rel-16 DMRS sequence for pi/2 BPSK PUCCH Format 3 for length ≤ 30.

## Agreements specific for DMRS for PUSCH

- When Rel-16 sequence for pi/2 BPSK PUSCH DMRS is configured, the Rel-16 sequence is used for all sequence lengths.

- Rel-15 NR DM-RS Type 1 mapping is used for the DM-RS sequences of PUSCH transmission with π/2-BPSK modulation.

  - Applies for Rel-16 for all sequence lengths

- For sequences with length 30 or larger, DMRS for ππ/2 BPSK modulation for PUSCH is generated based on Gold-sequence followed by π/2 BPSK modulation followed by transform precoding resulting in a DMRS Type 1 comb structure.

- If Rel-16 DMRS is configured by RRC for PUSCH with pi/2 BPSK modulation, the Rel-16 DMRS applies when scheduled by DCI formats 0_1 and 0_0 and using at least C-RNTI, CS-RNTI, SP-CSI-RNTI and MCS-C-RNTI, when applicable or PUSCH transmission with configured grant

  - Search space for DCI format 0_0: Rel-16 DMRS for at least USS is supported

- Rel.16 DMRS is not supported for MSG3 transmissions, Rel.15 behavior is used for MSG3

- For DCI format 1_0, SPS use port 0 and Rel-15 sequence applies

  - No specification impact necessary

- The PUSCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUSCH is up to two ports for one OFDM symbol, four ports (TD-OCC across OFDM symbols) for two OFDM symbols.

- For PUSCH DMRS with pi/2-BPSK modulation, two RRC parameters ScramblingID0 and ScramblingID1 can be configured by RRC as for CP-OFDM DMRS (16 bit per ID)

  - Used in cinit for length ≥30 as n_ID^{n_SCID=0}, n_ID^{n_SCID=1}

    - Cell ID default scrambling parameter(s) unless configured otherwise

    - Use c_init formula from Rel.15 CP-OFDM DMRS and reuse Rel-15 Gold sequence generator

  - For length<30, these parameters are used as N_ID^PUSCH is configured as either ScramblingID0 or ScramblingID1 depending on DCI

  - DCI indicates n_SCID={0,1} dynamically, without increasing DCI size to select the above parameter

    - Antenna port indication is re-used for n_SCID indication as follows:

      - DMRS port 1000,1002,1004 and 1006 are interpreted as port 1000,1002,1004 and 1006, respectively, with n_SCID=0

      - DMRS port 1001,1003,1005 and 1007 are interpreted as port 1000,1002,1004 and 1006, respectively, with n_SCID=1

- For initialization of Rel-16 π/2-BPSK modulated DM-RS using time domain Gold sequences, the OFDM symbol number l in the cinit formula for two-adjacent symbol DM-RS (maxLength=2) should correspond to the OFDM symbol number of the first DM-RS symbol of the pair of adjacent symbols

- For length 6 CGS; 8-PSK is used, the sequences are given by the following table.

- Table 1 Length 6 CGS

| u |  |
| --- | --- |
| 0 | -1 -7 -3 -5 -1 3 |
| 1 | -1 3 7 -3 7 3 |
| 2 | -1 3 1 5 -1 -5 |
| 3 | -7 -3 -7 5 -7 -3 |
| 4 | 7 5 -1 -7 -3 1 |
| 5 | 3 -3 1 5 -1 -1 |
| 6 | -7 -3 -7 -3 7 -5 |
| 7 | -7 -3 1 -5 -1 -5 |
| 8 | -7 -3 3 -3 -7 -3 |
| 9 | -7 -7 -1 1 -5 1 |
| 10 | -7 -3 -7 5 -1 5 |
| 11 | -7 -7 -3 1 5 -1 |
| 12 | 5 7 -3 -5 5 -5 |
| 13 | -3 7 -5 -1 -5 -1 |
| 14 | 5 -7 7 1 5 1 |
| 15 | -7 3 1 5 -1 3 |
| 16 | -7 -5 -1 -7 -5 5 |
| 17 | -7 1 -3 3 7 5 |
| 18 | -7 -7 3 5 1 5 |
| 19 | -7 -3 3 -1 3 -5 |
| 20 | -7 -5 5 3 -7 -1 |
| 21 | 1 5 1 5 3 7 |
| 22 | 1 -3 1 -5 -1 3 |
| 23 | 1 7 1 -5 -7 -1 |
| 24 | 1 -1 3 -1 -7 -3 |
| 25 | 1 -1 -5 -1 3 -3 |
| 26 | 1 -1 3 -1 3 7 |
| 27 | -5 3 7 5 3 7 |
| 28 | -7 1 -3 1 5 1 |
| 29 | 1 5 3 -7 5 -3 |