3GPP TSG-RAN WG1 #98	R1- 1909025

Prague, Czech Republic, August 26th– 30th, 2019

Agenda Item:	7.2.3.2

Source:	Ericsson

Title:	Preamble Format of IAB-specific RACH Configuration

Document for:	Discussion

# Introduction

In the RAN1 #97 Meeting, the following agreements on the extension of RACH occasions and periodicities were reached [1].

Agreements: The introduction of scaling factor for PRACH configuration period also scales the SSB-RACH association period and association pattern period accordingly. Assuming the scaling factor to be λ, for IAB nodes table 8.1-1 of 38.213 gets modified to the following: Mapping between scaled PRACH Configuration Period (x * λ) and SSB to PRACH Occasion Association Period Scaled PRACH configuration period (ms); Association period. 10; {1, 2, 4, 8, 16, 32, 64}. 20; {1, 2, 4, 8, 16, 32}. 40; {1, 2, 4, 8, 16}. 80; {1, 2, 4, 8}. 160; {1, 2, 4}. 320; {1, 2}. 640; {1}. The definition of the SSB-RACH association pattern period for IAB nodes should be updated as follows: An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec. Agreements: The signalling design for the IAB specific backhaul RACH configurations is up to RAN2. RAN1 has the following recommendations: - The ability to configure the IAB specific backhaul RACH configuration additionally to the Rel 15 RACH configuration for access UEs is supported. - If the IAB specific RACH configuration is not provided, RAN1 assumes that IAB node will use the configured Rel 15 RACH configuration for IAB node initial access. - Capability to configure IAB specific backhaul RACH configurations need to be provided for both CBRA & CFRA.

In this contribution, we want to ask for clarification about preamble formats and signaling of IAB-specific RACH.

# Clarifications on the Preamble Format of IAB-specific RACH Configuration

As one of the conclusions, the work item has confirmed that there will be IAB-specific RACH configurations. The discussions, however, have mainly focused on the specification of the extension of RACH occasions and periodicities. On the other hand, TR 38.874 [2] concludes to recommend having enhancements to support additional preamble formats for PRACH allowing for longer RTT, without impacting Rel-15 UEs.

For this reason, we believe and like to have the confirmation that the IAB-specific RACH configurations may also include a different preamble format, compared to the RACH configuration to be used by Rel-15 UEs for initial access in the same cell.

IAB-specific RACH configurations can have a different preamble format, compared to the RACH configuration to be used by Rel-15 UEs for initial access in the same cell.

In case the IAB-specific RACH configuration has the possibility for alternative preamble formats, i.e. different from the preamble format for Rel-15 UEs to be used for initial access, a typical situation would be to support larger range for the IAB access by the use of a longer preamble. Therefore, the IAB-specific RACH configuration including an alternative preamble format should also be broadcasted, preferably via SIB1 or a new IAB-specific SIB (to be determined by RAN2).

Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is broadcasted.

Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is included in SIB1 or a new IAB-specific SIB (to be determined by RAN2).

# Conclusion

In this paper, we propose the following:

Proposal 1	IAB-specific RACH configurations can have a different preamble format, compared to the RACH configuration to be used by Rel-15 UEs for initial access in the same cell.

- Proposal 2	Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is broadcasted.

- Proposal 3	Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is included in SIB1 or a new IAB-specific SIB (to be determined by RAN2).

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97.

3GPP TR 38.874 V16.0.0, Study on Integrated Access and Backhaul.