3GPP TSG-RAN WG1 #98		R1-1909043

Prague, Czech Republic, August 26th – 30th, 2019

Source: 	Apple Inc.

Title:                     Enhancement to initial access procedure for NR-U

Agenda item:	7.2.2.2.2

Document for:	Discussion and Decision

# 1 Introduction

At the RAN1 #97 meeting, good progress was made on enhancements to initial access procedure and the following agreements were made [1]:

Agreement: The mechanism to determine serving cell timing is as follows: - The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index - 10-bits SFN and half-frame indicator are indicated as in Rel-15 - PBCH payload size is not increased compared to Rel-15 - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination - If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE Agreement: LBT category for msg 3 initial transmission is provided to the UE in RAR Agreement: Reply to the RAN2 LS informing them of the following: - RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3: - LBT category for msg 3 initial transmission is provided to the UE in RAR - Multiple msg3 tx opportunities with a single or multiple RARs in the time domain is feasible from a RAN1 perspective but there is no consensus at this time in RAN1 to support this. RAN1 will continue discussions on the support of multiple msg3 tx opportunities. Agreement: For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE - FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index - FFS: How Q is indicated or determined - FFS: Restriction on the range of Q. Note: Neighbor cell RRM measurements will be addressed separately Agreement: For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window. - Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window - FFS: Whether and how DRS transmission window is configured to the UE

This contribution studies different FFS aspects of initial access procedure enhancements for NR-U and presents our views.

# 2. Discussion

## 2.1 Initial access procedure

QCL determination

In the RAN1 #97 meeting, it was agreed to reuse the Rel-15 framework to determine the frame timing based on the decoded SSB index (combination of 8 PBCH DMRS sequence and 3 values (2-bit) in PBCH payload) and the half-frame indicator. With regard to QCL relation between detected SSBs across DMRS windows, modulo (A, Q) function is agreed to determine the mapping between SSBs in different location and QCL relation (i.e. beam index). However, several FFS aspects remain open, including:

- The definition of A

- The mechanism to signal Q value

- Whether need to put some restrictions on the candidate values for setting the Q value?

Due to LBT requirement, the following was agreed in the RAN1 #96bis [2] to extend the number of SS/PBCH blocks candidate positions to Y = 20 for 30kHz and Y = 10 for 15kHz, respectively, to provide sufficient transmission opportunities for SS/PBCH blocks:

Agreement: The maximum DRS transmission window duration is 5 ms. - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS. - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions. Agreement: UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

Given the fact that the SSB index can be larger than 8 but the number of DMRS sequence is up to 7, it seems obvious to define A as the SS/PBCH block candidate index detected based on DMRS sequence and 3 LSB payload. In addition, Q denotes the maximum number of candidate beams operated by network. Regarding the range of Q value, we are supportive to extend the Rel-15 design to provide more flexibility together with feasible signaling overhead, i.e. restricting Q to 1,2,4, and 8.

Another FFS aspect related to QCL determination is how to signal the Q value. In general, modification on MIB content and especially increasing the payload should be avoided due to the backward compatible issue. As discussed in the past meeting, Q value is used to determine the associated RACH resources during the initial access procedure. It is therefore reasonable to signal the value of Q in the SIB1 information.

Based on the discussions above, we made the following proposal:

Proposal 1:

- Define ‘A’ as the SS/PBCH block candidate index for modulo (A, Q) function.

- The value of Q can be 1,2, 4 or 8 and indicated in SIB1

## 2.2 Random access procedure

In Rel-15 NR, the timing association between the SS/PBCH block and the PRACH transmission occasion is configured by SIB1 information. This allows network to derive the best downlink beam for the corresponding device based on the received preamble. On the other hand, fixed timing relative to SS/PBCH block may defer the PRACH transmission for NR-U due to LBT failure. Thus, it is beneficial in terms of PRACH transmission robustness to configure multiple PRACH resources across multiple LBT sub-bands. The best SS/PBCH block can be indicated by which frequency occasion or preamble the UE transmit Msg1 in.

Proposal 2:

- Support configuring multiple PRACH resources across multiple LBT sub-bands/CC for Msg1 transmission.

# 3. Conclusion

In this contribution, we discussed several FFS aspects related to initial access procedure enhancement on NR unlicensed band. Based on the discussions above, we made the following proposals:

Proposal 1:

- Define ‘A’ as the SS/PBCH block candidate index for modulo (A, Q) function.

- The value of Q can be 1,2, 4 or 8 and indicated in SIB1

Proposal 2:

- Support configuring multiple PRACH resources across multiple LBT sub-bands/CC for Msg1 transmission.

# References

- RAN1 Chairman’s Notes, RAN1 #97, May 2019

- RAN1 Chairman’s Notes, RAN1 #96bis, May 2019