3GPP TSG-RAN WG1 #98		R1-1909045

Prague, Czech Republic, August 26th – 30th, 2019

Source: 	Apple Inc.

Title:                     On enhancements for configuration grant transmissions

Agenda item:	7.2.2.2.4

Document for:	Discussion and Decision

# 1 Introduction

At the RAN1 #97 meeting, the following agreements were made on enhancement for configured grant (CG) transmissions [1]:

Agreement: For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured - FFS: How to indicate multiple PUSCHs within a slot. Agreement: UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

In this contribution, we provide our views on the remaining issues and potential solutions to enhance configuration-grant-based UL transmissions for NR-U.

# 2. Discussion

## 2.1 Time domain resource allocation

With latest agreements, it is possible to configure CG resources in consecutive slots without restriction of 1 slot CG periodicity setting by leveraging new offset parameter. One of FFS aspects for time domain resource allocation is how to indicate multiple PUSCHs within a slot to increase the possibility of CG-based UL transmission after LBT operation.

Figure 1: Multiple time domain resource allocations in a slot

We are supportive of enabling multiple PUSCHs within a slot to improve resource utilization of NR-U. This can be simply achieved by extending the slot-level signaling framework to additionally indicate the number of sub-slot allocations within the first of the consecutive slots. It should be noted that Rel-15 NR supports CG resource allocation not only slot-based but also on a sub-slot granularity i.e. 2-sybmols or 7-symbols. This can be reused for sub-slot resource indication without any design changes except limiting to the first slot. Figure 1 provides one example to configure two Type-B PUSCH transmission opportunities within a first slot of CG-PUSCH resources. Starting from the 2nd slot of COT, it is beneficial from resource utilization perspective to switch back to slot-based transmission.

Proposal 1:

- Support additional RRC configured sub-slot time domain resource allocation on top of slot-level resource allocation by indicating one of predefined length of sub-slot, e.g. 2-symbols or 7-symbols.

- UE may switch back to slot-based transmission after channel access is obtained.

## 2.2 On HARQ related operations

### 2.2.1 CBG-based retransmissions

One of FFS aspects for HARQ operation of NR-U design is how to support CBG-based retransmission. The following was agreed in RAN1 #96 meeting regarding retransmission for CBG-based PUSCH transmission.

Agreement: For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant. - FFS: CBG-based retransmission using a configured grant - Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

It remains open regarding whether or not to limit the CBG-based retransmission to be supported by using a dynamic UL grant.

CBG-based retransmission was defined in Rel-15 to enable HARQ operation on a per CBG basis instead of TB. With this approach, only the erroneously received CBGs are retransmitted and hence consumes significantly less resources than retransmitting the whole TB. This benefit of CBG-based retransmission cannot be explored by CG-based PUSCH transmission due to the preconfigured physical resources in time and frequency domain. In addition, it conveys CBG-based HARQ-ACK information in DFI format for each PUSCH transmission and substantially increase the DCI format size and signaling overhead. Furthermore, the UCI size also becomes larger to indicate the CBGTI-like information for each PUSCH. Therefore, based on the above considerations, we believe in most cases of CG-based PUSCH transmission support CBG-based retransmission cannot bring any practical gain, while introducing additional signaling overhead. As a consequence, we propose the following:

Proposal 2:

- Not support CBG-based retransmission using a UL configured grant.

### 2.2.2 DFI content

During the study item phase, it was concluded that support Downlink Feedback Information (DFI) is beneficial to indicate the pending HARQ-ACK feedback for prior CG-based transmission from the same UE [2]. In addition, it was agreed to support timer-based non-adaptive retransmission for CG-based PUSCH transmission of NR-U to improve the reliability. However, the details of DFI format and content remain open. We believe it is sufficient to use bit-map signaling to indicate the HARQ-ACK information associated with each HARQ process.

Proposal 3:

- Using bit-map based approach in DFI to indicate the HARQ-ACK information for each HARQ process.

Another FFS aspect related to DFI signaling is which DCI format is used to convey DFI information. As usual, the DFI DCI format should be size-aligned with one of other scheduling DCI so as to avoid increasing blind decoding attempts. NR supports up to 16 HARQ process for CG-PUSCH, which means the maximum payload size of DFI field for a single UE is up to 16 bits. To maximize the utilization of DCI format, it is beneficial to introduce a DCI format targeting to a group of UE and each UE is configured with separate DFI field for HARQ-ACK information indication, similar like DCI format 2-2 in NR.

Proposal 4:

- Consider using group-specific DCI format to convey DFI information targeting to a group of UEs.

- The group-specific DCI format is size-aligned with other Rel-15 UL grant formats to keep the same number of BDs as in Rel-15.

## 2.3 UCI content

The following was agreed in the past meetings [3] with regard to UCI contents:

Agreement: CG-UCI should at least include the following information: - HARQ ID - NDI - RV - COT sharing information, FFS details - FFS: other information including UE ID Agreement: When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following - The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE - FFS: - How the duration is signalled - Whether the UE should signal continued use of the COT for its own transmissions - LBT priority class

We believe the LBT priority class should be included in UCI content to let network know which priority class was used by UE to acquire this shared COT. Correspondingly, only DL-SCH with a same or higher priority is allowed to use this shared COT to ensure fair coexistence with incumbent systems, like Wi-Fi. Hence, we propose the following:

Proposal 5:

- Support to include the LBT priority class into UCI contents.

UE may access the channel after LBT outside of gNB-initiated COT before CG-PUSCH transmission but still have data in buffer after CG-PUSCH transmissions. Hence, it is useful to also allow gNB to use the shared UE-initiated COT to continuously schedule the same UE. This can be achieved by including SR bit into UCI contents. We therefore propose the following:

Proposal 6:

- Support to include the SR information into UCI contents to indicate the request of continuous PUSCH scheduling.

# 3. Conclusion

In this contribution, we discussed several FFS aspects related to CG-PUSCH transmissions on unlicensed band. Based on the discussions above, we made the following proposals:

Proposal 1:

- Support additional RRC configured sub-slot time domain resource allocation on top of slot-level resource allocation by indicating one of predefined length of sub-slot, e.g. 2-symbols or 7-symbols.

- UE may switch back to slot-based transmission after channel access is obtained.

Proposal 2:

- Not support CBG-based retransmission using a UL configured grant.

Proposal 3:

- Using bit-map based approach in DFI to indicate the HARQ-ACK information for each HARQ process.

Proposal 4:

- Consider using group-specific DCI format to convey DFI information targeting to a group of UEs.

- The group-specific DCI format is size-aligned with other Rel-15 UL grant formats to keep the same number of BDs as in Rel-15.

Proposal 5:

- Support to include the LBT priority class into UCI contents.

Proposal 6:

- Support to include the SR information into UCI contents to indicate the request of continuous PUSCH scheduling.

# References

- RAN1 Chairman’s Notes, RAN1 #97, May 2019

- 3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum”, v16.0.0, December 2018

- RAN1 Chairman’s Notes, RAN1 Ad Hoc 1901, January 2019

- RAN1 Chairman’s Notes, RAN1 #96, Feb. 2019