3GPP TSG RAN WG1 #98	                               				R1-1909049

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.8.4

Source:	Apple Inc.

Title:	Remaining Issues on Uplink Full Power Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #97 meeting, the following working assumptions on uplink full power transmission have been achieved. [1]

Working Assumption Support following scheme for UL full power Tx for UE capability 2 and 3: - A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability - A UE can be configured by the network to support full power transmission - Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’ - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission. - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15 - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported - FFS: Additional support of antenna selection TPMI precoders - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI - Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’ - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not - Rel-15 codebooks and codebook subsets are used - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE. - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least UEcap3, for SRS resource with more than 1 ports, - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects - The following cases are not precluded - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1] - FFS: number of SRS resources supported - 2 - 3 - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power - FFS: any rules for spatial filter update for the SRS resources with different number ports Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD Note: For single port, there is no SRI and TPMI Note: Support of Mode 1, Mode 2 have separate UE capability

In this contribution, we provide some discussion on remaining issues on mode 1 and mode 2 to support uplink full power transmission.

# Discussion

## Remaining issues on mode 1

For mode 1, the key issue is how to define the new codebook subset. According to the working assumption, this new codebook subset should include at least one precoder for coherent transmission, e.g. precoder 1211. Then one open issue is whether to include all precoders or only one of them. This could be related to the maximum phase offset between the SRS transmission and PUSCH transmission. If this phase offset could be small, to include all precoders for coherent transmission could be helpful for gNB to select the coarse direction for the transmission. On the other hand, if the phase offset is too large, it is hard for gNB to decide which precoder to select. The range of phase offset could be determined by UE implementation, which could be different for different UE. Therefore, which antenna combining based precoders should be selected should be based on UE capability.

Further, for capability 3 UE, it may be able to support full power transmission for some antenna selection based precoder, e.g. precoder 1210, which is cannot support full power transmission for the others. So whether to include antenna selection precoder or not should be based on UE capability.

Based on the discussion above, which precoder(s) should be included in the new codebook subset for mode 1 should be based on UE capability.

Proposal 1: Which precoder(s) should be included in the new codebook subset for mode 1 should be based on UE capability.

## Remaining issues on mode 2

For mode 2, one open issue is whether to support more than 2 SRS resources. It has been agreed that different number of ports can be configured for different SRS resources. Then for a 2 ports UE, gNB can configure 2 SRS resources: a 1-port SRS resource as well as a 2-port SRS resource. Then if 1-port SRS resource is indicated, UE can support full power transmission by antenna virtualization. If 2-port SRS resource is indicated, UE can support full power transmission for some precoders, e.g. rank 2 transmission or one antenna selection based precoder (for capability 3 UE only).

For 4 ports UE, one possible way is to configure multiple SRS resources with different number of ports, e.g. 4 SRS resources with different ports to support full power transmission for some capability 3 UE, e.g. UE with PA architecture like 20+20+17+17dBm. However this would increase the system overhead. To support that type of UE, another possible way is not to increase the maximum number of SRS resources, instead, gNB can configure a 1-port SRS resource as well as a 4-port SRS resource. UE can report the precoders that can support full power transmission when 4-port SRS resource is indicated. Then for full power transmission, gNB can indicate corresponding precoders. Further most likely full power transmission is required for cell edge UEs, therefore for full power transmission, in most cases, rank 1 transmission would be applied. Therefore, 1 port SRS should be indicated. It is not necessary to increase number of SRS resources.

Proposal 2: Do not support to increase number of SRS resources.

Another open issue is whether to define any rules to update the spatial relation info for SRS with different ports. Currently the spatial relation info update for SRS with different usage could be based on MAC CE or RRC signaling. It is not necessary to define an additional rule to update spatial relation info for SRS resources for codebook when different number of ports is configured.

Proposal 3: Do not support to introduce additional rule to update spatial relation info for SRS resources with different number of ports.

# Conclusion

In this contribution, we discussed the remaining issues for uplink full power transmission. Based on the discussion, the following proposals have been made:

Proposal 1: Which precoder(s) should be included in the new codebook subset for mode 1 should be based on UE capability.

Proposal 2: Do not support to increase number of SRS resources.

Proposal 3: Do not support to introduce additional rule to update spatial relation info for SRS resources with different number of ports.

# Reference

- Chairman notes, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, 13th – 17th, May 2019