3GPP TSG-RAN WG1 #98		R1-1909055

Prague, Czech Republic, August 26th – 30th, 2019

Source: 	Apple Inc.

Title:                     On UCI enhancements for NR URLLC

Agenda item:	7.2.6.2

Document for:	Discussion and Decision

# 1 Introduction

During RAN1#97 meeting [1], the following was agreed for UCI enhancements for NR URLLC:

Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot. Agreements: For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s) - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot. Agreements: - When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following: - FFS: For PUCCH-SpatialRelationInfo - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback. - FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList. - FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration. Working assumption: Support that SR priority (e.g. high or low priority) is known at PHY layer. - FFS how to use the priority information in handling prioritization/multiplexing of UL transmissions. - FFS how the SR priority is known

In this contribution, we discuss the open issues related to UCI collision enhancements and provide possible solutions on different designs aspects, such as enabling multiple PUCCHs transmission within a slot and handling UCI collision handling in intra-UE prioritization.

# 2. Discussion

## 2.1 Separate HARQ-ACK codebook constructions

In the past meetings [2][3], the following was agreed to achieve efficient HARQ-ACK feedback in case of mixed eMBB and URLLC services for a given UE:

Agreements: - For a R16 UE, at least two HARQ-ACK codebooks can be simultaneously constructed, intended for supporting different service types for a UE - FFS more details (including procedures when applicable) - FFS: How to identify a HARQ-ACK codebook - FFS applicability to semi-static HARQ-ACK codebook, or dynamic HARQ-ACK codebook, or both - FFS more than 2 - FFS whether or not CBG configuration is supported for Rel-16 URLLC Agreements: - Rules for the two HARQ-ACK codebooks for supporting different service types should be specified in R16 if the two HARQ-ACK codebooks are due to transmit in resources overlapping in time - FFS details, e.g., multiplexing and/or prioritizing or parallel tx – revisit later this week Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, a HARQ-ACK codebook can be identified based on some PHY indications/properties. - FFS in potential WI the details of the PHY identification

One of remaining FFS aspects is how UE determines the corresponding HARQ-ACK codebook when it receives mixed eMBB and URLLC PDSCH. Given the significantly different reliability and latency requirement, we believe it is desirable to separate the HARQ-ACK codebook for eMBB and URLLC PDSCH transmission to provide the respective reliability requirements with minimized control overhead. This can be achieved by explicitly indication by means of DCI format that schedules PDSCH transmissions. This indicated value should be used to determine the unit of K1 value, i.e. sub-slot or slot based. With regard to SPS PDSCH, the HARQ-ACK codebook can be simply configured as part of RRC configuration as it is mainly determined by the associated service type i.e. eMBB or URLLC. Rel-15 HARQ-ACK procedure can be reused for each HARQ-ACK codebook independently.

Proposal 1:

- Consider to explicitly indicate the associated HARQ-ACK codebook index by DCI that schedules PDSCH transmission.

- For SPS PDSCH, the HARQ-ACK codebook is indicated as part of RRC configuration.

There are several RRC configuration aspects related to two HARQ-ACK codebooks transmission such as whether to allow separate PUCCH-SpatialRelationInfo. The PUCCH-SpatialRelationInfo is used to configure the spatial setting for PUCCH transmission and the parameters for PUCCH power control, including referring serving cell ID, reference signal for path loss measurement, reference for spatial information (i.e. SSB or CSI-RS etc.) and power control parameters (i.e. P0, close loop index). Fundamentally, the reliability requirements can be hugely different depending on the associated service type, i.e. eMBB or URLLC. It is therefore beneficial to allow configurating separate power control parameters values for PUCCH channels associated with these two HARQ-ACK codebooks. Similar like PUSCH power control, DCI format can be used to dynamically select the proper power control set for a given PUCCH transmission based on the associated HARQ-ACK codebook.

Proposal 2:

- Support to separately configure power control related parameters in PUCCH-SpatialRelationInfo IE for two HARQ-ACK codebook transmissions on PUCCH.

- DCI format is used to dynamically indicate one power control parameter for a given PUCCH transmission.

## 2.2 UL transmission collisions handling

In our view, the Rel-15 collision handling procedure should be reused as much as possible at least for the case of collision between UL channels with a same priority order e.g. both associated with eMBB or URLLC service. For the new collision cases of Rel-16, i.e. collision between UL transmission with different priority orders (due to different latency and reliability requirements), the URLLC services related transmission should be generally prioritized to fulfil the stringent latency and reliability requirements of URLLC. In addition, as in earlier releases, the collision handling rule should target to minimize the dropping operation for HARQ-ACK of eMBB as much as possible to avoid throughput loss.

In the table below, the preferred UE behaviors were summarized for the possible collision cases in accordance to the rules mentioned at the start.

Table 1: Intra-UE prioritization rules to handle the UL transmission collisions

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reusing the Rel-15 SR and HARQ-ACK multiplexing rule. |  |  |  |
| CSI | Drop P-CSI, if positive SR; Otherwise, transmit P-CSI. Multiplexing SR and A-CSI. | Multiplexing CSI and HARQ-ACK if latency and reliability are met. Otherwise, drop CSI. |  |  |
| URLLC PUSCH | Reusing Rel-15 rule. | Reusing Rel-15 rule. | Multiplexing if timeline/reliability/latency can be met. |  |
| eMBB SR | Drop eMBB SR. | Drop eMBB SR | Reusing Rel-15 rule. | Drop eMBB SR as in Rel-15 |
| eMBB HARQ-ACK | Multiplex HARQ-ACK and SR based on Rel-15, if latency and reliability can be met; Otherwise, drop HARQ-ACK. | Multiplexing eMBB and URLLC HARQ-ACK if reliability and latency can be met. | Reusing Rel-15 rule. | Multiplexing based on Rel-15 rule if timeline/reliability/latency of URLLC can be met |
| eMBB PUSCH | Drop eMBB PUSCH. | Piggyback HARQ-ACK on eMBB PUSCH if timeline/latency/reliability requirement are met. | Reusing Rel-15 rule. | Drop eMBB PUSCH at least staring from the overlapped symbols. |

Proposal 3:

- Consider the UE behavior in Table 1 as a baseline to handle the collisions between UL transmissions.

## 2.3 UCI reliability enhancements

Referring to Table 1 above, one of key factors that determine whether or not to multiplex UL transmission e.g. HARQ-ACK or CSI on another overlapped transmission (e.g. PUSCH) with different priority is the associated reliability performance. The performance of UCI depends on the number of REs used for transmission, which are controlled by different RRC-configured beta-offset values for different UCI types.

Since in LTE, beta offset is used and dynamically signaled in DCI format to control the amount of resources used by UCI on PUSCH. We are supportive to extend the range to include a lowest value of 0.0 (i.e. essentially skip UCI). This allows network to drop UCI transmission in a flexible manner e.g. based on the eMBB or URLLC PUSCH.

Proposal 4:

- Support increasing the range of delta offset values to include value of ‘0’ for multiplexing UCI and PUSCH.

# 3. Conclusion

In this contribution, different FFS aspects related to UCI transmission for URLLC service were discussed. Based on the discussions in previous sections, we made the following proposals:

Proposal 1:

- Consider to explicitly indicate the associated HARQ-ACK codebook index by DCI that schedules PDSCH transmission.

- For SPS PDSCH, the HARQ-ACK codebook is indicated as part of RRC configuration.

Proposal 2:

- Support to separately configure power control related parameters in PUCCH-SpatialRelationInfo IE for two HARQ-ACK codebook transmissions on PUCCH.

- DCI format is used to dynamically indicate one power control parameter for a given PUCCH transmission.

Proposal 3:

- Consider the UE behavior in Table 1 as a baseline to handle the collisions between UL transmissions.

Proposal 4:

- Support increasing the range of delta offset values to include value of ‘0’ for multiplexing UCI and PUSCH.

# References

- RAN1 Chairman’s Notes, RAN1 #97, May 2019

- RAN1 Chairman’s Notes, RAN1 #95bis, May 2019

- RAN1 Chairman’s Notes, RAN1 #96, May 2019