3GPP TSG RAN WG1 #98	R1-1909059

Prague, Czech, Aug 26th – 30th, 2019

Agenda Item:	7.2.9.2

Source:	Apple Inc.

Title:	Cross Slot Scheduling Design for UE Power Saving

Document for:	Discussion, Decision

# Introduction

During the last RAN1#97 meeting, we made following agreements regarding the cross slot scheduling technique.

Agreements: For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured). Agreements: To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported. - FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication. - FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following: - Excluding the invalid TDRA entries - Re-interpret the selected K0 (K2) value Agreements: When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from: - An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s) Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure. - FFS: Exclude contention-free RACH - FFS: Exclude PUSCH scheduled with TC-RNTI Agreements: - To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported. Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to: - PUSCH scheduled by RAR UL grants for contention-free RACH procedure - PUSCH scheduled with TC-RNTI Conclusion: Companies are encouraged to check the following proposal for the application delay: For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot for K0, or slot for K2, where - X = max(Y, Z) - Y is the minimum applicable K0 value prior to the indicated change - Z = [1] - Z is the smallest feasible non-zero application delay that may depend on DL SCS - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP - FFS: Cross-carrier scheduling with different numerologies - FFS: interruption time, if any

In this contribution, we discuss the remaining issues on cross-slot scheduling technique in R16.

# Cross Slot Scheduling in R16

## Application Delay of Minimum Applicable Value

If a UE gets new minimum applicable value of K0/K2 from gNB signaling in a slot N, then the UE is supposed to apply the indicated value at some point in time, say, in application delay. After the application delay, both gNB and UE will use the new minimum applicable value. In case there is no signaling error between gNB and UE, they should have the same understanding on the minimum applicable value. Any PDSCH/PUSCH grant with K0/K2 value which is smaller than the current minimum applicable value is not expected by UE. In determining the application delay, the assumption on PDCCH decoding relaxation needs to be considered.

### Minimum Applicable Value for K0

Relaxation of PDCCH Decoding

During the past RAN1 discussions, one of the motivations behind cross slot scheduling is the relaxation of PDCCH decoding. Let minimum applicable value of K0 be K0_min. Then, the maximum available relax time depends on on K0_min value.

- For K0_min=0, the PDCCH candidate decoding should be finished in the same slot as soon as possible to receive potential PDSCH in the same slot.

- For K0_min>0, potential PDSCH could be received at least K0_min slots later, which allows the relaxed decoding of PDCCH received in slot N until slot N + K0_min - 1.

Note that it is implementation issue how much relax PDCCH processing for a given K0_min. But, the maximum relaxable time is determined by the current K0_min value. So, we have

PDCCH decoding delay ms<max1,K0_min*slot_length(ms)            (1)

Application Delay of Minimum Applicable Value for K0

Application delay is the time between the slot where DCI including new minimum applicable value is received and the slot where the indicated minimum value is applied as depicted in Figure 1. To apply the signaled new value, the decoding of DCI including the signaled new value should be decoded first. Thus, the application delay should be determined such that it is larger than current PDCCH decoding delay (which could be potentially relaxed), as

PDCCH decoding delay ms<application delay(ms).             (2)

Figure 1 The case application delay of derived new K0/K2 value

The issue is that the PDCCH decoding delay is implementation specific and could change based on needs of relaxed processing. In fact, it should be adjusted based on current application delay.

Above two equations (1) and (2) give

PDCCH decoding delayms<min⁡(application delay for K0ms,  max1,K0_min*slot_lengthms).  (3)

In theory, the range of possible choice of application delay is unlimited. It could be as small as 1 and no bound on the other side. However, as pointed out, the choice of too small application delay can limit the possibility of relaxed PDCCH processing. On the other hand, there is no need to have too large application delay beyond K0_min (in ms) since PDCCH decoding is finished in K0_min slots from the slot where DCI is received.

For this reason, it may be good to choose application delay as

max1,K0_min

in slot, where K0_min is the current K0_min. This will give largest margin for PDCCH relaxing.

Proposal 1

- The application delay of new minimum K0 is given as max(1, K0_min), where K0_min is the current minimum applicable value of K0.

### Minimum Applicable Value for K2

Relaxation of PDCCH Decoding and PUSCH Preparation

Unlike K0 case, the indicated minimum value for K2 could potentially allow the relaxed processing of PUSCH as well. One thing to note here is that once the PDCCH processing is relaxed with the indicated K0_min value, then the PDCCH processing time for PUSCH transmission is also processed in relaxed speed. This means to allow relaxed PUSCH processing, there should be additional time for processing PUSCH with relaxed speed. This will make indicated minimum indicated value for K2 (K2_min) value most likely be larger than the K0_min. Thus, to take advantage of relaxed PUSCH processing, K2_min value needs to be indicated separately from K1_min.

Observation 1

- To maximize the chance of PDCCH relaxation, minimum applicable value for K2 should be larger than that for K0, i.e., K0_min < K2_min.

A relative lengths of each time durations are shown in following Figure 2.

Figure 2 Relative comparison of PDCCH decoding time, K0_min, application delay for K0_min, PUSCH processing time, and K2_min

Note that if K2_min is configured to be smaller than K1_min, then the PDCCH processing time is to be limited by K2_min. This configuration is less likely.

Application Delay for Minimum Applicable Value for K2

We can do the same reasoning as above for K2. The only difference is that K2_min includes both PDCCH decoding delay and PUSCH processing time. We can rewrite above (3) with K0_min replaced with K2_min.

PDCCH decoding delayms<min(application delay for K2ms,  max1,K2_min*slot_lengthms).

However, the application delay for K2_min does not need to be as large as K2_min since PDCCH decoding delay is already smaller than K0_min.

Proposal 2

- The application delay of new minimum K2 is given as max(1, K0_min), where K0_min is the current minimum applicable value of K0.

## DCI Design

Currently, based on agreements, the indication for minimum applicable value for K0/K2 is to be supported at least inside Active Time. The scheduling DCI is to be configured to include additional field(s) for indicating minimum applicable K0/K2 value(s). The size of additional field should as small as possible to minimize the impact to the coverage of PDCCH and robust to the change of indicatable values for minimum applicable values for K0/K2. We propose to add X bit indicator indicating a row of configured table with 2X rows where each row includes the pairs of K0_min and K2_min values. Table 1 shows an example table when X=2 (i.e., two bit indicator).

Table 1 Example of minimum applicable value table with two rows

| bit value | meaning |
| --- | --- |
| 0 | first configured (K0_min, K2_min) |
| 1 | second configured (K0_min, K2_min) |
| 2 | third configured (K0_min, K2_min) |
| 3 | fourth configured (K0_min, K2_min) |

The value of X and table are configured by RRC signaling.

Proposal 3

- NR supports DCI to include an additional field of length X bits to indicate one row of minimum applicable value table which includes the pair of K0_min and K2_min values.

- The value of X and minimum applicable value table is RRC configured.

Per BWP Table

The required minimum K0/K2 value may be different for different BWP. For example, in default BWP with narrow bandwidth, the indicated K0/K2 could be large since the chance of new traffic arrival is relatively low. In such case, large relaxation could be used to maximize power saving gain. While in dedicated bandwidth part with large bandwidth, there is high chance that traffic arrives often. To support lower latency, the set of required minimum K0/K2 value could be smaller than that for default BWP.

Cross BWP scheduling

When cross BWP scheduling is performed with minimum applicable value indication, then the indicated minimum value should be applied to indicated (target) BWP. The different sizes of DCI field in two different BWPs could be handled by truncation rule or zero padding rule.

Proposal 4

- The indicated minimum applicable value for K0/K2 is used for indicated BWP when BWP indicator indicates different BWP than active BWP.

- Truncation and zero padding rule is applied in case the field length of active BWP and that of indicated BWP is different.

## UE assisted Minimum Applicable Values for K0 and K2

The power saving effect through processing relaxation is highly depends on hardware architecture. UE modem which does not support dynamic voltage scaling may not benefit much by processing relaxation. While a UE modem which is optimized with dynamic voltage scaling technique could benefit from relaxed processing. However, even UE modems supporting dynamic voltage scaling technique may have different architecture, which could results in different power consumption. Thus, it is UE itself which knows best about how much power saving gain it can expect with this technique. So, it makes sense that UE indicates network its preferred set of minimum K0/K2 values. It could be K0/K2 values itself or could be a table of K0/K2 pairs which UE want network to use.

Proposal 5

- NR supports UE preferred minimum applicable values for K0/K2.

More details on this topic is included in our companion paper [1].

# Conclusions

In this contribution, we have discussed remaining issues on cross slot scheduling techniques in R16. Based on discussion, we make following observations and proposals.

Proposal 1

- The application delay of new minimum K0 is given as max(1, K0_min), where K0_min is the current minimum applicable value of K0.

Observation 1

- To maximize the chance of PDCCH relaxation, minimum applicable value for K2 should be larger than that for K0, i.e., K0_min < K2_min.

Proposal 2

- The application delay of new minimum K2 is given as max(1, K0_min), where K0_min is the current minimum applicable value of K0.

Proposal 3

- NR supports DCI to include an additional field of length X bits to indicate one row of minimum applicable value table which includes the pair of K0_min and K2_min values.

- The value of X and minimum applicable value table is RRC configured.

Proposal 4

- The indicated minimum applicable value for K0/K2 is used for indicated BWP when BWP indicator indicates different BWP than active BWP.

- Truncation and zero padding rule is applied in case the field length of active BWP and that of indicated BWP is different.

Proposal 5

- NR supports UE preferred minimum applicable values for K0/K2.

# References

- R1-1909061 Traffic aware UE power saving techniques with UE assistance