3GPP TSG RAN WG1 #98	R1-1909060

Prague, CZ, Aug 26th – 30th, 2019

Agenda Item:	7.2.9.3

Source:	Apple Inc.

Title:	Maximum MIMO layer adaptation for UE power saving

Document for:	Discussion, Decision

# Introduction

During RP#84, we agreed following WID scope related to maximum MIMO layer adaptation.

2) Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation

In this paper, we discuss about issues to support max MIMO layer configuration.

# Release 15 Max MIMO Layer

In R15 spec, there are a few places where maximum MIMO layers is indicated : UE capability, PDSCH-ServingCellConfig, and OverheatingAssistance.

Max MIMO layer Indication in UE Capability

UE capability FeatureSetDownlinkPerCC parameters include maxNumberMIMO-LayersPDSCH. This parameter is used to indicate the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, in the bands where 4 RX is mandatory, it is mandatory for UE to report at least 4 MIMO layers in FR1 and 2 MIMO layers for FR2. This capability is conditionally mandator depending on bands [1].

Max MIMO layer Configuration in PDSCH-ServingCellConfig IE

When UE is configured with RRC message, max number MIMO layer for PDSCH is included in PDSCH-ServingCellConfig IE which is per serving cell. The included parameter is applied to all BWPs of the serving cell. Network can limit max MIMO layers using this parameter. Different serving cell could be configured with different maximum number of MIMO layers.

OverheatingAssistance IE in UEAssistanceInformation-v1540-IEs

The overheating assistance information can be sent in UEAssistanceInformation by UE to request network to restrict capability temporarily to notify UE overheating and indicate reduced capabilities which could help addressing UE overheating. The reduced capabilities include reduced maximum number of CCs, reduced maximum aggregate bandwidth in FR1/2, and reduced maximum number of MIMO layers for DL/UL in FR1/2.

The maximum MIMO layers for PDSCH in R15 is first limited by UE’s capability. And, then network configures UE with maximum number of MIMO layers to use by PDSCH-ServingCellConfig IE. In case overheating happens, UE can indicate overheating to network and then network can reduce the maximum MIMO layers to address overheating. In this case it is up to network whether to honor UE’s request or not.

# Release 16 Max MIMO Layer

## Different MIMO Layer for Initial/Default BWP

During RP#84 it was agreed to work on different MIMO layer configuration of the initial/default BWP. The motivation for this is that UE is less likely to have heavy traffic arrival in initial/default BWP. In a typical use case of BWP framework which is UE power saving, the default BWP would be configured with narrow bandwidth and dedicated BWP would be configured with larger bandwidth. Since the amount of data expected in default bandwidth is small, naturally the required MIMO layers to use would be also small. Thus, by this different max MIMO layer configuration for initial/default BWP, UE can save power while staying in those BWP.

Proposal 1

- Support different maximum number of MIMO layers for initial / default BWP.

## Per BWP Max MIMO Layer

As an extension or generalized version of above idea, per BWP max MIMO layer can be also considered. By this technique, network can configure different MIMO layers for different BWP depending on the needs. One use case would be that network configures two dedicate BWP with different MIMO layers and dynamically switch between two considering dynamic traffic arrival and UE power consumption.

For example, when a UE has good enough SINR to support required data rate, the UE may not even need max number of MIMO layers to support its traffic. In such case network can limit the max MIMO layers so that the UE turn off unnecessary RF chains. While, when a UE has low SINR (e.g., in cell edge), the UE may need more receive energy for its performance. In this case a BWP with larger bandwidth would help more. In this case, due to low SINR, network would mostly like use lower MIMO layers, so network could reduce the max number of MIMO layers, which could be translated to UE power saving. In this case, even the max MIMO layer is reduce, if the UE wants to improve its performance, then it could turn on additional RF chains. UE can control by implementation the tradeoff between performance and power saving. UE in the medium SINR regime can be configured with BWPs with different MIMO layers. Network can dynamically switch between e.g., two BWPs depending on its needs. UE power saving request, channel condition, traffic arrival rate, etc could be considered in the configuration of different MIMO layers. Network may also switch BWP based on rank reported from UE. Although current max MIMO layer is 8, if UE keep reporting 4, then it can switch BWP with max MIMO layer of 4. This will allow UE turn off unnecessary RF chains.

Proposal 2

- NR supports configuring maximum number of MIMO layers per BWP per CC.

## Max MIMO Layer Switching / Interruption Time

If per BWP max MIMO layer is supported, the max MIMO layer can be dynamically adjusted based on DCI indication. One thing to consider in this procedure is the relation between BWP switching delay and max MIMO layer switching delay (which is basically involved with RF on/off). If the max MIMO layer switching delay is smaller than the BWP switching delay, then there would be no impact in terms of data rx/tx restriction timing before / after BWP switching. Whereas, if the MIMO layer switching delay is larger than the BWP switching delay, then the first slot after BWP switching that UE can receive PDSCH or transmit PUSCH should be delayed than now until max MIMO layer switching is done.

Proposal 3

- When UE has received BWP switching indication requiring max MIMO layer switching, if the required maximum MIMO layer switching time is longer than BWP switching delay, then the UE is not expected to transmit or receive for max MIMO layer switching delay from the slot when UE received the DCI with BWP switching indication.

# Conclusions

In this paper, we have discussed the max MIMO layer configuration for initial/default BWP and per-BWP max MIMO configuration. Max MIMO layer changes is involved with RF on/off which may require non-negligible switching time. Thus, it is also necessary to take into max MIMO layer switching delay as well during BWP switching. Based on the discussion we make following proposals.

Proposal 1

- Support different maximum number of MIMO layers for initial / default BWP.

Proposal 2

- NR supports configuring maximum number of MIMO layers per BWP per CC.

Proposal 3

- When UE has received BWP switching indication requiring max MIMO layer switching, if the required maximum MIMO layer switching time is longer than BWP switching delay, then the UE is not expected to transmit or receive for max MIMO layer switching delay from the slot where UE received the DCI with BWP switching indication.

# References

- 38.306 User Equipment (UE) radio access capabilities

It is in RAN4’s WI scope to study switching and interruption times for dynamic max MIMO layer switching.