3GPP TSG-RAN WG1 #98	R1-1909062

Prague, Czech Republic, August 26th – 30th, 2019

Source: 	Apple Inc.

Title:                     Uplink power control for NR-NR Dual Connectivity

Agenda item:	7.2.13.1

Document for:	Discussion and Decision

# 1 Introduction

In RAN #81 meeting, a Work Item on DC and CA enhancements [1] was approved, which includes the UL power control aspects for support of asynchronous and synchronous NR-NR DC:

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4] - UE power control [RAN1] - RRC signaling to support of enhanced NR-NR DC [RAN2] - Core requirements to support enhanced NR-NR DC [RAN4] Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core.

In this contribution, we address some of the design issues related to uplink power control of NR-NR Dual Connectivity (DC) scenario to progress on this topic, focusing on physical layers aspects.

# 2. Discussion

## 2.1 On support of power sharing schemes

In Rel-15, NR-DC operation was supported with limiting the configuration of MCG and SCG to be fully in different frequency range (FR). UE performs independent transmission power control per CG without power sharing. As captured in [1], Rel-16 NR-DC targets the cases not covered in Rel-15. In general, two possible deployment scenarios were identified in past meetings to support NR-NR DC:

- Both MCG and SCG fully in a single FR (i.e. FR1or FR2), as shown in FIG.1A

- MCG or SCG in both FR1 and FR2, as shown in FIG.1B

Figure 1: Possible configurations of Rel-16 NR-NR DC

One key aspect on the physical layer of NR-NR DC is UL power control in case the UE transmits to both MeNB and SeNB simultaneously, especially when the total power exceeds the maximum allowed power defined for a certain FR. Many solutions have been proposed to address this power sharing issue, which can be categorized into two types, i.e. semi-static power sharing and dynamic power sharing. The main difference between semi-static and dynamic power sharing schemes is how the transmission power was shared across UL transmissions.

In the semi-static power sharing scheme, the power scaling between MgNB links and SgNB links is provided semi-statically at the level of maximum output level according to certain configuration parameters. That is, the maximum transmission power for the MgNB links and SgNB links are separately signalled with ensuring the sum does not exceed the total maximum output power of the UE. Sine the sum of MgNB and SgNB output power is guaranteed to not exceed PTotalFRX for FRX, the transmission power towards MgNB and SgNB are handled independently entirely.

For the dynamic power sharing scheme, the output power of the UE is shared dynamically between MgNB and SgNB links. When one gNB requires a certain amount of UL power, the entire reminder is available toward the UL of the others. When the sum of the output power of MgNB and SgNB channels exceeds PTotalFRX for a certain FRX, power scaling rule is applied across links to bring the sum down to PTotalFRX. In this option, the UL output power towards one gNB affects the UL power level towards the other gNB due to the joint transmission power determination on overlapping duration.

Generally speaking, the benefit of semi-static power sharing solution is simplicity, where each MgNB and SgNB link operates without need of tight coordination in terms of UL power control. On the other hand, some concerns have been raised in past on power allocation efficiency, coverage reduction and impacts on URLLC features support. Properties of semi-static power sharing schemes are summarized in Table 1 below.

Table 1: Properties of semi-static power sharing scheme for NR-NR Dual Connectivity

Contrarily, the dynamic power sharing option is advantage in avoiding coverage loss and UE transmission power underutilization. Several main concerns with this approach include possible throughput loss in some scenarios due to lack of scheduling coordination between MgNB and SgNB and phase discontinuity in case of power re-scaling in the middle of UL transmission resulting in the different power of DMRS and PUSCH. In addition, it may cause unpredictable network operation and the interruptions are not under the full control of the scheduling gNB and cannot be avoided or planned. More importantly, one strong concern about this approach is that it puts certain restriction on UE implementation and results in increased complexity e.g. due to tighter timeline to achieve a fast coordination interface across two CGs for power adjustment, such as going through all concurrent UL channels across CG to check the channel types and contents and then determining the priority order for power re-scaling operation.

In NR-NR DC, the MgNB and SgNB are connected via non-ideal backhaul and operate independently. The probability of power limitation at UE is significantly higher than that in CA case. UL throughput performance with two schemes highly depends on the deployment scenario e.g. whether there is some sort of coordination between gNBs and the traffic characteristic of MCG and SCG (e.g. both fully loaded, or one is lightly loaded with sparse traffic).

Based on the analysis above, it is clear that different power sharing scheme exhibits different pros and cons in terms of e.g. phase continuity, link adaptation operation, and more importantly the processing time budget requirement. It is feasible to support both of two schemes, which avoids any restriction on UE implementation and also offer a full freedom for network and operator to choose a proper power sharing scheme based on deployment scenario. Hence, we propose the following:

Proposal 1:

- Support both semi-static and dynamic power sharing for Rel-16 NR-NR dual connectivity by properly configuring the value of PMCGFRX and PSCGFRX for a same FRX.

## 2.2 Semi-static power sharing scheme

When power splitting is done in a semi-static manner by RRC signalling, overlapped transmissions do not need special handling in general, as it is guaranteed that PTotalNR-NR DC, FRX is never exceeded for FRX. The scaling and prioritization rule defined for Rel-15 carrier aggregation can be directly applied per CG.

Three options were proposed in the past meetings [2] to implement semi-static power sharing, as provided in Appendix. Comparing to Alt.1, Alt.2 is clearly more beneficial as it improves the power efficiency at the UE by conditionally utilizing the full power on the symbols that are semi-statically configured to be DL on the other CG. This can somehow address coverage shrink concern raised on the semi-static power sharing since the full power transmission is guaranteed in these symbols with DL on the other CG.

In Alt.3, the uplink slots are first divided into three sets by network. UE performs simultaneous towards MgNB and SgNB on the 1st slot set only. However, UE is allowed to transmit towards a single gNB only in the 2nd and 3rd slot sets. Similar as Alt.2, Alt.3 also solves coverage reduction problem. But, the throughput performance is further degraded compared to Alt.2. In principle, the same TDM patterns provided by Alt.3 to maintain coverage can be created with Alt.2, e.g. by exchanging the semi-static UL/DL configurations of each CG. In addition, Alt.2 does not incur any additional signaling overhead since the UL TDM pattern is implicitly determined based on the semi-static UL/DL configuration of other CG. Therefore, we proposed to adopt Alt.2 to be adopted for semi-static power sharing scheme, if supported.

Proposal 2: Semi-static power sharing is operated as follows:

- UE is configured with PMCGFRX+PSCGFRX≤PTotalNR-NR DC, FRX where PMCGFRX is the configured maximum transmission power for MCG for a single Frequency Range X (FRX) and PSCGFRX is the configured maximum transmission power for SCG for FRX, and PTotalNR-NR DC, FRX is the maximum transmission power for NR-NR DC operation on FRX.

- For the synchronous NR-NR DC scenario, for the transmission in one CG in FRX, UE checks the semi-statically configured transmission direction of the overlapped symbols of the other CG in the same FRX,

    - If the transmission direction of overlapped symbols is possible to be UL based on RRC configured UL/DL configuration, UE limits the actual maximum transmission power pCG to be configured maximum transmission power of this CG (i.e. PMCGFRX or PSCGFRX)

    - Otherwise, UE can be up to PMCGFRX' and PSCGFRX'. FFS on how to determine these two values.

## 2.3 Dynamic power sharing scheme

To protect the control plane connection to MgNB, as in LTE DC design, a minimum guaranteed power or equivalently maximum transmission power level for each CG should be introduced. Hence, the SRB or PUCCH transmission towards to MgNB are protected by this guaranteed power. The same target can be also achieved by setting maximum power of SCG PSCGFRX, which leaves the minimum guaranteed power level for MgNB is (PTotalNR-NR DC, FRX-PSCGFRX), with which the UE also guarantees a minimum power level for either SRB or PUCCH transmissions.

Proposal 3:

- Support to introduce configurable minimum or maximum transmission powers for MCG and SCG in a same FRX by higher layers.

For dynamic power sharing, in some cases, UE needs to re-scale the power in the middle of UL transmission due to the overlapping transmission of other CG with higher priority order, causing phase discontinuity problem. To reduce the probability of this kind of power adjustment operation, it was proposed to define more advanced “look-ahead” behaviors. As in LTE, ‘look-ahead’ operation requests UE to determine a transmission power at a given time instance by taking into account the overlapped PUSCH of other CG that is scheduled at a later time. This results in more stringent processing timing requirement at UE side.

In Rel-15, some kind of ‘look-ahead’ behavior was introduced for UCI multiplexing on PUSCH, which can serve as a starting point to define the corresponding operation of dynamic power sharing scheme for NR-NR DC. Given the concerns on potential restrictions of hardware structure, we believe it is reasonable to introduce UE capability for dynamic power sharing support. In addition, the case 2 power control mechanism defined for NR CA can be reused to dynamically share the leftover power of two CGs.

Proposal 4:

- Reuse the Rel-15 procedure of UCI multiplexing on PUSCH as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16.

- Introduce a UE capability to indicate the support of dynamic power sharing for Rel-16 NR-NR DC.

- Case 2 prioritization rules defined for CA are reused for NR-NR DC

# 3. Conclusion

In this contribution, we discussed the details of power sharing mechanisms for NR DC operation. We make following proposals:

Proposal 1:

- Support both semi-static and dynamic power sharing for Rel-16 NR-NR dual connectivity by properly configuring the value of PMCGFRX and PSCGFRX for a same FRX.

Proposal 2: Semi-static power sharing is operated as follows:

- UE is configured with PMCGFRX+PSCGFRX≤PTotalNR-NR DC, FRX where PMCGFRX is the configured maximum transmission power for MCG for a single Frequency Range X (FRX) and PSCGFRX is the configured maximum transmission power for SCG for FRX, and PTotalNR-NR DC, FRX is the maximum transmission power for NR-NR DC operation on FRX.

- For the synchronous NR-NR DC scenario, for the transmission in one CG in FRX, UE checks the semi-statically configured transmission direction of the overlapped symbols of the other CG in the same FRX,

    - If the transmission direction of overlapped symbols is possible to be UL based on RRC configured UL/DL configuration, UE limits the actual maximum transmission power pCG to be configured maximum transmission power of this CG (i.e. PMCGFRX or PSCGFRX)

    - Otherwise, UE can be up to PMCGFRX' and PSCGFRX'. FFS on how to determine these two values.

Proposal 3:

- Support to introduce configurable minimum or maximum transmission powers for MCG and SCG in a same FRX by higher layers.

Proposal 4:

- Reuse the Rel-15 procedure of UCI multiplexing on PUSCH as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16.

- Introduce a UE capability to indicate the support of dynamic power sharing for Rel-16 NR-NR DC.

- Case 2 prioritization rules defined for CA are reused for NR-NR DC

# References

- RP-182076	WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core), Ericsson, Nokia, Nokia Shanghai Bell, Huawei

- R1-1905839	Feature lead summary on Uplink Power Control for Supporting NR-NR Dual-Connectivity	Intel Corporation

# Appendix

## Semi-static power sharing scheme

The following candidates were proposed by companies for semi-static power sharing schemes. The following way forward was suggested by feature leader (FL) for semi-static power sharing scheme.

If supported, the possible candidates are considered for semi-static power sharing scheme: - The UE’s maximum allowed power P_tot is configured semi-statically between the two cell groups such that P_max_MCG + P_max_SCG <= P_tot. - The UE determines the maximum transmission power in a PUSCH transmission occasion as follows: - Alt.1 - The maximum transmit power per group is restricted by the configured maximum power of the each group. - Alt. 2 [12] - The maximum transmit power per group is restricted by the configured maximum power of the same group except for the case that the UE is certain that no uplink transmission is possible on the other cell group on the overlapping symbols by relying on the semi-statically indicated symbol directions. In such a case, the maximum transmit power of a cell group can be increased. - Alt.3 - A UE is semi-statically configured with a TDM pattern configuration providing three sets of slots: - On the first set of slots, the maximum configured power for the MCG and SCG are given such that P_max_MCG + P_max_SCG <= Ptot. - On the second set of slots, all available power is allocated to the MCG, i.e., P_max_MCG = P_tot. - On the third set of slots, all available power is allocated to the SCG, i.e., P_max_SCG = P_tot. - If a total UE transmit power for UL transmissions within a CG exceeds the maximum transmission power for the CG as determined above, UE allocates power to UL transmissions according to the following priority order - Alt.1: Following the uplink power control scheme of Rel. 15 NR CA - Alt.2: Introducing new priority scheme(s) - Candidate-1: To maintain a constant transmission power of a channel/signal in a CG. - Candidate-2: To account for the URLLC (or such high reliability) or random-access procedure, different PUCCH formats or PUSCH types - FFS: Support semi-static power sharing in both synchronous and asynchronous NR-DC cases or asynchronous NR-DC only or synchronous NR-DC only.

## Dynamic power sharing (DPS)

The following candidates were proposed by companies for dynamic power sharing (DPS) schemes. The following way forward was suggested by feature leader for dynamic power sharing schemes for NR-DC.

If supported, the following is considered for dynamic power sharing schemes for NR DC: - Alt.1: [3][4][6][8] - If a total UE transmit power for UL transmissions exceeds the maximum transmission power determined above, UE allocates power to UL transmissions according to the following priority order - Alt.1-1: Reuse the channel priority order defined for Rel-15 CA case 2. - Alt.1-2: Introducing new priority scheme(s) - Candidate-1: To maintain a constant transmission power of a channel/signal in a CG. - Candidate-2: To additionally consider the URLLC (or such high reliability) or random-access procedure, different PUCCH formats or PUSCH types - Alt.1-3: The sum transmit power per cell group is calculated independently. In case the UE is power limited, the sum power of SCG is scaled down. The sum power of MCG and SCG is then distributed across their channels by following the Rel. 15 NR CA within each CG independently. - FFS: UE is configured with maximum available power for each cell group, e.g., P_max_MCG, P_max_SCG and P_max_MCG + P_max_SCG > P_tal_max. - FFS how to handle the case where the UE is not power-limited for transmitting PUCCH, but it is limited for transmitting PUSCH with UCI. - Alt.2: [11] - UE is configured with transmit power limits P_limit_CG1 applicable to CG1 and P_limit_CG2 applicable to CG2. - To compute the transmit power for CG1 UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission, and if such PDCCH(s) are detected, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - UE also checks (based on RRC parameters) if any possible overlapping CG2 UL transmissions can be triggered by PDCCH(s) after T0-T_offset, and if such overlapping CG2 UL transmissions are possible, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - Otherwise, P_limit_CG1 is not applied and UE transmit power for the CG1 UL transmission can be up to ‘full power’ allowed by RAN4 requirements - T_offset can be similar to the T_proc specified in Rel15 for UCI multiplexing related UE procedures. - If NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2 > P_limit_total, and if pwr_CG1+ pwr_CG2> P_limit_total, UE scales down transmission power of SCG transmission(s) such that UE transmission power across MCG and SCG does not exceed P_limit_total. - Note: if NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2<= P_limit_total, the above SCG scaling behavior is not required. - P_limit_total is the power limit for all transmissions in both CG1 and CG2 and will be defined in RAN4 - Alt.3: [2][7][9][10] - UE is configured with a minimum guaranteed power for MCG and SCG separately. - To compute the transmit power for CG1 UL transmission starting at time T0, a UE considers a power for later scheduled overlapping transmissions T0-T_offset, - FFS on T_offset (e.g. subject to the same timelines as for determining a channel among overlapping channels for UCI multiplexing in Rel-15) - Alt.3-1: [2][7][9] - A power limited UE applies Rel-15 CA power allocation prioritization rules for transmissions in different CGs. In case of a same prioritization, power allocation is prioritized for transmissions on the MCG. - FFS for URLLC, different PUCCH formats or PUSCH types - Alt.3-2: [10] - For power determination of any given UL transmission on a CG in NN-DC, the UE does not adjust power to “past” UL transmission, and allocates remaining power for the higher-priority “concurrent” UL transmissions, while respecting the minimum guaranteed power (MGP) limits. Once the transmit power for a transmission is decided, the UE will not re-calculate/re-adjust the transmit power based on upcoming transmission (regardless of priority levels). - Alt.4: [5] - Two types of power ratios can be configured for each CG - Low power ratio L is used for guaranteeing the minimum residual power for each CG. - High power ratio H is used for restricting the maximum power portion when both CGs are requiring more power than its own high ratio - Defining a “cut-off” time of look-ahead operation as an offset before the first symbol T0 of one uplink transmission - Determine the CG prioritization by RRC time pattern or by the dynamic UL grant information. - If MCG is prioritized, the high power ratio for MCG is max{H_SCG , 1-H_SCG } and the high power ratio for SCG is min{H_SCG , 1-H_SCG }; - If SCG is prioritized, the high power ratio for MCG is min{H_MCG , 1-H_MCG } and the high power ratio for SCG is max{H_MCG , 1-H_MCG }; - Alt.5: - Reuse the principle of the uplink power sharing scheme of EN-DC: - MCG has a higher priority than SCG - FFS: Support DPS in both synchronous and asynchronous NR-DC cases or synchronous NR-DC only. - FFS: Maintain a constant transmission power of a channel/signal in a CG in power limited case. - FFS: Account for the URLLC (or such high reliability) or random-access procedure, different PUCCH formats or PUSCH types for defining priority order for power limited case.