3GPP TSG RAN WG1 #98		R1-1909063
Prague, Czech Republic, August 26th – 30th, 2019

Agenda item:	7.2.13.2

Source:	Apple Inc.

Title:	Remaining issues on enhancements to single Tx switched uplink solution for EN-DC

Document for:	Decision

# Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) operation, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity (EN-DC), requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is capable to transmit LTE UL and NR UL simultaneously, UE is allowed to signal one of two different UL operation modes, per agreement from RAN #77[1]:

- Dual UL transmission: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single UL transmission: in this mode, at any given time, UE is only required to transmit uplink in at most one RAT, i.e. either LTE or NR. We also refer to it as Single Tx Switched UL.

Specifically, which band combinations are allowed for single Tx switched UL is decided by RAN4 [1]. Compared to dual UL transmission, Single Tx Switched UL has several performance benefits especially for the UE in power limited scenario (i.e. cell edge) as well as implementation advantages [2].

The Single Tx Switched UL design should cover both the case of LTE TDD PCell (e.g. DC_41_n41) and LTE FDD PCell (e.g. DC_3_n77). So far, R15 specification has been completed for the LTE FDD PCell scenario.

In RAN4 #86bis meeting [3], it was agreed to allow Single Tx Switched UL operation for intra-band EN-DC with LTE TDD PCell in band 41, mainly due to large power reduction associated with dual Tx in intra-band.

In RAN #81 [16] and #84 [17], it was further agreed to include the following as one of the objectives for the R16 work item on “MR-DC and CA enhancements”[16]:

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1]. Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG - Enable the Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC, possibly including any conclusions on the previous objective (6) for LTE FDD Pcells [RAN1].

In RAN1 #96, a similar “SUO case 1” with DL-reference HARQ timing as in the EN-DC with LTE FDD PCell was agreed for EN-DC with LTE TDD PCell [4]:

Agreements: For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config - For DL HARQ timing, the DL-reference UL/DL configuration is applied - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6, - FFS: whether/how to support TDD pattern 0/6 for LTE PCell - UE is not expected to transmit on the MCG and SCG simultaneously - For type 2 UE (i.e., UE without dynamic power sharing capability): - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration - For type 1 UE (i.e., UE with dynamic power sharing capability): - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration - If there is a collision, - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH - In other UL subframes, UE behaviour to be expected to specified with details FFS - E.g., drop LTE PUSCH, drop NR PUSCH, etc. - FFS for the case of NR SRS & NR PRACH - Note: the impact of switching time (if non-zero) will be further studied. - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination) - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

In RAN1#96bis, some further clarifications and progress were made on the HARQ timing for the LTE DL CA scenario, as well as LTE PUCCH formats, as referenced below:

Agreements: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed: - For the LTE TDD PCell: use the PDSCH ACK timeline as in Table 10.1.3.1-1 in 36.213. - For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD- TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - FFS - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) Agreements: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, - Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported. - FFS PUCCH format and resource determination when the UE only receives DAI=1 - FFS whether or not there are any additional issues related to SPS

Based on above agreements so far, there are at least the following open issues to be further concluded for SUO case 1 in EN-DC with LTE TDD PCell:

- HARQ timing for LTE TDD SCell

- Supportable LTE TDD patterns (i.e. whether to include pattern 1 and 6)

- How to handle the case of “collision” between UL transmissions over MCG and SCG

- Impact of switching (if non-zero)

- Support of HARQ-offset

In this contribution, we provide our opinions on those remaining issues mentioned in the above agreement, as well as some other issues raised by contributions ([6]-[14]) to RAN1 #96bis.

# Discussions on remaining open issues for single-Tx in EN-DC

## Clarification on DL HARQ timing of LTE TDD SCell for “SUO case1” in EN-DC

According to the latest agreement from RAN1 #96bis, the LTE DL HARQ timing for SUO case 1 in EN-DC with LTE TDD PCell is clarified for both the PCell and FDD SCell, which is the same as in LTE CA:

- The PCell DL HARQ timing is the regular TDD HARQ timing based on the DL-ref config

- The FDD SCell DL HARQ timing is the same as SCell in LTE TDD-FDD CA case based on the DL-ref config

Where the “DL-ref config” refers to the tdm-Pattern configured to the LTE PCell by RRC. However, the DL HARQ timing for TDD SCells is still FSS as in the latest agreement.

For the TDD SCell with same DL/UL config, typically intra-band CA, it is straight-forward to use the same DL HARQ timing as the PCell. In case there is TDD SCell with different DL/UL config from the PCell, typically inter-band CA, simply using the regular TDD HARQ timing based on the DL-ref config may not always cover all the legitimate DL subframes in the SCell. To simply the design and guarantee HARQ timing is specified for all DL subframes in SCell regardless of the DL/UL config, the same HARQ timing as the FDD SCell can be used.

Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell:

- For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

## Supportable LTE TDD Patterns with “SUO case1” in EN-DC with TDD PCell

According to the latest agreement from RAN1 #96, the LTE PCell’s UL HARQ timing (i.e. gap between PUSCH to PUSCH retransmission) is the same as without the reference configuration.

Meanwhile, based on the existing LTE specifications, i.e. by combining Table 8-2 and Table 8.3-1 from TS 36.213, the PUSCH HARQ timing are always identical for the same UL subframe, at least among TDD Pattern 1,2,3,4 and 5 (notice that Pattern 0 and 6 are not used in any commercially deployed LTE TDD networks so far). This includes PUSCH to PHICH timing, and PHICH/DCI to PUSCH retransmission timing. Specifically, the PUSCH HARQ interval (i.e., delay between any UL PUSCH and the corresponding retransmission) is exactly 10ms for all UL subframes on the LTE PCell, at least for TDD Pattern 1, 2, 3, 4, 5. This is again consistent with the SUO case1 procedure in EN-DC with LTE FDD PCell, where the PUSCH HARQ interval is also 10ms.

However, for LTE TDD pattern 0 and 6, the PUSCH HARQ interval varies (e.g. 10, 11, 13, 14 ms) depending on the subframe number, as shown by Table 8-2 and Table 8.3-1 in TS 36.213. This makes it difficult to reuse the original PUSCH HARQ timing once a subset of UL subframes is configured using the DL-reference config.

Meanwhile, there seems to be no existing commercial deployment using these two configurations. Therefore, it is desirable to exclude LTE PCell using TDD pattern 0 and 6 for SUO case1 in EN-DC with LTE TDD PCell for expedite the R16 specification progress. Extra efforts to support TDD pattern 0 and 6 can be considered in future if commercial need indeed arises in future.

Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the case where LTE PCell’s TDD pattern (as configured in SIB1) being 0 or 6 is not supported in R16.

## UE handling of “collision” between UL transmissions over MCG and SCG

First, it was clearly noted in the RAN WID that “UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG”. Therefore, from a UE perspective, a collision should be considered as an error-case and should be avoided. In this sense, the corresponding UE behavior should typically be left for UE implementation so as to meet other regulation/specification requirements. This is desirable at least for type 2 UEs, which do not have fast communication between the LTE and NR modems.

On the other hand, it was also brought up that: in case network schedulers indeed have conflict UL scheduling on both RAT’s simultaneously, although not expected from UE perspective, it might still be desirable to have an expected UE behavior assuming UE has fast enough information exchange between LTE and NR modems (i.e. type 1 UE). Therefore, it was also proposed by some company that a type 1 UE should prioritize LTE UL transmission when “collision” happens between UL transmissions over MCG and SCG, when the type 1 UE is in single Tx switched uplink mode.

Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE.

- RAN1 to further decide whether LTE should always be prioritized for type 1 UE in this case.

## PUCCH formats

Another issue related to single Tx operation with case 1 DL HARQ timing is the potential PUCCH resource collision [6] between legacy UE and single Tx UE, if fallback operation is triggered, i.e., DAI=1. To avoid that, it was agreed in RAN1 #96bis that PUCCH formats 3/4/5 will be supported, and PUCCH fallback operation based on implicit resource indication is not supported.

The remaining open question is: what PUCCH format/resource to use when DAI = 1? If still use PUCCH format 3/4/5, there is no ARI bits available to indicate which of four configured PUCCH resources to use. If relying on fallback to format 1a/1b, as no implicit resource indication is supported, there will need to be extra configured PUCCH 1a/1b resource reserved for the UE in addition to the already reserved resource for PUCCH format 3/4/5. So the benefits of reducing resource overhead by fallback to format 1a/1b in legacy LTE TDD is no longer valid.

Therefore, the most efficient and simplest way of utilizing PUCCH resource when DAI=1 is to still use the configured resources for PUCCH format 3/4/5, but specify a default rule to select which one of the configured resources to use (e.g., always use the resource with the lowest index).

Proposal 4: For single UL operation in EN-DC with LTE TDD PCell, where UE is configured with DL-reference UL/DL configuration (i.e. SUO with case 1 HARQ timing), only LTE PUCCH formats 3/4/5 are supported

- when DAI=1, using the PUCCH resource with the lowest index.

## Configuration of RACH resources for UE operating in single Tx

One issue, which was raised by some contributions [8][9][12] to RAN1 #96bis, is the configuration of RACH resources for UE operating in SUO case1. Recall that, in R15, for single Tx in EN-DC for LTE FDD PCell, it was agreed that the RACH configuration for SUO case 1 UE is not different from legacy UEs, as follows:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

Considering the fact that RACH transmissions are rare and mostly happens before RRC connection, it is desirable to have similar conclusion for single-Tx in EN-DC with TDD PCell. In addition, at least type 1 UE would be capable to handle any overlapped transmission between LTE RACH and NR UL.

Proposal 5: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

# Application of single Tx switched UL mechanism to Dual Tx scenario

The WID for MR-DC and CA enhancements [19] was updated in RAN #84 to include additional objective of enabling R15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC. This was particularly motivated by the issue of NR DL de-sensing by UL harmonics from LTE transmission at lower frequency bands, which may happen for both single UL and dual UL capable UEs.

The fundamental mechanism is the same as the handling of single Tx for EN-DC, where the network limits the LTE UL transmissions to only a subset of the available LTE UL subframes. The subset of the UL subframes is configured through a DL-reference config by RRC parameter “tdm-PatternConfig”, which also specify the HARQ feedback timing for LTE DL packets.

The same or similar mechanism and signaling could potentially be applied for the harmonic issue as well, except that existing R15 specification limit the usage of the corresponding RRC signaling to a few scenarios. The signaling defined in R15 includes the following

- tdm-PatternConfig, as defined in 36.331: “UL/DL reference configuration indicating the time during which a UE configured with EN-DC is allowed to transmit. This field is used when power control or IMD issues require single UL transmission as specified in TS38.101-3 [85] and TS 38.213”

- tdm-Pattern, as defined in 38.306: “Indicates whether the UE supports the tdm-PatternConfig for single UL-transmission associated functionality, as specified in TS 36.331 [17]. Support is conditionally mandatory for UEs that do not support dynamic power sharing and for UEs that indicate single UL transmission for any BC, and optional otherwise”

- singleUL-Transmission, as defined in 38.306: “Indicates that the UE does not support simultaneous UL transmissions as defined in TS 38.101-3 [4]. The UE may only include this field for certain band combinations defined in TS 38.101-3 [4]. If included for a particular band combination, the field applies to all fallback band combinations of this band combination that are defined in TS 38.101-3 [4] as being allowed to include this field and does not apply to any other fallback band combinations defined in TS 38.101-3 [4]”

For network to configure a DL-reference config to the LTE PCell for the DL de-sensing issue, one possible solution would be modifying the existing description of “tdm-PatternConfig” usage scenarios, while keep the changes to remaining parts of the specification (e.g. 36.213 and 38.213) minimal. An alternative solution is to introduce a different parameter, e.g. “tdm-PatternConfig2”, to specifically differentiate the purpose of the DL-reference configuration. This will leave any specification related to existing single UL unchanged.

Proposal 6: For purpose of configuring a DL-reference config to the LTE PCell to mitigate DL de-sensing, RAN1 to decide among the following two options:

- Option 1: reuse existing “tdm-PatternConfig” defined in 36.331, but extend the description to also include the usage scenario of DL de-sensing from harmonics.

- Option 2: introduce a new signal “tdm-PatternConfig2” for the usage scenario fo DL de-sensing from harmonics.

For UE to indicate the capability to support DL-reference configuration on the LTE PCells, “tdm-Pattern” as defined in 38.306 can be reused, as such capability is independent of the usage scenarios. However, the description in 38.306 may need to be extended to also address the usage scenario of DL-desensing from harmonics.

Proposal 7: Modify the description of “tdm-Pattern” in 38.306 to include the usage scenario of DL de-sensing from harmonics.

For UE to indicate to the network that DL de-sensing need to be handled by configuring DL-reference config and restricting LTE UL to a subset of the subframes, a new capability bit (similar as “singleUL-Transmission”) need to be introduced per band combination.

Proposal 8: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes.

# Summary

In this contribution, we discuss some of the remaining issues from the latest agreements on single Tx switched UL. We have the following the proposals:

Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell:

- For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the case where LTE PCell’s TDD pattern (as configured in SIB1) being 0 or 6 is not supported in R16.

Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE.

- RAN1 to further decide whether LTE should always be prioritized for type 1 UE in this case.

Proposal 4: For single UL operation in EN-DC with LTE TDD PCell, where UE is configured with DL-reference UL/DL configuration (i.e. SUO with case 1 HARQ timing), only LTE PUCCH formats 3/4/5 are supported

- when DAI=1, using the PUCCH resource with the lowest index.

Proposal 5: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

Proposal 6: For purpose of configuring a DL-reference config to the LTE PCell to mitigate DL de-sensing, RAN1 to decide among the following two options:

- Option 1: reuse existing “tdm-PatternConfig” defined in 36.331, but extend the description to also include the usage scenario of DL de-sensing from harmonics.

- Option 2: introduce a new signal “tdm-PatternConfig2” for the usage scenario fo DL de-sensing from harmonics.

Proposal 7: Modify the description of “tdm-Pattern” in 38.306 to include the usage scenario of DL de-sensing from harmonics.

Proposal 8: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes.

# References

- RP-172085, “Signaling for single/dual UL Tx”, RAN2 Chairman, 3GPP RAN #77, Sapporo, Japan, Sept 11-15, 2017.

- R1-1902774, “Enhancements to single Tx switched uplink for EN-DC”, Apple, 3GPP RAN1#96, Athens, Greece, February 25th – March 1st, 2019

- “Chairman Note, RAN4#86bis”, 3GPP RAN4 #86bis, Melbourne, Australia, USA 16 - 20 April, 2018.

- “Chairman Notes, RAN1 #96”, 3GPP RAN1 #96, Athens, Greece, February 25th – March 1st, 2019.

- “Chairman Notes, RAN1 #96bis”, 3GPP RAN1 #96bis, Xi’an, China, April 8th-12th, 2019.

- R1-1904721, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1902820, “Potential enhancements to Single Tx Switched Uplink solution for EN-DC”, NTT DOCOMO Inc., 3GPP RAN1#96, Athens, Greece, February 25th – March 1st, 2019.

- R1-1903994, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904155, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904400, “Enhancements to single Tx switched UL for EN-DC”, Samsung, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904327, “Remaining issues for single uplink operation enhancement for EN-DC”, Intel, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1905143, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1905280, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- R1-1904638, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics, 3GPP RAN1#96bis, Xi’an, China, 8th – 12th April, 2019.

- RP-180680, “Solutions for single Tx TDM”, RAN #80, La Jolla, CA, U.S.A., June 11st-14th, 2018.

- “Chairman Note, RAN#81”, 3GPP RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- “Chairman Note, RAN#84”, 3GPP RAN #84, Newport Beach, U.S.A., 3rd – 6th June, 2019.

- PR-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- PR-191600, “Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #84, Newport Beach, U.S.A., 3rd – 6th June, 2019.

- “Chairman Notes, RAN1 Ad-Hoc #1901”, 3GPP RAN1 Ad-Hoc #1901, Taipei, Taiwan, 21st – 25th January 2019.