3GPP TSG RAN WG1 #98		                                    R1-1909065

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.6.2

Source:	InterDigital Inc.

Title:	UCI Enhancements for eURLLC

Document for:	Discussion and Decision

# Introduction

Work Items on physical layer enhancements for NR URLLC [1] and support for industrial IoT [2] were approved at RAN#83. The objectives of these WIs include specifying UCI enhancements for a UE that supports different service types. Several agreements related to HARQ-ACK reporting and UCI collisions were reached in RAN1#96bis and RAN1#97. In addition, an email discussion [97-NR-05] for the handling of various combinations of UCI types and data for URLLC and eMBB took place after RAN1#97.

This contribution summarizes our views on handling of UCI collisions. In particular, it is proposed that multiplexing eMBB with URLLC in the same transmission (when possible) is supported to allow maintaining good spectrum efficiency when URLLC traffic intensity increases. The contribution also addresses other issues related to UCI enhancements. Similar to HARQ-ACK, a separate PUCCH power control configuration for URLLC SR is proposed. Other details left FFS on HARQ-ACK reporting are also addressed.

# Handling of UCI collisions

During email discussion [97-NR-05] companies provided their views on the handling of overlap between URLLC data/HARQ-ACK/SR and other transmission (eMBB or CSI). In scenarios involving transmissions of different priority levels (URLLC vs eMBB or CSI), the choice is generally between dropping the lower priority (eMBB) transmission always, or multiplexing into the same transmission when the latency and reliability of the higher priority (URLLC) transmission is not compromised.

The latter option has the benefit of maintaining good spectral efficiency even when the level of URLLC traffic is relatively high. It enables the network to have control over the conditions under which multiplexing is enabled and adapt the properties of the transmissions as a function of traffic conditions. For example, when SR or HARQ-ACK for URLLC occurs more frequently the network can schedule eMBB PUSCH transmissions of short duration to enable multiplexing with URLLC SR or HARQ-ACK and avoid dropping, and otherwise schedule eMBB PUSCH transmissions of long duration.

Proposal 1: Support multiplexing of URLLC and eMBB on same transmissions when latency and reliability of URLLC transmission is not compromised.

Specific scenarios where this principle should be applied include:

- Scenario 12: eMBB HARQ-ACK overlapping with URLLC HARQ-ACK

- Scenario 14: eMBB HARQ-ACK overlapping with URLLC PUSCH

- Scenario 15: eMBB PUSCH overlapping with URLLC SR

- Scenario 16: eMBB PUSCH overlapping with URLLC HARQ-ACK

For scenarios 12 and 14, the latency requirement of URLLC is automatically satisfied by multiplexing on the URLLC resource for HARQ-ACK or PUSCH. However, there needs to be a condition to ensure that reliability is not compromised. For scenario 12 this can be done by configuring a maximum code rate, while for scenario 14 the amount of resources used by HARQ-ACK can be capped using existing mechanism (alpha parameter).

For scenarios 15 and 16, the latency requirement of URLLC is satisfied if the duration of PUSCH is sufficiently short. A simple rule could be that PUSCH needs to ends not later than the PUCCH resource that would carry SR or HARQ-ACK. For the reliability requirement one could have a condition that the required number of symbols for the UCI does not reach a threshold (e.g. corresponding to alpha).

Proposal 2: URLLC HARQ-ACK can be multiplexed on eMBB PUSCH only if PUSCH transmission does not end later than PUCCH resource that would carry URLLC HARQ-ACK.

Proposal 3: URLLC SR can be multiplexed on eMBB PUSCH only if PUSCH transmission does not end later than PUCCH resource that would carry URLLC SR.

Proposal 4: Support configuration of additional sets of beta factors for multiplexing HARQ-ACK/SR on PUSCH of different priority.

The following Table summarizes our views for each scenario.

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse R15 rules |  |  |  |
| CSI | Drop CSI if SR is positive and resource for SR overlaps with PUCCH resource for CSI. If CSI multiplexed on PUSCH, follow same rule as for SR overlapping with PUSCH | Drop CSI if resource for HARQ-ACK overlaps with PUCCH resource for CSI. If CSI multiplexed on PUSCH, follow same rule as for HARQ-ACK overlapping with PUSCH. |  |  |
| URLLC PUSCH | Reuse R15 rules | Reuse R15 rules | Multiplex CSI in PUSCH |  |
| eMBB SR | Drop eMBB SR if URLLC SR is positive | Reuse R15 rules at least if URLLC HARQ-ACK is on PF2, PF3 or PF4, otherwise drop SR | Reuse R15 rules | Drop eMBB SR |
| eMBB HARQ-ACK | Drop HARQ-ACK | Multiplex eMBB and URLLC HARQ-ACK on PUCCH resource indicated for URLLC if a maximum code rate applicable to URLLC is not exceeded, otherwise drop eMBB HARQ-ACK. | Reuse R15 rules | Multiplex HARQ-ACK in PUSCH. |
| eMBB PUSCH | Multiplex SR on PUSCH if the following conditions are satisfied, otherwise drop PUSCH at least for the overlapping portion when SR is positive: -PUSCH does not start earlier or end later than the applicable PUCCH resource for the SR -Number of coded modulation symbols per layer Q’SR would be less than a threshold | Multiplex HARQ-ACK on PUSCH if following conditions are satisfied, otherwise drop PUSCH at least for the overlapping portion: -PUSCH does not start earlier or end later than PUCCH resource for HARQ-ACK of URLLC -Number of coded modulation symbols per layer Q’ACK would be less than a threshold | Reuse R15 rules | Drop eMBB PUSCH at least for the overlapping portion |

For the identification of priority (e.g. URLLC vs eMBB) of a transmission, the following are proposed:

- For HARQ-ACK, the priority is indicated by the DCI scheduling corresponding PDSCH;

- For SR, the priority is explicitly configured as part of the corresponding SR configuration;

- For PUSCH (data), the priority is indicated by the scheduling DCI.

For SR, explicit RRC configuration of the priority is preferable to using the LCH priority since the number of priority levels for LCH (16) exceeds the number of priority levels required at the physical layer (e.g. 2). The parameter could be added to either SchedulingRequestResourceConfig or SchedulingRequestConfig.

For PUSCH, priority indication by DCI is preferable to using the LCH priority of included data, otherwise the priority of PUSCH is not known with certainty prior to decoding at the network side.

When priority is indicated by DCI, a solution that allows support for multiple (i.e. more than 2) levels of latency and reliability without much additional specification effort would also be very desirable from the perspective of extensibility. Either an explicit indication (e.g. in new field), using RNTI, or configuration of CORESET/search space would meet such requirement. For HARQ-ACK, the indication should also point to a slot/sub-slot HARQ-ACK configuration and associated PUCCH configuration.

Proposal 5: DCI scheduling PDSCH indicates the priority of corresponding HARQ-ACK.

Proposal 6: The priority of HARQ-ACK corresponds to a slot/sub-slot HARQ-ACK configuration and associated PUCCH configuration.

Proposal 7: DCI scheduling PUSCH indicates its priority.

Proposal 8: For DCI-based priority indication of HARQ-ACK or PUSCH, one of the following options is selected: RNTI, explicit field in DCI, or CORESET/search space.

Proposal 9: Confirm the working assumption that SR priority is known at PHY layer.

Proposal 10: RRC configures priority of SR for each SR (resource) configuration.

Proposal 11: RRC configures priority of PUSCH in the case of configured grant type 1.

# UCI enhancements

SR power control

In RAN1#97, it was agreed that when at least two HARQ-ACK codebooks are simultaneously constructed for support different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks (except FFS for PUCCH-SpatialRelationInfo).

One of the parameters is pucch-PowerControl. The agreement thus enables specific power control configuration for HARQ-ACK on PUCCH depending on the codebook linked to a service type, such that reliability requirement of URLLC is met. It should be noted that the parameter pucch-PowerControl itself can generally be applicable to other types of UCI than HARQ-ACK, including SR.

In RAN1#97, a working assumption that SR priority is known at PHY was taken. The priority of SR is used to handle prioritization when there is overlap with other UCI/data transmissions. In addition, to ensure that the reliability requirement of a high priority SR is met, it should also be possible to configure a specific power control configuration for that case, similar to HARQ-ACK. Since pucch-PowerControl parameter is already separately configured for different HARQ-ACK codebooks corresponding to different service types, it would make sense to reuse the same parameter as a function of the service type for SR.

Proposal 12: The pucch-PowerControl parameter applicable to HARQ-ACK of a given priority is also applicable to SR of same priority.

Value range and DCI field bitwidth of K1

For slot-based HARQ-ACK reporting, the parameter dl-DataToUL-ACK can have a maximum size of 8 values and a value range of 0 to 15 slots. In case of sub-slot-based HARQ-ACK reporting, one question is whether such setting is appropriate considering that the latency corresponding to a given value of K1 is shorter than for the case of slot-based. However, considering that sub-slot-based HARQ-ACK reporting is intended for low-latency services, there is no obvious reason why the span of K1 in absolute time would need to be the same as in slot-based. A value range of 0 to 15 for K1 and 8 possible values would seem to offer the same degree of scheduling flexibility for sub-slot-based as for the case of slot-based. In addition, increasing the number of possible values compared to slot-based would be incompatible with efforts to reduce the DCI size for URLLC.

Proposal 13: The maximum number of values for dl-DataToUL-ACK for sub-slot-based HARQ-ACK is 8 (same as slot-based).

Proposal 14: The value range of dl-DataToUL-ACK for sub-slot-based HARQ-ACK is 0 to 15 (same as slot-based).

Whether different PUCCH resource sets can be configured for different sub-slots within a slot

In case of a sub-slot configuration includes sub-slots with different number of symbols, imposing the same set of PUCCH resources irrespective of the sub-slot would be restrictive. In addition, for the first sub-slots of a slot it may be useful for the network to have the ability to select a longer resource (in case of coverage limitation).

Proposal 15: Different PUCCH resource sets can be configured for different sub-slots within a slot.

Whether PUCCH-SpatialRelationInfo is separately configured

The parameter PUCCH-SpatialRelationInfo consists of a set of possible combinations of parameters used for power control and spatial filtering (e.g., P0, reference signals), which can be applied to a PUCCH transmission. Since this parameters refers to other parameters configured as part of pucch-PowerControl, which is separately configured between different HARQ-ACK codebooks, it also needs to be separately configured.

Proposal 16: PUCCH-SpatialRelationInfo is separately configured.

Other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList

The PUCCH configuration contains parameter SchedulingRequestResourceConfig which consists of a set of resources available for SR transmission. The structure already allows that some resources are used for low priority (eMBB) or high priority (URLLC) SR. As discussed in previous section, adding a priority parameter in either SchedulingRequestResourceConfig or SchedulingRequestConfig enables proper handling in case of overlap with other UL transmissions and indicates the proper power control parameters (if agreed).

The multi-CSI-PUCCH-ResourceList parameter is relevant in case of multiplexing CSI with other UCIs. This could be revisited once decisions are made for the handling of UCI collisions.

# Conclusion

This contribution summarized our views on handling of UCI collisions and other UCI enhancements.

Proposal 1: Support multiplexing of URLLC and eMBB on same transmissions when latency and reliability of URLLC transmission is not compromised.

Proposal 2: URLLC HARQ-ACK can be multiplexed on eMBB PUSCH only if PUSCH transmission does not end later than PUCCH resource that would carry URLLC HARQ-ACK.

Proposal 3: URLLC SR can be multiplexed on eMBB PUSCH only if PUSCH transmission does not end later than PUCCH resource that would carry URLLC SR.

Proposal 4: Support configuration of additional sets of beta factors for multiplexing HARQ-ACK/SR on PUSCH of different priority.

Proposal 5: DCI scheduling PDSCH indicates the priority of corresponding HARQ-ACK.

Proposal 6: The priority of HARQ-ACK corresponds to a slot/sub-slot HARQ-ACK configuration and associated PUCCH configuration.

Proposal 7: DCI scheduling PUSCH indicates its priority.

Proposal 8: For DCI-based priority indication of HARQ-ACK or PUSCH, one of the following options is selected: RNTI, explicit field in DCI, or CORESET/search space.

Proposal 9: Confirm the working assumption that SR priority is known at PHY layer.

Proposal 10: RRC configures priority of SR for each SR (resource) configuration.

Proposal 11: RRC configures priority of PUSCH in the case of configured grant type 1.

Proposal 12: The pucch-PowerControl parameter applicable to HARQ-ACK of a given priority is also applicable to SR of same priority.

Proposal 13: The maximum number of values for dl-DataToUL-ACK for sub-slot-based HARQ-ACK is 8 (same as slot-based).

Proposal 14: The value range of dl-DataToUL-ACK for sub-slot-based HARQ-ACK is 0 to 15 (same as slot-based).

Proposal 15: Different PUCCH resource sets can be configured for different sub-slots within a slot.

Proposal 16: PUCCH-SpatialRelationInfo is separately configured.

# References

RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, Huawei, HiSilicon.

RP-190728, “New WID: Support of NR Industrial Internet of Things (IoT)”, Nokia, Nokia Shanghai Bell.

# Appendix

Agreements from RAN1#97:

Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot. Agreements: For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s) - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot. Agreements: - When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following: - FFS: For PUCCH-SpatialRelationInfo - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback. - FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList. - FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration. Conclusion: Further study the collision scenarios in the table below: - Companies are encouraged to fill in solutions, e.g. multiplexing, priorization, for each scenario. - A company can input “not related to RAN1” in one entry. - A company can input the priority of study for one entry. - Consider R15 as the starting point for collisions between two URLLC UCIs. - FFS: Collision between more than two channels. URLLC SR; URLLC HARQ-ACK; CSI; URLLC PUSCH. URLLC SR. URLLC HARQ-ACK. CSI. URLLC PUSCH. eMBB SR; (Example): Drop eMBB SR. eMBB HARQ-ACK. eMBB PUSCH. Email discussion till next meeting to fill-up the table – Jia (OPPO) Working assumption: Support that SR priority (e.g. high or low priority) is known at PHY layer. - FFS how to use the priority information in handling prioritization/multiplexing of UL transmissions. - FFS how the SR priority is known

Agreements from RAN1#96bis:

Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. - PDSCH transmission is not subject to sub-slot restrictions (if any) - FFS: PDSCH-to-sub-slot association. - FFS: Allowing PUCCH across sub-slot boundary or not. - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook. - FFS for Type I HARQ-ACK codebook. - R15 PUCCH resource overriding procedures is applied in unit of sub-slot. - Number or length of UL sub-slots in a slot is UE-specifically semi-statically configured. - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot. - FFS: K1 definition. - FFS: Details of PUCCH resource configuration and determination. FFS: Use “Codebook-less HARQ” as a complementary or not. FFS: If HARQ-ACK can be omitted in case latency requirement cannot be met. FFS: PDSCH groupings and PHY identification for separate HARQ-ACK constructions for different service types. Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, K1 is defined following R15 approach but in unit of sub-slot. Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and for dynamically-scheduled PDSCH, down-select from below for the PHY identification for identifying a HARQ-ACK codebook: - Opt.1: By DCI format - Opt.2: By RNTI - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field) - Opt.4: By CORESET/search space - FFS additional option(s) for Type I HARQ-ACK codebook FFS: For SPS PDSCH (including SPS release PDCCH)