3GPP TSG RAN WG1 #98	R1-1909100

Prague, CZ, August 26th – 30th, 2019

Source:	Sharp

Title:	UCI enhancements for URLLC

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss remaining issues for UCI enhancements for URLLC, focusing on PUCCH configurations for URLLC, and collision handling between different channels for data and/or control of the same or different service types.

# PUCCH resource configuration for URLLC

Sub-slot-based HARQ-ACK feedback can be used for URLLC. In RAN1 #97, it was agreed that parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for PUCCH-SpatialRelationInfo. Some details and FFS items should be further clarified in RAN1.

- The subslot configuration

The subslot for PUCCH configuration should be clarified. For subslot based HARQ-ACK feedback, at least a seven 2-symbol subslot, and a two 7-symbol subslot structure in a slot should be specified. Additionally, a 3 and 4 symbol structure can also be considered. Thus, up to 7 (e.g. 2, 4, 7) subslots can be configured in a slot. The subslot structure can be configured based on latency requirements and SCS settings.

- HARQ-ACK PUCCH resource configuration

It was agreed than for a given sub-slot configuration, a UE can be configured with PUCCH resource set(s). The PUCCH resources may be configured in all subslots or a subset of subslots within a slot, considering the trade-off between the allowed maximum number of PUCCH transmissions and PUCCH resource overhead.

For consistency and easy configuration, the same PUCCH resource sets should be configured for different sub-slots within a slot. A subslot based PUCCH resource should be confined within a subslot, and PUCCH resource across sub-slot boundary should be avoided. The number of URLLC PUCCH for HARQ-ACK feedback can be configured by higher layer signaling based on the SCS settings and subslot configurations.

- PUCCH reliability enhancements

The reliability of a the PUCCH resource for URLLC HARQ-ACK should be much higher than a PUCCH resource for eMBB. They can be differentiated by different PUCCH parameters, such as tranmit power, number of RBs, coding rates, etc. Furthermore, the subslot configuration can further limit the supported PUCCH formats for subslot based HARQ-ACK feedback, e.g. the durations of long PUCCH formats.

- PUCCH configuration for other UCI, such as SR and CSI.

The priority of a SR is known at PHY layer determined by at least the periodicity. To ensure low latency, the PUCCH resource for URLLC SR should also be configured at subslot level. The periodicity of URLLC SR can be reduced to 1 or 2 slots at least, and can be configured with a number of subslots instead of slots.

Furthermore, RAN1 should clarify the miss detection probability of a PUCCH for URLLC SR, which determines the PUCCH resource configuration, and impacts potential UCI multiplexing when channel collision occurs.

For CSI, RAN1 should first discuss whether it is necessary to introduce separate PUCCH resource configurations for CSI report of URLLC.

Proposal 1: The PUCCH for URLLC should be clarified with more details on the subslot structure, resource configuration, reliability enhancement, and PUCCH for different UCI types.

- Define 2-sym, 3/4-sym and 7-sym structure, and up to 7 subslots can be configured in a slot.

- Subslot based PUCCH is confined within a subslot with parameters to provide ultra-reliability.

# Collision handling of UL channels

As a general principle, URLLC should have higher priority than eMBB. Considering both UCI and data, the following priority can be defined from high to low: URLLC HARQ-ACK >= URLLC SR > URLLC PUSCH > eMBB HARQ-ACK >= eMBB SR > CSI eMBB HARQ-ACK > eMBB CSI > eMBB PUSCH.

For collisions between channels of the same service type, the Rel-15 rules should be reused as baseline.

For PUCCH collisions between URLLC UCI and eMBB UCI, UCI multiplexing between URLLC and eMBB should be supported under some timing and PUCCH resource restrictions. The conditions should be further clarified for different scenarios. If the conditions cannot be satisfied, the PUCCH carrying URLLC UCI should be transmitted and the PUCCH for eMBB should be dropped.

For collisions between URLLC UCI and eMBB PUSCH, URLLC HARQ-ACK on eMBB PUSCH should be specified with higher priority. Some timing, delay and performance restrictions may apply to determine whether URLLC HARQ-ACK multiplexing on eMBB PUSCH is possible. If the conditions cannot be satisfied, the URLLC UCI should be transmitted and the PUSCH for eMBB should be dropped.

For collisions between eMBB UCI and URLLC PUSCH, eMBB UCI may be dropped as a simple solution. Some timing restrictions may apply to determine whether eMBB UCI multiplexing on URLLC PUSCH is possible.

Since SR priority is known at PHY layer, SR priority determination and collision handling are critical aspects especially for grant based URLLC operation. And SR priority differentiation should be considered, esp. in UCI multiplexing between HARQ-ACK and SR on PUCCH.

Higher priority can be given to collision scenarios involving URLLC HARQ-ACK, esp.

- URLLC HARQ-ACK + eMBB HARQ-ACK

- URLLC HARQ-ACK + eMBB PUSCH

- URLLC HARQ-ACK + URLLC/eMBB SR

Since URLLC UL channels can be subslot based or at symbol level, the collision between an URLLC UL channel and an eMBB UL channel may be fully or partially overlapped. Especially, an URLLC UL channel may come after the start of an eMBB UL channel transmission. Thus, depending on timing relationships, an URLLC channel may drop an eMBB channel in total, or puncture an ongoing eMBB channel.

A summary of solutions with the above-mentioned principles is summarized in the table in the appendix for potential UL channel collision scenarios.

Proposal 2: The following principles can be applied to determine the methods of UL channel collisions

- For collisions between channels of the same service type, reuse Rel-15 rules as baseline.

- For collisions between URLLC and eMBB, URLLC channel should have higher priority.

- UCI multiplexing between URLLC and eMBB can be considered with some timing/latency/resource constraints. Details FFS.

- Higher priority can be given to collision scenarios involving URLLC HARQ-ACK.

# Conclusion

In this contribution, we first discuss remaining issues on URLLC PUCCH configuration, followed by the principles to handle channel collisions between control and data from the same or different service types. We propose that

Proposal 1: The PUCCH for URLLC should be clarified with more details on the subslot structure, resource configuration, reliability enhancement, and PUCCH for different UCI types.

- Define 2-sym, 3/4-sym and 7-sym structure, and up to 7 subslots can be configured in a slot.

- Subslot based PUCCH is confined within a subslot with parameters to provide ultra-reliability.

Proposal 2: The following principles can be applied to determine the methods of UL channel collisions

- For collisions between channels of the same service type, reuse Rel-15 rules as baseline.

- For collisions between URLLC and eMBB, URLLC channel should have higher priority.

- UCI multiplexing between URLLC and eMBB can be considered with some timing/latency/resource constraints. Details FFS.

- Higher priority can be given to collision scenarios involving URLLC HARQ-ACK.

# References

[1] Chairman’s notes RAN1 #97 final

# Appendix

Summary of proposed solutions for channel collision scenarios

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse R15 rules as baseline |  |  |  |
| CSI | Drop CSI | Drop CSI |  |  |
| URLLC PUSCH | Reuse R15 rules (Drop SR) | Reuse R15 rules | Drop CSI |  |
| eMBB SR | Drop eMBB SR | Reuse R15 rules if certain conditions are satisfied (timeline/latency), otherwise drop eMBB SR. | Reuse R15 rules | Drop eMBB SR |
| eMBB HARQ-ACK | Drop eMBB HARQ-ACK | Multiplex both HARQ-ACKs in one PUCCH if certain conditions are satisfied (timeline/latency/PUCCH capacity), otherwise drop eMBB HARQ-ACK | Reuse R15 rules | Drop eMBB HARQ-ACK as baseline. FFS reuse R15 rules if timeline is satisfied. |
| eMBB PUSCH | Drop PUSCH | URLLC HARQ-ACK is multiplexed on eMBB PUSCH under some timing constraints, otherwise, drop eMBB PUSCH | Reuse R15 rules | Drop eMBB PUSCH |