3GPP TSG RAN WG1 #98			R1-1909172

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	Discussion on Channel Structure for Two-Step RACH

Agenda Item:	7.2.1.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#97 meeting, the issues related to channel structure for 2-step RACH were discussed and RAN1 made following agreements [1].

Agreements: - The following parameters are defined per msgA PUSCH configuration: - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include: - MCS and/or TBS (to be further decided) - Number of FDMed POs - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain - Number of PRBs per PO - Number of DMRS symbols/ports/sequences (if support) per PO - FFS whether or not support repetitions for msgA PUSCH - FFS bandwidth of PRB-level guard band or duration of guard time - FFS PUSCH mapping type - Parameters specific to option 1, at least include: - Periodicity (msgA PUSCH configuration period) - FFS value range - Offset(s) (e.g., symbol, slot, subframe, etc.) - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc. - Frequency starting point - Parameters specific to option 2, at least include: - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc. - Number of symbols per PO - FFS explicit or implicit indication - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency) - FFS: Number of TDMed POs - Support multiple msgA PUSCH configurations for a UE - FFS the maximum number of configurations - FFS which parameters, if any, are common for all configurations - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact - FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH - FFS validation rule of msgA PUSCH Agreements: - The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be cell ID or configurable, to be FFS). - FFS details of the RNTI - FFS the inclusion of DMRS index.

In this contribution, the channel structure for 2-step RACH, especially regarding MsgA structure, is discussed.

- Discussion

In the WID, reuse of the Rel-15 NR PRACH preamble design and the Rel-15 NR PUSCH design is premised for MsgA, and also TDM between MsgA RACH occasion and MsgA PUSCH occasion is premised. Since MsgA RACH occasion and MsgA PUSCH occasion are located in different resources, the mapping between MsgA RACH occasion and MsgA PUSCH occasion should be considered. Although one simple way is that MsgA RACH occasion and MsgA PUSCH occasion are located in contiguous time resources, it is less flexible and the applicable case is limited since in some typical TDD operation UL slot is isolated and MsgA PUSCH transmission may not be able to follow MsgA preamble transmission within the slot. Thus, MsgA RACH occasion and MsgA PUSCH occasion should be able to be located in non-contiguous time resources and there can be TDD DL region between MsgA RACH occasion and corresponding MsgA PUSCH occasion.

Proposal 1: MsgA RACH occasion and corresponding MsgA PUSCH occasion can be located in non-contiguous time resources and there can be TDD DL region between MsgA RACH occasion and corresponding MsgA PUSCH occasion.

At the previous meetings, it was discussed how the resource location of MsgA PUSCH occasion is indicated/defined. In terms of latency, it should be desirable that MsgA RACH occasion and MsgA PUSCH occasion are located closely as much as possible. Considering the indication of RACH occasion for Rel-15, each RACH occasion may not be located with same interval at least for some RACH configurations in Rel-15 RACH configuration table. Thus, it might be better to consider the relative location at least in time domain of the MsgA PUSCH occasion with respect to the associated MsgA RACH occasion, so that MsgA PUSCH occasion can be located close from MsgA RACH occasion. Also, in order to have flexibility for gNB scheduling, the relative location should be configurable.

Proposal 2: The relative location in time domain of the MsgA PUSCH occasion with respect to the associated MsgA RACH occasion can be configured.

For Rel-15, the rule for valid/invalid RACH occasion based on TDD configuration and SSB location was defined. Similar rule for valid/invalid MsgA PUSCH occasion should be defined so that MsgA PUSCH occasion does not overlap with DL part and SSB, i.e., if possible MsgA PUSCH occasion overlaps with DL part in TDD configuration or SSB location, the MsgA PUSCH occasion becomes invalid. In addition, it should be studied whether or not MsgA PUSCH occasion needs to avoid overlap with MsgA RACH occasion. In case that the MsgA PUSCH occasion is associated with the overlapped MsgA RACH occasion, UE needs to transmit MsgA preamble and MsgA PUSCH simultaneously, and it may not be desirable due to UE complexity and PSD. In case that the MsgA PUSCH occasion is not associated with the overlapped MsgA RACH occasion, gNB may not be able to use appropriate Rx beam due to analogue beam forming.

Proposal 3: The validation rule for MsgA PUSCH occasion based on TDD configuration and SSB location should be defined, similarly as the rule for valid/invalid RACH occasion in Rel-15.

- FFS: whether or not the validation rule for MsgA PUSCH occasion based on location of MsgA RACH occasion is defined.

In Proposal 2, the relative location in time domain of the MsgA PUSCH occasion with respect to the associated MsgA RACH occasion is considered. In addition, considering FDMed and/or CDMed MsgA PUSCH occasions corresponding to MsgA RACH preambles in a MsgA RACH occasion, the mapping order for MsgA preamble and MsgA PUSCH should be determined. Similarly as SSB-to-RO mapping rule, the mapping order should be code domain, followed by frequency domain. In addition, if one-to-multiple mapping between MsgA RACH occasion and MsgA PUSCH occasions is supported, mapping rule similar to that for SSB-to-RO, i.e., code domain and then frequency domain followed by time domain, should be defined.

Proposal 4: The mapping order for MsgA preamble and MsgA PUSCH is following:

- First, in increasing order of code domain, i.e., MsgA PUSCH DMRS port/sequence or MsgA preamble index

- Second, in increasing order of frequency domain resource

- Third, in increasing order of time domain resource

At the previous meetings, configurable time gap between MsgA RACH occasion and MsgA PUSCH occasion required for NW/UE processing time was discussed. However, this issue can be resolved by the MsgA PUSCH occasion allocation, i.e., the relative location between MsgA PUSCH occasion and associated MsgA RACH occasion as in Proposal 2. The MsgA PUSCH occasion allocation is based on gNB configuration, which can ensure the required time gap for NW/UE processing time.

Observation 1: The required time gap between MsgA RACH occasion and MsgA PUSCH occasion for NW/UE processing time can be ensured by the relative location between MsgA PUSCH occasion and associated MsgA RACH occasion.

2-step RACH can be considered only for contention based random access, and collision probability needs to be considered. The MsgA preambles transmitted in a same MsgA RACH occasion, can be differentiated if different preamble indices are used. On the other hand, PUSCHs transmitted in a same MsgA PUSCH occasion can be differentiated if different DMRS ports or different DMRS sequence are used. However, in case of the DMRS based differentiation, some gNB cannot use the method since gNB receiver complexity would become high and also the possible number of DMRS ports/DMRS sequences would be less than the number of preamble indices. Thus, if only one-to-one mapping between MsgA preamble index in each MsgA RACH occasion and associated MsgA PUSCH resource unit is supported, MsgA PUSCH resource overhead would become significantly high or some of MsgA preamble index would be wasted when sufficient number of PUSCH resources cannot be prepared. Basically, the bottleneck might be a PUSCH resource overhead, and if sufficient number of preamble indices can be used for 2-step RACH to utilize multiple-to-one mapping based on the possible number of PUSCH resources, multiple-to-one mapping is beneficial. While the successful probability of 2-step RACH, i.e., successful MsgA PUSCH reception, is almost kept, the collision probability of MsgA preamble can be reduced even if MsgA PUSCH collides. Since some UEs which fails only MsgA PUSCH can fallback to 4-step RACH and can perform Msg3 transmission, RACH latency can be reduced and the collision probability of legacy 4-step RACH preamble can also be reduced. Therefore, at least one-to-one and multiple-to-one mapping should be supported.

Proposal 5: Confirm the following working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

In addition, one-to-multiple mapping is still FFS. One potential use case of one-to-multiple mapping is the scenario where only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter in case that uplink timing information by the MsgA preamble is not needed. In that case, the preamble is used only for reference to associated MsgA PUSCH, and one-to-multiple mapping is useful in order to reduce the collision probability of MsgA PUSCH. Such use case should be further considered, and if it will be identified as possible sceanrio, one-to-multiple mapping should be supported.

Proposal 6: It should be further considered on the scenario where only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter in case that uplink timing information by the MsgA preamble is not needed.

Proposal 7: If the scenario where only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter is identified as possible scenario, one-to-multiple mapping between MsgA preamble index in each MsgA RACH occasion and associated MsgA PUSCH resource unit should be supported.

- Conclusion

In this contribution, channel structure of MsgA for 2-step RACH was discussed. Based on the discussion, the following observation and proposals were made:

Proposal 1: MsgA RACH occasion and corresponding MsgA PUSCH occasion can be located in non-contiguous time resources and there can be TDD DL region between MsgA RACH occasion and corresponding MsgA PUSCH occasion.

Proposal 2: The relative location in time domain of the MsgA PUSCH occasion with respect to the associated MsgA RACH occasion can be configured.

Proposal 3: The validation rule for MsgA PUSCH occasion based on TDD configuration and SSB location should be defined, similarly as the rule for valid/invalid RACH occasion in Rel-15.

- FFS: whether or not the validation rule for MsgA PUSCH occasion based on location of MsgA RACH occasion is defined.

Proposal 4: The mapping order for MsgA preamble and MsgA PUSCH is following:

- First, in increasing order of code domain, i.e., MsgA PUSCH DMRS port/sequence or MsgA preamble index

- Second, in increasing order of frequency domain resource

- Third, in increasing order of time domain resource

Observation 1: The required time gap between MsgA RACH occasion and MsgA PUSCH occasion for NW/UE processing time can be ensured by the relative location between MsgA PUSCH occasion and associated MsgA RACH occasion.

Proposal 5: Confirm the following working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

Proposal 6: It should be further considered on the scenario where only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter in case that uplink timing information by the MsgA preamble is not needed.

Proposal 7: If the scenario where only UE ID is transmitted by MsgA PUSCH and the collision of MsgA preamble does not matter is identified as possible scenario, one-to-multiple mapping between MsgA preamble index in each MsgA RACH occasion and associated MsgA PUSCH resource unit should be supported.

References

- 3GPP RAN1#97, Chairman’s note, May 2019.