3GPP TSG RAN WG1 Meeting #98 R1-1909213 Prague, CZ, August 26th - 30th, 2019 CR-Form-v12.0. CHANGE REQUEST. 38.212; CR; DRAFT; rev; -; Current version:; 15.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft alignment CR on SRI. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2019-8-16. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; The abbreviation ‘SRI’ refers to not only SRI field in DCI but also SRS resource index within the SRS resource set. ‘indication’ in the description ‘SRI indication’ for SRI tables is redundant. ‘(s)’ is redundant for the table in where only one SRS resource is indicated.. -. Summary of change:; Change SRI(s) to SRS resource index or index(es) within the SRS resource. Remove ‘indication’. Remove ‘(s)’ in the case when only one SRS resource is indicated.. Consequences if not approved:; The same wording ‘SRI’ refers to different meanings. The wording ‘indication’ is redundant. ‘(s)’ is redundant for the table in where only one SRS resource is indicated.. Clauses affected:; 7.3.1.1.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: No impact. This CR's revision history:; This is the first version for this CR..

7.3.1.1.2	Format 0_1

<Unchanged part omitted>

Table 7.3.1.1.2-28: SRI for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRS resource index within the SRS resource set, | Bit field mapped to index | SRS resource index within the SRS resource set, | Bit field mapped to index | SRS resource index within the SRS resource set, |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

Table 7.3.1.1.2-29: SRI for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

Table 7.3.1.1.2-30: SRI for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14-15 | reserved |

Table 7.3.1.1.2-31: SRI for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, | Bit field mapped to index | SRS resource index(es) within the SRS resource set, |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14 | 0,1,2,3 |
|  |  |  |  | 15 | reserved |

Table 7.3.1.1.2-32: SRI for codebook based PUSCH transmission

| Bit field mapped to index | SRS resource index within the SRS resource set, |
| --- | --- |
| 0 | 0 |
| 1 | 1 |