3GPP TSG RAN WG1 Meeting #97 	R1-1909271

August 26th – 30th, 2019

Prague, Czech Republic.

Agenda item:	7.2.8.1

Source: 	Qualcomm Incorporated

Title: 	CSI Enhancement for MU-MIMO Support

Document for:	Discussion/Decision

# Introduction

In RAN1#97, high rank codebook for Rel-16 Type II CSI with FD compression and UCI design were finalized. The agreements and observation of offline email discussion were captured in [1-5]. In this contribution, we discuss remaining issues on parameter downselection, number of FD compression units, CSI omission, CBSR and UE capabilities.

# Parameter down-selection

## Combination of L, y0, v0 and β

In Rel-16 Type II codebook, the PMI on N3 subbands are provided by a product of three matrices, i.e., SD compression matix, coefficient matrix and FD compression matrix. These three matrices are parameterized by 3 values, i.e., number of SD bases L, (max) number of non-zero coefficients K0=2LMβ and the number of FD bases M=N3Ry0 for RI={1,2} or M=N3Rv0 for RI={3,4}. Based on the discussion so far, there are 3 candidate values proposed for each of them, leading to 22 different parameter combinations shown in Table 1 (v0 is not applicable to L=6 as it only supports upto Rank-2, y0=12 can only combine with β=14). Since the max number of CSI report supported by UE per type (P/SP/AP) per BWP is 4, it is necessary to perform downselection for the parameter combination.

Table 1. Parameter combination table, where y0,v0,β is shown in each row (for L=6, only y0,β) and the compression ratio is calculated via L×y0×β. The payload increases with combination of y0,v0,β for each L.

| Compression ratio | 1/8 | 1/8 | 1/4 | 1/4 | 1/4 | 3/8 | 3/8 | 1/2 | 3/4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L=2 | 14,18,14 | 14,14,14 | 14,18,12 | 14,14,12 | 12,14,14 | 14,18,34 | 14,14,34 | 12,14,12 | 12,14,34 |
| Compression ratio | 1/4 | 1/4 | 1/2 | 1/2 | 1/2 | 3/4 | 3/4 | 1 | 3/2 |
| L=4 | 14,18,14 | 14,14,14 | 14,18,12 | 14,14,12 | 12,14,14 | 14,18,34 | 14,14,34 | 12,14,12 | 12,14,34 |
| Compression ratio | 3/8 | - | 3/4 | - | 3/4 | 9/8 | - | - | - |
| L=6 | 14,14 | - | 14,12 | - | 12,14 | 14,34 | - | - | - |

To this end, two criteria was agreed during offline email discussion

- Criteria 1: Remove combinations whose total overhead exceeding the max of Rel-15 Type II

- Criteria 2: Remove combinations whose UPT lower than Rel-15 Type II for the same overhead. This criteria also applies to compare Rel-16 several combinations appearing redundant in terms of overhead.

Besides, in our view, the combinations downselection should consider the following issues

- The parameter combinations after downselection shall span a full range of the payload, so as to cover a wide range of UE as much as possible (i.e., cell-edge, medium and cell-center).

- The number combinations should be reduced as much as possible, e.g., 4 combinations. This is due to the max number of CSI report UE can support per type per BWP is 4. Having more combination may be useless as 1) a UE may be configured with other CSI reports with other codebook, e.g., Type I, and 2) for Rel-16 Type II CSI, there are some other RRC parameters, e.g., two phase quantization resolutions (i.e., 8PSK and 16PSK), two FD-unit resolution (R=1, 2) and three candidate values for window-based two-step FD bases selection.

- The discussion for L=2 and L=4 shall consider jointly for Rank-2 and Rank-4 cases. This is because if a UE does not support L=6, the parameter combinations should provide good UPT-overhead tradeoff.

Figure 1-4 provide simulation results considering Rel-15 Type II with L=4 as a baseline. For L=2, combinations of 
14,18,12, 14,18,34, 14,14,34, 12,14,12, 12,14,34 are shown, while All the combinations of L=4 and L=6 are shown. This is because the lowest two combinations for L=2 have too aggressive compression ratio, which leads to less than 80% of Rel-15 CSI performance. As shown from Figure 1 and 2, L=2 achieves better performance for low compression ratio region, while L=4 yields better performance at medium and high compression ratio region. Combination A with L,y0,v0, β=(2,14,18,12), B with L,y0,v0,β=(4,14,18,12) and C with L,y0,v0,β=(4,12,14,12) capture the envelop of the curve, which gives good performance at cell-edge, medium and cell-center. From Figure 3 and 4, it can be seen that combination A, B and C also captures the envelop for rank-2 considering L=2 and L=4 only. However, for a UE supporting L=6, combination D with L,y0,β=(6,14,12) gives the best UPT-overhead tradeoff as it provides 2.5% gain over combination C and provides similar performance to the last point of L=6 with around 150 less payload.

Figure 1. MU performance for L=2 and 4 with upto Rank-4, 8PSK phase quantization

Figure 2. MU performance for L=2 and 4 with upto Rank-4, 16PSK phase quantization

Figure 3. MU performance for L=2, 4 and 6 with upto Rank-2, 8PSK phase quantization

Figure 4. MU performance for L=2, 4 and 6 with upto Rank-2, 16PSK phase quantization

Hence, based on the discussion and simulation results, we propose

Proposal 1: The parameter combinations after downselection shall span a full range of the max payload, so as to cover a wide range of UE as much as possible (i.e., cell-edge, medium and cell-center)

Proposal 2: Strive to minimize the number combinations as much as possible, e.g., 4 combinations as L,y0,v0,β=(2,14,18,12), L,y0,v0,β=(4,14,18,12), L,y0,v0,β=(4,12,14,12) and L,y0,β=(6,14,12).

## Discussion on window size for two-stage FD basis selection

In the last meeting, for N3>19, a two-stage FD basis selection was agreed to further reduce overhead. That is, the UE would firstly select a window-based intermediate FD bases set for all layers, then secondly select FD bases for each layer from this intermediate set. The size of the intermediate set is denoted by N3'. It is also agreed that the value of N3' is obtained via scaling the number of FD bases by a factor α, i.e., N3'=α×M. There are three candidate values of α, i.e., 1.5, 2 and 2.5.

In our view, window size should be set large enough to cover the significant taps of each layer. The reason is as follows. Before choosing the window and its starting point, the UE has to align the strongest coefficient of each layer so that they are all located at FD basis 0. However, the PDP of each layer may be different, the main taps of layer 0 is to the right of FD basis 0, while the main taps of layer 1 may locate at the tail (i.e., left of FD basis 0) as shown in Figure 5. In this case, if the window size is not large enough, the main taps of layer 0 may not be captured.

Figure 5. Illustration of the effect of applying non-sufficient window size

Figure 6. MU performance with different value of α, where R=2, NSB=16, L=4 and (y0,v0,β) follow the last 7 combinations in Table 1.

A simulation results comparing different value of α is shown in Figure 6. As shown, α=1.5 is slightly inferior to α=2, while α=2 achieves similar performance to α=2.5. Given that there are already many RRC configurations of L,y0,v0,β, phase resolution and FD unit resolution R, it is preferred to support a single value of  α=2.

Hence, based on the discussion, we propose

Proposal 3: For the size of the intermediate set used in the 1st stage of two-stage FD bases report, support N3'=α×M where α=2 only.

# Number of FD Compression Units

Another remaining issue lies in the definition of N3 value (number of FD compression units) when the number of PMI SBs is greater than 13. In this case, to ease the computation efficiency, one argument lies in that the value of N3 being always powers of 2, 3, or 5 only, i.e., N3=2a⋅3b⋅5c, where a, b and c are non-negative integers. To this end, there are two alternatives extrapolation and segmentation were agreed in RAN1 #AH1901 and further described in RAN1 #97.

For extrapolation, denoting {Xk, 1≤k≤Ns} as the precoding coefficient of the unit k in the original Ns units (before extrapolation), ⌈N3-Ns2⌉ units are prepended to the original Ns units, and ⌊N3-Ns2⌋ units are appended to the original Ns units, resulting in:

- When θ=3: [Y1, Y2,  X1,X2,…,XNs, Y0 ];

- When θ=2: [Y1,  X1,X2,…,XNs, Y0 ];

- When θ=1: [Y0 , X1,X2,…,XNs]

Furthermore, there are 4 alternatives identified for the extrapolation methods:

- Alt 1.1: Average: Yt= (X1+X2+…+XNs)/Ns, t=0,1,…,θ-1:

- Alt 1.2: Raised cosine extrapolation: Yt= αtXNs+1-αtX1, t=0,1,…,θ-1

  - When θ=3: α0=0.8536,  α1=0.5,  α2=0.1464

  - When θ=2: α0=0.75,  α1=0.25

  - When θ=1: α0=0.5

- Alt 1.3: Repetition:

  - If θ=3 or 2, Y0=XNs, Y1=Y2=X1

  - If θ=1, Y0=X1

- Alt 1.4: Zero padding: Yt= 0,  t=0,1,…,θ-1

A simulation result comparing the extrapolation schemes with the baseline scheme (i.e., Alt3 with N3=NSB×R) is shown in Figure 7 considering two FD units are extrapolated. It can be seen that 1) the 4 extrapolation schemes achieve similar performance (around 1% difference), and 2) Alt3 achieves 70~110 bits less in payload with less than 1% performance loss compared to extrapolation. The payload difference is due to the fact that extrapolation leads to a larger N3 value than Alt3, which results in a larger number of FD bases and a large K0. Unfortunately, such an increase in the payload does not bring significant gain.

Figure 7. MU performance for different extrapolation method.

Besides, although extrapolation/segmentation yields N3 being N3=2a⋅3b⋅5c, these two schemes increase the UE implementation and spec complexity as either of them provides a different implementation compared to the case where number of PMI subbands is smaller than or equal to 13. From this perspective, these two alternatives yield neither good performance gain nor simplified design. Hence, unified framework with N3=NSB×R regardless of its value should be supported.

Proposal 4:  Support a unified framework to determine the value of N3 regardless its value, i.e., N3=NSB×R.

# CSI omission

In general, CSI omission can be avoided via network implementation. That is, when the gNB triggers CSI reports, it should allocate UL resource targeting that each CSI is reported with its max payload. In Rel-15 type II codebook, since there is a big difference between the payload of rank-1 and rank-2, if the gNB always allocate resource targeting rank-2 payload, there would be some waste in UL resource if the UE finally reports rank-1 CSI. Hence, in Rel-15, to allow more efficient CSI report configuration, CSI omission based on subband decimation was introduced. That is, the gNB does not allocate the UL resource based on the max payload of each CSI report, if the actual CSI payload exceeds the budget of UL resource, the UE omits the CSI on odd subbands. This mechanism is simple and does not require CQI/PMI recalculation.

However, in Rel-16 Type II codebook, since any single coefficient in Rel-16 type II is related to the PMI of all subbands, subband decimation is not reusable and omitting any coefficient may impact the subband CQI. Hence, from this aspect, it is ideal that the UE does not expect to be configured with CSI report whose payload exceeds the allocated UL resource. On the other hand, it is understandable that there still exists a big gap between the rank-1 payload and rank-2/3/4 payload in Rel-16 Type II codebook, CSI omission may be still beneficial to efficient CSI report trigger. Thus, as a secondary preference, the CSI omission rule for Rel-16 shall be studied, but consider design with no impact to CQIs/PMIs. The reason is as follows.

- The UE may process multiple CSI calculation in parallel, then decide whether CSI omission is needed afterwards. Since there would be very little time left for UE to transmit the CSI reports on PUSCH after all the CSI calculation, the CSI omission shall only comprise simple operations like adjusting/removing (portion of) the UL payload to meet the target coding rate, but not including any operation requiring CSI recalculation.

- Another reason for no CSI recalculation is that if UE updates rank or NNZC, the network would not know whether the lower rank/NNZC is due to CSI omission or a UE preferred CSI. Thus, it is important to have CSI omission without CSI recalculation so that the network is able to schedule more UL resource for the next CSI report trigger.

Hence, based on the discussion, we propose

Proposal 5: For Rel-16 Type II CSI, support one of the following

- UE does not expect to be configured with a CSI report whose payload exceeds the allocated UL resource (first priority);

- CSI omission for Rel-16 Type II codebook shall not require CSI recalculation, especially for CQI (second priority).

As aforementioned, any single coefficient in Rel-16 Type II CSI is related to the PMI of all subbands, so omitting any single coefficient would change the PMI on all subbands. This would lead to a mismatch between the CQI and the PMI after omission as the UE does not expect to update the CQI. From this perspective, apart from the wideband SD bases, all the other components in UCI part 2, i.e., FD bases selection, bitmap used to indicate NNZC locations, SCI and NNZC quantization, become useless. Besides, removing all those CSI components except for SD bases would lead to a lower coding rate than omitting a fraction of coefficients/bitmaps. Hence, based on the discussion, we propose

Proposal 6: For Rel-16 Type II CSI omission, support omitting all CSI components other than SD bases in UCI part 2.

# UE capability

In Rel-15, UE reports the max number of simultaneous resource and max number of simultaneous ports per codebook, and also reports the max number of simultaneous resource and max number of simultaneous ports per band-band combination regardless of codebook type. These types of capability reflect UE’s memory size to store intermediate results of CSI calculations. For a single CSI-RS resource associated with N CSI reports, it is also agreed that the CSI-RS resource and the CSI-RS ports within the resource are counted N times. The motivation is that two different CSI report may have different report quantity or different interference measurement, and these two CSI calculations may not share common intermediate results. So, the memory used to store intermediate results is doubled even if the two CSI reports share a common CSI-RS resource. Following the above logic, when R=2 is configured in Rel-16, there would be two different PMIs in one CQI. Thus, compared to R=1, the memory cost and the CSI calculation cost is doubled. Hence, based on the discussion, we propose

Proposal 7: For R=2, support the following rule for CSI processing criteria

- The number of resources and the number of ports within the resources are counted twice;

- The CPU occupation is 2 per resource.

In current UE capability signalling, a UE may report supporting max A1 resources for Type I, max A2 resources for Rel-15 Type II, and max B resources regardless of codebook type. This means that UE can support any combination of a1 Type I resource and a2 Rel-15 Type II resources as long as a1≤A1, a2≤A2 and a1+a2≤B. This mechanism may lead to an underreported capability of for Rel-15 Type II. An example is illustrated as in Table 2. Assume a Rel-15 UE reports B=8 resources regardless of codebook type, and it can support the following CSI report combinations in Table 2. As we can see, it can also support 7 resources for Rel-15 Type II if there is no Type I is triggered simultaneously. However, it cannot support 7 resources for Rel-15 Type II plus 1 resource for Type I simultaneously. To avoid configuration of a1=1 and a2=7, the UE has to underreport its Rel-15 Type II capability as A2=6.

Table 2. Examples of codebook combinations supported and non-supported by UE

|  |  | Type I | Rel-15 Type II | Rel-16 Type II | Support or not |
| --- | --- | --- | --- | --- | --- |
| For Rel-15 UE and Rel-16 UE | Comb0 | 8 | 0 | 0 | Yes |
|  | Comb1 | 0 | 7 | 0 | Yes |
|  | Comb2 | 1 | 7 | 0 | no |
|  | Comb3 | 2 | 6 | 0 | yes |
| For Rel-16 UE only | Comb4 | 0 | 0 | 4 | yes |
|  | Comb5 | 2 | 0 | 4 | yes |
|  | Comb6 | 0 | 7 | 1 | no |
|  | Comb7 | 0 | 6 | 2 | no |
|  | Comb8 | 4 | 2 | 2 | yes |
|  | Comb9 | 2 | 2 | 4 | no |

This problem is even more severe for a UE supporting Rel-16 Type II CSI. To avoid invalid configuration of Comb2, Comb6, Comb7 and Comb9, the UE may report supporting upto A1=8, A2=2 and A3=2 resources for Type I, Rel-15 Type II and Rel-16 Type II, respectively. However, this report also excludes the supportive of Comb1, Comb3, Comb4 and Comb5. To solve this issue, enhancement to UE capability signalling or restrictions on CSI triggering is needed. Hence, based on the discussion, we propose

Proposal 8: For Rel-16, enhancement to UE capability signalling or restrictions on CSI triggering needs to be studied to support codebook combinations.

# CBSR

For Rel-15 codebooks, codebook subset restriction was introduced for intra/inter-cell interference mitigation. For Type I codebook, a subset of SD bases and/or rank are disabled to mitigate the interference leakage to other UEs/cells. For Rel-15 Type II codebook, CBSR was further enhanced where a “soft” restriction is applied to a subset of SD bases. Specifically, for a certain SD basis i, a 2-bit restriction on its wideband amplitude, i.e., pl,i(1), is applied. That is, pl,i(1) should be smaller than or equal to a configured value from 1, 0.5, 0.25, 0). As the subband amplitude (pl,i(1)⋅pl,i(2)) is further determined by a differential value (pl,i(2)≤1) relative the wideband term, this restriction implies that the amplitude on each subband cannot exceed the configured restriction.

For Rel-16 Type II codebook, the amplitude of an SD basis across N3 subbands is m=0M-1pi,m,l(1)pi,m,l(2)fm,lH. Given this structure, it is not suitable to reuse the Rel-15 Type II CBSR due to the following reasons: 1) pi,m,l(1) is not beam-specific, instead, it is common to all coefficients associated with the same polarization and same layer; 2) the subband amplitude is jointly determined by the linear combination coefficients and the FD bases, so it is not possible to apply a restriction on the subband amplitude. On the other hand, one option to restrict the wideband amplitude pi,m,l(1)m=0M-1pi,m,l22 is to configure a pthr such that pi,m,l(1)pi,m,l(2)≤pthr and the total amplitude is pi,m,l(1)m=0M-1pi,m,l22≤pthrM. However, this scheme is much more expensive than the Rel-15 approach as it is very difficult to find two parameters jointly under a constrain on their product.

Besides, the motivation of CBSR on FD bases is not clear to us. The selection of FD bases is subject to the channel delay profile observed by the UE. Restricting UE to select FD bases from a configured subset does not mitigate interference, but may lose compression performance.

Hence, based on the discussion, we propose

Proposal 9: In Rel-16 Type II CSI with FD compression, support CBSR on rank and SD basis only.

# Conclusion

In this contribution, we discuss the remaining issues related to type II CSI enhancement. Based on the discussion, we propose

Proposal 1: The parameter combinations after downselection shall span a full range of the max payload, so as to cover a wide range of UE as much as possible (i.e., cell-edge, medium and cell-center)

Proposal 2: Strive to minimize the number combinations as much as possible, e.g., 4 combinations as L,y0,v0,β=(2,14,18,12), L,y0,v0,β=(4,14,18,12), L,y0,v0,β=(4,12,14,12) and L,y0,β=(6,14,12).

Proposal 3: For the size of the intermediate set used in the 1st stage of two-stage FD bases report, support N3'=α×M where α=2 only.

Proposal 4:  Support a unified framework to determine the value of N3 regardless its value, i.e., N3=NSB×R.

Proposal 5: For Rel-16 Type II CSI, support one of the following

- UE does not expect to be configured with a CSI report whose payload exceeds the allocated UL resource (first priority);

- CSI omission rule for Rel-16 Type II codebook shall not require CSI recalculation, especially for CQI.

Proposal 6: For Rel-16 Type II CSI omission, support omitting all CSI components other than SD bases in UCI part 2.

Proposal 7: For R=2, support the following rule for CSI processing criteria

- The number of resources and the number of ports within the resources are counted twice;

- The CPU occupation is 2 per resource.

Proposal 8: For Rel-16, enhancement to UE capability signalling or restrictions on CSI triggering needs to be studied to support codebook combinations.

Proposal 9: In Rel-16 Type II CSI with FD compression, support CBSR on rank and SD basis only.

# Appendix

Table 6, Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Number of RB per subbands |  | 8 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Maximum 12 layers for MU-MIMO |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50% |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |

# References

- Chairman’s notes RAN1#97 Final

- R1-1906965 Feature lead summary for MU-MIMO CSI, Samsung

- R1-1907783 Feature lead summary on offline email discussion for N3 issue, Samsung

- R1-1907883 Feature lead summary on Tue offline session for MU-MIMO CSI, Samsung

- R1-1907894 Feature lead summary on offline discussion for SCI-related issues, Samsung