3GPP TSG RAN WG1 #98			R1- 1909274

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.8.4

Source: 	Qualcomm Incorporated

Title: 	Full Tx power for UL transmissions

Document for:	Discussion/Decision

# Introduction

In RAN #80 meeting [1], the following WI was approved as one of work items for NR MIMO Enhancements.

  - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

In RAN1 #94bis meeting [2], the following agreements were made on this WI:

Consider the following potential solutions and other solutions (such as combination of the solutions below) for UL full power transmission. Decision will be made in RAN1#95:

Option 1: Refinement/adjustment of UL codebook is supported

•             1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

•             1-2: Introduce additional scaling factor for uplink codebook

Option 2: UE transparently apply a small cyclic or linear delay

Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

•             Note: Full rated PA refers to a PA having power not lower than that of the power class

Option 4: Up to UE implementation (no specification impact)

In RAN1 #95 meeting [3], the following agreements were made on this WI:

Agreement 1: Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

-	This specification support is a UE optional feature

-	FFS: Whether this applies for the entire codebook or subset of codebook

Agreement 2: Full TX power UL transmission, one additional option (option 5) is added

Option5: For the precoders with 0 entries, the linear value PPUSCH,b,f,c(i,j,qd,l)PPUSCH,b,f,c(i,j,qd,l) of a PUSCH transmission power is scaled by a ratio Rel-16.  The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

- UE is required to maintain consistent Rel-16αRel-16αRel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

Agreement 3: Full TX power UL transmission, option 4 is updated as follows

Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL (no specification impact)

For guidance in future RAN1 discussions:

Understanding of antenna virtualization for ease of discussion (not for specification):

-	A UE forms an antenna port by transmitting on one or more TX chains (each with a power amplifier)

In RAN1 AH-1901 meeting [4], the following agreement was made in this WI:

Agreement: Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified, or a subset will be specified

FFS: UE capability signaling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

Send LS to RAN4 to provide their view on PC 2 applicability of the new UE capability (Rakesh, vivo).

In Ran1 96 meeting, the following agreements were made

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

Agreement

Down select among the following two alternatives by RAN1#96bis. As part of UE capabilities signalled the following is included:

Alt1: UE capability signaling of supported one or group of TPMI precoder(s)

Alt3: UE capability signaling of power scaling schemes for full uplink power transmission

- Note: This does not imply any restriction on UE antenna virtualization

- FFS: Whether full uplink power transmission needs to be supported for all precoders

In RAN1 96bis, the following agreements were made

Agreement

For the the 2TX and 4TX case, the linear value of power after power scaling, is divided equally among the non-zero PUSCH ports

- The above applies for the cases including when UE transmitting at P_c_max

Agreement

Supported UE capabilities and supported scheme for UE capability 1

- Option 3

- FFS: Whether to additionally support Option 1-2

Agreement

Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- FFS: For UE capability 1, if any other information is necessary

- For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed

  - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced

  - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission

UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1. The signalling of above information does not imply any specific UE PA architecture implementation.

In RAN1 97, the follow working assumption were made:

Working Assumption

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - FFS: Additional support of antenna selection TPMI precoders

  - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

    - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UEcap3, for SRS resource with more than 1 ports,

    - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects

    - The following cases are not precluded

      - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1]

    - FFS: number of SRS resources supported

      - 2

      - 3

    - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power

  - FFS: any rules for spatial filter update for the SRS resources with different number ports

Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD

Note: For single port, there is no SRI and TPMI

Note: Support of Mode 1, Mode 2 have separate UE capability

With the most recent working assumption, the majority open issues on this topic are closed. In this contribution, we address the remaining issues in UL full power Tx.

In this contribution, the background and summary of the problem is introduced in Section 2. The UE capabilities to support UL full power transmissions are discussed in Section Error! Reference source not found.. The way to signal UE capabilities to support UL full power transmissions is discussed in Section 4. The solutions to support UL full power transmission with each UE capability is provided in Section Error! Reference source not found.. The design details and simulation results for proposed solutions are given in Section 7. Section Error! Reference source not found. discussed the view on a solution of using FDM to support UL full power transmission. Conclusions are drawn in Section 8.

# Background and summary of the problem

In NR Rel. 15, two key restrictions can potentially prevent a UE from transmitting PUSCH at full power. The first issue arises due to restrictions on transmit precoder codebook for UEs with non-coherent or partially coherent antennas (see Tables 7.3.1.1.2-2 to 7.3.1.1.2-5 in 38.212), and the second is due to the power allocation rule in 38.213, Section 7.1 This section focuses on addressing these two issues.

Issue 1: Restriction on TPMIs for UEs with non-coherent or partially-coherent antennas

In NR Rel. 15, for PUSCH transmission using codebooks, when UEs have non-coherent antennas or partially coherent antennas, the set of TPMIs are restricted so that the precoder for a layer only spans a coherent subset of antennas (see Tables 7.3.1.1.2-2 to 7.3.1.1.2-5 in 38.212). For example, if a UE has two non-coherent antennas, then the precoders are restricted to be either  or . Such a restriction can prevent such UEs from transmitting at full power. For example, suppose the UE is a class-3 UE, powered by 2 PAs, each with a 20 dBm output rating, then the UE cannot transmit at full power in the scenario described above.

Issue 2: Uplink power allocation

In NR Rel. 15, 38.213, Section 7.1 states that

“For PUSCH, a UE first scales a linear value  of the transmit power  on UL BWP , as described in Subclause 12, of carrier  of serving cell , with parameters as defined in Subclause 7.1.1, by the ratio of the number of antenna ports with a non-zero PUSCH transmission to the number of configured antenna ports for the transmission scheme. The resulting scaled power is then split equally across the antenna ports on which the non-zero PUSCH is transmitted.”

This rule requires scaling the linear value PPUSCH of the transmit power PPUSCH by the ratio of non-zero PUSCH ports to total configured ports, then divides the resulting power equally among non-zero PUSCH ports. Such an approach can prevent the UE from transmitting at full power. For example, consider a UE with 2 non-coherent antenna ports that is assigned the precoder , in this case, the first antenna port is assigned PPUSCH/2 transmit power (linear value) to transmit PUSCH. Thus, for a class-3 UE that is powered by 2 PAs, each with a 23 dBm output rating, the maximum transmit power for this precoder is 3 dB below the maximum power the UE can transmit at.

Resolving Issue 1 helps a class-3 UE with 2 PAs of 20 dBm output rating each to transmit at full power, while resolving Issue 2 helps a class-3 UE with 2 PAs having 23 dBm output rating each to transmit at full power. In what follows, we make proposals to address the above two issues.

# Extend the UE capabilities to other power class UEs

In RAN1 AH-1901 meeting [4], the 3 types of UE capabilities are agreed for power class 3 UEs. We additionally note that these three capabilities are useful for UEs of all power classes, and must not be restricted to only power class 3 UEs.

Further given that Issue 2, noted above in Section 2, impacts coherent UEs as well, we must ensure that these capabilities are clearly specified for coherent UEs as well, so that appropriate uplink power scaling factors can be chosen.

With the above in mind, we propose the following:

Proposal 1: Extend the definitions of the three capabilities to UEs of any power class and to UEs with coherent antenna capability.

# Signalling of UE capability to support UL full power transmissions

Due to the existence of 3 UE capability types and the two modes for capability 2 and 3 UEs, the UE capable signalling for full power needs to follow a hierarchy structure. The top level signalling can be just 1 bit to signal UE can support full power with all precoders. This signalling is sufficient for capability 1 UE and it needs no more additional lower level signalling for full power capability. For capability 2 or 3 UEs, they need to further signal they can support which mode, i.e., mode 1 only, mode 2 only, both, or none of the two modes. For UEs supports mode 1 only, no additional capability signalling is needed. NW knows that full power can be delivered with the introduced “new codebookSubset” while full power cannot be guaranteed with the “old antenna selection codebook”. For UEs supports mode 2, it was agreed in RAN1 #97 that further UE capability signalling per TPMI group is introduced. Our view is that RAN1 should aim to minimize the signalling overhead for mode 2.

Based on the above discussion, therefore, we make the follow proposal

Proposal 2: A single bit (level 1) is used to indicate that a UE supports full power by setting power scaling factor in power control equals to 1 always for all precoders or not.

- For a capability 1 UE, no additional full power capability signaling is needed.

- For a capability 2 or 3 UE, two more bits (level 2) to signal support mode 1 only, mode 2 only, both mode 1 and 2, none of mode 1 or 2.

  - For a UE supports mode 1 only or none of mode 1 or 2, no additional full power capability signaling is needed.

  - For a UE supports mode 2 only or both mode 1 and 2, RAN1 strive to minimize the number of additional bits (level 3) for full power capability signaling.

Note: the above UE full power capability signaling is per feature set per component carrier (per CC per band per band combination).

Next, we discuss how to design level 3 capability signalling for capability 2 or 3 UEs in mode 2. Obviously, we need design the signalling for 2 Tx and 4 Tx UEs separately. For 2 Tx UE, the design is trivial. Therefore, we can focus on the design for 4 Tx UEs. For a UE with 4 Tx, there are 12 rank 1 precoders (0-11 in Table 6.3.1.5-3 in 38.211), 7 rank 2 precoders (0-6 in Table 6.3.1.5-5 in 38.211), and 1 rank 3 precoder (0 in Table 6.3.1.5-6 in 38.211). If we signal full power capability per precoder, there are 20 bits needed per band in each band-band combination, which is too much overhead. Therefore, we must aim to reduce the bitmap length for overhead reduction.

First, we observe that the full power capability with multi-layer precoders can be inferred from single layer precoders. For example, if precoder  can support full power, so can . Another example, if precoder  can support full power, so can  or . For multi-layer precoder , we can create its “virtual” single layer precoder conceptually for full power capability signalling purpose, which is , although this virtual single layer precoder does not exist in Rel-15 codebook.

Based on this observation, we make the following proposal.

Proposal 3: For a capability 2 or 3 UE supports mode 2, no need to signal full power capability for a multi-layer precoders as it can be implicitly derived based on signaling for single layer precoders.

We then can focus on design full power capability signaling based on existing single layer precoders in Rel-15 codebook and created dual single layer precoders. By exhaustive search of the Rel-15 PUSCH codebook, we list all existing and virtual single layer precoders as below

1000, 0100, 0010, 0001, 1010, 0101, 1100, 1001, 0110, 0011, 11101111

The colour code to read the list is following. The first 6 black entries are from existing single layer precoders in NR Rel-15 codebook. The 4 red entries are virtual single layer precoders collapsed from two layers precoders. The green entry is a virtual single layer precoder collapsed from three layers precoders. For completeness of the single layer precoder list, we included the last grey entry. But one should notice the full power with last entry does not need to be signalled, as it needs to be supported by default, if a UE declares it can support full power as a Rel-16 feature.

One should notice that even with proposal 2, 11 bits is needed per band for each band-band combination if a UE signals full power capability per precoder. To further reduce the overhead, we can consider two types of UE. Type A UE is an advanced UE which can dynamically switch the linkage between PA and antenna ports. For Type A UE, if it can support full power with 1000, it can also support full power with  0100, 0010, 0001. Similarly, if it can support full power with 1010, it should also able to support full power with 0101, 1100, 1001, 0110, 0011. Because type A UE can switch connection between PA and antenna ports dynamically, what matters to that UE is how many 1s exits in the precoder, while it does care where the 1s are located/associated with which ports. Therefore, for type A UE, 2 bits is enough to signal full power capability in mode 2 for precoders with 1,2,3, or, 4 nonzero entries, i.e., precoder 1000, 1010, 1110, and 1111.

Another UE, which is type B UE, is a UE cannot dynamically switch connection between PA and antenna ports. To align the number of bits used for capability signaling, for type B UE, we also propose use 2 bits to signal full power capability for precoder 1000, 1010, 1110, and 1111. The advantage of this scheme is the reduced signaling overhead and aligned number of bits between type A and type B UEs. The limitation of this scheme is that type B UE cannot guarantee to deliver full power with precoders other than 1000, 1010, 1110, and 1111. But given that the goal of this Rel-16 WI is already achieve by allow eNB to use these precoders to request full power UL transmission, extending type B UE to support full power with other precoders can be considered as a secondary optimization for later releases.

Based on the above discussion, we have the following proposal for 4 Tx UEs.

Proposal 4: For a capability 2 or 3 UE supports mode 2, if the UE is with 4 Tx, additional 3 bits (level 3) are used for full power capability signaling.

- 2 bits(LSB) to indicate support full power with precoder 1000, 1010, 1110, and 1111 or not.

- 1 bit(MSB) to indicate if gNB can extend full power support from the signaled precoder to other precoders with the same number of nonzero entries.

We can have the similar signaling for 2 Tx UEs. Note that in this case there are only two TPMIs of interest 10. and 01. Without any loss of functionality, it can be mandated that a 2 Tx UE indicating support for mode 2 supports full power using precoder 10. This simplifies signaling and in fact requires no additional signaling for such UEs. If necessary, an additional bit signaling can indicate if this UE can also support full power on the 01 precoder. Thus 10 becomes the default precoder support full power transmission.

Proposal 5: For a capability 2 or 3 UE supports mode 2, if the UE is with 2 Tx, additional 2 bits (level 3) are used for full power capability signaling.

- 1 bit(LSB) to indicate support full power with precoder 10 or not.

- 1 bit(MSB) to indicate if gNB can extend full power support from the signaled precoder to other precoders with the same number of nonzero entries.

# Remaining issues for UE capability 1: UEs having PAs with full power rating

We split this discussion into two parts.

UE with Capability 1 and coherent antennas: For such UEs, any TPMI can be used to transmit at full power. Since it is known that all PAs can transmit at full power, the first step of the power allocation rule where the linear value PPUSCH of the transmit power PPUSCH  is scaled by the ratio of non-zero PUSCH ports to total configured ports can be omitted. The linear value PPUSCH can be directly divided equally among the the non-zero PUSCH ports. No further changes are necessary.

UE with Capability 1 and non-coherent or partially-coherent antennas:  Once again, any TPMI from codebookSubset “partialAndNonCoherent” or “noncoherent” can be used to transmit at full power by omitting the first step of the power allocation rule.

Proposal 6: For a capability 1 UE, full power UL transmissions can be supported by setting the power scaling factor to 1 for the linear value of power PPUSCH, before equally divide it among the non-zero PUSCH ports.

# Remaining issue for UE capability 2 and 3 in mode 1

In mode 1 for capability 2 or 3 UEs, the remaining issue is that the details of “new codebookSubset” is still open. Given that the principle of mode 1 is to allow a non-coherent UE to transmit full power using full coherent codebooks via transparent small delay CDD in UE implementation, it is sufficient to support just a subset of the non-antenna selection precoder(s) to deliver full power. There is no need to support additional antenna selection precoders with full power. Furthermore, as a matter of factor, without defining additional SRS ports, it is not feasible to support antenna selection precoders with full power for capability 2 UE. For capability 3 UE, it is possible to support some antenna selection precoders with full power. But the performance of antenna selection precoders suffer significantly in case of hand-blocking. While, non-antenna selection precoder can deliver much more robust performance in case of hand-block.

Based on the above reasoning, we have the following proposal.

Proposal 7: For a capability 2 or 3 UE in mode 1, the introduced new codebookSubset includes a subset of the non-antenna selection precoders in Rel-15 codebookSubset “fullyAndPartialAndNonCoherent”.

- For a UE with noncoherent antennas, introduce a new codebookSubset = “noncoherentWithCyclicDelay” in 38.212 Section 7.1 to include:

  - TPMIs 0 to 2 in 38.211 Table 6.3.1.5-1 (Single-layer transmission using two antenna ports).

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding).

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - TPMI 0 to 6 in 38.211 Table 6.3.1.5-5 (Two layer transmission using four antenna ports without transform precoding),

  - TPMI 0 to 1 in 38.211 Table 6.3.1.5-6 (Three layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “nonCoherent” in 38.212

- For a UE with partial and noncoherent antennas, introduce a new codebookSubset = “partialAndNonCoherentWithCyclicDelay” in 38.212 Section 7.1 to include:

  - TPMIs 0 to 19, in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding).

  - TPMIs 0 to 19 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “partialAndNonCoherent” in 38.212.

# Remaining issue for UE capability 2 and 3 in mode 2

For capability 2 and 3 UEs in mode 2, one of the open issues is deciding how many additional SRS resources are needed due to the introduced new SRS port. Without losing generality, we start to look at this open issue with 2 Tx UEs.

For a 2 Tx UE, there are three options on expanding the number of SRS resource per resource set, as shown in Figure 2. Option 1 is keeping the number of SRS resources equalling to 2 per resource set which is the same as in Rel-15. The advantage of option 1 is no increment in DCI signalling. The disadvantage of option 1 is that there is no Tx beam diversity in FR2. With option 1, for the same set of SRS ports, eNB cannot configure two SRS resources with different Tx spatial filter coefficients.  In order to keep Tx beam diversity, we need extend the number of SRS resources per resource set to more than 2. If we take this direction, then option 2 does not make much sense. With option 2, although we paid the price of increasing SRI from 1-bit to 2-bits, but we did not fully explore the potential Tx diversity. Option 3 is more reasonable than option 2. A Rel-16 gNB can configure 4 SRS resources per resource set where 2 resources for the rank 2 MIMO transmissions when the UE is in cell centre. Another 2 SRS resources for rank 1 full Tx transmissions when the UE is in cell edge. Furthermore, no matter in cell centre or cell edge, UE can use two different spatial filters to take the benefit of Tx diversity factor of 2, which is the same as in Rel-15.

Figure 1: Options of number of SRS resources per resource set in mode 2

For a 4 Tx UE, more than 2 SRS resources are necessary per resource set, as the additionally introduced ports in mode 2 can be 2 ports each with 20dBm or single port with 23dBm. UE needs to sound 4 ports each with 17dBm for rank-4 MIMO, 2 ports each with 20dBm for rank-2 MIMO, and single port with 23dBm for rank 1 transmission. Therefore, at least 3 SRS resources are needed. Given 3 SRS resources already increases SRI from 1-bit to 2-bits, it is reasonable to allow 4 SRS resources per resource set.

An additional aspect to consider for 4 Tx. UEs is on the number of ports the new SRS resources can have. Since a 4 Tx UE may be able to virtualize its 4 transmit chains to form 2 full power virtual ports, as in the case of a power class 3 UE with 4 20 dBm PAs, these additional SRS resources can have more than 1 port. The UE’s ability to synthesize more than 2 or more virtual ports that can each simultaneously transmit at full power can be signalled by the UE so as to help the gNB with the SRS resource configuration.

Figure 2 Options for SRS Resource configurations for 4 Tx. UEs. that support mode 2.

Regarding the open issue “FFS: any rules for spatial filter update for the SRS resources with different number ports”, our view is that spatial filter for each SRS resources is up to gNB configuration, which should be transparent to RAN1 specification. Therefore, no rule needs to be specified for spatial filter update for the SRS resources with different number ports.

Therefore, based on the above discussion, we have the following proposal.

Proposal 8: For a capability 2 or 3 UE in mode 2, per SRS resource set, Rel-16 supports up to 4 SRS resources where each can be configured with different number of SRS ports.

Proposal 9: For 4 Tx UEs that support mode 2, one additional bit of capability signaling is introduced to indicate whether they are able to synthesize 1 or 2 virtual ports that can independently transmit at full power.

- If this bit is set to ‘0’, then the UE is capable of synthesizing only 1 virtual port, and if this bit is set to ‘1’, the UE is capable of synthesizing 2 virtual ports.

# Conclusions

In summary, we have the following proposals to address the remaining issues for full power uplink transmissions for Rel-16 MIMO enhancements.

Proposal 1: Extend the definitions of the three capabilities to UEs of any power class and to UEs with coherent antenna capability.

Proposal 2: A single bit (level 1) is used to indicate that a UE supports full power by setting power scaling factor in power control equals to 1 always for all precoders or not.

- For a capability 1 UE, no additional full power capability signaling is needed.

- For a capability 2 or 3 UE, two more bits (level 2) to signal support mode 1 only, mode 2 only, both mode 1 and 2, none of mode 1 or 2.

  - For a UE supports mode 1 only or none of mode 1 or 2, no additional full power capability signaling is needed.

  - For a UE supports mode 2 only or both mode 1 and 2, RAN1 strive to minimize the number of additional bits (level 3) for full power capability signaling.

Note: the above UE full power capability signaling is per feature set per component carrier (per CC per band per band combination).

Proposal 3: For a capability 2 or 3 UE supports mode 2, no need to signal full power capability for a multi-layer precoders as it can be implicitly derived based on signaling for single layer precoders.

Proposal 4: For a capability 2 or 3 UE supports mode 2, if the UE is with 4 Tx, additional 3 bits (level 3) are used for full power capability signaling.

- 2 bits(LSB) to indicate support full power with precoder 1000, 1010, 1110, and 1111 or not.

- 1 bit(MSB) to indicate if gNB can extend full power support from the signaled precoder to other precoders with the same number of nonzero entries.

Proposal 5: For a capability 2 or 3 UE supports mode 2, if the UE is with 2 Tx, additional 2 bits (level 3) are used for full power capability signaling.

- 1 bit(LSB) to indicate support full power with precoder 10 or not.

- 1 bit(MSB) to indicate if gNB can extend full power support from the signaled precoder to other precoders with the same number of nonzero entries.

Proposal 6: For a capability 1 UE, full power UL transmissions can be supported by setting the power scaling factor to 1 for the linear value of power PPUSCH, before equally divide it among the non-zero PUSCH ports.

Proposal 7: For a capability 2 or 3 UE in mode 1, the introduced new codebookSubset includes a subset of the non-antenna selection precoders in Rel-15 codebookSubset “fullyAndPartialAndNonCoherent”.

- For a UE with noncoherent antennas, introduce a new codebookSubset = “noncoherentWithCyclicDelay” in 38.212 Section 7.1 to include:

  - TPMIs 0 to 2 in 38.211 Table 6.3.1.5-1 (Single-layer transmission using two antenna ports).

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding).

  - TPMIs 0 to 4, 8, and 12 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - TPMI 0 to 6 in 38.211 Table 6.3.1.5-5 (Two layer transmission using four antenna ports without transform precoding),

  - TPMI 0 to 1 in 38.211 Table 6.3.1.5-6 (Three layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “nonCoherent” in 38.212

- For a UE with partial and noncoherent antennas, introduce a new codebookSubset = “partialAndNonCoherentWithCyclicDelay” in 38.212 Section 7.1 to include:

  - TPMIs 0 to 19, in 38.211 Table 6.3.1.5-2 (Single layer transmission using four antenna ports with transform precoding).

  - TPMIs 0 to 19 in 38.211 Table 6.3.1.5-3 (Single layer transmission using four antenna ports without transform precoding).

  - For Tables not listed above, set of allowed TPMI remains same as that for codebookSubset “partialAndNonCoherent” in 38.212.

Proposal 8: For a capability 2 or 3 UE in mode 2, per SRS resource set, Rel-16 supports up to 4 SRS resources where each can be configured with different number of SRS ports.

Proposal 9: For 4 Tx UEs that support mode 2, one additional bit of capability signaling is introduced to indicate whether they are able to synthesize 1 or 2 virtual ports that can independently transmit at full power.

- If this bit is set to ‘0’, then the UE is capable of synthesizing only 1 virtual port, and if this bit is set to ‘1’, the UE is capable of synthesizing 2 virtual ports.

# References

- 3GPP TSG RAN Meeting #80, RP-181453 “WI Proposal on NR MIMO Enhancements,” La Jolla, USA, June 11 – 14, 2018.

- 3GPP TSG RAN WG1 Meeting #94b, “RAN1 Chairman’s notes,” Chengdu, China, October 8th – 12th, 2018.

- 3GPP TSG RAN WG1 Meeting #95, “RAN1 Chairman’s notes,” Spokane, USA, November 21-25, 2018.

- 3GPP TSG RAN WG1 Meeting AH 1901, “RAN1 Chairman’s notes,” Taipei, Taiwan, 21st – 25th January, 2019.