3GPP TSG-RAN WG1 Meeting #98							R1-1909301

Prague, Czech Republic, August 26th – 30th, 2019

Agenda Item:	7.2.2.2.4

Source:	Ericsson

Title:	Configured grant enhancement

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we provide our view on aspects related to configured UL grant for NR-U.

# 2	Discussion

## 2.1	Multiple Active CG Configurations

Enabling multiple starting positions using existing CG configurations is already supported in NR Rel-15. However, if TypeB PUSCH mapping (mini-slots) is used for all UL transmission opportunities, overhead (DMRS and UCI) may be higher than desirable. Hence, one simple approach is to support a mechanism by which a CG-UE can switch to slot-based transmissions after channel access is obtained.

Such a mechanism has already been agreed RAN1#95 [2] within the NR URLLC SI whereby a UE can be provided with multiple active configured grants:

Agreements:

  - Multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency

    - FFS details

    - Note: it is understood that the above may be related to RAN2-led work on intra-UE multiplexing

Multiple active configured grant configuration can be a means to allow the UE to switch to slot-based transmissions after initiating the COT to minimize DMRS and UCI overhead. The gNB can activate three separate configurations for a UE, for example

- Configuration A:

  - CG-window = 8 Slots; S = 0, L = 7

- Configuration B:

  - CG-window = 8 Slots; S = 7, L = 7

- Configuration C:

  - CG-window = 8 Slots; S = 0, L = 14

The UE can first make use of Configuration A and/or B at the beginning of a COT. During the COT, the UE may switch to slot-based transmissions according to Configuration C. While this type of behavior may be useful, it can also be considered as a non-essential optimization. Based on this we observe the following

If multiple configured grant configurations per BWP of a serving cell are configured with different periodicities, a UE may switch from the higher periodicity to the lower periodicity after channel access is obtained.

NR-U supports multiple active configured grant configurations per BWP

## 2.2		Time domain resource configuration for configured grants

During RAN1#97 [1], the following agreement was made:

Agreement:

For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

In our view, two alternatives can be considered to indicate the PUSCH time allocation

Alternative 1

For simplicity, the time domain resource assignment in configured grant repeats over the multiple slots within the CG-allocated slots. The same symbol allocation and mapping type is used across CG-slots.

For example, with this time domain resource configuration extension, the set of three CG configuration discussed in the previous section allows UE to have two possible starting points and the ability to switch to slot based transmission after initiating the COT.

As a second example, a finer starting point grid can be created with a combination of L=3, 4 and 14 CG configurations to enable four possible starting points and the ability to switch to slot-based transmission after initiating the COT.

Alternative 2

The time domain resource assignment in configured grant repeats consecutively throughout the multiple slots within the CG-allocated slots. S defines the first symbol of the first allocated CG-PUSCH, the remaining PUSCH follows consecutively and immediately after the first PUSCH with no gaps in between. All CG-PUSCHs within the CG-allocated slots are of the same length. To limit the complexity, it should be guaranteed that no CG-PUSCH crosses slot boundary.

For example, with this time domain resource configuration extension, CG configuration A and configuration B as discussed in the previous section together will UE to have two possible starting points and the ability to switch to slot-based transmission after initiating the COT.

However, with the restriction of a CG-PUSCH not crossing a slot boundary and the CG transmission not leaving unused OFDM symbols in a slot (which will require subsequent LBT to regain COT), L=3 and 4 CG configurations cannot be used under this extension.

Basically, alternative 2 may require fewer CG configurations to enable the same multiple starting points as compared to Alternative 1. However, for practical deployment cases, the difference is likely to be only one. Furthermore, Alternative 2 introduces more specifics complexity with potentially more corner error cases but still being more restrictive. On balance, alternative 1 extension to the time domain resource configuration is preferred.

The time domain resource assignment in configured grant repeats over the multiple slots within the CG-allocated slots. The same symbol allocation and mapping type is used across CG-slots.

## 2.3	DFI Content

During the SI phase, the following agreements were made [2] related to explicit HARQ feedback for UL CG transmissions:

The following modifications to the configured grant procedures are beneficial.

-	…

-	Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

…

It is identified to be beneficial to support DFI to include pending HARQ ACK feedback for prior configured grant transmissions from the same UE.

…

UE may autonomously initiate retransmission for a HARQ process that was initially transmitted via configured grant mechanism for NR-unlicensed when it receives NACK feedback via DFI for the corresponding HARQ process.

As stated in the above, explicit HARQ feedback for CG PUSCH transmissions has been agreed for unlicensed operation. This has been referred to as downlink feedback information (DFI). This is in contrast to NR Rel-15 which supports only implicit ACK feedback. To enable implicit ACK feedback, the UE configures a timer when it performs an UL transmission with configured grant. If the timer expires and the UE has not received a dynamic grant scheduling an UL retransmission, then it treats this as an explicit ACK of the initial transmission. The issue for unlicensed operation, is that due to hidden nodes not sensed by the UE prior to an UL CG transmission, the gNB may fail to detect the UL transmission. Hence it is not safe for the UE to assume ACK if the timer expires. For this reason, the following (opposite) behavior was agreed in RAN1#96 whereby the UE assumes NACK upon timer expiration instead of ACK:

Agreement:

For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

With this pair of agreements, the UE may perform retransmissions on UL configured grant resources based on either timer expiration or explicit DFI carrying HARQ feedback. What has not yet been agreed is the DFI content and means to signal it.

For DFI content we propose the following:

Support DFI containing a HARQ-ACK bitmap with one bit per configured HARQ process.

## 2.4		CBG-Based Retransmissions

The following agreement was made in RAN1#96 regarding retransmissions in the case that PUSCH is configured with multiple CBGs:

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

CBG-based transmission enables individual ACK/NACK for each CBG and the UE retransmits only the failed CBGs. This can reduce unnecessary retransmissions of part of a TB. This agreements states that the individual CBGs are retransmitted using a dynamically scheduled grant. CBG-based retransmissions on UL CG resources is FFS.

We observed that if CBG-based retransmissions are allowed on UL CG resources, several problems arise. Firstly, the UCI on top of indicating RV, NDI, and HARQ ID, would also need to include a indicates the transmitted CBG(s) in every PUSCH. Secondly, the length of the DFI would increase significantly in order to provide HARQ feedback per CBG instead of just per TB. Lastly, if only a subset of CBGs are retransmitted, some UL CG resources would be wasted. Similar to multiple starting positions, CBG is a non-essential feature, since the main operation region of CG transmission is low load where interference and congestion are not severe.

Based on this we propose the following:

If CBG is enabled for UL transmissions with configured grant (CG), support CBG-based retransmissions using ONLY dedicated scheduled UL resources. CBG-based retransmissions are not supported on an UL CG.

## 2.5	Starting Time Offsets

During RAN1 AH 1901, the following agreement was made:

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

For LTE-LAA, the UE starting time offset falls within the first symbol of the subframe. UCI is always mapped from the second symbol and indicates if PUSCH starts at the first or second symbol of the subframe. For NR-U, the situation is a bit more complicated since the PUSCH starting position can be at multiple positions in the slot depending on the configured periodicity and time domain allocation (start and length indicator values). Also, DMRS can be configured in the first symbol of the slot.

If DMRS patterns for Type B mapping are not modified to support UE starting offset, the UE starting time offset should be an offset value until the boundary of the first symbol of the CG resource. To align the design with Type A mapping, the same procedure is used in case of Type A mapping. Instead of puncturing the first symbol of the CG resource, the UE extends the first symbol to the earlier symbol depending on the offset value. The UCI is mapped using the Rel-15 UCI multiplexing rules. The UE does not need to indicate the first symbol used for PUSCH as it was done for FeLAA.

The UE starting time offset is an offset value until the boundary of the first symbol of the CG resource.

    - The UE extends the first symbol of the CG transmission by an offset value through use of a longer CP

## 2.6	UCI Mapping and Content

The following agreements about the UCI content has been made until now:

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

Agreement:

When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

- The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

- FFS:

  - How the duration is signalled

  - Whether the UE should signal continued use of the COT for its own transmissions

  - LBT priority class

One of the FFS points is signaling the LBT priority class in UCI. In our view this is needed since it informs the gNB what priority class value was used to acquire the channel. Hence, if the gNB shares the COT with the UE, it will know that it can transmit DL data only with equal or higher priority class values.

Support UCI for UL configured grant that includes the LBT priority class value used by the UE to access the channel.

The details on how to signal the duration of the COT is preferably postponed until the discussion about COT sharing requirements is finalized. If slot level duration is supported, our preference is to indicate in UCI:

- The duration in unit of slots. Used UL duration + indicated DL slots do not exceed MCOT.

- COT sharing indication: 1 bit indicating if slot n+X is an applicable slot for UL to DL sharing). X is configured by the eNB as part of RRC configuration.

The ending symbol for the last CG-PUSCH before the switch is configured by the gNB. This way there is no need to indicate the ending symbol to the gNB in UCI. Also, the gap between UL and DL in the UE shared COT is under gNB control. Based on this gap, the gNB can derive if CAT1 or CAT2 is to be used.

Additionally, to increase the robustness of UCI specially in case of intra-cell collisions and to identify the UE at the eNB, UCI may include UE ID and CRC scrambled by specific RNTI.

CG-UCI should include CRC and UE ID.

As for the PUSCH starting position, it was agreed that multiple starting positions based on UE autonomously puncturing some symbols at the beginning of the PUSCH is not supported. Also, as explained in section 2.6, the LBT offset should be based on extending the first PUSCH symbol instead of puncturing it. If so, from gNB perspective, there is no ambiguity about the PUSCH starting symbol. The UE always follows the SLIV indicated by the gNB. There is no need to indicate PUSCH starting symbol in UCI. There is also no strong motivation to support partial ending symbol other than that configured by SLIV or preconfigured by gNB in case of COT sharing as described in the previous section.

PUSCH starting/ending symbol is not indicated in UCI.

The gNB does not know when the UE will transmit on the configured UL resources. The processing burden on the gNB to detect the UL configured transmission should not be increased. In NR, the DMRS and UCI is mapped as early in the PUSCH as possible to give more processing time for the gNB to demod/decode the configured UL transmission. The same principle should be kept in NR-U.  As discussed in the previous section, it is desirable to keep the Rel-15 UCI multiplexing rules as they are. To simplify the spec changes, the CG-UCI is considered part of HARQ-ACK UCI. The same Rel-15 multiplexing rules used for HARQ-ACK UCI are applied to multiplex the CG-UCI on the CG-PUSCH.

The same Rel-15 multiplexing rules used for HARQ-ACK UCI are applied to multiplex the CG-UCI on the CG-PUSCH. CG-UCI is part of HARQ-ACK UCI.

## 2.7	Indication of MCS for Configured Grant PUSCH Transmissions

In NR Rel-15, the MCS to be used for CG PUSCH transmissions is provided to the UE through RRC configuration (Type1 CG) or via DCI during (re)-activation of the configured grant (Type2 CG). For the latter, the UE provides an acknowledgement to the gNB through an empty MAC-CE message that the (re-)activation was received. In this way, a robust method is ensured for avoiding a mismatch between the gNB and UE’s understanding of the granted MCS in the case that the DCI containing the (re-)activation of the CG happens to be lost.

During the NR-U WI, there have been proposals to change the NR Rel-15 mechanism such that the MCS can be changed and indicated to the UE on a faster basis by signaling the MCS in the same message carrying downlink feedback information (DFI), e.g., by DCI. A drawback of this approach is that it does not include the built-in robustness of the NR Rel-15 mechanism where the MCS is carried in the DCI (re-)activation message which is acknowledged by MAC-CE. In principle, one could consider introducing a new MAC-CE message to acknowledge that the DCI carrying DFI is received; however, this is not an efficient design as DFI signaling is quite frequent.

No new mechanism for indicating CG PUSCH transmission MCS (e.g., by indication of MCS in the DCI carrying DFI) is introduced for Rel-15 NR-U.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If multiple configured grant configurations per BWP of a serving cell are configured with different periodicities, a UE may switch from the higher periodicity to the lower periodicity after channel access is obtained.

Based on the discussion in the previous sections we propose the following:

Proposal 1	NR-U supports multiple active configured grant configurations per BWP

Proposal 2	The time domain resource assignment in configured grant repeats over the multiple slots within the CG-allocated slots. The same symbol allocation and mapping type is used across CG-slots.

Proposal 3	Support DFI containing a HARQ-ACK bitmap with one bit per configured HARQ process.

Proposal 4	If CBG is enabled for UL transmissions with configured grant (CG), support CBG-based retransmissions using ONLY dedicated scheduled UL resources. CBG-based retransmissions are not supported on an UL CG.

Proposal 5	The UE starting time offset is an offset value until the boundary of the first symbol of the CG resource.

i.	The UE extends the first symbol of the CG transmission by an offset value through use of a longer CP

Proposal 6	Support UCI for UL configured grant that includes the LBT priority class value used by the UE to access the channel.

Proposal 7	CG-UCI should include CRC and UE ID.

Proposal 8	PUSCH starting/ending symbol is not indicated in UCI.

Proposal 9	The same Rel-15 multiplexing rules used for HARQ-ACK UCI are applied to multiplex the CG-UCI on the CG-PUSCH. CG-UCI is part of HARQ-ACK UCI.

Proposal 10	No new mechanism for indicating CG PUSCH transmission MCS (e.g., by indication of MCS in the DCI carrying DFI) is introduced for Rel-15 NR-U.

# 4	References

Chairman’s Notes, RAN1 WG #97, April 2019

Chairman’s Notes, RAN1 WG #95, November 2018

3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum”, v16.0.0, December 2018.