3GPP TSG RAN WG1 Meeting #98				R1-1909351

Prague, Czech, August 26th – 30th, 2019

Source 	: CAICT

Title 	: UCI enhancements for URLLC

Agenda Item	: 7.2.6.2

Document for	: Discussion / Decision

Introduction

The work item of “Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)” was approved in the previous RAN #83 meeting [1] and there are the following objectives for UCI enhancements:

- Specification of UCI enhancements [RAN1]

  - More than one PUCCH for HARQ-ACK transmission within a slot

  - At least two HARQ-ACK codebooks simultaneously constructed, intended for supporting different service types for a UE

The following agreements about UCI enhancements were reached in the previous meetings:

Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. - PDSCH transmission is not subject to sub-slot restrictions (if any) - FFS: PDSCH-to-sub-slot association. - FFS: Allowing PUCCH across sub-slot boundary or not. - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook. - FFS for Type I HARQ-ACK codebook. - R15 PUCCH resource overriding procedures is applied in unit of sub-slot. - Number or length of UL sub-slots in a slot is UE-specifically semi-statically configured. - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot. - FFS: K1 definition. - FFS: Details of PUCCH resource configuration and determination. FFS: Use “Codebook-less HARQ” as a complementary or not. FFS: If HARQ-ACK can be omitted in case latency requirement cannot be met. FFS: PDSCH groupings and PHY identification for separate HARQ-ACK constructions for different service types. Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, K1 is defined following R15 approach but in unit of sub-slot. Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and for dynamically-scheduled PDSCH, down-select from below for the PHY identification for identifying a HARQ-ACK codebook: - Opt.1: By DCI format - Opt.2: By RNTI - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field) - Opt.4: By CORESET/search space - FFS additional option(s) for Type I HARQ-ACK codebook FFS: For SPS PDSCH (including SPS release PDCCH) Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot. Agreements: For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s) - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot. Agreements: - When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following: - FFS: For PUCCH-SpatialRelationInfo - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback. - FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList. - FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration.

In this contribution, we will discuss more details about the UCI enhancements to for URLLC.

Discussion

Multiple PUCCHs for HARQ-ACK within a slot

With multiple PUCCHs for HARQ-ACK within a slot supported, how to multiplex HARQ-ACK should be considered. To our opinion, HARQ-ACK bits within a same time unit is to be multiplexed to one codebook. Within each time unit, the similar multiplexing rule as the current rule for slot based PUCCH could be reused. The following Fig.2 is an example. PUCCH-1, PUCCH-2, PUCCH-3 are all indicated at slot level for HARQ-ACK feedback. These HARQ-ACK could be multiplexed to one PUCCH. PUCCH-4 and PUCCH-5 are at the level of subslot(7OS) and multiplexed to one PUCCH.

Fig. 1 HARQ-ACK multiplexing with different time units

Since sub-slot-based HARQ-ACK feedback procedure is to be defined for supporting multiple PUCCHs for HARQ-ACK within a slot, the HARQ-ACK codebook determination, multiplexing, and etc would be based on the level of subslot. There are no obvious requirements to support PUCCH across sub-slot boundary.

Within each time unit, it is possible the PUCCHs are for HARQ-ACK of PDSCHs of different service types which are with different reliability requirements. If the service types could be identified in PHY, it is possible to multiplex the HARQ-ACKs into multiple codebooks according to the service type. When PUCCHs within one time unit for different services are overlapped, the PUCCH for low latency service should be prioritized. When multiple PUCCHs in different time units are overlapped, for example in the time unit of subslot and slot, the PUCCH in shorter time unit would have higher priority to satisfy the original intention of gNB scheduling the urgent service to shorter time units.

Proposal 1: Multiplex the HARQ-ACKs within the same time unit into multiple codebooks according to the service type identified in PHY indications.

- When PUCCHs within one time unit for different services are overlapped, the PUCCH for low latency service should be prioritized.

- If multiple PUCCHs in different time units are overlapped, the PUCCH in shorter time unit would have higher priority.

HARQ-ACK codebook constructions for different service types

Currently, whether HARQ-ACK codebook type depends on the configured parameter pdsch-HARQ-ACK-Codebook. In general, dynamic codebook size determination provides more efficient resource usage than semi-static codebook size determination since HARQ-ACK feedback is only for the actual transmitted PDSCH. However, since the HARQ-ACK codebook size determination depends on the DAI bits in the PDCCH, the reliability of PDCCH would impact the accuracy of codebook size determination. On the other hand, semi-static HARQ-ACK codebook size determination is robust in the codebook size determination while it is inefficient considering resource usage.

For URLLC, since the reliability of PDCCH should have been enhanced to a high level, the accuracy of dynamic HARQ-ACK codebook determination could be guaranteed. Also considering the high requirements reliability of HARQ-ACK, dynamic codebook size is more proper for URLLC. For eMBB, semi-static codebook determination is needed sometimes since the reliability of PDCCH could not reach a certain degree. However, it is much restricted to always fix the dynamic codebook for URLLC while semi-static codebook for eMBB considering great diversity of service requirements. It is proposed to support configuring the type of HARQ-ACK codebook according to the service type independently.

Proposal 2: Support configuring the type of HARQ-ACK codebook according to the service type independently.

Conclusion

In this contribution, we discussed details about UCI enhancements for URLLC. The following proposals are reached:

Proposal 1: Multiplex the HARQ-ACKs within the same time unit into multiple codebooks according to the service type identified in PHY indications.

- When PUCCHs within one time unit for different services are overlapped, the PUCCH for low latency service should be prioritized.

- If multiple PUCCHs in different time units are overlapped, the PUCCH in shorter time unit would have higher priority.

Proposal 2: Support configuring the type of HARQ-ACK codebook according to the service type independently.

- Reference

- RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”

- 3GPP TR 38.824 V2.0.1, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”