3GPP TSG RAN WG1 Meeting #98																R1-1909358

Prague, Czech, August 26 – 30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR UL transmission in LTE UL subframe in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The EN-DC power control agreements in RAN1#91 are showing below, Agreement: For LTE/NR NSA operation, If this UE supports dual UL operation and also supports single UL operation with Case 1 HARQ timing, RRC signaling can configure a UE to operate in one of the following modes: Dual UL operation Single UL operation with Case 1 HARQ timing Single UL operation with Case 2 HARQ timing For UE supporting single UL operation and with Case 1 HARQ timing if UE does not support power scaling for LTE-NR DC with P_LTE+P_NR>Pcmax, UE shall support the following two operations: Operation A with Case1: P_LTE + P_NR > Pcmax, in which case the UE assumes that no NR UL transmission takes place in an UL subframe/slot that is designated as LTE UL in the Case 1 reference TDD configuration Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified) The operation A vs operation B configuration is implicitly determined based on P_LTE and P_NR Note that the above agreement does not affect the current status on the optional/mandatory support of power scaling for LTE-NR DC with P_LTE + P_NR > Pcmax Note that the above agreement can become obsolete if power scaling for LTE-NR DC is mandated to all UEs The Operation B in above agreements is not captured in the specifcation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If _LTE + P_NR <= Pcmax and UE is configured with reference TDD configuration, UL transmission can be scheduled in any NR UL slot. Small editorial formatting alignment to make the somewhat hard-to-follow ordering and tabulation of the conditions less confusing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If P_LTE + P_NR <= Pcmax, and UE is configured with case1 HARQ timing. It’s not clear whether the NR UL transmission is alllowed in LTE UL subframe. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Isolated impact analysis - If the gNB implements the CR and the UE does not, if the gNB configures the UE with PLTE+PNR≤PTotalEN-DC, and with TDD reference configuration for LTE, the UE behaviour when scheduled to transmit NR during the UL phase of the reference configuration is unclear. - If the UE implements the CR and the gNB does not, if the gNB configures the UE with PLTE+PNR≤PTotalEN-DC, and with TDD reference configuration for LTE, the gNB may anyway assume the behaviour according to the CR, or alternatively assume that the UE cannot be scheduled with NR UL during the UL phase of the reference configuration. There are no interoperability issues in either case. |  |  |  |  |  |  |  |  |

## 7.6	Dual connectivity

### 7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Subclauses 7.1 through 7.5 using  as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Subclauses 7.1 through 7.5.

A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration.

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]), and

-	if the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC,

the UE does not expect to transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and

-	if UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG in FR1, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot  of the SCG if  would need to be reduced by more than the value provided by XSCALE in order for  in any portion of slot  of the SCG. The UE is required to transmit in slot  of the SCG if  would not need to be reduced by more than the value provided by XSCALE in order for  in all portions of slot .

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]).

If a UE is configured with PLTE+PNR≤PTotalEN-DC,  and If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]), the UL transmission can be scheduled in any UL slot on the SCG in FR1.